3GPP TSG-RAN5 Meeting #106	R5-250793r1

# Athens, Greece, 17th Feb 2024 – 21th Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5425 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case related to AC-Barring per PLMN 8.1.2.6a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI12_Test |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As discussion paper(R5-242949) mentioned, Japan will use following 3GPP features to add the affected user to Relieving Carrier. -Access Control per PLMN So we need to introduce test cases related to ac-BarringPerPLMN-List for stability of connectivity. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of new test case related to AC-Barring per PLMN 8.1.2.6a |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE behavior after receiving some parameters on ac-BarringPerPLMN-List will not be guaranteed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/36.523-2 CR 1489 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-250793r1 : - Pre-test conditions of UE is replaced with “None.” - Step 7 and 8 on the original test procedure of 8.1.2.6a is deleted. - IE of cellReservedForOperatorUse is added to Table 8.1.2.6a.3-3-1. |  |  |  |  |  |  |  |  |

[Start of change]

8.1.2.6a	RRC connection establishment / non-zero percent access probability for MO data, no restriction for MO signalling / AC-Barring per PLMN

8.1.2.6a.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {
  when { UE reselects a new cell which belongs to different TA and broadcasts a SystemInformationBlockType2 message including ac-BarringForMO-Data-r12 indicating no restriction for MO signalling }

then { UE transmits an RRCConnectionRequest message }

}

(2)

with { UE in E-UTRA RRC_IDLE state having received a SystemInformationBlockType2 message including ac-BarringForMO-Data-r12 indicating non zero percent access probability for MO data }

ensure that {
  when { UE has user data pending }

then { UE does not transmit any RRCConnectionRequest message or UE transmits an RRCConnectionRequest message }

}

8.1.2.6a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331 clauses 5.3.3.2 and 5.3.3.11. Unless otherwise stated these are Rel-12 requirements.

[TS 36.331, clause 5.3.3.2]

The UE initiates the procedure when upper layers request establishment of an RRC connection while the UE is in RRC_IDLE.

Upon initiation of the procedure, the UE shall:

1>	if SystemInformationBlockType2 includes ac-BarringPerPLMN-List and the ac-BarringPerPLMN-List contains an AC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35]):

2>	select the AC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers;

2>	in the remainder of this procedure, use the selected AC-BarringPerPLMN entry (i.e. presence or absence of access barring parameters in this entry) irrespective of the common access barring parameters included in SystemInformationBlockType2;

1>	else

2>	in the remainder of this procedure use the common access barring parameters (i.e. presence or absence of these parameters) included in SystemInformationBlockType2;

...

1>	else if the UE is establishing the RRC connection for mobile originating calls:

2>	perform access barring check as specified in 5.3.3.11, using T303 as "Tbarring" and ac-BarringForMO-Data as "AC barring parameter";

2>	if access to the cell is barred:

3>	if SystemInformationBlockType2 includes ac-BarringForCSFB or the UE does not support CS fallback:

4>	inform upper layers about the failure to establish the RRC connection and that access barring for mobile originating calls is applicable, upon which the procedure ends;

...

1>	else if the UE is establishing the RRC connection for mobile originating signalling:

2>	perform access barring check as specified in 5.3.3.11, using T305 as "Tbarring" and ac-BarringForMO-Signalling as "AC barring parameter";

2>	if access to the cell is barred:

3>	inform upper layers about the failure to establish the RRC connection and that access barring for mobile originating signalling is applicable, upon which the procedure ends;

...

1>	apply the default physical channel configuration as specified in 9.2.4;

1>	apply the default semi-persistent scheduling configuration as specified in 9.2.3;

1>	apply the default MAC main configuration as specified in 9.2.2;

1>	apply the CCCH configuration as specified in 9.1.1.2;

1>	apply the timeAlignmentTimerCommon included in SystemInformationBlockType2;

1>	start timer T300;

- initiate transmission of the RRCConnectionRequest message in accordance with 5.3.3.3;

NOTE 2:	Upon initiating the connection establishment procedure, the UE is not required to ensure it maintains up to date system information applicable only for UEs in RRC_IDLE state. However, the UE needs to perform system information acquisition upon cell re-selection.

[TS 36.331, clause 5.3.3.11]

1>	if timer T302 or "Tbarring" is running:

2>	consider access to the cell as barred;

1>	else if SystemInformationBlockType2 includes "AC barring parameter":

2>	if the UE has one or more Access Classes, as stored on the USIM, with a value in the range 11..15, which is valid for the UE to use according to TS 22.011 [10] and TS 23.122 [11], and

NOTE:	ACs 12, 13, 14 are only valid for use in the home country and ACs 11, 15 are only valid for use in the HPLMN/ EHPLMN.

2>	for at least one of these valid Access Classes the corresponding bit in the ac-BarringForSpecialAC contained in "AC barring parameter" is set to zero:

3>	consider access to the cell as not barred;

2>	else:

3>	draw a random number 'rand' uniformly distributed in the range: 0 ≤ rand < 1;

3>	if 'rand' is lower than the value indicated by ac-BarringFactor included in "AC barring parameter":

4>	consider access to the cell as not barred;

3>	else:

4>	consider access to the cell as barred;

1>	else:

2>	consider access to the cell as not barred;

1>	if access to the cell is barred and both timers T302 and "Tbarring" are not running:

2>	draw a random number 'rand' that is uniformly distributed in the range 0 ≤ rand < 1;

2>	start timer "Tbarring" with the timer value calculated as follows, using the ac-BarringTime included in "AC barring parameter":

"Tbarring" = (0.7+ 0.6 * rand) * ac-BarringTime.

8.1.2.6a.3	Test description

8.1.2.6a.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 11 are in different TA.

-	Cell 1 and Cell 11 as specified in TS 36.508[18] clause 4.4.1.2 are configured as shown in Table 8.1.2.6a.3.1–1.

Table 8.1.2.6a.3.1-1: PLMN identifiers

| Cell | PLMN names | MCC | MNC |
| --- | --- | --- | --- |
| 1,11 | PLMN1 | 001 | 01 |
|  | PLMN2 | 001 | 02 |

UE:

-	None.

Preamble:

-	The UE is in Idle mode (state 2) according to TS 36.508[18] after the UE is in state Loopback Activated (state 4) with condition UE TEST LOOP MODE B on Cell 1 (serving cell) according to [18].

8.1.2.6a.3.2	Test procedure sequence

Table 8.1.2.6a.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are assigned to satisfy RCell 1 < RCell 11. |

Table 8.1.2.6a.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 1 and Cell 11 level according to the row "T1" in table 8.1.2.6a.3.2-1. | - | - | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionRequest on Cell 11? | --> | RRCConnectionRequest | 1 | P |
| 3-6 | Steps 2 to 5 of the generic test procedure in TS 36.508 [18] subclause 6.4.2.7 are performed on Cell 11. Note: The UE performs a TAU procedure. | - | - | - | - |
| 7 | The SS initializes an internal counter K to 0 and L to 0. | - | - | - | - |
| - | EXCEPTION: Step 8 to 12b8 shall be repeated maximum 50 times unless K > 0 and L >0. (Note 1) | - | - | - | - |
| 8 | The SS transmits one IP packet to the UE on the DRB associated with the default EPS bearer context on Cell 11. | - | - | - | - |
| 9 | Wait for 1 s after the IP packet has been transmitted in step 8. | - | - | - | - |
| 10 | SS transmits an RRCConnectionRelease message to release the RRC connection. | <-- | RRCConnectionRelease | - | - |
| 11 | Wait for 5 s for the UE to enter E-UTRA RRC_IDLE state. | - | - | - | - |
| - | EXCEPTION: Steps 12a1 to 12b8 describe behaviours which vary depending on whether the UE transmits an RRCConnectionRequest message or not. | - | - | - | - |
| 12a1 | IF the UE does not transmit any RRCConnectionRequest message within 5s (Note 3) THEN the SS increments the counter K by 1. | - | - | - | - |
| - | EXCEPTION: Steps 12a9 to 12a10 can occur in any order | - | - | - | - |
| 12a2-12a9 | Steps 2 to 9 of the generic radio bearer establishment procedure (TS 36.508 4.5.3.3-1) are executed (Note 2). | - | - | - | - |
| - | EXCEPTION: Step 12a10 describes behaviour that depends on UE implementation; step 12a10 takes place if the UE has user data pending. | - | - | - | - |
| 12a10 | IF the UE has user data pending THEN the UE loop backs the IP packet received in step 9 on the DRB associated with the default EPS bearer context on Cell 11 within 10 s. | - | - | - | - |
| 12b1 | ELSE IF the UE transmits an RRCConnectionRequest message on Cell 11 THEN the SS increments the counter L by 1. | --> | RRCConnectionRequest | - | - |
| - | EXCEPTION: Steps 12b7 to 12b8 can occur in any order | - | - | - | - |
| 12b2-12b7 | Steps 4 to 9 of the generic radio bearer establishment procedure (TS 36.508 4.5.3.3-1) are executed. | - | - | - | - |
| 12b8 | The UE loop backs the IP packet received in step 9 on the DRB associated with the default EPS bearer context on Cell 11. | - | - | - | - |
| 13 | Check: Is the counter K > 0 and L > 0? | - | - | 2 | P |
| Note 1: There is an extremely low probability that the UE originating calls are barred, or not barred, 50 times consecutively. (The probability is 1.78´10-15). Therefore, 50 times is enough number of trials for this test case. Note 2: After UE enters RRC_CONNECTED T303 is stopped. Note 3: The UE starts T303. |  |  |  |  |  |

8.1.2.6a.3.3	Specific message contents

Table 8.1.2.6a.3.3-1: SystemInformationBlockType1 for Cell 1 and 11 (preamble and all steps, Table 8.1.2.6a.3.2-2)

| Derivation path: TS 36.508[18], Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| PLMN-IdentityList SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-IdentityInfo { | 2 entries |  |  |
| PLMN-IdentityInfo[1] SEQUENCE { |  | entry 1 |  |
| plmn-Identity[1] | 00101 | PLMN1 |  |
| cellReservedForOperatorUse[1] | notReserved |  |  |
| } |  |  |  |
| PLMN-IdentityInfo[2] SEQUENCE { |  | entry 2 |  |
| plmn-Identity[2] | 00211 | PLMN2 |  |
| cellReservedForOperatorUse[2] | Reserved |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.6a.3.3-2: SystemInformationBlockType2 for Cell 11 (preamble and all steps, Table 8.1.2.6a.3.2-2)

| Derivation Path: TS 36.508, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| ac-BarringPerPLMN-List-r12 SEQUENCE (SIZE (1.. maxPLMN-r11)) OF AC-BarringPerPLMN-r12 { | 1 entry |  |  |
| AC-BarringPerPLMN-r12[1] SEQUENCE { |  | entry 1 |  |
| plmn-IdentityIndex-r12 | 1 | PLMN1 |  |
| ac-BarringInfo-r12 SEQUENCE { |  |  |  |
| ac-BarringForEmergency-r12 | FALSE |  |  |
| ac-BarringForMO-Signalling-r12 | Not present |  |  |
| ac-BarringForMO-Data-r12 SEQUENCE { |  |  |  |
| ac-BarringFactor | p50 |  |  |
| ac-BarringTime | s64 |  |  |
| ac-BarringForSpecialAC | '11111'B |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.6a.3.3-3: CLOSE UE TEST LOOP (preamble, Table 8.1.2.6a.3.2-2)

| Derivation Path: 36.508, Table 4.7A-3, condition UE TEST LOOP MODE B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE test loop mode B LB setup |  |  |  |
| IP PDU delay | '0000 1000'B | 8 seconds |  |

Table 8.1.2.6a.3.3-4: RRCConnectionRequest (step 2, Table 8.1.2.6a.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| establishmentCause | mo-Signalling |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.6a.3.3-5: RRCConnectionRequest (step 12b1, Table 8.1.2.6a.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| establishmentCause | mo-Data |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

[End of change]