3GPP TSG-RAN5 Meeting #106	R5-250794r1

# Athens, Greece, 17th Feb 2024 – 21th Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5426 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case related to AC-Barring per PLMN 13.5.2d |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI12_Test |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As discussion paper(R5-242949) mentioned, Japan will use following 3GPP features to add the affected user to Relieving Carrier. -Access Control per PLMN So we need to introduce test cases related to ac-BarringPerPLMN-List for stability of connectivity. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of new test case related to AC-Barring per PLMN 13.5.2d |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE behavior after receiving some parameters on ac-BarringPerPLMN-List will not be guaranteed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/36.523-2 CR 1489 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-250794r1: - Pre-test conditions of UE is replaced with “None.” - Test procedure of 13.5.2d is replaced with “Same as test case 13.5.2a except using Tx of 128s in step 7.” - IE of cellReservedForOperatorUse is added to Table 13.5.2d.3-3-1. |  |  |  |  |  |  |  |  |

[Start of change]

13.5.2d	MTSI MO video call / SSAC in Connected mode / 0% access probability for MTSI MO video call / AC-Barring per PLMN

13.5.2d.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state having received a SysteminformationBlockType2 message including a ssac-BarringForMMTEL-Video-r12 set to 0% accessibility }

ensure that {
  when { the user initiates a MTSI MO video call }

then { the UE does not initiate a MTSI MO video call }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state having received a SysteminformationBlockType2 message including a ssac-BarringForMMTEL-Video-r12 and having failed to initiate a video call }

ensure that {
  when { ssac-BarringForMMTEL-Video-r12 is removed and the user initiates a MTSI MO video call during ac-BarringTime is running }

then { the UE does not initiate a MTSI MO video call }

}

(3)

with { UE in E-UTRA RRC_CONNECTED state having received a SysteminformationBlockType2 message including a ssac-BarringForMMTEL-Video-r12 and having failed to initiate a video call }

ensure that {
  when { the user initiates a MTSI MO video call after expiring ac-BarringTime }

then { the UE initiates a MTSI MO video call  }

}

13.5.2d.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.173 clause J.2.1.1 and TS 36.331 clause 5.3.3.10. Unless otherwise stated these are Rel-12 requirements.

[TS 24.173, clause J.2.1.1]:

The following information is provided by lower layer:

-	BarringFactorForMMTEL-Voice: barring rate for MMTEL voice;

-	BarringTimeForMMTEL-Voice: barring timer for MMTEL voice;

-	BarringFactorForMMTEL-Video: barring rate for MMTEL video; and

-	BarringTimeForMMTEL-Video: barring timer for MMTEL video.

Upon request from a user to establish a multimedia telephony communication session as described in subclause 5.2, the UE shall:

1)	if the multimedia telephony communication session to be established is an emergency session, then skip the rest of steps below and continue with session establishment as described in subclause 5.2;

2)	retrieve SSAC related information mentioned above from lower layers;

3)	if video is offered in the multimedia telephony communication session:

A)	if back-off timer Tx is running, reject the multimedia telephony communication session establishment and skip the rest of steps below; or

B)	else, then:

I)	draw a new random number "rand1" that is uniformly distributed in the range 0 ≤ rand1 < 1; and

II)	if the random number "rand1" is lower than BarringFactorForMMTEL-Video, then skip the rest of steps below and continue with session establishment as described in subclause 5.2;

III)	else, then;

i)	draw a new random number "rand2" that is uniformly distributed in the range 0 ≤ rand2 < 1; and

ii)	start back-off timer Tx with the timer value calculated using the formula:

Tx =  (0,7 + 0,6*rand2) * BarringTimeForMMTEL-Video; and

iii)	reject the multimedia telephony communication session establishment and skip the rest of steps below;

[TS 36.331 clause 5.3.3.10]:

Upon request from the upper layers, the UE shall:

1>	if SystemInformationBlockType2 includes ac-BarringPerPLMN-List and the ac-BarringPerPLMN-List contains an AC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35]):

2>	select the AC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers;

2>	in the remainder of this procedure, use the selected AC-BarringPerPLMN entry (i.e. presence or absence of access barring parameters in this entry) irrespective of the common access barring parameters included in SystemInformationBlockType2;

1>	else:

2>	in the remainder of this procedure use the common access barring parameters (i.e. presence or absence of these parameters) included in SystemInformationBlockType2;

1>	set the local variables BarringFactorForMMTEL-Voice and BarringTimeForMMTEL-Voice as follows:

2>	if ssac-BarringForMMTEL-Voice is present:

3>	if the UE has one or more Access Classes, as stored on the USIM, with a value in the range 11..15, which is valid for the UE to use according to TS 22.011 [10] and TS 23.122 [11], and

NOTE:	ACs 12, 13, 14 are only valid for use in the home country and ACs 11, 15 are only valid for use in the HPLMN/ EHPLMN.

3>	if, for at least one of these Access Classes, the corresponding bit in the ac-BarringForSpecialAC contained in ssac-BarringForMMTEL-Voice is set to zero:

4>	set BarringFactorForMMTEL-Voice to one and BarringTimeForMMTEL-Voice to zero;

3>	else:

4>	set BarringFactorForMMTEL-Voice and BarringTimeForMMTEL-Voice to the value of ac-BarringFactor and ac-BarringTime included in ssac-BarringForMMTEL-Voice, respectively;

2>	else set BarringFactorForMMTEL-Voice to one and BarringTimeForMMTEL-Voice to zero;

1>	set the local variables BarringFactorForMMTEL-Video and BarringTimeForMMTEL-Video as follows:

2>	if ssac-BarringForMMTEL-Video is present:

3>	if the UE has one or more Access Classes, as stored on the USIM, with a value in the range 11..15, which is valid for the UE to use according to TS 22.011 [10] and TS 23.122 [11], and

3>	if, for at least one of these Access Classes, the corresponding bit in the ac-BarringForSpecialAC contained in ssac-BarringForMMTEL-Video is set to zero:

4>	set BarringFactorForMMTEL-Video to one and BarringTimeForMMTEL-Video to zero;

3>	else:

4>	set BarringFactorForMMTEL-Video and BarringTimeForMMTEL-Video to the value of ac-BarringFactor and ac-BarringTime included in ssac-BarringForMMTEL-Video, respectively;

2>	else set BarringFactorForMMTEL-Video to one and BarringTimeForMMTEL-Video to zero;

1>	forward the variables BarringFactorForMMTEL-Voice, BarringTimeForMMTEL-Voice, BarringFactorForMMTEL-Video and BarringTimeForMMTEL-Video to the upper layers;

13.5.2d.3	Test description

13.5.2d.3.1	Pre-test conditions

System Simulator:

-	Cell 1 as specified in TS 36.508[18] clause 4.4.1.2 are configured as shown in Table 13.5.2c.3.1–1.

Table 13.5.2c.3.1-1: PLMN identifiers

| Cell | PLMN names | MCC | MNC |
| --- | --- | --- | --- |
| 1 | PLMN1 | 001 | 01 |
|  | PLMN2 | 002 | 11 |

UE:

-	None.

Preamble:

-	The UE is in state Registered for PLMN1, Generic RB Established (state 3) on Cell 1 according to [18].

13.5.2d.3.2	Test procedure sequence

Table 13.5.2d.3.2-1: Main behaviour

Same as test case 13.5.2a except using Tx of 128s in step 7.

13.5.2d.3.3	Specific message contents

Table 13.5.2d.3-3-1: SystemInformationBlockType1 for Cell 1 (preamble and all steps, Table 13.5.2d.3.2-1)

| Derivation path: TS 36.508[18], Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| PLMN-IdentityList SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-IdentityInfo { | 2 entries |  |  |
| PLMN-IdentityInfo[1] SEQUENCE { |  | entry 1 |  |
| plmn-Identity[1] | 00101 | PLMN1 |  |
| cellReservedForOperatorUse[1] | notReserved |  |  |
| } |  |  |  |
| PLMN-IdentityInfo[2] SEQUENCE { |  | entry 2 |  |
| plmn-Identity[2] | 00211 | PLMN2 |  |
| cellReservedForOperatorUse[2] | Reserved |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 13.5.2d.3.3-2: SystemInformationBlockType2 for Cell 1 (preamble, Table 13.5.2d.3.2-1)

| Derivation Path: TS 36.508 [18], Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| ac-BarringPerPLMN-List-r12 SEQUENCE (SIZE (1.. maxPLMN-r11)) OF AC-BarringPerPLMN-r12 { | 1 entry |  |  |
| AC-BarringPerPLMN-r12[1] SEQUENCE { |  | entry 1 |  |
| plmn-IdentityIndex-r12 | 1 | PLMN1 |  |
| ac-BarringInfo-r12 SEQUENCE { |  |  |  |
| ac-BarringForEmergency-r12 | FALSE |  |  |
| } |  |  |  |
| ssac-BarringForMMTEL-Video-r12 SEQUENCE { |  |  |  |
| ac-BarringFactor | p00 |  |  |
| ac-BarringTime | s128 |  |  |
| ac-BarringForSpecialAC | '11111'B |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 13.5.2d.3.3-3: SystemInformationBlockType2 for Cell 1 (step 3, Table 13.5.2d.3.2-1)

| Derivation Path: TS 36.508 [18], Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| ac-BarringPerPLMN-List-r12 SEQUENCE (SIZE (1.. maxPLMN-r11)) OF AC-BarringPerPLMN-r12 { |  |  |  |
| AC-BarringPerPLMN-r12[1] SEQUENCE { |  | entry 1 |  |
| plmn-IdentityIndex-r12 | 1 | PLMN1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 13.5.2d.3.3-4: Paging (step3, Table 13.5.2d.3.2-1)

| Derivation Path: TS 36.508[18], Table 4.6.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| systemInfoModification | true |  |  |
| } |  |  |  |

[End of change]