# Title:	Test Tolerances for Random access tests in FR2

# Source:	Anritsu, Rohde & Schwarz

# 1. Introduction

The purpose of this document is to present the methodology and principles used for the development of Test Tolerances for Contention Based Random Access Test.

The test cases in question, from TS 38.533, are:

- 5.3.2.2.1, Contention based random access test in FR2 for PSCell in EN-DC

- 5.3.2.2.2, Non-contention based random access test in FR2 for PSCell in EN-DC

- 7.3.2.2.1, Contention based random access test in FR2 for NR standalone

- 7.3.2.2.2, Non-Contention based random access test in FR2 for NR standalone

This document describes the process to derive the Test Tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. Prototype test case in Annex A of TS 38.133

The Test Requirements are defined in the following extracts from TS 38.133 v15.28.0 Annex A.

A standalone test case is chosen, to show only the NR-related parameters.

<< Extract from TS 38.133 >>

##### A.7.3.2.2.1	Contention based random access test in FR2 for NR Standalone

A.7.3.2.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the behavior of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits. This test will verify the requirements in Clause 6.2.2.2 and Clause 7.1.2 in an AWGN model.

For this test one cell is used, with the configuration of Cell 1 configured as PCell or SCell in FR2. Supported test parameters are shown in Table A.7.3.2.2.1.1-1. UE capable of SA with PCell or SCell in FR2 needs to be tested by using the parameters in Table A.7.3.2.2.1.1-2 and Table A.7.3.2.2.1.1-3.

Table A.7.3.2.2.1.1-1: Supported test configurations for contention based random access test in FR2 for NR Standalone

| Config | Description |
| --- | --- |
| 1 | NR PSCell/SCell 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.3.2.2.1.1-2: General test parameters for contention based random access test in FR2 for NR Standalone

| Parameter |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- |
| SSB Configuration | Config 1 |  | SSB.1 FR2 | As defined in A.3.10 |
| CSI-RS for tracking | Config 1 |  | TRS.2.1 TDD |  |
| Duplex Mode for Cell 1 | Config 1 |  | TDD |  |
| TDD Configuration | Config 1 |  | TDDConf.3.1 | As defined in A.3.1.4 |
| BWchannel | Config 1 | MHz | 100: NRB,c = 66 |  |
| Data RBs allocated | Config 1 |  | 24 |  |
| OCNG Pattern Note 1 |  |  | OP.3 | As defined in A.3.2.1. |
| PDSCH Reference Channel Note 2 | Config 1 |  | SR.3.1 TDD | As defined in A.3.1.1. |
| RMSI CORESET Reference Channel | Config 1 |  | CR.3.1 TDD | As defined in A.3.1.2 |
| NR RF Channel Number |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  | dB |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  | dB |  |  |
| ss-PBCH-BlockPower |  | dBm/ SCS | +20 +ΔUL | As defined in TS 38.331 [2]. ΔUL is derived from the uplink calibration process Note 3 |
| Configured UE transmitted power () |  | dBm | maximum value configurable for certain power class | As defined in clause 6.2.4 in TS 38.101-2 [19] |
| PRACH Configuration |  |  | FR2 PRACH configuration 1 | As defined in A.3.8.3, with exceptions as defined below |
| rsrp-ThresholdSSB |  | dBm | RSRP_69 +ΔDL | RSRP_69 corresponds to -88dBm. ΔDL is derived from the downlink calibration process Note 4 |
| preambleReceivedTargetPower |  | dBm | -100 | As defined in TS 38.331 [2] |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. Note 3: The ΔUL value is calculated as -ROUND(PPRACH0 -1), where PPRACH0 is the measured first PRACH power with -80.6dBm/SCS applied, preambleReceivedTargetPower = -100dBm and ss-PBCH-BlockPower = 20dBm. These values are used during the uplink calibration process carried out before the test case is run, with the UE configured to send PRACH. Note 4: The ΔDL value is calculated as (RSRP_REP – RSRP_76), where RSRP_REP is the SS-RSRP Reported value in Table 10.1.6.1-1 with -80.6dBm/SCS applied. These values are used during the downlink calibration process carried out before the test case is run, with the UE configured to report SS-RSRP. For a Reported value RSRP_x, x is treated as a positive integer value. |  |  |  |  |

Table A.7.3.2.2.1.1-3: OTA-related test parameters for contention based random access test in FR2 for NR Standalone

| Parameter |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- |
| AoA setup |  |  | Setup 1 | As defined in A.3.15.1 |
| Assumption for UE beamsNote 3 |  |  | Rough |  |
| SSB with index 0 | Es Note1 | dBm/SCS | -80.6 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB |
|  | SSB_RP | dBm/SCS | -80.6 |  |
|  | Es/IotBB | dB | 21.09 |  |
|  | Io | dBm/95.04 MHz | -56.01 | Io in symbols containing SSB index 0 |
| SSB with index 1 | Es Note1 | dBm/SCS | -95.0 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB |
|  | SSB_RP | dBm/SCS | -95.0 |  |
|  | Es/IotBB | dB | 6.69 |  |
|  | Io | dBm/95.04 MHz | -70.41 | Io in symbols containing SSB index 1 |
| Propagation Condition |  | - | No external noise (Note 4) |  |
| Note 1: No articial noise is applied in this test. Note 2: Void. Note 3: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 4: The downlink connection between the System Simulator and the UE is without Additive White Gaussian Noise, and has no fading or multipath effects as specified in TS 38.521-2 B.0 [29]. |  |  |  |  |

A.7.3.2.2.1.2	Test Requirements

Contention based random access is triggered by not explicitly assigning a random access preamble via dedicated signalling in the downlink.

A.7.3.2.2.1.2.1	Random Access Preamble Transmission

To test the UE behavior specified in Clause 6.2.2.2.1.1 the System Simulator shall receive the Random Access Preamble which belongs to one of the Random Access Preambles associated with the SSB with index 0, which has SS-RSRP above the configured rsrp-ThresholdSSB.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2.2. The power of the first preamble shall be 0.6 dBm to be received at TE with an accuracy specified in clause 6.3.4.2 of TS 38.101-2 [19]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-2 [19].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Clause 7.1.2.

A.7.3.2.2.1.2.2	Random Access Response Reception

To test the UE behavior specified in Clause 6.2.2.2.1.2 the System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 3 preambles have been received by the System Simulator. In response to the first 2 preambles, the System Simulator shall transmit a Random Access Response not corresponding to the transmitted Random Access Preamble.

The UE may stop monitoring for Random Access Response(s) and shall transmit the msg3 if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2.2. The power of the first preamble shall be 0.6 dBm to be received at TE with an accuracy specified in clause 6.3.4.2 of TS 38.101-2 [19]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-2 [19].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Clause 7.1.2.

A.7.3.2.2.1.2.3	No Random Access Response Reception

To test the UE behavior specified in clause 6.2.2.2.1.3 the System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 3 preambles have been received by the System Simulator. The System Simulator shall not respond to the first 2 preambles.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if no Random Access Response is received within the RA Response window.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2.2. The power of the first preamble shall be 0.6 dBm to be received at TE with an accuracy specified in clause 6.3.4.2 of TS 38.101-2 [19]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-2 [19].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Clause 7.1.2.

A.7.3.2.2.1.2.4	Receiving an UL grant for msg3 retransmission

To test the UE behavior specified in clause 6.2.2.2.1.4 the System Simulator shall provide an UL grant for msg3 retransmission following a successful Random Access Response.

The UE shall re-transmit the msg3 upon the reception of an UL grant for msg3 retransmission.

A.7.3.2.2.1.2.5	Reception of an Incorrect Message over Temporary C-RNTI

To test the UE behavior specified in Clause 6.2.2.2.1.5 the System Simulator shall send a message addressed to the temporary C-RNTI with a UE Contention Resolution Identity included in the MAC control element not matching the CCCH SDU transmitted in msg3 uplink message.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires unless the received message includes a UE Contention Resolution Identity MAC control element and the UE Contention Resolution Identity included in the MAC control element matches the CCCH SDU transmitted in the uplink message.

A.7.3.2.2.1.2.6	Reception of a Correct Message over Temporary C-RNTI

To test the UE behavior specified in Clause 6.2.2.2.1.5 the System Simulator shall send a message addressed to the temporary C-RNTI with a UE Contention Resolution Identity included in the MAC control element matching the CCCH SDU transmitted in the msg3 uplink message.

The UE shall send ACK if the Contention Resolution is successful.

A.7.3.2.2.1.2.7	Contention Resolution Timer expiry

To test the UE behavior specified in Clause 6.2.2.2.1.6 the System Simulator shall not send a response to a msg3.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if the Contention Resolution Timer expires.

<< clauses skipped >>

### A.3.8.3	PRACH configurations in FR2

#### A.3.8.3.1	FR2 PRACH configuration 1

FR2 PRACH configuration 1 in this clause provides the typical PRACH configuration for SSB-based contention based random access in FR2.

Table A.3.8.3.1-1: Parameters for FR2 PRACH configuration 1

| Field | Value | Comment |
| --- | --- | --- |
| prach-ConfigurationIndex | 190 | Preamble Format C2, with 10ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-4 in TS 38.211 [6]. |
| msg1-SubcarrierSpacing | Same as UL carrier SCS |  |
| totalNumberOfRA-Preambles | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | No group B. |
| prach-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB | oneFourth, n48 | OneFourth: 1 SSB associated with 4 RACH occasions n48: 48 contention based preambles per SSB |
| msg1-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| rsrp-ThresholdSSB | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [2]. |
| ra-ContentionResolutionTimer | sf48 | 48 sub-frames |
| powerRampingStep | dB2 |  |
| preambleReceivedTargetPower | dBm-120 |  |
| preambleTransMax | n6 | Max number of RA preamble transmission performed before declaring a failure is 6 |
| ra-ResponseWindow | sl10 | 10 slots |
| zeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 20 ms, as defined in table 7.2-1 in TS 38.321 [7]. |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

# 3. Discussion

The test consists of seven subtests with one cell configured; the difference between the Subtests is related to the UE behaviour scenarios for the random access procedure. Of the Subtests, Tests 1, 2, 3, 5 and 7 require test system measurement uncertainties to be specified and Test Tolerances to be applied to the Test Requirements. Tests 4 and 6 are functional tests and are not covered here.

The UE shall calculate PRACH transmission power according to the PRACH power formula, and apply this power level at the first preamble or additional preambles. The transmit timing of all PRACH transmissions shall be within the accuracy specified in TS 38.133. Overall there are four types of requirement:

- Choosing the correct SSB index according to levels and rsrp-ThresholdSSB – see section 4

- Absolute power of first PRACH preamble – see section 5

- Relative power of subsequent two PRACH preambles – see section 6

- Timing of each PRACH preamble – see section 7

Choosing the correct SSB index and the Downlink calibration process

The signal level for SSB index 0 is chosen to be decisively above the signalled rsrp-ThresholdSSB, and the signal level for SSB index 1 to be decisively below the signalled rsrp-ThresholdSSB. As there is a wide variability of mapping between applied SSB_RP and reported SS-RSRP, the Downlink calibration process determines a value ΔDL, which is used to modify the signalled value of rsrp-ThresholdSSB.

The diagram below shows a typical scenario, with the left Y-axis showing applied values, and the right Y-axis showing reported or signalled values. The difference between them is quantified by ΔDL.

The Test Tolerance analysis assumes that the Downlink calibration process has been carried out to determine the value ΔDL.

Absolute power of first PRACH preamble and the Uplink calibration process

The UE determines the uplink power based on the UE-measured downlink power and the UE’s estimate of path loss, with detail given in section 5. As there is a wide variability of mapping between applied SSB_RP and reported SS-RSRP, the uplink power could be anywhere within a very wide range, and there may not be enough uplink range to measure the UE power steps. The Uplink calibration process determines a value ΔUL, which is used to modify the signalled value of ss-PBCH-BlockPower.

PPRACH = preambleReceivedTargetPower + ss-PBCH-BlockPower – measured RSRP

The diagram below shows why the power of the first PRACH preamble needs to be placed at the optimum start point, to ensure that two subsequent power steps can be measured.

The Test Tolerance analysis assumes that the Uplink calibration process has been carried out to determine the value ΔUL, to modify the signalled value of ss-PBCH-BlockPower and set the power of the first PRACH preamble at the optimum start point.

# 4. Calculation of Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters are copied from Tables A.7.3.2.2.1.1-1, A.7.3.2.2.1.1-2 and A.7.3.2.2.1.1-3 in TS 38.133. The key parameters are selected as the minimum set to define the cell power levels, and which are subject to a test system uncertainty which may affect the verdict of the test. Signalled parameters including the threshold are also copied, and the angle of arrival of the radiated signals is also recorded.

A number of UE core requirement values and their associated parameters are also entered in this section, taking into account the angle of arrival of the radiated signals. The requirements are entered because they affect the radiated signal received by the UE, or affect how the UE sees the received signals at baseband.

The key parameters appear in section a) of the accompanying spreadsheet.

b) Derived parameters

A number of derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

Some UE core requirements in section a) are converted into UE-related derived parameters, in a form that allows them to be used in later sections of the spreadsheet. The rationale and method for UE gain G midpoint is given in TR 38.903 clause A.2.6.

c) Uncertainties

The Test system provides SSB index 0 and SSB index 1, on 1 frequency, without AWGN. We propose to control the following parameters:

- SSB index 0 Ês averaged over BWConfig uncertainty: ±6.0 dB

- SSB index 0 Ês for PRBs #RBJ-RBJ+19 uncertainty: ±6.0 dB

- SSB index 1 Ês averaged over BWConfig uncertainty: ±6.0 dB

- SSB index 1 Ês for PRBs #RBJ-RBJ+19 uncertainty: ±6.0 dB

In this test the UE measures the SS-RSRP for SSB index 0 and SSB index 1 over specific Physical Resource Block (PRB) numbers #RBJ to #RBJ+19 for 120kHz SCSSSB. Two sets of parameters are therefore given.

This choice forms a minimum set, so the superposition principle can be applied if necessary.

UE-related uncertainties are considered, together with selected UE core requirements that have an allowed range, and are therefore handled as a UE uncertainty:

- UE SS-RSRP relative accuracy: Value taken from TS 38.133

- Spherical coverage gain variation: Value derived, using method defined in TR 38.903 A.2.1

- UE gain G variation: Value derived, using method defined in TR 38.903 A.2.6

The Test system also measures the UE’s uplink power. We propose to control the following parameters:

- Uplink absolute power measurement uncertainty: ±6.0 dB

- Uplink relative power measurement uncertainty: ±FFS dB

The UE measurement accuracy needs to be considered because when the UE makes the comparison with the threshold, it has some inaccuracy which could affect the SSB index selection.

The UE measures the SS-RSRP of each SSB index and compares the results to the signalled rsrp-ThresholdSSB value. As the signalled rsrp-ThresholdSSB value is obtained from the Downlink calibration process, the UE relative measurement accuracy given in TS 38.133 is relevant:

### 10.1.3	Intra-frequency RSRP accuracy requirements for FR2

#### 10.1.3.1	Intra-frequency SS-RSRP accuracy requirements

<< clauses skipped >>

##### 10.1.3.1.2	Relative SS-RSRP Accuracy

The relative accuracy of SS-RSRP is defined as the SS-RSRP measured from one cell compared to the SS-RSRP measured from another cell on the same frequency, or between any two SS-RSRP levels measured on the same cell in FR2.

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.3.1.2-1: SS-RSRP Intra frequency relative accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 2 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 1 |  | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |
| 6 | 9 | ≥-6 | Same value as SSB_RP in Table B.2.2-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. Note 4: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. |  |  |  |  |  |

±6dB accuracy is selected for normal conditions. The relative accuracy requirement refers to RSRP side conditions in TS 38.133 Annex B.2.2:

Table B.2.2-2: Conditions for intra-frequency measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE power class |  |  |  | UE power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-6 |
|  |  | n258 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 |  |  |
|  |  | n259 |  |  | -108.5 |  |  |  |
|  |  | n260 | -125.3+Y1 |  | -109.5 | -125.8+Y4 |  |  |
|  |  | n261 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-6 |
|  |  | n258 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 |  |  |
|  |  | n259 |  |  | -95.7 |  |  |  |
|  |  | n260 | -117.3+Z1 |  | -96.9 | -113.8+Z4 |  |  |
|  |  | n261 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 |  |  |
| Note 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. Note 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. Note 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

The SSB Ês/Iot side condition is ≥ -6dB in both tables. Band n259 gives the highest SSB_RP side condition of -108.5dBm/120kHz, and we allow up to 1dB for ∆MBP,n.

d) Controlled parameters critical to verdict

In many RRM test cases there is not a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict.

It is therefore essential to identify those parameters determining the test verdict, and also the side conditions that should be met for the UE to be tested correctly. The 9 controlled parameters listed in the accompanying spreadsheet have been derived by study of the test case and by careful reading of the relevant clauses in TS 38.133. The reason for each parameter being critical to the test verdict is given briefly in the “Comment” column of section d) in the accompanying spreadsheet. Information about the value to be achieved is given in the “Requirement” column of section g) in the spreadsheet.

Having identified the parameters critical to the test verdict which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is just the ratio (effect on a critical parameter y / a test equipment uncertainty x), and is usually in dB/dB.

Where there is a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict, the sensitivity factors are derived by inspection as one, or if no effect, as zero.

Where there is not a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict, the calculation of sensitivity factors is done using a method selected from TR 38.903. SSB index 0 and SSB index 1 Es/Iot at UE baseband and applied Io are calculated according to TR 38.903 A.2.7.2 and A.2.7.3 respectively, but taking into account that SSB index 0 and SSB index 1 occupy different symbols and do not overlap.

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, taking into account the uncertainties and sensitivity factors for each parameter that can be set by the test equipment, and the UE SS-RSRP relative measurement accuracy where relevant. This process follows the superposition principle. More details and explanation can be found in section 4 of TR 38.903.

When the test system uncertainties and the UE SS-RSRP reporting accuracy are combined, the test system uncertainties are added arithmetically to the UE S-RSRP reporting accuracy. If all the uncertainties were combined root-sum square, the resulting smaller test limits could cause a conformant test system to fail a conformant UE, which would be unacceptable.

e) Determine parameters to offset

In the downlink calibration process which is carried out before the test case is run, the UE is configured to report SS-RSRP. The ΔDL value is calculated as (RSRP_REP – RSRP_76), where RSRP_REP is the SS-RSRP Reported value in Table 10.1.6.1-1 with -80.6dBm/SCS applied. For a Reported value RSRP_x, x is treated as a positive integer value.

In the test case, rsrp-ThresholdSSB is signalled with the value (RSRP_69 +ΔDL). With this value selection of SSB index 0 will be reliable, and no other offsets are required. Also, SSB index 1 will not be selected.

In the uplink calibration process which is carried out before the test case is run, the UE is configured to send PRACH. ΔUL value is calculated as -ROUND(PPRACH0 -1), where PPRACH0 is the measured first PRACH power with -80.6dBm/SCS applied, preambleReceivedTargetPower = -100dBm and ss-PBCH-BlockPower = 20dBm.

In the test case, ss-PBCH-BlockPower is signalled with the value (+20 +ΔUL). With this value the first PRACH power should be centred in the uplink range, and no other offsets are required.

f) Parameters modified by Test Tolerances

Based on the decision in e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, modified by the Test Tolerances (applied offsets).

Re-derived parameters are calculated using the same methods as were used in step b).

g) Check controlled parameters Min/Max

Using a format similar to that in step d), the nominal value of each controlled parameter is recalculated, as some will have changed from the original due to the application of the Test Tolerances in step f).

The minimum and maximum values, due to variability from uncertainties, of controlled parameters is then calculated and compared against the requirements (Es/IotBB, Applied SSB_RP, Applied Io, UE measured SS-RSRP relative to threshold). The critical requirement for each parameter is given in the “Requirement” column of section g) in the accompanying spreadsheet. The cases closest to limit are identified by turquoise cells in the spreadsheet. If all the requirements are met, then the chosen offsets are acceptable.

It can be seen that with the uncertainty values and Test Tolerances proposed, the requirements are met.

# 5. Calculation of Test Requirements for first PRACH preamble Absolute power

5.1 UE calculation of first PRACH preamble power

To determine the PRACH power, the UE uses the formula specified in TS 38.213 clause 7.4:

A UE determines a transmission power for a physical random access channel (PRACH), , on active UL BWP  of carrier  of serving cell  based on DL RS for serving cell  in transmission occasion  as

[dBm],

where  is the UE configured maximum output power defined in [8-1, TS 38.101-1], [8-2, TS38.101-2] and [38.101-3] for carrier  of serving cell  within transmission occasion ,  is the PRACH target reception power PREAMBLE_RECEIVED_TARGET_POWER provided by higher layers [11, TS 38.321] for the active UL BWP  of carrier  of serving cell , and  is a pathloss for the active UL BWP  of carrier  based on the DL RS associated with the PRACH transmission on the active DL BWP of serving cell  and calculated by the UE in dB as referenceSignalPower – higher layer filtered RSRP in dBm, where RSRP is defined in [7, TS 38.215] and the higher layer filter configuration is defined in [12, TS 38.331]. If the active DL BWP is the initial DL BWP and for SS/PBCH block and CORESET multiplexing pattern 2 or 3, as described in Clause 13, the UE determines  based on the SS/PBCH block associated with the PRACH transmission.

If a PRACH transmission from a UE is not in response to a detection of a PDCCH order by the UE, or is in response to a detection of a PDCCH order by the UE that triggers a contention based random access procedure, or is associated with a link recovery procedure where a corresponding index  is associated with a SS/PBCH block, as described in Clause 6, referenceSignalPower is provided by ss-PBCH-BlockPower.

The test case defines Configured UE transmitted power PCMAX,f,c to be signalled as +23 dBm for PC3, +35 dBm for PC1, and this turns out to be much higher than the pathloss term, so the formula simplifies to:

PPRACH = PPRACH,target + PL [dBm]

Substituting the formula for PL, noting that PPRACH,target is preambleReceivedTargetPower and that referenceSignalPower is provided by ss-PBCH-BlockPower:

PPRACH = preambleReceivedTargetPower + ss-PBCH-BlockPower – higher layer filtered RSRP [dBm]

In this test case preambleReceivedTargetPower is signalled as -100dBm, referenceSignalPower is ss-PBCH-BlockPower signalled as (+20 +ΔUL), and the higher layer RSRP as reported by the UE is (-80.6 + ΔDL) for SSB index 0. Typically, if the UE has gain G at the midpoint value of 5dB for PC3 and 25dB for PC1, ΔDL =5dB and ΔUL =5dB for PC3 and 25dB for PC1, so the UE will calculate:

- PPRACH = -100 dBm + (+20 +ΔUL – (-80.6 + ΔDL)) = +0.6 dBm

We note that in the UE calculation of PPRACH the power is directly dependent on UE reported SS-RSRP, so any uncertainty in the test system downlink signal level will directly affect the UE PRACH power.

5.2 Overall PRACH absolute power Test requirements

The overall uncertainty is derived by taking into account both the downlink and uplink uncertainties. The spreadsheet calculates nominal first preamble power, and also the overall uncertainty. When the UE estimates uplink power based on downlink levels and path loss, the accuracy is given in TS 38.101-2:

### 6.3.4	Power control

#### 6.3.4.1	General

The requirements on power control accuracy apply under normal conditions and are defined as a directional requirement. The requirements are verified in beam locked mode on beam peak direction.

#### 6.3.4.2	Absolute power tolerance

The absolute power tolerance is the ability of the UE transmitter to set its initial output power to a specific value for the first sub-frame (1 ms) at the start of a contiguous transmission or non-contiguous transmission with a transmission gap larger than 20 ms. The tolerance includes the channel estimation error RSRP estimate.

The minimum requirements specified in Table 6.3.4.2-1 apply in the power range bounded by the minimum output power as specified in sub-clause 6.3.1 ('Pmin') and the maximum output power as specified in sub-clause 6.2.1 as minimum peak EIRP ('Pmax'). The intermediate power point 'Pint' is defined in table 6.3.4.2-2

Table 6.3.4.2-1: Absolute power tolerance

| Power Range | Tolerance |
| --- | --- |
| Pint ≥ P ≥ Pmin | ± 14.0 dB |
| Pmax ≥ P > Pint | ± 12.0 dB |

Table 6.3.4.2-2: Intermediate power point

| Power Parameter | Value |
| --- | --- |
| Pint | Pmax – 12.0 dB |

The UE absolute power tolerance includes any effect of RSRP inaccuracy, so the UE RSRP reporting accuracy is not used in the calculation.

In the test case the uplink power is modified by the uplink calibration process, which reduces the range which would otherwise be allowed by the UE Absolute power tolerance.

The test requirements are relative to the target PPRACH power +0.6 dBm.

# 6. Calculation of Test Requirements for subsequent PRACH preambles Relative power

The UE Relative power tolerance is given in TS 38.101-2:

#### 6.3.4.3	Relative power tolerance

The relative power tolerance is the ability of the UE transmitter to set its output power in a target sub-frame (1 ms) relatively to the power of the most recently transmitted reference sub-frame (1 ms) if the transmission gap between these sub-frames is 20 ms.

The minimum requirements specified in Table 6.3.4.3-1 apply when the power of the target and reference sub-frames are within the power range bounded by the minimum output power as defined in sub-clause 6.3.1 and Pint as defined in sub-clause 6.3.4.2. The minimum requirements specified in Table 6.3.4.3-2 apply when the power of the target and reference sub-frames are within the power range bounded by Pint as defined in sub-clause 6.3.4.2 and the measured PUMAX as defined in sub-clause 6.2.4.

For a test pattern that is either a monotonically increasing or monotonically decreasing power sweep over the range specified for Tables 6.3.4.3-1 and 6.3.4.3-2, 3 exceptions are allowed for each of the test patterns. For these exceptions, the power tolerance limit is a maximum of ±11.0 dB.

Table 6.3.4.3-1: Relative power tolerance, Pint ≥ P ≥ Pmin

| Power step ∆P (Up or down) (dB) | All combinations of PUSCH and PUCCH, PUSCH/PUCCH and SRS transitions between sub-frames, PRACH (dB) |
| --- | --- |
| ΔP < 2 | ±5.0 |
| 2 ≤ ΔP < 3 | ±6.0 |
| 3 ≤ ΔP < 4 | ±7.0 |
| 4 ≤ ΔP < 10 | ±8.0 |
| 10 ≤ ΔP < 15 | ±10.0 |
| 15 ≤ ΔP | ±11.0 |
| NOTE: The requirements apply with ue-BeamLockFunction enabled. |  |

Table 6.3.4.3-2: Relative power tolerance, PUMAX ≥ P > Pint

| Power step ∆P (Up or down) (dB) | All combinations of PUSCH and PUCCH, PUSCH/PUCCH and SRS transitions between sub-frames, PRACH (dB) |
| --- | --- |
| ΔP < 2 | ± 3.0 |
| 2 ≤ ΔP < 3 | ± 4.0 |
| 3 ≤ ΔP < 4 | ± 5.0 |
| 4 ≤ ΔP < 10 | ± 6.0 |
| 10 ≤ ΔP < 15 | ± 8.0 |
| 15 ≤ ΔP | ± 9.0 |
| NOTE 1: The requirements apply with ue-BeamLockFunction enabled. NOTE 2: For PUSCH to PUSCH transitions with the allocated resource blocks fixed in frequency and no transmission gaps other than those generated by downlink subframes, guard periods: for a power step ΔP = 1 dB, the relative power tolerance for transmission is ± 1.0 dB. |  |

FR2 PRACH configuration 1 specifies powerRampingStep as 2dB, so the UE PRACH step tolerance is ±6dB or ±4dB depending on the level. The test system uncertainty value for a relative power measurement on the uplink is ±FFS dB.

The test system uncertainty is added arithmetically to the UE Relative power tolerance. If the uncertainties were combined root-sum square, the resulting smaller test limits could cause a conformant test system to fail a conformant UE, which would be unacceptable.

Normal conditions, Pint ≥ P ≥ Pmin: (±6 dB ±FFS dB)

Normal conditions, PUMAX ≥ P > Pint: (±4 dB ±FFS dB)

The test system needs to choose which limit to apply. The tighter limits in Table 6.3.4.3-2 apply when “the power of the target and reference sub-frames are within the power range bounded by Pint as defined in sub-clause 6.3.4.2 and the measured PUMAX as defined in sub-clause 6.2.4”, so the absolute power of both PRACHs being compared must fall in the upper part of the range, above Pint. Pint is defined in TS 38.101-2:

Table 6.3.4.2-2: Intermediate power point

| Power Parameter | Value |
| --- | --- |
| Pint | Pmax – 12.0 dB |

Pmax is defined in TS 38.101-2 clause 6.2.1, and the value depends on the UE power class and operating band. The test system has an absolute uplink power measurement uncertainty of ±6 dB, so to ensure the actual power transmitted by the UE is > (Pmax – 12.0 dB), the measured value should be > (Pmax – 6.0 dB). The tighter relative power limit is only applied when the measured power of both PRACHs being compared is > (Pmax – 6dB), where Pmax is selected according to power class and operating band. Otherwise, the looser limit is applied.

# 7. Calculation of Test Requirements for Transmit Timing

The UE Timing requirement is given in TS 38.133:

### 7.1.2	Requirements

The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 7.1.2-1. This requirement applies:

-	when it is the first transmission in a DRX cycle for PUCCH, PUSCH and SRS, or it is the PRACH transmission, or it is the msgA transmission..

The UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the reference cell minus . The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the reference cell. NTA for PRACH is defined as 0.

(in Tc units) for other channels is the difference between UE transmission timing and the downlink timing immediately after when the last timing advance in clause 7.3 was applied. NTA for other channels is not changed until next timing advance is received. The value ofdepends on the duplex mode of the cell in which the uplink transmission takes place and the frequency range (FR). is defined in Table 7.1.2-2.

Table 7.1.2-1: Te Timing Error Limit

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 12*64*Tc |
|  |  | 30 | 10*64*Tc |
|  |  | 60 | 10*64*Tc |
|  | 30 | 15 | 8*64*Tc |
|  |  | 30 | 8*64*Tc |
|  |  | 60 | 7*64*Tc |
| 2 | 120 | 60 | 3.5*64*Tc |
|  |  | 120 | 3.5*64*Tc |
|  | 240 | 60 | 3*64*Tc |
|  |  | 120 | 3*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Uplink SCS is assumed to be the same as the DL SCS. Tc is defined in TS 38.211:

## 4.1	General

Throughout this specification, unless otherwise noted, the size of various fields in the time domain is expressed in time units  where  Hz and . The constant  where ,  and .

Check limits:

Tightest requirement for FR2 in NR is 3*64*Tc = 192*Tc, and Anritsu proposes a measurement uncertainty of [48]*Tc.

Formula: Upper limit + TT, Lower limit – TT:

In Configuration 1, selecting 120kHz SCS: ± 3.5*64*Tc ± [48]*Tc gives ± 224 ±[48]*Tc

# 8. Treatment of other configurations within the same test case

Not applicable

# 9. Treatment of related test cases

38.533 also contains a similar SA non-contention based test case in clause 7.3.2.2.2, which has the same signal levels and the same signalled parameters related to level, so we propose to apply the same level uncertainties and Test Tolerances. There is no need to provide a tab on the spreadsheet as all the numeric values are identical.

38.533 also contains similar EN-DC test cases in clauses 5.3.2.2.1 and 5.3.2.2.2, which have the same NR signal levels and the same signalled parameters related to level, so we propose to apply the same level uncertainties and Test Tolerances. There is no need to provide a tab on the spreadsheet as all the numeric values are identical.

# 10. Treatment of other UE Power Classes

The initial analysis was done for UE Power class 3, as defined in TS 38.101-2, Table 6.2.1.0-1.

As the calculations use different values for UE Power class 1, separate spreadsheet tabs are provided. The following parameters are changed, shown as underlined cells on both tabs of the spreadsheet:

- Configured UE transmitted power, section a)

- UE Refsens (Rx Beam peak direction), section a)

- UE Spherical coverage (%ile directions), section a)

- UE rough/fine beam gain ∆ Rx Beam peak, Y, section a)

- UE rough/fine beam gain ∆ Spherical Coverage, Z, section a)

- UE gain G, Rx beam peak, section a)

- Cell 1 SSB_RP index 0 and index 1 minimum requirement, section g) and associated Source/Reference

- n259 is removed, as it is not specified for PC1 in 38.133, B2.2