3GPP TSG-RAN5 Meeting #106	R5-250888r2

# Athens, Greece, 17th - 21st February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-1 | CR | 0595 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Annex C update with new TT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_pos-UEConTest |  |  |  |  |  | Date: |  |  | 2025-01-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test Tolerance for Rel16 FR1 test cases 14.2.x and 16.2.X have been updated Test Tolerance for Rel17 FR1 test cases 14.2.x, 14.4.x, 15.2.x, 16.2.X, 16.4.x and 16.5.x have been added |  |  |  |  |  |  |  |  |
| Summary of change: |  | Annex C for test cases 14.2.x, 14.4.x, 15.2.x, 16.2.X, 16.4.x and 16.5.x been updated according to latest TT analysis |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Annex C will not be aligned with latest TT analysis |  |  |  |  |  |  |  |  |
| Clauses affected: |  | C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.903 CR 0944, 0945, 0946, 0947, 0948, 0949, 0950, 0951, 0952, 0953, 0954, 0955, 0956, 0957, 0958, 0959, 0960, 0961 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RRM TT |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Values in Table C.4.7 have been aligned to those in the TT analysis. TR38.309 CR numbers added to cover pages r2: Values in Annex C have been aligned to those in the TT analysis for Rel17 test cases 15.2.x |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

### C.1.6	NR PRS-based measurement requirements

Table C.1.6-1: Maximum Test System Uncertainty for NR PRS-based Measurement Requirements

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 14.2.1 NR RSTD measurement period test case for single positioning frequency layer in FR1 SA | Noc ±1.5 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig PRS Ês3 / Noc ±0.3 dB averaged over BWConfig Ês3 / Noc ±0.3 dB averaged over BWConfig Response Time = 300ms | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN PRS Ês3 / Noc and Ês3 / Noc are the ratios of cell 3 signal / AWGN Signal-to-noise ratio uncertainty ±0.3 dB |
| 14.2.2 NR RSTD measurement period test case for dual positioning frequency layers in FR1 SA | Same as 14.2.1 |  |
| 14.2.3 NR RSTD measurement period test case for single positioning frequency layer in FR2 SA | Noc ±5.65 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig PRS Ês3 / Noc ±0.3 dB averaged over BWConfig Ês3 / Noc ±0.3 dB averaged over BWConfig Response Time = 300ms | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN PRS Ês3 / Noc and Ês3 / Noc are the ratios of cell 3 signal / AWGN Signal-to-noise ratio uncertainty ±0.3 dB |
| 14.2.4 NR RSTD measurement period test case for dual positioning frequency layers in FR2 SA | Same as 14.2.3 |  |
| 14.2.5 NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA | Same as 14.2.3 |  |
| 14.2.6 NR RSTD measurement reporting delay test case for single positioning frequency layer without measurement gap in FR1 SA | Same as 14.2.3 |  |
| 14.2.7 NR RSTD measurement reporting delay test case for single positioning frequency layer with Rx TEG in FR1 SA | Same as 14.2.3 |  |
| 14.2.8 NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA | Same as 14.2.3 |  |
| 14.2.9 NR RSTD measurement reporting delay test case for single positioning frequency layer without measurement gap in FR2 SA | Same as 14.2.3 |  |
| 14.2.10 NR RSTD measurement reporting delay test case for single positioning frequency layer with Rx TEG in FR2 SA | Same as 14.2.3 |  |
| 14.3.1 NR RSTD measurement accuracy test case for single positioning frequency layer in FR1 SA | Noc ±1.5 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig Cell Timing Difference = 32 Tc | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN |
| 14.3.2 NR RSTD measurement accuracy test case for dual positioning frequency layers in FR1 SA | Same as 14.3.1 |  |
| 14.3.3 NR RSTD measurement accuracy test case for single positioning frequency layer in FR2 SA | Noc ±5.65 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig Cell Timing Difference = 32 Tc | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN |
| 14.3.4 NR RSTD measurement accuracy test case for dual positioning frequency layer in FR2 SA | Same as 14.3.3 |  |
| 14.3.5 NR RSTD measurement accuracy test case for single positioning frequency layer with reduced number of samples in FR1 SA | Same as 14.3.3 |  |
| 14.3.6 NR RSTD measurement accuracy test case for single positioning frequency layer with Rx TEG in FR1 SA | Same as 14.3.3 |  |
| 14.3.7 NR RSTD measurement accuracy test case for single positioning frequency layer with reduced number of samples in FR2 SA | Same as 14.3.3 |  |
| 14.3.8 NR RSTD measurement accuracy test case for single positioning frequency layer with Rx TEG in FR2 SA | Same as 14.3.3 |  |
| 14.4.1 NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA | Same as 14.3.3 |  |
| 14.4.2 NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA | Same as 14.3.3 |  |
| 14.4.3 NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA | Same as 14.3.3 |  |
| 14.4.4 NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA | Same as 14.3.3 |  |
| 14.5.1 NR RSTD measurement accuracy test case for single positioning frequency layer in FR1 SA | Same as 14.3.3 |  |
| 14.5.2 NR RSTD measurement accuracy for single positioning frequency layer with reduced number of samples in FR1 SA | Same as 14.3.3 |  |
| 14.5.3 NR RSTD measurement accuracy test case for single positioning frequency layer in FR2 SA | Same as 14.3.3 |  |
| 14.5.4 NR RSTD measurement accuracy for single positioning frequency layer with reduced number of samples in FR2 SA | Same as 14.3.3 |  |
| 15.2.1 UE Rx-Tx time difference measurement period for single positioning frequency layer in FR1 SA | Noc ±1.5 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig Response Time = ± 300 ms | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN Tc = 1/(480000 x 4096), the basic timing unit defined in TS 38.211 [53] |
| 15.2.2 UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR1 SA | Same as 15.2.1 | Same as 15.2.1 |
| 15.2.3 UE Rx-Tx time difference measurement period for single positioning frequency layer in FR2 SA | Noc ±5.65 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig Response Time = ± 300 ms | PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN Tc = 1/(480000 x 4096), the basic timing unit defined in TS 38.211 [53] |
| 15.2.4 UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR2 SA | Same as 15.2.3 | Same as 15.2.3 |
| 15.2.5 UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA | Same as 15.2.1 | Same as 15.2.1 |
| 15.2.6 UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer without measurement gap in FR1 SA | Same as 15.2.1 | Same as 15.2.1 |
| 15.2.7 UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer with multiple RxTx TEGs in FR1 SA | Same as 15.2.1 | Same as 15.2.1 |
| 15.2.8 UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA | Same as 15.2.3 | Same as 15.2.3 |
| 15.2.9 UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer without measurement gap in FR2 SA | Same as 15.2.3 | Same as 15.2.3 |
| 15.2.10 UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer with RxTx TEG in FR2 SA | Same as 15.2.3 | Same as 15.2.3 |
| 15.3.1 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA | Noc ±1.5 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig Cell Timing Difference = 32 Tc | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN |
| 15.3.2 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR2 SA | Noc ±5.65 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig Cell Timing Difference = 32 Tc | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN |
| 15.3.5 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer with reduced number of samples in FR2 SA | Same as 15.3.2 |  |
| 15.3.6 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer with RxTx TEG in FR2 SA | Same as 15.3.2 |  |
| 15.4.1 UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer in FR1 SA | Same as 15.2.1 | Same as 15.2.1 |
| 15.4.2 UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA | Same as 15.2.1 | Same as 15.2.1 |
| 15.4.3 UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer in FR2 SA | Same as 15.2.3 |  |
| 15.4.4 UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA | Same as 15.2.3 |  |
| 15.5.3 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR2 SA | Noc ±5.65 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig Cell Timing Difference = 33 Tc | Same as 15.3.2 |
| 15.5.4 UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA | Noc ±5.65 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig Cell Timing Difference = 33 Tc | Same as 15.3.2 |
| 16.2.1 PRS-RSRP measurement period test case for single positioning frequency layer in FR1 SA | Noc ±1.5 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN Signal-to-noise ratio uncertainty ±0.3 dB |
| 16.2.2 PRS-RSRP measurement period test case for dual positioning frequency layer in FR1 SA | Noc ±1.5 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc2 ±0.3 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc2 and Ês2 / Noc2 are the ratios of cell 2 signal / AWGN Signal-to-noise ratio uncertainty ±0.3 dB |
| 16.2.3 PRS-RSRP measurement period test case for single positioning frequency layer in FR2 SA | Noc ±5.65 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig Response Time = ± 300 ms | PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN Tc = 1/(480000 x 4096), the basic timing unit defined in TS 38.211 [53] |
| 16.2.4 PRS-RSRP measurement period test case for dual positioning frequency layer in FR2 SA | Same as 16.2.3 | Same as 16.2.3 |
| 16.2.5 PRS-RSRP measurement reporting delay test case for single positioning frequency with reduced number of samples in FR1 SA | Same as 16.2.1 | Same as 16.2.1 |
| 16.2.6 PRS-RSRP measurement reporting delay test case for single positioning frequency layer without measurement gap in FR1 SA | Same as 16.2.1 | Same as 16.2.1 |
| 16.2.7 PRS-RSRP measurement reporting delay test case for single positioning frequency with reduced number of samples in FR2 SA | Same as 16.2.3 | Same as 16.2.3 |
| 16.2.8 PRS-RSRP measurement reporting delay test case for single positioning frequency layer without measurement gap in FR2 SA | Same as 16.2.3 | Same as 16.2.3 |
| 16.3.1 PRS-RSRP measurement accuracy with PRS in FR1 | Noc ±1.5 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN Signal-to-noise ratio uncertainty ±0.3 dB |
| 16.3.2 PRS-RSRP measurement accuracy with PRS in FR2 | Noc ±5.65 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN |
| 16.3.3 PRS-RSRP measurement accuracy with PRS in FR1 with reduced sample number | Same as 16.3.1 | Same as 16.3.1 |
| 16.3.4 PRS-RSRP measurement accuracy with PRS in FR2 with reduced sample number | Same as 16.3.2 | Same as 16.3.2 |
| 16.4.1 PRS-RSRP measurement reporting delay test case for single positioning frequency layer in FR1 SA | Same as 16.2.1 | Same as 16.2.1 |
| 16.4.2 PRS-RSRP measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA | Same as 16.2.1 | Same as 16.2.1 |
| 16.4.3 PRS-RSRP measurement reporting delay test case for single positioning frequency layer in FR2 SA | Same as 16.2.3 | Same as 16.2.3 |
| 16.4.4 PRS-RSRP measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA | Same as 16.2.3 | Same as 16.2.3 |
| 16.5.1 PRS-RSRP measurement accuracy test case for single positioning frequency layer with PRS in FR1 SA | Same as 16.3.1 | Same as 16.3.1 |
| 16.5.2 PRS-RSRP measurement accuracy test case for single positioning frequency layer with PRS in FR1 with reduced sample number | Same as 16.3.1 | Same as 16.3.1 |
| 16.5.3 PRS-RSRP measurement accuracy test case for single positioning frequency layer with PRS in FR2 SA | Same as 16.3.2 | Same as 16.3.2 |
| 16.5.4 PRS-RSRP measurement accuracy test case for single positioning frequency layer with PRS in FR2 with reduced sample number | Same as 16.3.2 | Same as 16.3.2 |
| 17.2.1 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer in FR1 SA | Noc ±1.5 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN Signal-to-noise ratio uncertainty ±0.3 dB |
| 17.2.2 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA | Same as 17.2.1 | Same as 17.2.1 |
| 17.2.3 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer without measurement gap in FR1 SA | Same as 17.2.1 | Same as 17.2.1 |
| 17.2.4 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer in FR2 SA | Noc ±5.65 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN |
| 17.2.5 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA | Same as 17.2.4 | Same as 17.2.4 |
| 17.2.6 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer without measurement gap in FR2 SA | Same as 17.2.4 | Same as 17.2.4 |
| 17.3.1 PRS-RSRPP measurement accuracy test case for single positioning frequency layer in FR1 SA | Same as 17.2.1 | Same as 17.2.1 |
| 17.3.2 PRS-RSRPP measurement accuracy test case for single positioning frequency layer with reduced number of samples in FR1 SA | Same as 17.2.1 | Same as 17.2.1 |
| 17.3.3 PRS-RSRPP measurement accuracy test case for single positioning frequency layer in FR2 SA | Same as 17.2.4 | Same as 17.2.4 |
| 17.3.4 PRS-RSRPP measurement accuracy test case for single positioning frequency layer with reduced number of samples in FR2 SA | Same as 17.2.4 | Same as 17.2.4 |
| 17.4.1 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer in FR1 SA | Same as 17.2.1 | Same as 17.2.1 |
| 17.4.2 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA | Same as 17.2.1 | Same as 17.2.1 |
| 17.4.3 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer in FR2 SA | Same as 17.2.4 | Same as 17.2.4 |
| 17.4.4 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA | Same as 17.2.4 | Same as 17.2.4 |
| 17.5.1 PRS-RSRPP measurement accuracy test case for single positioning frequency layer in FR1 SA | Same as 17.2.1 | Same as 17.2.1 |
| 17.5.2 PRS-RSRPP measurement accuracy test case for single positioning frequency layer with reduced number of samples in FR1 SA | Same as 17.2.1 | Same as 17.2.1 |
| 17.5.3 PRS-RSRPP measurement accuracy test case for single positioning frequency layer in FR2 SA | Same as 17.2.4 | Same as 17.2.4 |
| 17.5.4 PRS-RSRPP measurement accuracy test case for single positioning frequency layer with reduced number of samples in FR2 SA | Same as 17.2.4 | Same as 17.2.4 |

# < Unchanged sections omitted >

### C.2.5	NR PRS-based Measurement requirements

Table C.2.5-1: Test Parameter Relaxations for NR PRS-based Measurement requirements

| Clause | Test Parameter Relaxation |  |
| --- | --- | --- |
| 14.2.1 NR RSTD measurement period test case for single positioning frequency layer in FR1 SA | Response time | 300ms |
| 14.2.2 NR RSTD measurement period test case for dual positioning frequency layers in FR1 SA | Response time | 300ms |
| 14.2.3 NR RSTD measurement period test case for single positioning frequency layer in FR2 SA | Response time | 300ms |
| 14.2.4 NR RSTD measurement period test case for dual positioning frequency layers in FR2 SA | Response time | 300ms |
| 14.2.5 NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA | Response time | 300ms |
| 14.2.6 NR RSTD measurement reporting delay test case for single positioning frequency layer without measurement gap in FR1 SA | Response time | 300ms |
| 14.2.7 NR RSTD measurement reporting delay test case for single positioning frequency layer with Rx TEG in FR1 SA | Response time | 300ms |
| 14.2.8 NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA | Response time | 300ms |
| 14.2.9 NR RSTD measurement reporting delay test case for single positioning frequency layer without measurement gap in FR2 SA | Response time | 300ms |
| 14.2.10 NR RSTD measurement reporting delay test case for single positioning frequency layer with Rx TEG in FR2 SA | Response time | 300ms |
| 14.3.1 NR RSTD measurement accuracy test case for single positioning frequency layer in FR1 SA | Cell Timing Difference | 32 Tc |
| 14.3.2 NR RSTD measurement accuracy test case for dual positioning frequency layers in FR1 SA | Cell Timing Difference | 32 Tc |
| 14.3.3 NR RSTD measurement accuracy test case for single positioning frequency layer in FR2 SA | Same as 14.3.1 | Same as 14.3.1 |
| 14.3.4 NR RSTD measurement accuracy test case for dual positioning frequency layer in FR2 SA | Same as 14.3.2 | Same as 14.3.2 |
| 14.3.5 NR RSTD measurement accuracy test case for single positioning frequency layer with reduced number of samples in FR1 SA | Cell Timing Difference | 32 Tc |
| 14.3.6 NR RSTD measurement accuracy test case for single positioning frequency layer with Rx TEG in FR1 SA | Cell Timing Difference | 32 Tc |
| 14.3.7 NR RSTD measurement accuracy test case for single positioning frequency layer with reduced number of samples in FR2 SA | Cell Timing Difference | 32 Tc |
| 14.3.8 NR RSTD measurement accuracy test case for single positioning frequency layer with Rx TEG in FR2 SA | Cell Timing Difference | 32 Tc |
| 14.4.1 NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA | Response time | 300ms |
| 14.4.2 NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA | Response time | 300ms |
| 14.4.3 NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA | Response time | 300ms |
| 14.4.4 NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA | Response time | 300ms |
| 14.5.1 NR RSTD measurement accuracy test case for single positioning frequency layer in FR1 SA | Cell Timing Difference | 32 Tc |
| 14.5.2 NR RSTD measurement accuracy for single positioning frequency layer with reduced number of samples in FR1 SA | Cell Timing Difference | 32 Tc |
| 14.5.3 NR RSTD measurement accuracy test case for single positioning frequency layer in FR2 SA | Cell Timing Difference | 32 Tc |
| 14.5.4 NR RSTD measurement accuracy for single positioning frequency layer with reduced number of samples in FR2 SA | Cell Timing Difference | 32 Tc |
| 15.2.1 UE Rx-Tx time difference measurement period for single positioning frequency layer in FR1 SA | Response time | 300ms |
| 15.2.2 UE Rx-Tx time difference measurement period for dual positioning frequency layer in FR1 SA | Response time | 300ms |
| 15.2.3 UE Rx-Tx time difference measurement period for single positioning frequency layer in FR2 SA | Response time | 300ms |
| 15.2.4 UE Rx-Tx time difference measurement period for dual positioning frequency layer in FR2 SA | Response time | 300ms |
| 15.2.5 UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA | Response time | 300ms |
| 15.2.6 UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer without measurement gap in FR1 SA | Response time | 300ms |
| 15.2.7 UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer with multiple RxTx TEGs in FR1 SA | Response time | 300ms |
| 15.3.1 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA | Cell Timing Difference | 32 Tc |
| 15.3.2 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR2 SA | Cell Timing Difference | 32 Tc |
| 15.3.5 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer with reduced number of samples in FR2 SA | Cell Timing Difference | 32 Tc |
| 15.3.6 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer with RxTx TEG in FR2 SA | Cell Timing Difference | 32 Tc |
| 15.4.1 UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer in FR1 SA | Response time | 300ms |
| 15.4.2 UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA | Response time | 300ms |
| 15.4.3 UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer in FR2 SA | Response time | 300ms |
| 15.4.4 UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA | Response time | 300ms |
| 15.5.3 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR2 SA | Cell Timing Difference | 33 Tc |
| 15.5.4 UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA | Cell Timing Difference | 33 Tc |
| 16.2.1 PRS-RSRP measurement period test case for single positioning frequency layer in FR1 SA | Response time | 300ms |
| 16.2.2 PRS-RSRP measurement period test case for dual positioning frequency layer in FR1 SA | Response time | 300ms |
| 16.2.3 PRS-RSRP measurement period test case for single positioning frequency layer in FR2 SA | Response time | 300ms |
| 16.2.4 PRS-RSRP measurement period test case for dual positioning frequency layer in FR2 SA | Response time | 300ms |
| 16.2.5 PRS-RSRP measurement reporting delay test case for single positioning frequency with reduced number of samples in FR1 SA | Response time | 300ms |
| 16.2.6 PRS-RSRP measurement reporting delay test case for single positioning frequency layer without measurement gap in FR1 SA | Response time | 300ms |
| 16.3.1 PRS-RSRP measurement accuracy with PRS in FR1 | Test 1: Reported absolute PRP values Reported relative PRP values Test 2: Reported absolute PRP values Reported relative PRP values | Test 1: Reported absolute PRP values: Cell 1 ±3.5, Cell 2: ±8.5. Reported relative PRP values: Cell 1 ±3.5, Cell 2: ±9.5. Test 2: Reported absolute PRP values: Cell 1 ±3.5, Cell 2: ±6. Reported relative PRP values: Cell 1 ±3.5, Cell 2: ±6.5. |
| 16.3.2 PRS-RSRP measurement accuracy with PRS in FR2 | Test 1: Reported absolute PRP values Reported relative PRP values Test 2: Reported absolute PRP values Reported relative PRP values | Test 1: Reported absolute PRP values: ±8.5 Reported relative PRP values: ±10 Test 2: Reported absolute PRP values: ±6 Reported relative PRP values: ±7.5 |
| 16.3.3 PRS-RSRP measurement accuracy with PRS in FR1 with reduced sample number | Test 1: Reported absolute PRP values Reported relative PRP values Test 2: Reported absolute PRP values Reported relative PRP values | Test 1: Reported absolute PRP values: Cell 1 ±3.5, Cell 2: ±4.5. Reported relative PRP values: Cell 1 ±3.5, Cell 2: ±6.5. Test 2: Reported absolute PRP values: Cell 1 ±3.5, Cell 2: ±4.5. Reported relative PRP values: Cell 1 ±3.5, Cell 2: ±6.5 |
| 16.3.4 PRS-RSRP measurement accuracy with PRS in FR2 with reduced sample number | Test 1: Reported Cellllabsolute PRP values Reported relative PRP values Reported absolute PRP values Reported relative PRP values | Test 1: Reported absolute PRP values: ±8.5 Reported relative PRP values: ±10 Reported absolute PRP values: ±6 Reported relative PRP values: ±7.5 |
| 16.4.1 PRS-RSRP measurement reporting delay test case for single positioning frequency layer in FR1 SA | Response time | 300ms |
| 16.4.2 PRS-RSRP measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA | Response time | 300ms |
| 16.4.3 PRS-RSRP measurement reporting delay test case for single positioning frequency layer in FR2 SA | Response time | 300ms |
| 16.4.4 PRS-RSRP measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA | Response time | 300ms |
| 16.5.1 PRS-RSRP measurement accuracy test case for single positioning frequency layer with PRS in FR1 SA | Test 1: Cell1 reported absolute PRP values Cell 1 reported relative PRP values Cell2 reported absolute PRP values Cell 2 reported relative PRP values Test 2: Cell1 reported absolute PRP values Cell 1 reported relative PRP values Cell2 reported absolute PRP values Cell 2 reported relative PRP values | Test 1: Cell 1 reported absolute PRP values: ±3.5 Cell 1 reported relative PRP values: ±3.5 Cell 2 reported absolute PRP values: ±8.5 Cell 2 reported relative PRP values: ±9.5 Test 2: Cell 1 reported absolute PRP values: ±3.5 Cell 1 reported relative PRP values: ±3.5 Cell 2 reported absolute PRP values: ±6 Cell 2 reported relative PRP values: ±6.5 |
| 16.5.2 PRS-RSRP measurement accuracy test case for single positioning frequency layer with PRS in FR1 with reduced sample number | Same as 16.5.1 | Same as 16.5.1 |
| 16.5.3 PRS-RSRP measurement accuracy test case for single positioning frequency layer with PRS in FR2 SA | Test 1: Cell1 reported absolute PRP values Cell 1 reported relative PRP values Cell2 reported absolute PRP values Cell 2 reported relative PRP values Test 2: Cell1 reported absolute PRP values Cell 1 reported relative PRP values Cell2 reported absolute PRP values Cell 2 reported relative PRP values | Test 1: Cell 1 reported absolute PRP values: ±5 Cell 1 reported relative PRP values: ±5 Cell 2 reported absolute PRP values: ±8.5 Cell 2 reported relative PRP values: ±10 Test 2: Cell 1 reported absolute PRP values: ±5 Cell 1 reported relative PRP values: ±5 Cell 2 reported absolute PRP values: ±6 Cell 2 reported relative PRP values: ±7.5 |
| 16.5.4 PRS-RSRP measurement accuracy test case for single positioning frequency layer with PRS in FR2 with reduced sample number | Same as 16.5.3 | Same as 16.5.3 |
| 17.2.1 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer in FR1 SA | Response time | 300ms |
| 17.2.2 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA | Response time | 300ms |
| 17.2.3 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer without measurement gap in FR1 SA | Response time | 300ms |
| 17.2.4 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer in FR2 SA | Response time | 300ms |
| 17.2.5 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA | Response time | 300ms |
| 17.2.6 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer without measurement gap in FR2 SA | Response time | 300ms |
| 17.3.1 PRS-RSRPP measurement accuracy test case for single positioning frequency layer in FR1 SA | Test 1: Cell1 reported absolute PRS-RSRPP values Cell2 reported absolute PRS-RSRPP values Test 2: Cell1 reported absolute PRS-RSRPP values Cell2 reported absolute PRS-RSRPP values | Test 1: Cell 1 reported absolute PRS-RSRPP values: ±4.1 Cell 2 reported absolute PRS-RSRPP values: ±5.8 Test 2: Cell 1 reported absolute PRS-RSRPP values: ±4.1 Cell 2 reported absolute PRS-RSRPP values: ±4.9 |
| 17.3.2 PRS-RSRPP measurement accuracy test case for single positioning frequency layer with reduced number of samples in FR1 SA | Cell1 reported absolute PRS-RSRPP values Cell2 reported absolute PRS-RSRPP values | Cell 1 reported absolute PRS-RSRPP values: ±3.9 Cell 2 reported absolute PRS-RSRPP values: ±4.2 |
| 17.3.3 PRS-RSRPP measurement accuracy test case for single positioning frequency layer in FR2 SA | Test 1: Cell1 reported absolute PRS-RSRPP values Cell2 reported absolute PRS-RSRPP values Test 2: Cell1 reported absolute PRS-RSRPP values Cell2 reported absolute PRS-RSRPP values | Test 1: Cell 1 reported absolute PRS-RSRPP values: ±6 Cell 2 reported absolute PRS-RSRPP values: ±7.3 Test 2: Cell 1 reported absolute PRS-RSRPP values: ±6 Cell 2 reported absolute PRS-RSRPP values: ±6.2 |
| 17.3.4 PRS-RSRPP measurement accuracy test case for single positioning frequency layer with reduced number of samples in FR2 SA | Cell1 reported absolute PRS-RSRPP values Cell2 reported absolute PRS-RSRPP values | Cell 1 reported absolute PRS-RSRPP values: ±5.9 Cell 2 reported absolute PRS-RSRPP values: ±5.6 |
| 17.4.1 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer in FR1 SA | Response time | 300ms |
| 17.4.2 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA | Response time | 300ms |
| 17.4.3 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer in FR2 SA | Response time | 300ms |
| 17.4.4 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA | Response time | 300ms |
| 17.5.1 PRS-RSRPP measurement accuracy test case for single positioning frequency layer in FR1 SA | Same as 17.3.1 | Same as 17.3.1 |
| 17.5.2 PRS-RSRPP measurement accuracy test case for single positioning frequency layer with reduced number of samples in FR1 SA | Same as 17.3.2 | Same as 17.3.2 |
| 17.5.3 PRS-RSRPP measurement accuracy test case for single positioning frequency layer in FR2 SA | Same as 17.3.3 | Same as 17.3.3 |
| 17.5.4 PRS-RSRPP measurement accuracy test case for single positioning frequency layer with reduced number of samples in FR2 SA | Same as 17.3.4 | Same as 17.3.4 |

# < Unchanged sections omitted >

## C.4	Derivation of Test Requirements (This clause is informative)

# < Unchanged sections omitted >

Table C.4.5: Derivation of Test Requirements for NR RSTD Measurement tests

| Test | Minimum Requirement in TS 38.133 | Test Parameter Relaxation (TPR) | Test Requirement in TS 37.571-1 |
| --- | --- | --- | --- |
| 14.2.1 NR RSTD measurement period test case for single positioning frequency layer in FR1 SA | Response Time between 1 s and 210 s depending on UE capabilities | 300 ms | Time + TPR: between 1.3 s and 210.3 s depending on UE capabilities |
| 14.2.2 NR RSTD measurement period test case for dual positioning frequency layers in FR1 SA | Response Time between 2 s and 104 s depending on UE capabilities | 300 ms | Time + TPR: between 2.3 s and 104.3 s depending on UE capabilities |
| 14.2.3 NR RSTD measurement period test case for single positioning frequency layer in FR2 SA | Response Time between 6 s and 250 s depending on UE capabilities | 300 ms | Time + TPR: between 6.3 s and 250.3 s depending on UE capabilities |
| 14.2.4 NR RSTD measurement period test case for dual positioning frequency layers in FR2 SA | Response Time between 11 s and 210 s depending on UE capabilities | 300 ms | Time + TPR: between 11.3 s and 210.3 s depending on UE capabilities |
| 14.2.5 NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA | Response Time between 1 s and 28 s depending on UE capabilities | 300 ms | Time + TPR: between 1.3 s and 28.3 s depending on UE capabilities |
| 14.2.6 NR RSTD measurement reporting delay test case for single positioning frequency layer without measurement gap in FR1 SA | Response Time between 1 s and 14 s depending on UE capabilities | 300 ms | Time + TPR: between 1.3 s and 14.3 s depending on UE capabilities |
| 14.2.7 NR RSTD measurement reporting delay test case for single positioning frequency layer with Rx TEG in FR1 SA | Response Time between 1 s and 870 s depending on UE capabilities | 300 ms | Time + TPR: between 1.3 s and 870.3 s depending on UE capabilities |
| 14.2.8 NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA | Response Time between 2 s and 31 s depending on UE capabilities | 300 ms | Time + TPR: between 2.3 s and 31.3 s depending on UE capabilities |
| 14.2.9 NR RSTD measurement reporting delay test case for single positioning frequency layer without measurement gap in FR2 SA | Response Time between 5 s and 25 s depending on UE capabilities | 300 ms | Time + TPR: between 5.3 s and 25.3 s depending on UE capabilities |
| 14.2.10 NR RSTD measurement reporting delay test case for single positioning frequency layer with Rx TEG in FR2 SA | Response Time between 11 s and 500 s depending on UE capabilities | 300 ms | Time + TPR: between 11.3 s and 500.3 s depending on UE capabilities |
| 14.3.1 NR RSTD measurement accuracy test case for single positioning frequency layer in FR1 SA | Cell Timing Difference = 3µs | 32 Tc | Time + TPR: 3µs ± 32 Tc |
| 14.3.2 NR RSTD measurement accuracy test case for dual positioning frequency layers in FR1 SA | Cell Timing Difference = 3µs | 32 Tc | Time + TPR: 3µs ± 32 Tc |
| 14.3.3 NR RSTD measurement accuracy test case for single positioning frequency layer in FR2 SA | Test 1: Cell Timing Difference = 3µs± 88 Tc Test 2: Cell Timing Difference = 3µs± 50 Tc | 32 Tc | Test 1: Time + TPR: 3µs ± 120 Tc Test 2: Time + TPR: 3µs ± 82 Tc |
| 14.3.4 NR RSTD measurement accuracy test case for dual positioning frequency layer in FR2 SA | Test 1: Cell Timing Difference = 3µs± 328 Tc Test 2: Cell Timing Difference = 3µs± 278 Tc | 32 Tc | Test 1: Time + TPR: 3µs ± 360 Tc Test 2: Time + TPR: 3µs ± 310 Tc |
| 14.3.5 NR RSTD measurement accuracy test case for single positioning frequency layer with reduced number of samples in FR1 SA | Test Configuration 1 and 2 Test 1: Cell Timing Difference = 3µs±202 Tc Test 2: Cell Timing Difference = 3µs± 110 Tc Test Configuration 3 Test 1: Cell Timing Difference = 3µs±116 Tc Test 2: Cell Timing Difference = 3µs± 72 Tc | 32 Tc | Test Configuration 1 and 2 Test 1: Time + TPR: 3µs ± 234 Tc Test 2: Time + TPR: 3µs ± 142 Tc Test Configuration 3 Test 1: Time + TPR: 3µs ± 148 Tc Test 2: Time + TPR: 3µs ± 104 Tc |
| 14.3.6 NR RSTD measurement accuracy test case for single positioning frequency layer with Rx TEG in FR1 SA | Test Configuration 1 and 2 Test 1: Cell Timing Difference = 3µs±284 Tc Test 2: Cell Timing Difference = 3µs± 110 Tc Test Configuration 3 Test 1: Cell Timing Difference = 3µs±179 Tc Test 2: Cell Timing Difference = 3µs± 72 Tc | 32 Tc | Test Configuration 1 and 2 Test 1: Time + TPR: 3µs ± 316 Tc Test 2: Time + TPR: 3µs ± 142 Tc Test Configuration 3 Test 1: Time + TPR: 3µs ± 211 Tc Test 2: Time + TPR: 3µs ± 104 Tc |
| 14.3.7 NR RSTD measurement accuracy test case for single positioning frequency layer with reduced number of samples in FR2 SA | Test 1: Cell Timing Difference = 3µs± 61 Tc Test 2: Cell Timing Difference = 3µs± 50 Tc | 32 Tc | Test 1: Time + TPR: 3µs ± 93 Tc Test 2: Time + TPR: 3µs ± 82 Tc |
| 14.3.8 NR RSTD measurement accuracy test case for single positioning frequency layer with Rx TEG in FR2 SA | Test 1: Cell Timing Difference = 3µs±88 Tc Test 2: Cell Timing Difference = 3µs± 50 Tc | 32 Tc | Test 1: Time + TPR: 3µs ± 120 Tc Test 2: Time + TPR: 3µs ± 82 Tc |
| 14.4.1 NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA | Response Time between 6 s and 109 s depending on UE capabilities | 300 ms | Time + TPR: between 6.3 s and 109.3 s depending on UE capabilities |
| 14.4.2 NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA | Response Time between 2 s and 29 s depending on UE capabilities | 300 ms | Time + TPR: between 2.3 s and 29.3 s depending on UE capabilities |
| 14.4.3 NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA | Response Time between 21 s and 124 s depending on UE capabilities | 300 ms | Time + TPR: between 21.3 s and 124.3 s depending on UE capabilities |
| 14.4.4 NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA | Response Time between 6 s and 32 s depending on UE capabilities | 300 ms | Time + TPR: between 6.3 s and 32.3 s depending on UE capabilities |
| 14.5.1 NR RSTD measurement accuracy test case for single positioning frequency layer in FR1 SA | Test Configuration 1 and 2 Test 1: Cell Timing Difference = 3µs±284 Tc Test 2: Cell Timing Difference = 3µs± 110 Tc Test Configuration 3 Test 1: Cell Timing Difference = 3µs±179 Tc Test 2: Cell Timing Difference = 3µs± 72 Tc | 32 Tc | Test Configuration 1 and 2 Test 1: Time + TPR: 3µs ± 316 Tc Test 2: Time + TPR: 3µs ± 142 Tc Test Configuration 3 Test 1: Time + TPR: 3µs ± 211 Tc Test 2: Time + TPR: 3µs ± 104 Tc |
| 14.5.2 NR RSTD measurement accuracy for single positioning frequency layer with reduced number of samples in FR1 SA | Test Configuration 1 and 2 Test 1: Cell Timing Difference = 3µs±202 Tc Test 2: Cell Timing Difference = 3µs± 110 Tc Test Configuration 3 Test 1: Cell Timing Difference = 3µs±116 Tc Test 2: Cell Timing Difference = 3µs± 72 Tc | 32 Tc | Test Configuration 1 and 2 Test 1: Time + TPR: 3µs ± 234 Tc Test 2: Time + TPR: 3µs ± 142 Tc Test Configuration 3 Test 1: Time + TPR: 3µs ± 148 Tc Test 2: Time + TPR: 3µs ± 104 Tc |
| 14.5.3 NR RSTD measurement accuracy test case for single positioning frequency layer in FR2 SA | Cell Timing Difference = 3µs | 32 Tc | Time + TPR: 3µs ± 32 Tc |
| 14.5.4 NR RSTD measurement accuracy test case for single positioning frequency layer in FR2 SA | Sub-test 1: Noc: -98dBm/15kHz PRS Ês1 / Noc: -1.44dB PRS Ês2 / Noc: -3.65dB Cell Timing Difference = 3µs± 93 Tc Sub-test 2: Noc: -98dBm/15kHz PRS Ês1 / Noc: -1.44dB PRS Ês2 / Noc: -3.65dB Cell Timing Difference = 3µs± 82 Tc | -2.6dB 0.88dB 0.93dB Via mapping -2.6dB 0.88dB 0.93dB Via mapping | Noc: -100.6dBm/15kHz PRS Ês1 / Noc: -0.56dB PRS Ês2 / Noc: -2.72dB Test 1: Time + TPR: 3µs ± 125 Tc Noc: -100.6dBm/15kHz PRS Ês1 / Noc: -0.56dB PRS Ês2 / Noc: -2.72dB Test 1: Time + TPR: 3µs ± 114 Tc |

Table C.4.6: Derivation of Test Requirements for UE Rx–Tx time difference Measurement tests

| Test | Minimum Requirement in TS 38.133 | Test Parameter Relaxation (TPR) | Test Requirement in TS 37.571-1 |
| --- | --- | --- | --- |
| 15.2.1 UE Rx-Tx time difference measurement period for single positioning frequency layer in FR1 SA | Response Time between 1 s and 103 s depending on UE capabilities | 300ms | Time + TPR: between 1.3 s and 103.3 s depending on UE capabilities |
| 15.2.2 UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR1 SA | Response Time between 2.3 s and 53.3 s depending on UE capabilities | 300ms | Time + TPR: between 2.3 s and 53.3 s depending on UE capabilities |
| 15.2.3 UE Rx-Tx time difference measurement period for single positioning frequency layer in FR2 SA | Response Time between 6 s and 170 s depending on UE capabilities | 300ms | Time + TPR: between 6.3 s and 170.3 s depending on UE capabilities |
| 15.2.4 UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR2 SA | Response Time between 11 s and 84 s depending on UE capabilities | 300ms | Time + TPR: between 11.3 s and 84.3 s depending on UE capabilities |
| 15.2.5 UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA | Response Time between 1 s and 52 s depending on UE capabilities | 300ms | Time + TPR: between 1.3 s and 52.3 s depending on UE capabilities |
| 15.2.6 UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer without measurement gap in FR1 SA | Response Time between 1 s and 205 s depending on UE capabilities | 300ms | Time + TPR: between 1.3 s and 205.3 s depending on UE capabilities |
| 15.2.7 UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer with multiple RxTx TEGs in FR1 SA | Response Time between 1 s and 164 s depending on UE capabilities | 300ms | Time + TPR: between 1.3 s and 164.3 s depending on UE capabilities |
| 15.2.8 UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA | Response Time between 2 s and 21 s depending on UE capabilities | 300ms | Time + TPR: between 2.3 s and 21.3 s depending on UE capabilities |
| 15.2.9 UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer without measurement gap in FR2 SA | Response Time Sub-test 1: between 3 s and 21 s depending on UE capabilities Sub-test 2: between 6 s and 83 s depending on UE capabilities | 300ms | Sub-test 1: Time + TPR: between 3.3 s and 21.3 s depending on UE capabilities Sub-test 2: Time + TPR: between 6.3 s and 83.3 s depending on UE capabilities |
| 15.2.10 UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer with RxTx TEG in FR2 SA | Response Time between 11 s and 170 s depending on UE capabilities | 300ms | Time + TPR: between 11.3 s and 170.3 s depending on UE capabilities |
| 15.3.1 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA | Config 1 and Config 2: Sub-test 1: Cell Timing Difference = 3µs± 238 Tc Sub-test 2: Cell Timing Difference = 3µs± 86 Tc Config 3: Sub-test 1: Cell Timing Difference = 3µs± 137 Tc Sub-test 2: Cell Timing Difference = 3µs± 39 Tc | 32 Tc | Config 1 and Config 2: Sub-test 1: Cell Timing Difference = 3µs± 270 Tc Sub-test 2: Cell Timing Difference = 3µs± 118 Tc Config 3: Sub-test 1: Cell Timing Difference = 3µs± 169 Tc Sub-test 2: Cell Timing Difference = 3µs± 71 Tc |
| 15.3.2 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR2 SA | Sub-test 1: Cell Timing Difference = 3µs± 44 Tc Sub-test 2: Cell Timing Difference = 3µs± 24 Tc | 32 Tc | Sub-test 1: Cell Timing Difference = 3µs± 76 Tc Sub-test 2: Cell Timing Difference = 3µs± 56 Tc |
| 15.3.5 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer with reduced number of samples in FR2 SA | Cell Timing Difference = 3µs± 33 Tc | 32 Tc | Time + TPR: 3µs ± 65 Tc |
| 15.3.6 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer with RxTx TEG in FR2 SA | Sub-test 1: Cell Timing Difference = 3µs± 56 Tc Sub-test 2: Cell Timing Difference = 3µs± 26 Tc | 32 Tc | Sub-test 1: Cell Timing Difference = 3µs± 88 Tc Sub-test 2: Cell Timing Difference = 3µs± 58 Tc |
| 15.4.3 UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer in FR2 SA | Response Time between 21 s and 83 s depending on UE capabilities | 300 ms | Time + TPR: between 21.3 s and 83.3 s depending on UE capabilities |
| 15.4.4 UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA | Response Time between 6 s and 22 s depending on UE capabilities | 300 ms | Time + TPR: between 6.3 s and 22.3 s depending on UE capabilities |
| 15.5.3 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR2 SA | Sub-test 1: Cell Timing Difference = 3µs± 44 Tc Sub-test 2: Cell Timing Difference = 3µs± 24 Tc | 33 Tc | Sub-test 1: Cell Timing Difference = 3µs± 77 Tc Sub-test 2: Cell Timing Difference = 3µs± 57 Tc |
| 15.5.4 UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA | Cell Timing Difference = 3µs± 33 Tc | 33 Tc | Cell Timing Difference = 3µs± 66 Tc |

Table C.4.7: Derivation of Test Requirements for PRS-RSRP Measurement tests

| Test | Minimum Requirement in TS 38.133 | Test Parameter Relaxation (TPR) | Test Requirement in TS 37.571-1 |
| --- | --- | --- | --- |
| 16.2.1 PRS-RSRP measurement period test case for single positioning frequency layer in FR1 SA | Response Time between 1 s and 210 s depending on UE capabilities | 300 ms | Time + TPR: between 1.3 s and 210.3 s depending on UE capabilities |
| 16.2.2 PRS-RSRP measurement period test case for dual positioning frequency layer in FR1 SA | Response Time between 2 s and 104 s depending on UE capabilities | 300 ms | Time + TPR: between 2.3 s and 104.3 s depending on UE capabilities |
| 16.2.3 PRS-RSRP measurement period test case for single positioning frequency layer in FR2 SA | Response Time between 6 s and 170 s depending on UE capabilities | 300ms | Time + TPR: between 6.3 s and 170.3 s depending on UE capabilities |
| 16.2.4 PRS-RSRP measurement period test case for dual positioning frequency layer in FR2 SA | Response Time between 11 s and 84 s depending on UE capabilities | 300ms | Time + TPR: between 11.3 s and 84.3 s depending on UE capabilities |
| 16.2.5 PRS-RSRP measurement reporting delay test case for single positioning frequency with reduced number of samples in FR1 SA | Response Time between 1 s and 52 s depending on UE capabilities | 300ms | Time + TPR: between 1.3 s and 52.3 s depending on UE capabilities |
| 16.2.6 PRS-RSRP measurement reporting delay test case for single positioning frequency layer without measurement gap in FR1 SA | Response Time between 1 s and 7 s depending on UE capabilities | 300ms | Time + TPR: between 1.3 s and 205.3 s depending on UE capabilities |
| 16.2.7 PRS-RSRP measurement reporting delay test case for single positioning frequency with reduced number of samples in FR2 SA | Response Time between 2 s and 21 s depending on UE capabilities | 300ms | Time + TPR: between 2.3 s and 21.3 s depending on UE capabilities |
| 16.2.8 PRS-RSRP measurement reporting delay test case for single positioning frequency layer without measurement gap in FR2 SA | Response Time Sub-test 1: between 2 s and 21 s depending on UE capabilities Sub-test 2: between 6 s and 83 s depending on UE capabilities | 300ms | Sub-test 1: Time + TPR: between 2.3 s and 21.3 s depending on UE capabilities Sub-test 2: Time + TPR: between 6.3 s and 83.3 s depending on UE capabilities |
| 16.3.1 PRS-RSRP measurement accuracy with PRS in FR1 | Absolute power, log: -98 dBm/15Hz PRS Es/Noc Cell#1: -2 PRS Es/Noc Cell#2: -10 Test 1 Reported absolute PRP1 values: ±3.5 Reported relative PRP1 values: ±3.5 Reported absolute PRP2 values: ±8.5 Reported reletive PRP2 values: ±9.5 Test2 Reported absolute PRP1 values: ±3.5 Reported relative PRP1 values: ±3.5 Reported absolute PRP2 values: ±6 Reported reletive PRP2 values: ±6.5 | 0 0 0 Via mapping | Absolute power, log: -98 dBm/15Hz PRS Es/Noc Cell#1: -2 PRS Es/Noc Cell#2: -10 Test 1 Reported absolute PRP1 ：PRS_RSRP_51 to PRS_RSRP_62 Reported relative PRP1：DIFFRSRP_27 to DIFFRSRP_30 Reported absolute PRP2：PRS_RSRP_38 to PRS_RSRP_59 Reported relative PRP2：DIFFRSRP_21 to DIFFRSRP_30 Test 2 Reported absolute PRP1：PRS_RSRP_51 to PRS_RSRP_62 Reported relative PRP1：DIFFRSRP_27 to DIFFRSRP_30 Reported absolute PRP2：PRS_RSRP_41 to PRS_RSRP_56 Reported relative PRP2：DIFFRSRP_24 to DIFFRSRP_30 |
| 16.3.2 PRS-RSRP measurement accuracy with PRS in FR2 | Test 1： Reported absolute PRP1 values: ±8.5 Reported relative PRP1 values: ±10 Reported absolute PRP2 values: ±8.5 Reported reletive PRP2 values: ±10 Test 2： Reported absolute PRP1 values: ±6 Reported relative PRP1 values: ±7.5 Reported absolute PRP2 values: ±6 Reported reletive PRP2 values: ±7.5 | Via mapping | Test 1： Reported absolute PRP1 ：PRS_RSRP_54 to PRS_RSRP_85 Reported relative PRP1 ：DIFFRSRP_22 to DIFFRSRP_30 Reported absolute PRP2 ：PRS_RSRP_46 to PRS_RSRP_77 Reported relative PRP2 ：DIFFRSRP_20 to DIFFRSRP_30 Test 2： Reported absolute PRP1 ：PRS_RSRP_54 to PRS_RSRP_85 Reported relative PRP1 ：DIFFRSRP_24 to DIFFRSRP_30 Reported absolute PRP2 ：PRS_RSRP_46 to PRS_RSRP_77 Reported relative PRP2 ：DIFFRSRP_23 to DIFFRSRP_30 |
| 16.3.3 PRS-RSRP measurement accuracy with PRS in FR1 with reduced sample number | Absolute power, log: -98 dBm/15Hz PRS Es/Noc Cell#1: 2.23 PRS Es/Noc Cell#2: -1.73 Test 1 Reported absolute PRP1 values: ±3.5 Reported relative PRP1 values: ±3.5 Reported absolute PRP2 values: ±6 Reported reletive PRP2 values: ±6.5 Test2 Reported absolute PRP1 values: ±3.5 Reported relative PRP1 values: ±3.5 Reported absolute PRP2 values: ±4.5 Reported reletive PRP2 values: ±5 | 0 0.69 0.80 Via mapping | Absolute power, log: -98 dBm/15Hz PRS Es/Noc Cell#1: 2.92 PRS Es/Noc Cell#2: -0.93 Test 1 Reported absolute PRP1 ：PRS_RSRP_56 to PRS_RSRP_66 Reported relative PRP1：DIFFRSRP_27 to DIFFRSRP_30 Reported absolute PRP2：PRS_RSRP_50 to PRS_RSRP_65 Reported relative PRP2：DIFFRSRP_24 to DIFFRSRP_30 Test 2 Reported absolute PRP1：PRS_RSRP_59 to PRS_RSRP_69 Reported relative PRP1：DIFFRSRP_27 to DIFFRSRP_30 Reported absolute PRP2：PRS_RSRP_55 to PRS_RSRP_67 Reported relative PRP2：DIFFRSRP_25 to DIFFRSRP_30 |
| 16.3.4 PRS-RSRP measurement accuracy with PRS in FR2 with reduced sample number | Reported absolute PRP1 values: ±5 Reported relative PRP1 values: ±5 Reported absolute PRP2 values: ±8.5 Reported reletive PRP2 values: ±10 | Via mapping | Reported absolute PRP1 ：PRS_RSRP_66 to PRS_RSRP_76 Reported relative PRP1 ：DIFFRSRP_25 to DIFFRSRP_30 Reported absolute PRP2 ：PRS_RSRP_61 to PRS_RSRP_73 Reported relative PRP2 ：DIFFRSRP_23 to DIFFRSRP_30 |
| 16.4.1 PRS-RSRP measurement reporting delay test case for single positioning frequency layer in FR1 SA | Response Time between 6 s and 53 s depending on UE capabilities | 300ms | Time + TPR: between 6.3 s and 207.3 s depending on UE capabilities |
| 16.4.2 PRS-RSRP measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA | Response Time between 2 s and 15 s depending on UE capabilities | 300ms | Time + TPR: between 2.3 s and 53.3 s depending on UE capabilities |
| 16.4.3 PRS-RSRP measurement reporting delay test case for single positioning frequency layer in FR2 SA | Response Time between 21 s and 83 s depending on UE capabilities | 300ms | Time + TPR: between 21.3 s and 83.3 s depending on UE capabilities |
| 16.4.4 PRS-RSRP measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA | Response Time between 6 s and 22 s depending on UE capabilities | 300ms | Time + TPR: between 6.3 s and 22.3 s depending on UE capabilities |
| 16.5.1 PRS-RSRP measurement accuracy test case for single positioning frequency layer with PRS in FR1 SA | Test Configuration 1 and 2 Test 1： Reported absolute PRP1 values: ±3.5 Reported relative PRP1 values: ±3.5 Reported absolute PRP2 values: ±8.5 Reported reletive PRP2 values: ±9.5 Test 2： Reported absolute PRP1 values: ±3.5 Reported relative PRP1 values: ±3.5 Reported absolute PRP2 values: ±6 Reported reletive PRP2 values: ±6.5 Test Configuration 3 Test 1： Reported absolute PRP1 values: ±3.5 Reported relative PRP1 values: ±3.5 Reported absolute PRP2 values: ±8.5 Reported reletive PRP2 values: ±9.5 Test 2： Reported absolute PRP1 values: ±3.5 Reported relative PRP1 values: ±3.5 Reported absolute PRP2 values: ±4.5 Reported reletive PRP2 values: ±5 | Via mapping | Test Configuration 1 and 2 Test 1： Reported absolute PRP1 ：PRS_RSRP_51 to PRS_RSRP_62 Reported relative PRP1 ：DIFFRSRP_27 to DIFFRSRP_30 Reported absolute PRP2 ：PRS_RSRP_38 to PRS_RSRP_59 Reported relative PRP2 ：DIFFRSRP_21 to DIFFRSRP_30 Test 2： Reported absolute PRP1 ：PRS_RSRP_51 to PRS_RSRP_62 Reported relative PRP1 ：DIFFRSRP_27 to DIFFRSRP_30 Reported absolute PRP2 ：PRS_RSRP_41 to PRS_RSRP_56 Reported relative PRP2 ：DIFFRSRP_24 to DIFFRSRP_30 Test Configuration 3 Test 1： Reported absolute PRP1 ：PRS_RSRP_54 to PRS_RSRP_65 Reported relative PRP1 ：DIFFRSRP_27 to DIFFRSRP_30 Reported absolute PRP2 ：PRS_RSRP_41 to PRS_RSRP_62 Reported relative PRP2 ：DIFFRSRP_21 to DIFFRSRP_30 Test 2： Reported absolute PRP1 ：PRS_RSRP_54 to PRS_RSRP_65 Reported relative PRP1 ：DIFFRSRP_27 to DIFFRSRP_30 Reported absolute PRP2 ：PRS_RSRP_45 to PRS_RSRP_58 Reported relative PRP2 ：DIFFRSRP_25 to DIFFRSRP_30 |
| 16.5.2 PRS-RSRP measurement accuracy test case for single positioning frequency layer with PRS in FR1 with reduced sample number | Test Configuration 1 and 2 Test 1： Reported absolute PRP1 values: ±3.5 Reported relative PRP1 values: ±3.5 Reported absolute PRP2 values: ±8.5 Reported reletive PRP2 values: ±9.5 Test 2： Reported absolute PRP1 values: ±3.5 Reported relative PRP1 values: ±3.5 Reported absolute PRP2 values: ±6 Reported reletive PRP2 values: ±6.5 Test Configuration 3 Test 1： Reported absolute PRP1 values: ±3.5 Reported relative PRP1 values: ±3.5 Reported absolute PRP2 values: ±8.5 Reported reletive PRP2 values: ±9.5 Test 2： Reported absolute PRP1 values: ±3.5 Reported relative PRP1 values: ±3.5 Reported absolute PRP2 values: ±4.5 Reported reletive PRP2 values: ±5 | Via mapping | Test Configuration 1 and 2 Test 1： Reported absolute PRP1 ：PRS_RSRP_57 to PRS_RSRP_67 Reported relative PRP1 ：DIFFRSRP_27 to DIFFRSRP_30 Reported absolute PRP2 ：PRS_RSRP_48 to PRS_RSRP_68 Reported relative PRP2 ：DIFFRSRP_21 to DIFFRSRP_30 Test 2： Reported absolute PRP1 ：PRS_RSRP_57 to PRS_RSRP_67 Reported relative PRP1 ：DIFFRSRP_27 to DIFFRSRP_30 Reported absolute PRP2 ：PRS_RSRP_50 to PRS_RSRP_65 Reported relative PRP2 ：DIFFRSRP_24 to DIFFRSRP_30 Test Configuration 3 Test 1： Reported absolute PRP1 ：PRS_RSRP_60 to PRS_RSRP_70 Reported relative PRP1 ：DIFFRSRP_27 to DIFFRSRP_30 Reported absolute PRP2 ：PRS_RSRP_51 to PRS_RSRP_71 Reported relative PRP2 ：DIFFRSRP_21 to DIFFRSRP_30 Test 2： Reported absolute PRP1 ：PRS_RSRP_60 to PRS_RSRP_70 Reported relative PRP1 ：DIFFRSRP_27 to DIFFRSRP_30 Reported absolute PRP2 ：PRS_RSRP_55 to PRS_RSRP_67 Reported relative PRP2 ：DIFFRSRP_25 to DIFFRSRP_30 |
| 16.5.3 PRS-RSRP measurement accuracy test case for single positioning frequency layer with PRS in FR2 SA | Test 1： Reported absolute PRP1 values: ±5 Reported relative PRP1 values: ±5 Reported absolute PRP2 values: ±8.5 Reported reletive PRP2 values: ±10 Test 2： Reported absolute PRP1 values: ±5 Reported relative PRP1 values: ±5 Reported absolute PRP2 values: ±6 Reported reletive PRP2 values: ±7.5 | Via mapping | Test 1： Reported absolute PRP1 ：PRS_RSRP_59 to PRS_RSRP_94 Reported relative PRP1 ：DIFFRSRP_20 to DIFFRSRP_30 Reported absolute PRP2 ：PRS_RSRP_52 to PRS_RSRP_86 Reported relative PRP2 ：DIFFRSRP_20 to DIFFRSRP_30 Test 2： Reported absolute PRP1 ：PRS_RSRP_61 to PRS_RSRP_93 Reported relative PRP1 ：DIFFRSRP_23 to DIFFRSRP_30 Reported absolute PRP2 ：PRS_RSRP_53 to PRS_RSRP_85 Reported relative PRP2 ：DIFFRSRP_23 to DIFFRSRP_30 |
| 16.5.4 PRS-RSRP measurement accuracy test case for single positioning frequency layer with PRS in FR2 with reduced sample number | Test 1： Reported absolute PRP1 values: ±5 Reported relative PRP1 values: ±5 Reported absolute PRP2 values: ±8.5 Reported reletive PRP2 values: ±10 Test 2： Reported absolute PRP1 values: ±5 Reported relative PRP1 values: ±5 Reported absolute PRP2 values: ±6 Reported reletive PRP2 values: ±7.5 | Via mapping | Test 1： Reported absolute PRP1 ：PRS_RSRP_66 to PRS_RSRP_76 Reported relative PRP1 ：DIFFRSRP_25 to DIFFRSRP_30 Reported absolute PRP2 ：PRS_RSRP_59 to PRS_RSRP_76 Reported relative PRP2 ：DIFFRSRP_20 to DIFFRSRP_30 Test 2： Reported absolute PRP1 ：PRS_RSRP_66 to PRS_RSRP_76 Reported relative PRP1 ：DIFFRSRP_25 to DIFFRSRP_30 Reported absolute PRP2 ：PRS_RSRP_61 to PRS_RSRP_73 Reported relative PRP2 ：DIFFRSRP_23 to DIFFRSRP_30 |

<End of changes >