Title:	UE Rx-Tx time difference measurement period for single positioning frequency layer in FR1 SA

Source:	Rohde &Schwarz

# 1. Introduction

This document relates to UE Rx-Tx time difference measurement period test case for single positioning frequency layer in FR1 SA in TS 37.571-1. The test case is:

- 15.2.6, UE Rx-Tx time difference measurement period for single positioning frequency layer in FR1 SA without measurement gap in FR1 SA

This document describes the process to derive the Test Tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. Test cases in TS 38.133

The test conditions are defined in the following extract from TS 38.133 v17.16.0.

# A.6.6.14.4	UE Rx-Tx time difference measurement without gaps in FR1 SA

A.6.6.14.4.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx measurement meets the requirements specified in clause 9.9.4.6 in AWGN propagation condition in FR1 in standalone scenario. There are two sub-tests in the test, sub-test 1 is to verify the delay requirements with Nsample=1, and sub-test 2 is to verify the delay requirements with Nsample=4.

The supported test configurations in listed in Table A.6.6.14.4.1-1.

Table A.6.6.14.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). All cells are on the same RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. Cell 1 and Cell 2 mute PRS transmission during T1 and transmit PRS during T2.

The NR-Multi-RTT-ProvideAssistanceData and nr-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the multi-RTT assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the beginning of PRS processing window containing the PRS resources.

The UE is configured with PRS processing window before T2.

The UE is configured to transmit SRS during T2.

The general test parameters and cell specific test parameters are as given in Table A.6.6.14.4.1-2 and Table A.6.6.14.4.1-3 respectively.

Table A.6.6.14.4.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| RF Channel Number |  | 1, 2, 3 | 1 | For both Cell 1 and Cell 2 |
| BWchannel | MHz | 1 | 20: NRB,c = 106 |  |
|  |  | 2 | 20: NRB,c = 106 |  |
|  |  | 3 | 50: NRB,c = 133 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| PRS processing window |  | 1,2,3 | PPW.1 | As defined in A.3.33 |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | OFF |  |
| Time offset between serving and neighbour cells | s | 1 | 3 Note1 |  |
|  |  | 2 | 3 Note1 |  |
|  |  | 3 | 3 or 2.34 Note1 |  |
| Expected RSTD | s | 1,2,3 | 0 |  |
| Expected RSTD uncertainty | s | 1,2,3 | Same as time offset between serving and neighbour cells |  |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 10 |  |
| NOTE 1: The value is up to the UE capability. The possible UE capability value: (1/4 symbol, 1/2 symbol, CP length, half of slot). When the UE reported value is > 3us, the time offset between serving and neighbour cells is set to 3us; when the UE reported value is < 3us, the time offset between serving and neighbour cells is set to the UE reported value. |  |  |  |  |

Table A.6.6.14.4.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Sub-test 1 | Sub-test 2 | Sub-tets 1 | Sub-test 2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 2 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 3 | PRS.2.2 FR1 |  | PRS.2.2 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| SRS configuration |  | 1 | POS-SRS.1 |  | N/A |  |
|  |  | 2 | POS-SRS.1 |  | N/A |  |
|  |  | 3 | POS-SRS.2 |  | N/A |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | 0 | -2.41 | -6 | -12.12 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | 2.23 | -2 | -1.73 | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -95.77 | -100 | -99.73 | -108 |
|  |  | 2 | -95.77 | -100 | -99.73 | -108 |
|  |  | 3 | -92.77 | -97 | -96.73 | -105 |
| Io | dBm/19.08 MHz | 1 | -61.71 | -64.57 | -61.71 | -64.57 |
|  | dBm/19.08 MHz | 2 | -61.71 | -64.57 | -61.71 | -64.57 |
|  | dBm/47.88 MHz | 3 | -57.73 | -60.59 | -57.73 | -60.59 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

A.6.6.14.4.2	Test requirements

The UE Rx-Tx time difference measurement time fulfils the requirements specified in clause 9.9.4.6.

The UE shall perform and report the UE Rx-Tx time difference measurements for Cell 1 and Cell 2 within the specified UE Rx-Tx time difference measurement time starting from the beginning of time interval T2.

- NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the time duration above because of TTI insertion uncertainty of the measurement report in DCCH.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported UE Rx-Tx measurement for each correct event shall be within the UE Rx-Tx reporting range specified in clause 10.1.25.3.1.

# 3. Discussion

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). All cells are on the same RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. Cell 1 and Cell 2 mute PRS transmission during T1 and transmit PRS during T2.

The NR-Multi-RTT-ProvideAssistanceData and nr-Multi-RTT-RequestLocationInformation as defined in TS 37.355 shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the multi-RTT assistance data and location information request. The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources. The UE is configured to transmit SRS during T2.

The UE shall perform and report the UE Rx-Tx time difference measurements for Cell 1 and Cell 2 within the specified UE Rx-Tx time difference measurement time starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported UE Rx-Tx measurement for each correct event shall be within the UE Rx-Tx reporting range specified in clause TS 38.133 clause 10.1.25.3.1.

# 4. Calculation of Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requi`rements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which original or derived parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters are selectively copied from Table A.6.6.14.4.1-3 in TS 38.133, which are subcarrier spacing, channel bandwidth, RBs in channel bandwidth, Noc, Cell 1 PRS Es/Noc and Cell 2 PRS Es/Noc. All other parameters such as Io, PRS Es/Iot and PRS-RSRP are derived, and not independently settable by the Test Equipment.

The key parameters appear in section a) of the accompanying spreadsheet.

b) Derived parameters

A number of derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

c) Uncertainties

The SS provides AWGN and 2 intra-frequency cells to the UE under AWGN conditions in FR1 in standalone scenario. We propose to control the following parameters:

- AWGN absolute power, Noc ±1.5 dB averaged over BWConfig

- Ratio of cell 1 PRS signal / AWGN, PRS Ês1 / Noc ±0.3 dB averaged over BWConfig

- Ratio of cell 2 PRS signal / AWGN, PRS Ês2 / Noc ±0.3 dB averaged over BWConfig

- LPP Response Time, 0.3 seconds

The uncertainties appear in section c) of the accompanying spreadsheet.

The absolute level uncertainties of Noc are chosen to ensure the analysis and Test Requirements are valid for frequency bands up to 6GHz.

The LPP Response Time uncertainty has been chosen to be in-line with all other LPP test cases defined in TS 37.571-1 and there is no necessity to derive the test tolerance in the spreadsheet. LPP Response Time is as defined in TS 37.571-1 clause 4.14.3.

d) Controlled parameters critical to verdict

In many RRM test cases, and particularly intra-frequency multi-cell tests, there is not a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict.

It is therefore essential to identify those parameters determining the test verdict. In this test case there are two aspects to consider:

- The uncertainties in the stimulus set by the Test system (Downlink signal)

- The uncertainties in the UE reporting delay (UE Rx-Tx time difference measurements)

The stimulus set by the Test system should be within the constraints (side conditions) for the UE measurement. For Cell 1, they are the PRS Es/Iot range and the PRP power range for the reference cell over which the UE is expected to provide UE Rx-Tx time difference measurements. For Cell 2, they are the PRS Es/Iot range and the PRP power range for the neighbour cell over which the UE is expected to provide UE Rx-Tx time difference measurements. The Io power shall also be within the range over which the UE is expected to provide UE Rx-Tx time difference measurements. The controlled parameters listed in the accompanying spreadsheet have been derived by study of the test case and by careful reading of the relevant clauses in TS 38.133. The reason for each parameter being critical to the test verdict is given briefly in the “Comment” column of section d) in the accompanying spreadsheet. Information about the value to be achieved is given later in the “Comment” column of section g) in the spreadsheet.

Having identified the parameters critical to the test verdict which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is just the ratio (effect on a critical parameter y / a test equipment uncertainty x), and is usually in dB/dB. Often it can be derived by inspection as one or zero. For example, an error of 1dB in the absolute AWGN level Noc would cause 1dB error in the Cell 1 PRP, so the sensitivity factor is 1.000 for all tests. However the same error of 1dB in the absolute AWGN level Noc would cause no change to Cell 2 Es/Iot, because all other powers are specified relative to Noc, so the sensitivity factor is zero.

In some cases, the sensitivity factor is an intermediate value. For example, the Cell 2 Es/Noc has an effect on Cell 1 Es/Iot which depends on ratios of the powers making up the total. In such cases the sensitivity factor value between 0 and 1 result. It is important to calculate these correctly to obtain the overall uncertainty.

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, taking into account the uncertainties and sensitivity factors for each parameter that can be set by the test equipment. This process follows the superposition principle. More details and explanation can be found in section 4 of TS 38.903. Uncertainties are calculated separately for T1 and T2.

For the test system uncertainties the normal procedure of combining uncorrelated uncertainties root-sum-square is followed.

e) Determine parameters to offset

During T2, the uncertainties take Cell 1 PRS Es/Iot outside the allowed ranges.

We observe that for the test configuration that the test equipment uncertainties could take the UE outside the allowed range, so the Es/Noc of Cell 1 is therefore increased by an amount sufficient to achieve Es/Iot >=-3dB.

The RSRP and Io values for both cells remain inside the allowed range, and are checked in step g).

The UE Rx-Tx time differnece measurement reporting delay will be offset by the LPP Response Time uncertainty as specified in other LPP test cases defined in TS 37.571-1.

f) Parameters modified by Test Tolerances

Based on the decision in e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, but this time modified by the Test Tolerances.

For test 1, The PRS Es/Noc of Cell 1 and Cell 2 are increased by an amount sufficient to achieve Cell 1 Es/Iot >=-0dB and Cell 2 Es/Iot >=-6dB. The actual offsets required are 0.87dB for Cell 1 and 1.03dB.for Cell 2.

For test 2, no offset is required.

These values are found empirically by observing the minimum/maximum parameter values in step g).

g) Check controlled parameters Min/Max

For the Test System downlink signal, the nominal value of each controlled parameter is equal to the original since there has been no application of the Test Tolerances in step f).

The minimum and maximum values, due to variability from uncertainties, of controlled parameters is then calculated and compared against the requirements (PRS Es/Iot range, PRP power range, and Io power range). It is not necessary to calculate all parameters during each test, so a selection is made of those critical to the test verdict. The critical requirement for each parameter is given briefly in the “Comment” column of section g) in the accompanying spreadsheet. The cases closest to limit are identified by turquoise cells in the spreadsheet. If all the stimulus requirements are met, then the chosen stimulus offsets are acceptable.

The stimulus requirements defined in TS 38.133 clause 10.1.25 are reproduced below. The stimulus requirements defined in TS 38.133 clause 10.1.25 indicate that the UE Rx-Tx Time Difference measurement accuracy for all measured cells and the Io range.

### 10.1.25	UE Rx-Tx Time Difference Measurements

<< Some text skipped >>

The accuracy requirements in Table 10.1.25.2-1 for FR1 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

PRP|dBm according to Annex B.2.14 for a corresponding Band.

AWGN propagation condition.

Table 10.1.25.2-1: UE Rx-Tx time difference measurement accuracy in FR1 in AWGN

| Accuracy | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | Minimum PRS bandwidth | PRS SCS | PRS resource repetition (TrepPRS*LPRS/KcombPRSNote 3 | NR operating band groupsNote 2 | IoNote 4 range |  |
|  |  |  |  |  |  | Minimum IoNote 1 | Maximum Io |
| TcNote 5 | dB | RB | kHz |  |  | dBm / SCSPRS | dBm/BW |
| ± 78+ | -3 | ≥24 | 15 | ≥4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -126.5 |  |
|  |  |  |  |  | NR_TDD_FR1_C | -126 |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 |  |
|  |  |  |  |  | NR_FDD_FR1_F | -124.5 |  |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -124 |  |
|  |  |  |  |  | NR_FDD_FR1_H | -123.5 |  |
| ± 59+ |  | ≥52 |  | ≥1 | Note 6 | NOTE 6 | NOTE 6 |
| ± 30+ |  | >104 |  | ≥1 | Note 6 | NOTE 6 | NOTE 6 |
| ± 57+ |  | ≥24 | 30 | ≥4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -123.5 |  |
|  |  |  |  |  | NR_TDD_FR1_C | -123 |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 |  |
|  |  |  |  |  | NR_FDD_FR1_F | -121.5 |  |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -121 |  |
|  |  |  |  |  | NR_FDD_FR1_H | -120.5 |  |
| ± 30+ |  | ≥48 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 15+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 29+ |  | ≥24 | 60 | ≥4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -120.5 |  |
|  |  |  |  |  | NR_TDD_FR1_C | -120 |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 |  |
|  |  |  |  |  | NR_FDD_FR1_F | -118.5 |  |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -118 |  |
|  |  |  |  |  | NR_FDD_FR1_H | -117.5 |  |
| ± 15+ |  | ≥ 64 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 7+ |  | ≥ 132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 101+ | -13 | ≥24 | 15 | ≥4 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 75+ |  | ≥52 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 37+ |  | >104 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 58+ |  | ≥24 | 30 | ≥4 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 39+ |  | ≥48 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 16+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 36+ |  | ≥24 | 60 | ≥4 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 16+ |  | ≥ 64 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 8+ |  | ≥ 132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in Section 3.5. NOTE 3: TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34]. NOTE 4: The Io is defined in PRS slots. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same slot. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 7: is the margin determined from Table 10.1.25.2-5. |  |  |  |  |  |  |  |

Table 10.1.25.2-5: Margin for UE Rx-Tx time difference measurement accuracy in FR1

| Min(PRS BW, SRS BW) (RB) |  |  | Margin (Tc Note 1) |
| --- | --- | --- | --- |
| SCS = 15 kHz | SCS = 30 kHz | SCS = 60 kHz |  |
| ≥ 24 | N/A | N/A | 160 |
| ≥ 52 | ≥ 24 | N/A | 80 |
| ≥ 104 | ≥ 48 | ≥ 24 | 56 |
| N/A | ≥ 132 | ≥ 64 | 24 |
| N/A | N/A | ≥ 132 | 24 |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 2: If SRS and PRS have different SCS, the margin corresponding to the smallest RS BW in MHz applies. |  |  |  |

The side conditions are defined in TS 38.133 Annex B.2.14 and are reproduced below.

## B.2.14	Conditions for NR PRS-based measurements

This clause defines the following conditions for NR PRS-based measurements and corresponding procedures performed based on PRS: PRP and PRS Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.14-1 for FR1 NR cells.

The conditions are defined in Table B.2.14-2 for FR2 NR cells.

Table B.2.14-1: Conditions for NR PRS-based measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum PRP1,2 |  |  | PRS Ês/Iot |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | dBm / SCSPRS |  |  | dB |  |
|  |  | SCSPRS = 15 kHz | SCSPRS = 30 kHz | SCSPRS = 60 kHz |  |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -124 | -121 | -6 Note2 -13 Note3 -3 Note4 | -6 Note5 -3 Note6 0 Note7 |
|  | NR_FDD_FR1_B | -126.5 | -123.5 | -120.5 |  |  |
|  | NR_TDD_FR1_C | -126 | -123 | -120 |  |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 | -119.5 |  |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 | -119 |  |  |
|  | NR_FDD_FR1_F | -124.5 | -121.5 | -118.5 |  |  |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G | -124 | -121 | -118 |  |  |
|  | NR_FDD_FR1_H | -123.5 | -120.5 | -117.5 |  |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. NOTE 2: PRS Ês/Iot for RSTD measurement reference cell PRS resource. NOTE 3: PRS Ês/Iot for RSTD measurement neighbor cell PRS resource, PRS-RSRP measurement, PRS-RSRPP measurement and UE Rx-Tx time difference measurement. NOTE 4: PRS Ês/Iot for PRS-RSRP measurement, PRS-RSRPP measurement and UE Rx-Tx time difference measurement. NOTE 5: PRS Ês/Iot for RSTD measurement neighbor cell PRS resource, PRS-RSRP measurement, PRS-RSRPP measurement and UE Rx-Tx time difference measurement when performed with reduced number of samples. NOTE 6: PRS Ês/Iot for RSTD measurement reference cell PRS resource when performed with reduced number of samples. NOTE 7: PRS Ês/Iot for PRS-RSRP measurement, PRS-RSRPP measurement and UE Rx-Tx time difference measurement when performed with reduced number of samples. |  |  |  |  |  |  |

We note that for the 20MHz Channel BW test cases, the tightest applicable requirements are given by Band n25 in NR_FDD_FR1_H. The highest PRP side condition is -123.5dBm/15kHz and NR_TDD_FR1_G. The highest PRP side condition is -121dBm/15kHz for SCS 30Khz

Io shall be no less than -123.5 dBm/15kHz (for configuration 1 & 2), -121dBm/SCS (for configuration 3) and no larger than -50 dBm/Ch BW (according to NOTE 6 of Table 10.1.25.2-1.)

# 5. Treatment of other configurations within the same test case

TS 37.71-1 test case 15.2.6 also contains configurations with different subcarrier spacings and different channel bandwidths. As the calculations use different values, a separate spreadsheet tab is provided. The following parameters are changed:

- Subcarrier spacing, section a)

- Channel bandwidth, section a)

- RBs in Channel bandwidth, section a)

- Noc, section a)

- NR Cell 1 and NR Cell 2 PRP minimum requirement, section g) and associated Source/Reference

- Io minimum requirement, section g) and associated Source/Reference

# 6. Calculation of the response time for configuration 1

The response time is determined by the core requirements in the following extract from TS 38.133:

9.9.4.6	Measurement Period Requirements without Measurement Gaps

When physical layer receives last of NR-Multi-RTT-ProvideAssistanceData message and NR-Multi-RTT-RequestLocationInformation message from LMF via LPP [34], UE shall be able to measure multiple (up to the UE capability specified in clause 9.9.4.3) UE Rx-Tx time difference measurements as defined in TS 38.215 [4] in configured positioning frequency layers within the measurement period TUERxTx_wo_gap,Total ms.

TUERxTx_wo_gap, Total=i=1LTUERxTx_wo_gap,i+L-1*maxTeffect,i, if any of the positioning frequency layers are in Case 1, or

TUERxTx_wo_gap, Total=maxTUERxTx_wo_gap,i+Tuncertainty,i, if all the positioning frequency layers are in Case 2.

Where,

i is the index of positioning frequency layer, and

L is total number of positioning frequency layers, and

Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i, and

Tuncertainty,i is the time from the start of the first PPW occasion for positioning frequency layer i to the start of measurement period TUERxTx_wo_gap, Total.

A positioning frequency layer is in Case 1 if UE reports ppw-durationOfPRS-Processing1-r17 for the band containing the positioning frequency layer, and a positioning frequency layer is in Case 2 if UE reports ppw-durationOfPRS-Processing2-r17 for the band containing the positioning frequency layer.

TUERxTx_wo_gap,i is the measurement period for UE Rx-Tx time difference measurement in positioning frequency layer i as specified below.

TUERxTx_wo_gap,i=kmultiTEG,i*NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast,i

Where

Teffect,i is the periodicity of the UE Rx-Tx time difference measurement in positioning frequency layer i as defined further in this clause.

kmultiTEG,i is the scaling factor for measurement of same PRS resource with multiple Rx TEGs.

kmultiTEG,i=1 if UE is not supported measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 or not requested by LMF to measure a PRS resource with multiple Rx TEGs via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 or measureSameDL-PRS-ResourceWithDifferentRxTxTEGs-r17 [34] in NR-Multi-RTT-RequestLocationInformation;

otherwise,

kmultiTEG,i =NTEG,i, if UE is not capable of receiving same DL PRS resource simultaneously from multiple Rx TEGs, and

kmultiTEG,i =NTEG,ikTEG,simul,i if UE is capable of receiving the same DL PRS resource simultaneously from multiple Rx TEGs.

where

NTEG,i is the number of Rx TEGs or RxTx TEGs with which UE is requested to measure a PRS resource indicated via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 or measureSameDL-PRS-ResourceWithDifferentRxTxTEGs-r17 [34] in NR-Multi-RTT-RequestLocationInformation, and in case ‘n0’ is indicated, NTEG,i is the maximum number of Rx TEGs with which UE can support to measure the same PRS resource as reported in NR-UE-TEG-Capability, and

kTEG,simul,i is the number of Rx TEGs UE can measure simultaneously which is reported via measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul.

NRxBeam,i is the scaling factor for Rx beam sweeping, and NRxBeam,i=1 if positioning frequency layer i is in FR1.  If positioning frequency layer i is in FR2, NRxBeam,i is equal to the value reported by the UE in supportedLowerRxBeamSweepingFactor-FR2 if the UE supports the capability for the band containing positioning frequency layer i, and the LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-Multi-RTT-RequestLocationInformation. NRxBeam,i is equal to 8, otherwise.

Lavailable_PRS,i is the time duration of available PRS resources in the positioning frequency layer i, to be measured during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26].  For calculation of Lavailable_PRS,i,

only the unmuted PRS resources that meet the applicability conditions and fully or partially overlapped with PRS processing window are considered, if positioning frequency layer i is in Case 1, or

only the PRS resources unmuted and fully or partially overlapped with the first (PPWL-T2) ms of PPW are considered, if positioning frequency layer i is in Case 2, where PPWL is the PPW length and T2 corresponds to ppw-durationOfPRS-ProcessingSymbolsT2.

NPRS,islot is the maximum number of DL PRS resources of positioning frequency layer i configured in a slot,

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to ppw-durationOfPRS-ProcessingSymbolsN  in TS 37.355 [34] if positioning frequency layer i is in Case 1, or corresponding to ppw-durationOfPRS-ProcessingSymbolsN2 in TS 37.355 [34] if positioning frequency layer i is in Case 2,

-	N’ is UE capability for number of DL PRS resources that it can process in a slot corresponding to ppw-maxNumOfDL-PRS-ResProcessedPerSlot as specified in clause 6.4.3 of TS 37.355 [34],

-	Nsample is the number of UE Rx-Tx time difference measurement samples:

-	Nsample= 4 if the UE is not capable of supportedDL-PRS-ProcessingSamples-RRC-CONNECTED defined in [34].

-	Nsample= 1 if the UE is capable of supportedDL-PRS-ProcessingSamples-RRC-CONNECTED defined in [34] and LMF requests the UE to perform positioning measurements with reduced number of samples by reducedDL-PRS-ProcessingSamples [34] and the following conditions are met:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample=  2 if the UE is capable of supportedDL-PRS-ProcessingSamples-RRC-CONNECTED defined in [34] and the LMF requests the UE to perform positioning measurements with reduced number of samples by reducedDL-PRS-ProcessingSamples [34] but the following conditions are not met:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 4 otherwise.

-	Tlast,i is the measurement duration for the last UE Rx-Tx time difference measurement sample in the positioning layer i, including the sampling time and processing time.

If positioning frequency layer i is in Case 1 and all of the PRS resources to be measured are available in the same PPW occasion during Tavailable, then Tlast,i = Ti +PPWL, else

if positioning frequency layer i is in Case 2 and all of the PRS resources to be measured are available in the same PPW occasion during Tavailable, then  Tlast,i = PPWL;

otherwise, Tlast,i = Ti + Tavailable_PRS,i.

-	Teffect,i is periodicity of UE Rx-Tx time difference measurement in positioning frequency layer i:

Teffect,i= TiTavailablePRS,i*TavailablePRS,i

where

Ti	corresponds to ppw-durationOfPRS-ProcessingSymbolsT  in TS 37.355 [34] if positioning frequency layer i is in Case 1, or corresponds to the sum of ppw-durationOfPRS-ProcessingSymbolsT2 and ppw-durationOfPRS-ProcessingSymbolsN2 in TS 37.355 [34] if positioning frequency layer i is in Case 2,

Tavailable_PRS,i= LCMTPRS,i,PPWRPi, the least common multiple between TPRS,i and PPWRPi

PPWRPi is the PRS processing window repetition periodicity in positioning frequency layer i.

TPRS,i is the PRS resource periodicity in positioning frequency layer i. If the positioning frequency layer i has more than one DL PRS resource sets with different PRS periodicities with muting,  TperPRS with muting=Nmuting*TperPRS, the least common multiple of TperPRS with muting among DL PRS resource sets is used to derive TPRS,i, where

TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1

Note:	For the purpose of calculating TPRS,i, only the PRS resources that meet the applicability conditions and fully or partially covered by the PRS processing window are considered.

The time TUERxTx_wo_gap,i starts from the first PRS processing window instance aligned with DL PRS resources in the assistance data after both the NR-Multi-RTT-RequestLocationInformation message and NR-Multi-RTT-ProvideAssistanceData message from LMF via LPP [34] are delivered to the physical layer of UE.

The UE Rx-Tx time difference measurement period is restarted if HO occurs during the measurement period and after SRS reconfiguration on the target cell is complete.

The measurement requirements do not apply for a PRS resource:

-	if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i or

-	if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

If during the measurement period of one or more positioning frequency layers, the PRS processing window is reconfigured or reactivated either per UE request or not per UE request, the measurement period can be longer.

The requirements in this section apply, provided no PRS symbols are dropped during the measurement period TUERxTx_wo_gap,i within PRS processing window due to collisions with other signals; otherwise, a longer measurement period may be used.

When PRS-RSRP is configured for multi-RTT, the UE Rx-Tx time difference measurements and PRS-RSRP measurements are performed over the same measurement period.

The requirements in clause 9.9.4.6 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-Multi-RTT-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

When PSCell or SCell addition or release does not cause SRS reconfiguration during the measurement period, UE continues the UE Rx-Tx time difference measurement, and the measurement period requirements apply.

When PSCell or SCell addition or release causes SRS reconfiguration during the measurement period, UE shall restart the UE Rx-Tx time difference measurement after the SRS reconfiguration on the target cell is complete.

When SRS is reconfigured without cell change during the measurement period, UE shall restart the UE Rx-Tx time difference measurement after the SRS reconfiguration on the target cell is complete.

If UE uplink transmission timing changes due to the network-configured Timing Advance command during the UE Rx-Tx measurement period, then the UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

If UE uplink transmission timing changes due to the change in the NTA_offset defined in Table 7.1.2-2 during the UE Rx-Tx measurement period, then the UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

If UE uplink transmission timing changes due to the UE autonomous timing adjustment defined in clause 7.1.2 during the UE Rx-Tx measurement period, then:

-	UE Rx-Tx measurement period requirements in this clause shall apply for a cell, which is also the downlink reference cell (defined in section 7.1.1) for SRS transmission.

-	UE Rx-Tx measurement period requirements in this clause shall not apply for a cell, which is not the downlink reference cell (defined in section 7.1.1) for SRS transmission. The UE Rx-Tx time difference measurement period may be restarted in such case.

If any positioning frequency layer is in Case 2, the requirements in this clause apply provided that the PPWL corresponding to the positioning frequency layer is larger than (T2+X) ms.

The requirements in this clause apply provided that a single positioning frequency layer is configured for measurement in each PPW.

.

5.1 LPP response time calculation procedure for case 1

Since the test case is for single positioning frequency layer, i = 1 and L = 1, so TUERxTx_wo_gap, Total=TUERxTx,_wo_gap 1.

Analysing the equation for TUERxTx_wo_gap,i:

NRxBeam,i = 1

kmultiTEG,i=1

NPRS,islot=4

For SCS 15KHz:

LavailablePRS,i=1010*1*14*4*1*4=1.14ms

For SCS=30KHz

LavailablePRS,i=1020*1*14*4*1*4=0.571ms

Nsample= 4.

N is a duration of DL PRS symbols in ms corresponding to ppw-durationOfPRS-ProcessingSymbolsN

N’ is UE capability for number of DL PRS resources that it can process in a slot as indicated by ppw-maxNumOfDL-PRS-ResProcessedPerSlot

Therefore, the first part of the equation:

1*1*4N'LavailablePRS,iN*4-1

Based on the value of N’:

| N’ | 4N' |
| --- | --- |
| n1 | 4 |
| n2 | 2 |
| >=n4 | 1 |

Based on the value of N for SCS  15KHz, using value of LavailablePRS,i: 1.143sec

| N | 1.143N |
| --- | --- |
| nDot125 | 10 |
| nDot25 | 4 |
| nDot5 | 2 |
| >=n2 | 1 |

Based on the value of N for SCS 30KHz, using value of LavailablePRS,i: 0.571sec

| N | 0.571N |
| --- | --- |
| nDot125 | 5 |
| nDot25 | 2 |
| >=nDot1 | 1 |

Solving for Case 1:

Tlast,i = Ti +PPWL

Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i

Tavailable_PRS,i= LCMTPRS,i,PPWRPi; where Tprs = 160 ms, and the repetition periodicity PPWRPi=160ms. Therefore, Tavailable_PRS,i=160

#### A.3.31.2.1.	PRS pattern 1 in FR2: SCS=120 KHz

Table A.3.31.2.1-1: PRS.1 FR2: PRS Pattern 1 for SCS=120 KHz

| PRS Parameters | Values |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel | PRS.1.1 FR2 | PRS.1.2 FR2 | PRS.1.3 FR2 |  | PRS.1.4 FR2 |  |
| Resource index in resource set | 0 | 0 | 0 | 1 | 0 | 1 |
| PRS periodicity | 160ms |  |  |  |  |  |
| PRS Resource set slot offset Note 1 | 10 ms |  |  |  |  |  |
| PRS Resource slot offset (slot) Note 1 | 0 | 4 | 0 |  | 4 |  |
| PRS RE offset Note 1 | 0 |  | 0 | 1 | 0 | 1 |
| SCS | 120kHz |  |  |  |  |  |
| PRS comb size | 2 | 4 | 2 |  | 4 |  |
| Number of PRS symbol | 4 | 4 | 4 |  | 4 |  |
| Repetion factor | 2 | 1 | 2 |  | 1 |  |
| PRS resource time gap (slot) | 1 | 1 | 1 |  | 1 |  |
| RB numbers containing PRS within channel BW | 0-31 | 0-127 | 0-31 |  | 0-127 |  |
| PRS Start PRB | 0 |  |  |  |  |  |
| Note 1: Unless otherwise specified in the test case |  |  |  |  |  |  |

A.3.33	PRS Processing Window (PPW) configurations

Table A.3.33-1: Reference PPW configuration

| PPW Parameters | Unit | Values |
| --- | --- | --- |
| Reference PPW |  | PPW.1 |
| Periodicity | slot | Note 1 |
| Offset | slot | Note 2 |
| Length | ms | 10 |
| Type |  | Same as the value reported in prsProcessingType-r17 |
| Priority |  | st1 |
| Note 1: Same as PRS resource set periodicity as used in the test case. Note 2: Same as PRS resource with smallest offset as used in the test case. |  |  |

Ti corresponds to ppw-durationOfPRS-ProcessingSymbolsT in TS 37.355  if positioning frequency layer i is in Case 1

Therefore, Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i  and Tlast,i = Ti + PPWL take following values depending on Ti:

| ppw-durationOfPRS-ProcessingSymbolsT-r17 | Teffect | Tlast |
| --- | --- | --- |
| ms1 | 160 | 11 |
| ms2 | 160 | 12 |
| ms4 | 160 | 14 |
| ms8 | 160 | 18 |
| ms16 | 160 | 26 |
| ms20 | 160 | 30 |
| ms30 | 160 | 40 |
| ms40 | 160 | 50 |
| ms80 | 160 | 90 |
| ms160 | 160 | 170 |
| ms320 | 320 | 330 |
| ms640 | 640 | 650 |
| ms1280 | 1280 | 1290 |

For case 1: N sample -4:

Finally, it results in the following equation:

1*4N'LavailablePRS,iN*4-1*Teffect+Tlast

The remaining parameters cannot be univocally solved, since they depend on the UE capabilities.

For SCS 15Khz

Solving the equation for worst-case results in: (4*10*4-1)*1280+1290= 204810ms.

Solving the equation for best-case results in: (1*1*4-1)*160+11= 491ms.

For SCS 30Khz

Solving the equation for worst-case results in: (4*5*4-1)*1280+1290= 102410ms.

Solving the equation for best-case results in: (1*1*4-1)*160+11= 491ms.

Given the wide range of response time possibilities, it is not possible to conclude on a unique value for the response time. The SS will have to calculate the response time based on the UE capabilities.

Example of the response time calculation:

For SCS: 15KHz:

For the best-case result of 0.491s, the SS should round up to the next integer (1 second) and configure this value in the LPP Response Time IE for the RequestLocationInformation message. Then, it should add the MU value of 300ms. The reporting delay limit for the evaluation is 1.3s.

For the worst-case result of 204.08s, the SS should round up to the next integer (205 second) and configure this value in the LPP Response Time IE for the RequestLocationInformation message.

For the worst-case result of 205s, it is not possible to transmit this value in the standard LPP Response Time, since its maximum value is 128 seconds.

ResponseTime ::= SEQUENCE {

time								INTEGER (1..128),

...,

[[	responseTimeEarlyFix-r12		INTEGER (1..128)		OPTIONAL		-- Need ON

]],

[[	unit-r15				ENUMERATED { ten-seconds, ... , ten-milli-seconds-v1700 }

OPTIONAL		-- Need ON

]]

}

Therefore, we need to use the value of unit-r15 = ten-seconds, then round up the result to the next multiple of ten seconds ( 210s in this case) and divide by ten to obtain that value that should be transmitted in the time field (21).

For SCS: 30KHz:

For the best-case result of 0.491s, the SS should round up to the next integer (1 second) and configure this value in the LPP Response Time IE for the RequestLocationInformation message. Then, it should add the MU value of 300ms. The reporting delay limit for the evaluation is 1.3s.

For the worst-case result of 102.4s, the SS should round up to the next integer 103 seconds and configure this value in the LPP Response Time IE for the RequestLocationInformation message. Then, it should add the MU value of 300ms. The reporting delay limit for the evaluation is 103.3s

Solving the equation further for Nsamsple=1:

The remaining parameters cannot be univocally solved, since they depend on the UE capabilities.

For SCS 15Khz

Solving the equation for worst-case results in: (4*10*1-1)*1280+1290= 51210ms.

Solving the equation for best-case results in: (1*1*1-1)*160+11=11ms.

For SCS 30Khz

Solving the equation for worst-case results in: (4*5*1-1)*1280+1290= 25610ms.

Solving the equation for best-case results in: (1*1*1-1)*160+11=11ms.

For SCS: 15KHz:

For the best-case result of 0.011s, the SS should round up to the next integer (1 second) and configure this value in the LPP Response Time IE for the RequestLocationInformation message. Then, it should add the MU value of 300ms. The reporting delay limit for the evaluation is 1.3s.

For the worst-case result of 51.21, the SS should round up to the next integer (52 second) and configure this value in the LPP Response Time IE for the RequestLocationInformation message. Then, it should add the MU value of 300ms. The reporting delay limit for the evaluation is 52.3s.

For SCS: 30KHz:

For the best-case result of 0.011s, the SS should round up to the next integer (1 second) and configure this value in the LPP Response Time IE for the RequestLocationInformation message. Then, it should add the MU value of 300ms. The reporting delay limit for the evaluation is 1.3s.

For the worst-case result of 25.6s, the SS should round up to the next integer 26 seconds and configure this value in the LPP Response Time IE for the RequestLocationInformation message. Then, it should add the MU value of 300ms. The reporting delay limit for the evaluation is 26.3s.

5.2 LPP response time calculation procedure for case 2

Since the test case is for single positioning frequency layer,  TUERxTx_wo_gap, Total=maxTUERxTx_wo_gap,1+Tuncertainty,i,

Analysing for TUERxTxwogap,1:

NRxBeam,i = 1

kmultiTEG,i=1

NPRS,islot=4

For SCS 15KHz:

LavailablePRS,i=1010*1*14*4*1*4=1.14ms

For SCS=30KHz

LavailablePRS,i=1020*1*14*4*1*4=0.571ms

Nsample= 4.

N is ppw-durationOfPRS-ProcessingSymbolsN2 in TS 37.355 if positioning frequency layer i is in Case 2

corresponds of the sum of ppw-durationOfPRS-ProcessingSymbolsT2 and ppw-durationOfPRS-ProcessingSymbolsN2 in TS 37.355 [34] if positioning frequency layer i is in Case 2. So the smallest Ti = 4.125ms, the biggest Ti = 20ms.

Therefore, Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i  and Tlast,i = PPWL take following values depending on Ti:

| Ti | Teffect | Tlast |
| --- | --- | --- |
| 4.125ms | 160 | 10 |
| 20ms | 160 | 10 |

TPRS-RSRP,i=NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast

Solving the equation further for Nsamsple=1:

The remaining parameters cannot be univocally solved, since they depend on the UE capabilities.

For SCS 15Khz

Solving the equation for worst-case results in: (4*10*1-1)*1280+1290= 51210ms.

Solving the equation for best-case results in: (1*1*1-1)*160+11=11ms.

For SCS 30Khz

Solving the equation for worst-case results in: (4*5*1-1)*1280+1290= 25610ms.

Solving the equation for best-case results in: (1*1*1-1)*160+11=11ms.

For SCS: 15KHz:

For the best-case result of 0.011s, the SS should round up to the next integer (1 second) and configure this value in the LPP Response Time IE for the RequestLocationInformation message. Then, it should add the MU value of 300ms. The reporting delay limit for the evaluation is 1.3s.

For the worst-case result of 51.21, the SS should round up to the next integer (52 second) and configure this value in the LPP Response Time IE for the RequestLocationInformation message. Then, it should add the MU value of 300ms. The reporting delay limit for the evaluation is 52.3s.

For SCS: 30KHz:

For the best-case result of 0.011s, the SS should round up to the next integer (1 second) and configure this value in the LPP Response Time IE for the RequestLocationInformation message. Then, it should add the MU value of 300ms. The reporting delay limit for the evaluation is 1.3s.

For the worst-case result of 25.6s, the SS should round up to the next integer 26 seconds and configure this value in the LPP Response Time IE for the RequestLocationInformation message. Then, it should add the MU value of 300ms. The reporting delay limit for the evaluation is 26.3s.