# Title:	Test Tolerances for PRS-RSRP measurement accuracy test in FR1

# Source:	Rohde & Schwarz

# 1. Introduction

The purpose of this document is to present the methodology and principles used for the development of Test Tolerances for PRS-RSRP measurement accuracy tests for FR1.

The test case in question, from TS 37.571-1, is:

16.5.2 The purpose of this test is to verify that the PRS-RSRP measurement accuracy in RRC_INACTIVE is within the specified limits with reduced number of samples

This document describes the process to derive the Test Tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. Prototype test case in Annex A of TS 38.133

The Test Requirements are defined in the following extracts from TS 38.133 v17.16.0 Annex A.

<< Extract from TS 38.133 >>

#### A.6.9.2.2	SA: measurement accuracy with PRS in FR1 with reduced number of samples in RRC_INACTIVE state

##### A.6.9.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that the PRS-RSRP measurement accuracy is within the specified limits provided that PRS is transmitted within the initial BWP of the UE. UE can support supportedDL-PRS-ProcessingSamples-RRC-Inactive, and the LMF indicates the UE to perform positioning measurements with reduced number of samples NsampleNsample via reducedDL-PRS-ProcessingSamples. This test will verify the requirements in clauses 10.1.24.2.1 and 10.1.24.2.2.

##### A.6.9.2.2.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Supported test configurations are shown in table A.6.9.2.2.2-1. Both absolute and relative accuracy of PRS-RSRP measurements are tested by using the parameters in A.6.9.2.2.2-2. In all test cases, Cell 1 is the PCell.

Table A.6.9.2.2.2-1: PRS-RSRP supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table A.6.9.2.2.2-2: PRS-RSRP test parameters

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Cell ID |  |  | 489 | 0 | 489 | 0 |
| SSB ARFCN |  |  | freq1 |  | freq1 |  |
| Duplex mode | Config 1 |  | FDD |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |
| TDD configuration | Config 1 |  | Not Applicable |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |
| BWchannel | Config 1 | MHz | 20: NRB,c = 106 |  |  |  |
|  | Config 2 |  | 20: NRB,c = 106 |  |  |  |
|  | Config 3 |  | 50: NRB,c = 133 |  |  |  |
| BWP BW | Config 1 |  | 20: NRB,c = 106 |  |  |  |
|  | Config 2 |  | 20: NRB,c = 106 |  |  |  |
|  | Config 3 |  | 50: NRB,c = 133 |  |  |  |
| Downlink initial BWP configuration |  |  | DLBWP.0.1 |  |  |  |
| Uplink initial BWP configuration |  |  | ULBWP.0.1 |  |  |  |
| DRX Cycle |  | s | 1.28 |  |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR2.1 TDD |  | SR2.1 TDD |  |
| RMSI CORESET Reference Channel | Config 1 |  | CR.1.1 FDD | - | CR.1.1 FDD | - |
|  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR2.1 TDD |  | CR2.1 TDD |  |
| Control channel RMC | Config 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  | Config 3 |  | CCR2.1 TDD |  | CCR2.1 TDD |  |
| PRS configuration | Config 1 |  | PRS.1.3 FR1 | PRS.1.3 FR1 | PRS.1.4 FR1 | PRS.1.4 FR1 |
|  | Config 2 |  | PRS.1.3 FR1 | PRS.1.3 FR1 | PRS.1.4 FR1 | PRS.1.4 FR1 |
|  | Config 3 |  | PRS.2.3 FR1 | PRS.2.3 FR1 | PRS.2.4 FR1 | PRS.2.4 FR1 |
| PRS BW | Config 1 |  | 52 PRBs | 52 PRBs |  |  |
|  | Config 2 |  | 52 PRBs | 52 PRBs |  |  |
|  | Config 3 |  | 48 PRBs | 48 PRBs |  |  |
| PRS Resource slot offset (slot) | Config 1,2,3 | slot | 0 | 4 | 0 | 4 |
| SSB configuration | Config 1 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |
|  | Config 2 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |
|  | Config 3 |  | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 |
| Time offset with Cell 1 | Config 1 | ms | - | 3 | - | 3 |
|  | Config 2,3 | s | - | 3 | - | 3 |
| SMTC configuration | Config 1 |  | SMTC.2 |  |  |  |
|  | Config 2,3 |  | SMTC.1 |  |  |  |
| OCNG Patterns |  |  | OCNG pattern 1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2 | kHz | 15 kHz |  |  |  |
|  | Config 3 |  | 30 kHz |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |  |
| Note2 | Config 1,2 | dBm/15Khz | -98 |  | -98 |  |
|  | Config 3 |  |  |  |  |  |
| Note2 | Config 1,2 | dBm/SCS | -98 |  | -98 |  |
|  | Config 3 |  | -95 |  | -95 |  |
|  |  | dB | 0 | -6 | 0 | -6 |
|  |  | dB | 2.23 | -1.73 | 2.23 | -1.73 |
| PRS-RSRP Note3 | Config 1, 2 | dBm/SCS | -95.77 | -99.73 | -95.77 | -99.73 |
|  | Config 3 |  | -92.77 | -96.73 | -92.77 | -96.73 |
| IoNote3 | Config 1,2 | dBm/19.08MHz | -61.71 |  | -61.71 |  |
|  | Config 3 | dBm/47.88MHz | -57.73 |  | -57.73 |  |
| Propagation condition |  |  | AWGN |  |  |  |
| Antenna configuration |  |  | 1x2 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: PRS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |

##### A.6.9.2.2.3	Test Requirements

In each test, the absolute PRS-RSRP measurement for each cell shall fulfil the absolute accuracy requirement in clause 10.1.24.2.1. The relative PRS-RSRP measurement between the two PRS resources within the same cell shall fulfil the relative accuracy requirement in clause 10.1.24.2.2.

# 3. Discussion

The test has 2 cells and 2 subtests. The difference between Test 1 and Test 2 is only in the PRS configuration. The power levels remain constant for both subtests. However, the PRS configuration has an impact on the reporting accuracy. Thus, a separate calculation of the reporting limits is provided for each test.

The PRS Ês/Iot value for all cells is set near the bottom of the range of PRS Ês/Iot given in the core requirement TS 38.133.

There are three test configurations according to Table A.6.9.2.2.2-1. Test Configurations 1 and 2 have identical power settings, so they will be analysed together. Test Configuration 3 has different power settings and it will be analysed in a separate tab in the accompanying excel sheet.

# 4. Calculation of Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters are copied from Table A.6.9.2.2.2-2 in TS 38.133. The key parameters are selected as the minimum set to define the cell power levels, and which are subject to a test system uncertainty which may affect the verdict of the test. Signalled parameters including the threshold are also copied. For this test, the key parameters are:

- Noc

- Cell 1 PRS Es / Noc

- Cell 2 PRS Es / Noc

All other parameters such as PRS Es/Iot, etc. are derived and not independently settable by the test system.

The key parameters appear in section a) of the accompanying spreadsheet.

b) Derived parameters

A number of derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

c) Uncertainties

The Test system provides 2 cells on 1 frequency, with AWGN. We propose to control the following parameters:

- AWGN absolute power on carrier frequency, Noc uncertainty: ±1.5 dB

- Ratio of cell 1 signal / AWGN, PRS Ês1 / Noc ±0.3 dB averaged over PRS BWConfig

- Ratio of cell 2 signal / AWGN, PRS Ês2 / Noc ±0.3 dB averaged over PRS BWConfig

The uncertainties appear in section c) of the accompanying spreadsheet.

We note also that the outcome of this test is a range of allowed RSTD values reported by the UE. The UE reporting accuracy is therefore taken into account when determining the test limits, although it is not an uncertainty of the test system itself.

This choice forms a minimum set, so the superposition principle can be applied if necessary.

The absolute level of Noc is specified for frequency bands up to 6GHz.

The PRS-RSRP accuracy requirements are defined in the following extract from TS 38.133:

##### 10.1.24.2.1Absolute PRS RSRP accuracy

The absolute accuracy requirements for PRS-RSRP measurement for FR1 defined in Table 10.1.24.2.1-1 are valid under the following conditions:

The absolute accuracy requirements for PRS-RSRP measurement for FR1 defined in Table 10.1.24.2.1-3 are valid under the following conditions:

-	Conditions defined in 38.101-1 Clause 7.3 for reference sensitivity are fulfilled.

-	PRP 1,2|dBm according to Annex B.2.14 for a corresponding Band

-	UE supports positioning measurements with reduced number of samples, and LMF indicates UE to perform positioning measurements with reduced number of samples

-	AWGN channel

Table 10.1.24.2.1-3: PRS-RSRP absolute accuracy for FR1 with reduced sample number

| Accuracy |  | Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | PRS Ês/Iot | PRS BW | Repetition factor (TrepPRS*LPRS/KcombPRS) | Io Note 6 range |  |  |  |  |  |
|  |  |  |  |  | NR operating band groups Note 7 | Minimum Io Note 1 dBm / SCSPRS |  |  |  | Maximum Io |
| dB | dB | dB | PRB | - |  | dBm / SCSPRS |  |  |  | dBm/BWChannel |
|  |  |  |  |  |  | dBm/15kHz Note 5 |  | dBm/30kHz Note 5 | dBm/60kHz Note 5 |  |
| ±3.5 | ±8 | ≥0 | ≥48 | All | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -124 |  | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -126.5 | -123.5 |  | -120.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -126 | -123 |  | -120 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 |  | -119.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 |  | -119 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -124.5 | -121.5 |  | -118.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -124 | -121 |  | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -123.5 | -120.5 |  | -117.5 | -50 |
| ±8.5 | ±13 | ≥-6 | 48 ≤ BW ≤ 52 | All | Note 3 |  |  |  |  |  |
| ±6 | ±10.5 |  | 52< BW≤ 104 | All | Note 3 |  |  |  |  |  |
| ±4.5 | ±9 |  | BW >104 | All | Note 3 |  |  |  |  |  |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA or DL-AoD assistance data defined in [34]. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth ≥ 48 RB. NOTE 4: The serving cell, the reference cell, and the measured neighbour cell i are on the same carrier frequency. NOTE 5: The condition level is increased by ∆>0, when applicable, as described in Sections B.3.2 and B.3.3. NOTE 6: The Io is defined in PRS positioning subframes. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same subframe. NOTE 7: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |  |  |

We observe that the UE shall report with ±3.5 dB accuracy for Cell 1 for both tests, while for Cell 2, the accuracy shall be ±8.5 for Test 1 (PRS BW = 52 RB) and ±6 for config 1/2 Test 2 (PRS BW = 104RB) and config 3 cell 2 test 2 ±4.5 (PRS BW= 132 RB)

##### 10.1.24.2.2	Relative PRS RSRP accuracy

The relative accuracy of PRS-RSRP is defined as accuracy of the difference between two PRS-RSRP measurements.

The relative PRS-RSRP accuracy requirements apply for the cases when PRS-RSRP is measured from PRS resources in the same PRS resource set in FR1 or FR2, and measured with same Rx beam in case of FR2.

The absolute accuracy requirements for PRS-RSRP measurement for FR1 defined in Table 10.1.24.2.2-3 are valid under the following conditions:

-	Conditions defined in 38.101-1 Clause 7.3 for reference sensitivity are fulfilled.

-	PRP 1,2|dBm according to Annex B.2.14 for a corresponding Band

-	UE supports positioning measurements with reduced number of samples, and LMF indicates UE to perform positioning measurements with reduced number of samples

-	AWGN channel

Table 10.1.24.2.2-3: PRS-RSRP relative accuracy for FR1 with reduced sample number

| Accuracy |  | Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | PRS Ês/Iot | PRS BW | Repetition factor (TrepPRS*LPRS/KcombPRS) | Io Note 6 range |  |  |  |  |
|  |  |  |  |  | NR operating band groups Note 7 | Minimum Io Note 1 dBm / SCSPRS |  |  | Maximum Io |
| dB | dB | dB | PRB | - |  | dBm / SCSPRS |  |  | dBm/BWChannel |
|  |  |  |  |  |  | dBm/15kHz Note 5 | dBm/30kHz Note 5 | dBm/60kHz Note 5 |  |
| [±3.5] | ±5.0 | ≥0 | ≥48 | All | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -124 | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -126.5 | -123.5 | -120.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -126 | -123 | -120 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 | -119.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 | -119 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -124.5 | -121.5 | -118.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -124 | -121 | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -123.5 | -120.5 | -117.5 | -50 |
| ±9.5 | ±11.0 | ≥-6 | 48 ≤ BW ≤ 52 | All | Note 3 |  |  |  |  |
| ±6.5 | ±8.0 |  | 52< BW≤ 104 | All | Note 3 |  |  |  |  |
| ±5.0 | ±6.5 |  | BW >104 | All | Note 3 |  |  |  |  |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA or DL-AoD assistance data defined in [34]. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth ≥ 48 RB. NOTE 4: The serving cell, the reference cell, and the measured neighbour cell i are on the same carrier frequency. NOTE 5: The condition level is increased by ∆>0, when applicable, as described in Sections B.3.2 and B.3.3. NOTE 6: The Io is defined in PRS positioning subframes. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same subframe. NOTE 7: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |  |

For relative PRS accuracy: We observe that the UE shall report with ±3.5 dB accuracy for Cell 1 for both tests, while for Cell 2, the accuracy shall be ±9.5 for Test 1 (PRS BW = 52 RB) and ±6.5 for config 1&2 Test 2 (PRS BW = 104RB) and config 3 cell 2 test 2 ±5 (PRS BW= 132 RB)

The UE measures the PRS signal as reference for the RSTD measurement. The minimum conditions to measure the PRS signal are given in TS 38.133:

## B.2.14	Conditions for NR PRS-based measurements

This clause defines the following conditions for NR PRS-based measurements and corresponding procedures performed based on PRS: PRP and PRS Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.14-1 for FR1 NR cells.

The conditions are defined in Table B.2.14-2 for FR2 NR cells.

Table B.2.14-1: Conditions for NR PRS-based measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum PRP1,2 |  |  | PRS Ês/Iot |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | dBm / SCSPRS |  |  | dB |  |
|  |  | SCSPRS = 15 kHz | SCSPRS = 30 kHz | SCSPRS = 60 kHz |  |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -124 | -121 | -6 Note2 -13 Note3 -3 Note4 | -6 Note5 -3 Note6 0 Note7 |
|  | NR_FDD_FR1_B | -126.5 | -123.5 | -120.5 |  |  |
|  | NR_TDD_FR1_C | -126 | -123 | -120 |  |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 | -119.5 |  |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 | -119 |  |  |
|  | NR_FDD_FR1_F | -124.5 | -121.5 | -118.5 |  |  |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G | -124 | -121 | -118 |  |  |
|  | NR_FDD_FR1_H | -123.5 | -120.5 | -117.5 |  |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. NOTE 2: PRS Ês/Iot for RSTD measurement reference cell PRS resource. NOTE 3: PRS Ês/Iot for RSTD measurement neighbor cell PRS resource, PRS-RSRP measurement, PRS-RSRPP measurement and UE Rx-Tx time difference measurement. NOTE 4: PRS Ês/Iot for PRS-RSRP measurement, PRS-RSRPP measurement and UE Rx-Tx time difference measurement. NOTE 5: PRS Ês/Iot for RSTD measurement neighbor cell PRS resource, PRS-RSRP measurement, PRS-RSRPP measurement and UE Rx-Tx time difference measurement when performed with reduced number of samples. NOTE 6: PRS Ês/Iot for RSTD measurement reference cell PRS resource when performed with reduced number of samples. NOTE 7: PRS Ês/Iot for PRS-RSRP measurement, PRS-RSRPP measurement and UE Rx-Tx time difference measurement when performed with reduced number of samples. |  |  |  |  |  |  |

The PRS Ês/Iot side condition is ≥ 0dB for the reference cell (Cell 1) and >-6 dB for the neighbour cells (Cell 2). Band group NR_FDD_FR1_H gives the highest PRP side condition of -123.5dBm/15kHz. And Band group NR_TDD_FR1_G gives the highest PRP side condition of -121dBm/30kHz

d) Controlled parameters critical to verdict

In many RRM test cases, and particularly intra-frequency multi-cell tests, there is not a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict.

It is therefore essential to identify those parameters determining the test verdict. The 8 controlled parameters listed in the accompanying spreadsheet have been derived by study of the test case and by careful reading of the relevant clauses in TS 38.133. The reason for each parameter being critical to the test verdict is given briefly in the “Comment” column of section d) in the accompanying spreadsheet. Information about the value to be achieved is given later in the “Source and Reference” column of section g) in the spreadsheet.

Having identified the parameters critical to the test verdict which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is just the ratio (effect on a critical parameter y / a test equipment uncertainty x), and is usually in dB/dB. Often it can be derived by inspection as one or zero. For example, an error of 1dB in the absolute level Noc would cause 1dB error in the Io on RF Channel 2, so the sensitivity factor is 1.000. However the same error of 1dB in the absolute level Noc would cause no change to NR Cell 2 Es/Iot, because all other powers are specified relative to Noc, so the sensitivity factor is zero.

In some cases, the sensitivity factor is an intermediate value. For example, the NR Cell 1 Es/Noc has an effect on NR Cell 1 Es/Iot which depends on ratios of the powers making up the total. In such cases a sensitivity factor value between 0 and 1 results. It is important to calculate these correctly to obtain the overall uncertainty.

For example,

- The effect of NR Cell 2 Es/Noc uncertainty on NR Cell 1 Es/Iot is x 0.027

Although in this case the sensitivity factor value of 0.027 is close to 0, it would be wrong to approximate because any change in geometry factors could make the approximation invalid.

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, taking into account the uncertainties and sensitivity factors for each parameter that can be set by the test equipment. This process follows the superposition principle. More details and explanation can be found in section 4 of TS 38.903.

For the test system uncertainties the normal procedure of combining uncorrelated uncertainties root-sum-square is followed.

e) Determine parameters to offset

We observe that all values are within limits.

f) Parameters modified by Test Tolerances

Based on the decision in e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, modified by the Test Tolerances (applied offsets).

The PRS Es/Noc of Cell 1 and Cell 2 are increased by an amount sufficient to achieve Cell 1 Es/Iot >=-0dB and Cell 2 Es/Iot >=-6dB. The actual offsets required are 0.95dB for Cell 1 and 0.97dB.for Cell 2.

Re-derived parameters are calculated using the same methods as were used in step b).

g) Check controlled parameters Min/Max

Using a format similar to that in step d), the nominal value of each controlled parameter is recalculated, as some will have changed from the original due to the application of the Test Tolerances in step f).

The minimum and maximum values, due to variability from uncertainties, of controlled parameters is then calculated and compared against the requirements (PRS Es/Iot range, PRP power range, and the Io power range). It is not necessary to calculate all parameters during each test, so a selection is made of those critical to the test verdict. The critical requirement for each parameter is given briefly in the “Comment” column of section g) in the accompanying spreadsheet. The cases closest to limit are identified by turquoise cells in the spreadsheet. If all the stimulus requirements are met, then the chosen stimulus offsets are acceptable.

It can be seen that with the uncertainty values and Test Tolerances proposed, the requirements are met.

# 5. Calculation of the PRS-RSRP reporting range for 16.5.4

The reporting limits are based on the PRS-RSRP. The applied value of the PRP for Cell 1 is -92.04dBm/SCS and for Cell 2 is -95.82dBm/SCS.

Table 10.1.24.3.1-1: Measurement report mapping for PRS-RSRP

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| PRS_RSRP_0 | PRS-RSRP<-156 | dBm |
| PRS_RSRP_1 | -156≤PRS-RSRP<-155 | dBm |
| PRS_RSRP_2 | -155≤PRS-RSRP<-154 | dBm |
| PRS_RSRP_3 | -154≤PRS-RSRP<-153 | dBm |
| PRS_RSRP_4 | -153≤PRS-RSRP<-152 | dBm |
| PRS_RSRP_5 | -152≤PRS-RSRP<-151 | dBm |
| PRS_RSRP_6 | -151≤PRS-RSRP<-150 | dBm |
| PRS_RSRP_7 | -150≤PRS-RSRP<-149 | dBm |
| PRS_RSRP_8 | -149≤PRS-RSRP<-148 | dBm |
| PRS_RSRP_9 | -148≤PRS-RSRP<-147 | dBm |
| PRS_RSRP_10 | -147≤PRS-RSRP<-146 | dBm |
| PRS_RSRP_11 | -146≤PRS-RSRP<-145 | dBm |
| PRS_RSRP_12 | -145≤PRS-RSRP<-144 | dBm |
| PRS_RSRP_13 | -144≤PRS-RSRP<-143 | dBm |
| PRS_RSRP_14 | -143≤PRS-RSRP<-142 | dBm |
| PRS_RSRP_15 | -142≤PRS-RSRP<-141 | dBm |
| PRS_RSRP_16 | -141≤PRS-RSRP<-140 | dBm |
| PRS_RSRP_17 | -140≤PRS-RSRP<-139 | dBm |
| PRS_RSRP_18 | -139≤PRS-RSRP<-138 | dBm |
| … | … | … |
| PRS_RSRP_111 | -46≤PRS-RSRP<-45 | dBm |
| PRS_RSRP_112 | -45≤PRS-RSRP<-44 | dBm |
| PRS_RSRP_113 | -44≤PRS-RSRP<-43 | dBm |
| PRS_RSRP_114 | -43≤PRS-RSRP<-42 | dBm |
| PRS_RSRP_115 | -42≤PRS-RSRP<-41 | dBm |
| PRS_RSRP_116 | -41≤PRS-RSRP<-40 | dBm |
| PRS_RSRP_117 | -40≤PRS-RSRP<-39 | dBm |
| PRS_RSRP_118 | -39≤PRS-RSRP<-38 | dBm |
| PRS_RSRP_119 | -38≤PRS-RSRP<-37 | dBm |
| PRS_RSRP_120 | -37≤PRS-RSRP<-36 | dBm |
| PRS_RSRP_121 | -36≤PRS-RSRP<-35 | dBm |
| PRS_RSRP_122 | -35≤PRS-RSRP<-34 | dBm |
| PRS_RSRP_123 | -34≤PRS-RSRP<-33 | dBm |
| PRS_RSRP_124 | -33≤PRS-RSRP<-32 | dBm |
| PRS_RSRP_125 | -32≤PRS-RSRP<-31 | dBm |
| PRS_RSRP_126 | -31≤PRS-RSRP | dBm |

Therefore, the reported value for cell 1 is PRS_RSRP_65 and for cell 2 is PRS_RSRP_61.

The reporting limits, based on given uncertainties and the UE reporting limits are summarized in the following table:

PRS RSRP accuracy requirements for the reported values

| Test Configuration | Configuration | Report Mapping | Lowest reported value | Highest reported value |
| --- | --- | --- | --- | --- |
| Sub-test 1 | Config 1& 2 | Absolute PRS RSRP accuracy for Cell 1 | 57 | 67 |
|  |  | Relative PRS RSRP accuracy for Cell 1 | 27 | 30 |
|  |  | Absolute PRS RSRP accuracy for Cell 2 | 48 | 68 |
|  |  | Relative PRS RSRP accuracy for Cell 2 | 21 | 30 |
|  | Config 3 | Absolute PRS RSRP accuracy for Cell 1 | 60 | 70 |
|  |  | Relative PRS RSRP accuracy for Cell 1 | 27 | 30 |
|  |  | Absolute PRS RSRP accuracy for Cell 2 | 51 | 71 |
|  |  | Relative PRS RSRP accuracy for Cell 2 | 21 | 30 |
| Sub-test 2 | Config 1& 2 | Absolute PRS RSRP accuracy for Cell 1 | 57 | 67 |
|  |  | Relative PRS RSRP accuracy for Cell 1 | 27 | 30 |
|  |  | Absolute PRS RSRP accuracy for Cell 2 | 50 | 65 |
|  |  | Relative PRS RSRP accuracy for Cell 2 | 24 | 30 |
|  | Config 3 | Absolute PRS RSRP accuracy for Cell 1 | 60 | 70 |
|  |  | Relative PRS RSRP accuracy for Cell 1 | 27 | 30 |
|  |  | Absolute PRS RSRP accuracy for Cell 2 | 55 | 67 |
|  |  | Relative PRS RSRP accuracy for Cell 2 | 25 | 30 |