3GPP TSG-RAN5 Meeting #106	R5-250930r1

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5435 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IoT NTN enhancement test case 22.3.1.15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | UEs may not instruct lower layers to report the remaining GNSS validity duration since the position remains the same during the test case execution. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The conformance requirements and test procedure sequence were extended to clarify UE behavior. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.1.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R1: Specified the new location of the UE. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

22.3.1.15	NB-IoT / NTN / GNSS validity duration reporting

22.3.1.15.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state and GNSS is valid }

ensure that {

when { UE receives a GNSS Measurement Command MAC CE }

then { UE transmits a GNSS Validity Duration Report MAC CE upon a new GNSS position becomes valid }

}

22.3.1.15.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.331 clause 5.5.9, TS 36.321, clauses 5.4.10, 6.1.3.22 and 6.1.3.23. Unless otherwise stated these are Rel-18 requirements.

[TS 36.331, clause 5.5.9]

For BL UEs or UEs in CE or NB-IoT UEs that are connected to NTN, GNSS measurement can be triggered aperiodically by the GNSS Measurement Command MAC CE (see TS 36.321 [6]), or triggered by the UE autonomously if enabled by the network, or triggered by the UE using available idle periods.

The UE shall:

1>	if an indication to perform GNSS measurement is received from lower layers:

2>	perform GNSS measurement using the measurement gap with a gap length indicated by lower layers, as specified in TS 36.213 [23];

2>	stop timer T390, if running;

…

1>	upon starting GNSS measurement:

2>	stop timer T318, if running;

1>	upon indication that a new GNSS position becomes valid:

2>	instruct lower layers to report the remaining GNSS validity duration (see TS 36.321 [6]);

…

[TS 36.321, clause 5.4.10]

For an NB-IoT UE, a BL UE or a UE in enhanced coverage in a non-terrestrial network, an indication may be sent by upper layers to report the remaining GNSS validity duration.

If the MAC entity receives an indication from upper layers to report the remaining GNSS validity duration:

-	stop the timeAlignmentTimer associated with the pTAG, if running.

-	initiate a Random Access procedure (see clause 5.1).

NOTE:	When in RRC_CONNECTED, if the UE autonomously starts and completes GNSS acquisition using available idle periods, the UE reports the remaining GNSS validity duration but it is up to UE implementation whether to stop timeAlignmentTimer and initiate a Random Access procedure.

If the GNSS validity duration reporting procedure has been triggered and not cancelled:

-	if the MAC entity has UL resources allocated for new transmission for this TTI, and;

-	if the allocated UL resources can accommodate the GNSS Validity Duration Report MAC control element plus its subheader, as a result of logical channel prioritization:

-	instruct the Multiplexing and Assembly procedure to generate the GNSS Validity Duration Report MAC control element as defined in clause 6.1.3.23.

[TS 36.321, clause 6.1.3.22]

The GNSS Measurement Command MAC Control Element is identified by a MAC PDU subheader with LCID as specified in table 6.2.1-1.

It has a fixed size and consists of a single octet defined as follow (Figure 6.1.3.22-1):

-	T: The Type field is a flag indicating UE is triggered by network to perform GNSS measurement or just a GNSS measurement gap length is provided for UE-autonomous measurement. The T field set to "0" indicates UE needs to perform GNSS measurement with the GNSS measurement gap length configured in this MAC CE. The T field set to "1" indicates the GNSS measurement gap length configured in this MAC CE needs to be stored and used for the subsequent UE-autonomous GNSS measurement;

-	R: Reserved bit, set to 0;

-	GNSS Measurement Gap Length: the field corresponds to the configuration of GNSS measurement gap length as shown in Table 6.1.3.22-1. The length of the field is 4 bits.

Figure 6.1.3.22-1: GNSS Measurement Command MAC control element

Table 6.1.3.22-1: Values of GNSS Measurement Gap Length field

| Codepoint/Index | Value [seconds] |
| --- | --- |
| 0000 | 1 |
| 0001 | 2 |
| 0010 | 3 |
| 0011 | 4 |
| 0100 | 5 |
| 0101 | 6 |
| 0110 | 7 |
| 0111 | 13 |
| 1000 | 19 |
| 1001 | 25 |
| 1010 | 31 |
| 1011-1111 | Reserved |

[TS 36.321, clause 6.1.3.23]

The GNSS Validity Duration Report MAC Control Element is identified by a MAC PDU subheader with LCID as specified in table 6.2.1-2.

It has a fixed size and consists of a single octet defined as follows (Figure 6.1.3.23-1):

-	R: Reserved bit, set to 0;

-	GNSS Validity Duration: the field corresponds to the remaining GNSS validity duration defined in the TS 36.331 [8].

Figure 6.1.3.23-1: GNSS Validity Duration Report MAC control element

22.3.1.15.3	Test description

22.3.1.15.3.1	Pre-test conditions

System Simulator:

-	Ncell 1 is configured according to Table 8.1.4.2-1 and Table 8.1.4.2-2 in TS 36.508 [18].

-	System information combination 8 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used.

UE:

-	The preconfigured UE location is defined in TS 36.508 [18] Clause 4.13.

Preamble

-	The UE is in state 2-NB according to TS 36.508 [18] clause 8.1.5.1.

22.3.1.15.3.2	Test procedure sequence

Table 22.3.1.15.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits MAC PDU containing a GNSS Measurement Command MAC CE with T field set to "0" and GNSS Measurement Gap Length set to "0100". | <-- | MAC PDU (GNSS Measurement Command MAC CE) | - | - |
| 1A | The SS transmits Update UE location information message. | <-- | UPDATE UE LOCATION INFORMATION | - | - |
| 1B | The UE transmits a preamble on NPRACH. | --> | NPRACH Preamble | - | - |
| 1C | The SS transmits Random Access Response including UL grant suitable for transmitting GNSS Validity Duration Report MAC CE. | <-- | Random Access Response | - | - |
| 2 | Check: Does UE send a MAC PDU to report the remaining GNSS measurement validity duration? | --> | MAC PDU (GNSS Validity Duration Report MAC CE) | 1 | P |

22.3.1.15.3.3	Specific message contents

Table 22.3.1.15.3.3-1: UPDATE UE LOCATION INFORMATION (Steps 1A, Table 22.3.1.15.3.2-1)

| Derivation Path: TS 36.509 [25] Table 6.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ellipsoidPointWithAltitude | '23AC F356 7176 0000'H | UE Location: - Latitude: 25.08439433 - Longitude: 121.56076999 - Altitude: 0 |  |
| horizontalVelocity | '00 0000'H | Speed: 0 km/s |  |
| gnss-TOD-msec | '00 0000'H |  |  |

[End of change]