3GPP TSG-RAN5 Meeting #106	R5-250940r1

# Athens, Greece, 17th Feb. – 21st Feb. 2025

Title:							Analysis on common UL configuration for FR1 intra-band contiguous 2UL CA with non-contiguous RB allocation

Source:			Anritsu, Keysight

Agenda Item:			5.4.20

Document for:			Endorsement

# 1.	Introduction

In the previous meeting, errors of the current common UL configuration for FR1 intra-band contiguous 2UL CA with non-contiguous RB allocation in Table 6.1A-1b [1] was discovered by [2], and the following RF Action Point was created. This paper provides our analysis on the correct common UL configuration.

| Action ID | sWG | Action | Responsible | Relevant Tdoc | Deadline | Status |
| --- | --- | --- | --- | --- | --- | --- |
| AP#105.22 | RF | Evaluate and apply any corrections needed to the common uplink configuration for FR1 intra-band contiguous CA (non-contiguous RB) in TS 38.521-1 Clause 6.1. | Apple, Anritsu, Keysight | R5-247730 | RAN5#107 | Open |

# 2.	Discussion

In the minimum conformance requirements for MPR in 6.2A.2.0.4 [1], Inner, Outer1, and Outer2 RB allocations are defined as below.

<Inner RB allocation>

In contiguous CA, a non-contiguous RB allocation is a non-contiguous Inner RB allocation if the following conditions are met:

RBStart,Low ≤ RBStart_CA ≤ RBStart,High and NRB_alloc ≤ ceil((BWChannel_CA / 3 – BWgap ) / 0.18MHz),

where

NRB_alloc = (NRB1 - RBStart1)∙ 2^µ1 + (RBStart2 + LCRB2 ) ∙ 2^µ2, RBStart_CA = RBStart1∙2^1

RBStart,Low = max(1, floor(NRB_alloc + (BWgap – BWGB,low)/0.18MHz))

RBStart,High = floor((BWChannel_CA – 2 ∙ BWgap – BWGB,low)/0.18MHz – 2 ∙ NRB_alloc)

BWGB,low =Foffset,low – (NRB1∙12+1)∙SCS1/2

BWgap is the bandwidth of the gap between NRB1 and NRB2 possible allocations of CC1 and CC2 respectively.

<Outer1 RB allocation>

In contiguous CA, a non-contiguous RB allocation is a non-contiguous outer 1 RB allocation if the following conditions are met:

RBStart,Low ≤ RBStart_CA ≤ RBStart,High and NRB_alloc ≤ ceil((3 BWChannel_CA / 5 – BWgap) / 0.18MHz)

where

RBStart,Low = max(1, 2 ∙ NRB_alloc – floor( (BWChannel_CA – 2 ∙ BWgap + BWGB,low)/0.18MHz)),

RBStart,High = floor((2 ∙ BWChannel_CA – 3 ∙ BWgap – BWGB,low) / 0.18MHz – 3 ∙ NRB_alloc)

NRB_alloc, RBStart_CA, BWgap and BWGB,low are as defined for the Inner region.

<Outer2 RB allocation>

In contiguous CA, a non-contiguous allocation is an Outer 2 allocation if it is neither a non-contiguous Inner allocation nor an Outer 1 allocation.

RB allocation is judged as Inner, Outer1, or Outer2 from the relationships between RBStart,Low, RBStart_CA, RBStart,High, and the relationships between NRB_alloc, BWChannel_CA, BWgap. Due to the characteristics of these parameters, there are two complicated points for the definition of RB allocation.

First, the conditions for Inner, Outer1, and Outer2 depend on the actual operating band since the derivation of parameters other than RBStart_CA and NRB_alloc involves μ and nominal channel spacing. As shown below, μ = 0, 1, or 2 corresponds to the largest SCS supported in the operating band rather than SCS1 and SCS2, in order to avoid an issue that BWChannel_CA calculated from SCS1 and SCS2 exceeds CBW1 + CBW2 [3]. And for nominal channel spacing, the following two types of formulas are defined for channel raster = 15 kHz or 100 kHz to optimize performance in each operating band [4]. As a result, there are 6 different patterns of the conditions.

### 5.3A.3	Minimum guard band and transmission bandwidth configuration for CA

The aggregated channel bandwidth, BWChannel_CA, is defined as

BWChannel_CA = Fedge,high - Fedge,low (MHz).

The lower bandwidth edge Fedge, low and the upper bandwidth edge Fedge,high of the aggregated channel bandwidth are used as frequency reference points for transmitter and receiver requirements and are defined by

Fedge,low = FC,low - Foffset,low

Fedge,high = FC,high + Foffset,high

The lower and upper frequency offsets depend on the transmission bandwidth configurations of the lowest and highest assigned edge component carrier and are defined as

Foffset,low = (NRB,low*12 + 1)*SCSlow/2 + BWGB (MHz)

Foffset,high = (NRB,high*12 - 1)*SCShigh/2 + BWGB (MHz)

BWGB = max(BWGB,Channel(k))

NRB,low and NRB,high are the transmission bandwidth configurations according to Table 5.3.2-1 for the lowest and highest assigned component carrier, SCSlow and SCShigh are the sub-carrier spacing for the lowest and highest assigned component carrier respectively. SCSlow, SCShigh, NRB,low, NRB,high, and BWGB,Channel(k) use the largest μ value among the subcarrier spacing configurations supported in the operating band for both of the channel bandwidths according to Table 5.3.5-1 and BWGB,Channel(k) is the minimum guard band for carrier k according to Table 5.3.3-1 for the said μ value. In case there is no common μ value for both of the channel bandwidths, μ=1 is used for SCSlow, SCShigh, NRB,low, NRB,high, and BWGB,Channel(k).

### 5.4A.1	Channel spacing for CA

For intra-band contiguous carrier aggregation with two or more component carriers, the nominal channel spacing between two adjacent NR component carriers is defined as the following unless stated otherwise:

For NR operating bands with a 100 kHz channel raster:

while for NR operating bands with 15 kHz channel raster:

Observation 1: The conditions for Inner, Outer1, and Outer2 have 6 different patterns depending on μ and channel raster of the operating band.

However, the 6 different patterns will not apply to all the CABW combinations given common μ to use will depend on the BW of each component carrier. For example, looking into Table 5.3.5-1 [1], it can be observed that:

- μ = 0 will only apply if testing 5 MHz + 5 MHz, a CABW combination not even in current Table 6.1A-1b [1].

- μ = 1 will only apply in case of:

  - CABW combinations implying CCx = 5 MHz and CCy > 5 MHz for any band.

  - 10 MHz < CABW < 25 MHz in case of band n5 and n71.

  - 25 MHz < CABW < 35 MHz in case of band n71. (Band n71 not yet for IBC CA in RAN5).

- μ = 2 otherwise.

Hence, there are two options to determine the common UL configuration for FR1 intra-band contiguous 2UL CA with non-contiguous RB allocation.

Option 1: For each CABW combination, determine the common UL configuration optimized for each of the patterns of conditions that can apply to the given CABW.

Option 2: For each CABW combination, determine the common UL configuration that can be used across all operating bands. Define one pattern of RB allocation in Table 6.1A-1b [1] which satisfies the patterns of conditions that can apply to the given CABW.

Considering the following advantages, we prefer the Option 2.

- Selection of UL configuration during tests is simple.

- There is no additional work in the future even if a new operating band is added.

- It can avoid the risk that the appropriate pattern of μ and channel raster could be different among readers.

Proposal 1: Select Option 2 to determine the common UL configuration for FR1 intra-band contiguous 2UL CA with non-contiguous RB allocation.

The second complicated point is that the threshold of the condition depends on the actual RB allocation, so there may be multiple RB allocations that are just before the threshold. For the common UL configuration, we propose to use the outermost RB allocation as far away from the center of the band as possible that is a valid allocation for all the patterns of conditions that can apply to the given CABW as per Option 2 proposed above.

Proposal 2: For each CABW combination, adopt the first RB allocation which is a valid allocation for all the patterns of conditions that can apply to the given CABW by moving RB inwards from the outermost possible RB for the common UL configuration for FR1 intra-band contiguous 2UL CA with non-contiguous RB allocation.

To calculate the common UL configuration, the attached excel file was provided as an example of algorithm which select the common UL configuration from all of 6 patterns of conditions. It should be updated to select the common UL configuration from the patterns of conditions that can apply to the given CABW. Note that most parameters were calculated according to the formulas in 5.3A.3, 5.4A.1, and 6.2A.2.0.4 [1], but BWChannel_CA and BWgap were calculated by the following formulas.

- BWChannel_CA = Nominal channel spacing + Foffset,high + Foffset,low

In 5.3A.3 [1], it is defined as BWChannel_CA = Fedge,high – Fedge,low = (FC,high + Foffset,high) – (FC,low – Foffset,low). However, FC,high and FC,low are defined for each CA band combination uniquely. For general calculation, the above formula “BWChannel_CA = Nominal channel spacing + Foffset,high + Foffset,low” was used instead of the formula in 5.3A.3 [1] based on an assumption of “Nominal channel spacing = FC,high – FC,low“.

- BWgap = Nominal channel spacing – (Max NRB1 * 12 * SCS1 + Max NRB2 * 12 * SCS2)/2 (MHz), where Max N[RB1,RB2] is defined in Table 5.3.2-1 [1].

There is no concrete formula in [1]. The above formula is based on an assumption that the difference between the center of NRB1 and the center of NRB2 equals Nominal channel spacing.

# 3.	Conclusion

RAN5 is asked to endorse the following proposals.

Proposal 1: Select Option 2 to determine the common UL configuration for FR1 intra-band contiguous 2UL CA with non-contiguous RB allocation.

Proposal 2: For each CABW combination, adopt the first RB allocation which is a valid allocation for all the patterns of conditions that can apply to the given CABW by moving RB inwards from the outermost possible RB for the common UL configuration for FR1 intra-band contiguous 2UL CA with non-contiguous RB allocation.

# 4.	References

[1] TS 38.521-1, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone”, V18.5.0 (2024-12)

[2] R5-247730, “On Common UL config for FR1 intra-band non-contiguous CA scenarios”, Apple Benelux B.V., RAN5#105

[3] R4-2005657, “WF on intra-band UL contiguous CA emission requirement”, Huawei, HiSilicon, Qualcomm, Skyworks, Nokia, RAN4#94bis-e

[4] R4-1800309, “Intra-band contiguous CA channel spacing”, ZTE Corporation, RAN4 AH-1801