3GPP TSG-RAN5 Meeting #106	R5-250967r1

# Athens, Greece, 17th Feb 2025 – 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0955 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct test case order in Annex F |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2025-02-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test case order defined in section 6 is not followed in Table F.1.1.3-2 and Table F.1.3.3-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct test case order in Table F.1.1.3-2 and Table F.1.3.3-1 to follow test case order definition in section 6. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case order in Table F.1.1.3-2 and Table F.1.3.3-1 will not be aligned with test case definition in section 6. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1.1.3, F.1.3.3, F.1.1.2, F.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Changes impacting following WI codes: TEI16_Test , NR_L1enh_URLLC-UEConTest, NR_eMIMO-UEConTest, NR_perf_enh-UEConTest, TEI17_Test , NR_demod_enh2-UEConTest, NR_DL1024QAM_FR1-UEConTest |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1 correcting TC ID for 9.4B.1.1A SDR for 1024QAM |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

### F.1.1.2	Measurement of Demod Performance requirements

This clause defines the maximum test system uncertainty for Demod Performance requirements. The maximum test system uncertainty allowed for the measurement uncertainty contributors are defined in Table F.1.1.2-1.

Table F.1.1.2-1: Maximum measurement uncertainty values for the test system for FR1 (up to 6 GHz) and Channel BW ≤ 40 MHz

| MU contributor | Unit | Value | Comment |
| --- | --- | --- | --- |
| AWGN flatness and signal flatness, max deviation for any Resource Block, relative to average over BWconfig | dB | ±2.0 | Same as in LTE |
| Signal to noise ratio uncertainty | dB | ±0.3 | Same as in LTE |
| Signal to noise ratio variation | dB | ±0.5 | Same as in LTE |
| Fading profile power uncertainty for 1Tx | dB | ±0.5 | Same as in LTE |
| Fading profile power uncertainty for 2Tx | dB | ±0.7 | Same as in LTE |
| Fading profile power uncertainty for 4Tx | dB | ±0.7 | Same as in LTE |

The maximum test system uncertainty for test cases defined in section 5 is defined in Table F.1.1.2-2.

Table F.1.1.2-2: Maximum test system uncertainty for FR1 demodulation performance test cases

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 5.2.1.1.1 1 Rx FDD FR1 PDSCH performance for RedCap | ± 0.9 dB for > 10Hz doppler ± 1 dB for 10Hz doppler | Same as 5.2.2.1.1_1 |
| 5.2.1.1.2 1 Rx FDD FR1 PDSCH performance for eRedCap | ± 0.9 dB for > 10Hz doppler ± 1 dB for 10Hz doppler | Same as 5.2.2.1.1_1 |
| 5.2.1.2.1 1Rx TDD FR1 PDSCH performance for RedCap | ± 0.9 dB for > 10Hz doppler ± 1 dB for 10Hz doppler | Same as 5.2.2.1.1_1 |
| 5.2.1.2.2 1 Rx FDD FR1 PDSCH performance for eRedCap | ± 0.9 dB for > 10Hz doppler ± 1 dB for 10Hz doppler | Same as 5.2.2.1.1_1 |

<<< Skip unchanged rows >>

| 5.3.3.1.2 4Rx FDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA | Same as 5.3.2.1.2 | Same as 5.3.2.1.2 |
| --- | --- | --- |
| 5.3.3.1.3 4Rx FDD FR1 PDCCH 1 Tx antenna performance for power saving | Same as 5.3.2.1.1 | Same as 5.3.2.1.1 |
| 5.3.3.1.4 4RX FDD Minimum requirements for PDCCH with intra-slot repetition | Same as 5.3.2.1.2 | Same as 5.3.2.1.2 |
| 5.3.3.2.1 4Rx TDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA | Same as 5.3.2.1.1 | Same as 5.3.2.1.1 |
| 5.3.3.2.2 4Rx TDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA | Same as 5.3.2.1.2 | Same as 5.3.2.1.2 |
| 5.3.3.2.3 4Rx TDD FR1 PDCCH 1 Tx antenna performance for power saving | Same as 5.3.2.1.1 | Same as 5.3.2.1.1 |
| 5.3.3.2.4 4RX TDD Minimum requirements for PDCCH with intra-slot repetition | Same as 5.3.2.1.2 | Same as 5.3.2.1.2 |
| 5.5A.1.1 FR1 SDR performance for CA (2DL CA) | Same as 5.5.1 | Same as 5.5.1 |
| 5.5A.1.2 FR1 SDR performance for CA (3DL CA) | Same as 5.5.1 | Same as 5.5.1 |
| 5.5A.1.3 FR1 SDR performance for CA (4DL CA) | Same as 5.5.1 | Same as 5.5.1 |
| 5.5A.1.4 FR1 SDR performance for CA (5DL CA) | Same as 5.5.1 | Same as 5.5.1 |
| 5.5.1 FR1 Sustained downlink data rate performance for single carrier | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.5 dB, 4.2GHz < f ≤ 6GHz Downlink EVM ≤ 3% | 3% EVM is equivalent to a Test system downlink SNR of 30.5dB. The noise from the Test system is then sufficiently below that required for the UE to demodulate the signal with the required % success rate. Under these conditions the UE throughput is limited by the Reference measurement channel and the UE capability, and not by the Test system EVM. |
| 5.5.2 FR1 Sustained downlink data rate performance for single carrier with DL1024QAM | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.5 dB, 4.2GHz < f ≤ 6GHz Downlink EVM ≤ 2.5% | 2.5% EVM is equivalent to a Test system downlink SNR of 32.04dB. The noise from the Test system is then sufficiently below that required for the UE to demodulate the signal with the required % success rate. Under these conditions the UE throughput is limited by the Reference measurement channel and the UE capability, and not by the Test system EVM. |
| 5.5.3 FR1 Sustained downlink data rate performance for RedCap | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.5 dB, 4.2GHz < f ≤ 6GHz Downlink EVM ≤ 3% | 3% EVM is equivalent to a Test system downlink SNR of 30.5dB. The noise from the Test system is then sufficiently below that required for the UE to demodulate the signal with the required % success rate. Under these conditions the UE throughput is limited by the Reference measurement channel and the UE capability, and not by the Test system EVM. |
| 5.6.2.1.1_1 2Rx FDD FR1 PDSCH performance with link adaptation – 2x2 MIMO for SA | FFS | FFS |
| 5.6.2.2.1_1 2Rx TDD FR1 PDSCH performance with link adaptation – 2x2 MIMO for SA | FFS | FFS |
| 5.6.3.1.1_1 4Rx FDD FR1 PDSCH performance with link adaptation – 2x4 MIMO for SA | FFS | FFS |
| 5.6.3.2.1_1 4Rx TDD FR1 PDSCH performance with link adaptation – 2x4 MIMO for SA | FFS | FFS |
| 9.4B.1.1 Sustained downlink data rate performance for EN-DC within FR1 | E-UTRA CC: ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz NR CC: Same as 5.5.1 | Same as 5.5.1 |
| 9.4B.1.1A Sustained downlink data rate performance for EN-DC within FR1 with Dl 1024QAM | E-UTRA CC: ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz NR CC: Same as 9.4B.1.1 | 2.5% EVM is equivalent to a Test system downlink SNR of 32.04dB. The noise from the Test system is then sufficiently below that required for the UE to demodulate the signal with the required % success rate. Under these conditions the UE throughput is limited by the Reference measurement channel and the UE capability, and not by the Test system EVM. |
| 9.4B.1.1_1 Sustained downlink data rate performance for EN-DC within FR1 (2 NR CCs) | Same as 9.4B.1.1 | Same as 9.4B.1.1 |
| 9.4B.1.1_2 Sustained downlink data rate performance for EN-DC within FR1 (3 NR CCs) | Same as 9.4B.1.1 | Same as 9.4B.1.1 |
| 9.4B.1.1_3 Sustained downlink data rate performance for EN-DC within FR1 (3 NR CCs) | Same as 9.4B.1.1 | Same as 9.4B.1.1 |
| 11.1.2.1.1_1 2Rx FR1 PSSCH performance - single active PSSCH link | ± 0.8 dB | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB |
| 11.1.3.1.1_1 2Rx FR1 PSCCH performance - single active PSSCH link | Same as 11.1.2.1.1_1 | Same as 11.1.2.1.1_1 |
| 11.1.5.1.1_1 2Rx FR1 PSCCH performance - single active PSSCH link | Same as 11.1.2.1.1_1 | Same as 11.1.2.1.1_1 |
| 11.1.6.1.1_1 2Rx FR1 Power imbalance performance - two active PSSCH link | ± 0.6 dB | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1, and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 11.1.7.1.1_1 2Rx FR1 HARQ buffer soft combining performance - maximum number of HARQ processes | Same as 11.1.2.1.1_1 | Same as 11.1.2.1.1_1 |
| 11.1.8.1.1_1 2Rx FR1 PSCCH decoding capability - maximum number of received PSCCHs | Downlink absolute power uncertainty, averaged over BWConfig ±1.0 dB Downlink EVM ≤ 3% | 3% EVM is equivalent to a Test system downlink SNR of 30.5dB. The noise from the Test system is then sufficiently below that required for the UE to demodulate the signal with the required % success rate. Under these conditions the UE throughput is limited by the Reference measurement channel and the UE capability, and not by the Test system EVM. |
| 11.1.9.1.1_1 2Rx FR1 PSFCH decoding capability - maximum number of received PSFCHs | Same as 11.1.8.1.1_1 | Same as 11.1.8.1.1_1 |

### F.1.1.3	Measurement of Channel State Information reporting

This clause defines the maximum test system uncertainty for channel state information reporting requirements. The maximum test system uncertainty allowed for the measurement uncertainty contributors are defined in Table F.1.1.3-1.

Table F.1.1.3-1: Maximum measurement uncertainty values for the test system for FR1 (up to 6 GHz) and Channel BW ≤ 40 MHz

| MU contributor | Unit | Value | Comment |
| --- | --- | --- | --- |
| AWGN flatness and signal flatness, max deviation for any Resource Block, relative to average over BWconfig | dB | Same as in table F.1.1.2-1 |  |
| Signal to noise ratio uncertainty | dB | Same as in table F.1.1.2-1 |  |
| Signal to noise ratio variation | dB | Same as in table F.1.1.2-1 |  |
| Fading profile power uncertainty for 1Tx | dB | Same as in table F.1.1.2-1 |  |
| Fading profile power uncertainty for 2Tx | dB | Same as in table F.1.1.2-1 |  |
| Fading profile power uncertainty for 4Tx | dB | Same as in table F.1.1.2-1 |  |

The maximum test system uncertainty for test cases defined in section 6 is defined in Table F.1.1.3-2.

Table F.1.1.3-2: Maximum test system uncertainty for FR1 channel state information reporting test cases

| Subclause | Maximum Test System Uncertainty |  | Derivation of Test System Uncertainty |  |  |
| --- | --- | --- | --- | --- | --- |
| 6.2.1.1.1.1 1Rx FDD FR1 periodic CQI reporting under AWGN conditions for RedCap | Same as 6.2.2.1.1.1 |  | Same as 6.2.2.1.1.1 |  |  |
| 6.2.1.1.1.2 1Rx FDD FR1 periodic CQI reporting under AWGN conditions for eRedCap | Same as 6.2.2.1.1.1 |  | Same as 6.2.2.1.1.1 |  |  |
| 6.2.1.1.2.1 1Rx FDD FR1 periodic wideband CQI reporting under fading conditions for RedCap | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |  |  |
| 6.2.1.1.2.2 1Rx FDD FR1 periodic wideband CQI reporting under fading conditions for eRedCap | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |  |  |
| 6.2.1.2.1.1 1Rx TDD FR1 periodic CQI reporting under AWGN conditions for RedCap | Same as 6.2.2.2.1.1 |  | Same as 6.2.2.2.1.1 |  |  |
| 6.2.1.2.1.2 1Rx TDD FR1 periodic CQI reporting under AWGN conditions for eRedCap | Same as 6.2.2.2.1.1 |  | Same as 6.2.2.2.1.1 |  |  |
| 6.2.1.2.2.1 1Rx TDD FR1 periodic wideband CQI reporting under fading conditions for RedCap | Same as 6.2.1.1.2.1 |  | Same as 6.2.1.1.2.1 |  |  |
| 6.2.2.1.1.1 2Rx FDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA | +/- 0.3 dB |  | Overall system uncertainty for AWGN conditions comprises: Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |  |  |
| 6.2.2.1.1.2 2Rx FDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA | Same as 6.2.2.1.1.1 |  | Same as 6.2.2.1.1.1 |  |  |
| 6.2.2.1.1.3 2Rx FDD FR1 periodic CQI reporting with Table 4 under AWGN conditions for both SA and NSA | Same as 6.2.2.1.1.1 |  | Same as 6.2.2.1.1.1 |  |  |
| 6.2.2.1.1.4 2Rx FDD FR1 periodic CQI reporting under AWGN conditions for RedCap | Same as 6.2.2.1.1.1 |  | Same as 6.2.2.1.1.1 |  |  |
| 6.2.2.1.2.1 2Rx FDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA | +/- 0.8 dB |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty ±0.3 dB 2. Fading profile power uncertainty for 2Tx ±0.7 dB Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |  |  |
| 6.2.2.1.2.2 2Rx FDD FR1 aperiodic subband CQI reporting under fading conditions for both SA and NSA | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |  |  |
| 6.2.2.1.2.3 2Rx FDD FR1 Wideband CQI reporting with inter-cell interference for both SA and NSA | Ês1/Noc ± 0.8 dB Ês2/Noc ± 0.3 dB |  | Overall system uncertainty for fading conditions on Cell 1 comprises two quantities: 1. Signal-to-noise ratio uncertainty ±0.3 dB 2. Fading profile power uncertainty ±0.7 dB for 2Tx Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |  |  |
| 6.2.2.1.2.4 2Rx FDD FR1 periodic wideband CQI reporting under fading conditions for RedCap | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |  |  |
| 6.2.2.2.1.1 2Rx TDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA | Same as 6.2.2.1.1.1 |  | Same as 6.2.2.1.1.1 |  |  |
| 6.2.2.2.1.2 2Rx TDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA | Same as 6.2.2.1.1.1 |  | Same as 6.2.2.1.1.1 |  |  |
| 6.2.2.2.1.3 2Rx TDD FR1 CQI reporting for PCell on band with shared spectrum access under AWGN conditions for both SA and NSA | Same as 6.2.2.1.1.1 |  | Same as 6.2.2.1.1.1 |  |  |
| 6.2.2.2.1.4 2Rx TDD FR1 periodic CQI reporting with Table 4 under AWGN conditions for both SA and NSA | Same as 6.2.2.1.1.1 |  | Same as 6.2.2.1.1.1 |  |  |
| 6.2.2.2.1.5 2Rx TDD FR1 periodic CQI reporting under AWGN conditions for RedCap | Same as 6.2.2.1.1.1 |  | Same as 6.2.2.1.1.1 |  |  |
| 6.2.2.2.2.1 2Rx TDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |  |  |
| 6.2.2.2.2.2 2Rx TDD FR1 aperiodic subband CQI reporting under fading conditions for both SA and NSA | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |  |  |
| 6.2.2.2.2.3 2Rx TDD FR1 Wideband CQI reporting with inter-cell interference for both SA and NSA | Same as 6.2.2.1.2.3 |  | Same as 6.2.2.1.2.3 |  |  |
| 6.2.2.2.2.4 2Rx TDD FR1 periodic wideband CQI reporting under fading conditions for RedCap | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |  |  |
| 6.2.3.1.1.1 4Rx FDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA | Same as 6.2.2.1.1.1 |  | Same as 6.2.2.1.1.1 |  |  |
| 6.2.3.1.1.2 4Rx FDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA | Same as 6.2.2.1.1.1 |  | Same as 6.2.2.1.1.1 |  |  |
| 6.2.3.1.1.3 4Rx FDD FR1 periodic CQI reporting with Table 4 under AWGN conditions for both SA and NSA | Same as 6.2.2.1.1.1 |  | Same as 6.2.2.1.1.1 |  |  |
| 6.2.3.1.2.1 4Rx FDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |  |  |
| 6.2.3.1.2.2 4Rx FDD FR1 aperiodic subband CQI reporting under fading conditions for both SA and NSA | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |  |  |
| 6.2.3.1.2.3 4Rx FDD FR1 Wideband CQI reporting with inter-cell interference for both SA and NSA | Same as 6.2.2.1.2.3 |  | Same as 6.2.2.1.2.3 |  |  |
| 6.2.3.2.1.1 4Rx TDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA |  | Same as 6.2.2.1.1.1 |  | Same as 6.2.2.1.1.1 |
| 6.2.3.2.1.2 4Rx TDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA |  | Same as 6.2.2.1.1.1 |  | Same as 6.2.2.1.1.1 |
| 6.2.3.2.1.4 4Rx TDD FR1 periodic CQI reporting with Table 4 under AWGN conditions for both SA and NSA |  | Same as 6.2.2.1.1.1 |  | Same as 6.2.2.1.1.1 |
| 6.2.3.2.1.3 4Rx TDD FR1 CQI reporting for PCell on band with shared spectrum access under AWGN conditions for both SA and NSA |  | Same as 6.2.2.1.1.1 |  | Same as 6.2.2.1.1.1 |
| 6.2.3.2.2.1 4Rx TDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |  |  |
| 6.2.3.2.2.2 4Rx TDD FR1 aperiodic subband CQI reporting under fading conditions for both SA and NSA | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |  |  |
| 6.2.3.2.2.3 4Rx TDD FR1 Wideband CQI reporting with inter-cell interference for both SA and NSA | Same as 6.2.2.1.2.3 |  | Same as 6.2.2.1.2.3 |  |  |
| 6.2A.3.1.1 2Rx CQI reporting accuracy under AWGN conditions for CA (2DL CA) |  | Same as 6.2.2.1.1.1 for each CC |  | Same as 6.2.2.1.1.1 |
| 6.2A.3.1.2 2Rx CQI reporting accuracy under AWGN conditions for CA (3DL CA) |  | Same as 6.2.2.1.1.1 for each CC |  | Same as 6.2.2.1.1.1 |
| 6.2A.3.1.3 2Rx CQI reporting accuracy under AWGN conditions for CA (4DL CA) |  | Same as 6.2.2.1.1.1 for each CC |  | Same as 6.2.2.1.1.1 |
| 6.3.1.1.1 1Rx FDD Single PMI with 4TX TypeI-SinglePanel Codebook for RedCap |  | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |
| 6.3.1.2.1 1Rx TDD Single PMI with 4TX TypeI-SinglePanel Codebook for RedCap |  | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |
| 6.3.2.1.1 2Rx FDD FR1 Single PMI with 4Tx Type I- SinglePanel codebook for both SA and NSA | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |  |  |
| 6.3.2.1.2 2Rx FDD FR1 Single PMI with 8Tx Type I – SinglePanel codebook for both SA and NSA | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |  |  |
| 6.3.2.1.3 2Rx FDD FR1 Multiple PMI with 16Tx Type I – SinglePanel Codebook for both SA and NSA | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |  |  |
| 6.3.2.1.4 2Rx FDD FR1 Single PMI with 32Tx Type1 - SinglePanel codebook for both SA and NSA |  | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |
| 6.3.2.1.5 2Rx FDD FR1 Multiple PMI with 16Tx TypeII codebook for both SA and NSA |  | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |
| 6.3.2.1.6 2Rx FDD FR1 Multiple PMI with 16Tx Enhanced TypeII codebook for both SA and NSA |  | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |
| 6.3.2.1.7 2Rx FDD FR1 Single PMI with 8TX TypeI-SinglePanel codebook for both SA and NSA |  | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |
| 6.3.2.1.9 2Rx FDD FR1 Multiple PMI with 8 ports Enhanced Type II Codebook for CJT for both SA and NSA |  | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |
| 6.3.2.2.1 2Rx TDD FR1 Single PMI with 4Tx TypeI – SinglePanel codebook for both SA and NSA |  | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |
| 6.3.2.2.2 2Rx TDD FR1 Single PMI with 8Tx TypeI – SinglePanel codebook for both SA and NSA |  | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |
| 6.3.2.2.3 2Rx TDD FR1 Single PMI with 16Tx Type1 – SinglePanel codebook for both SA and NSA |  | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |
| 6.3.2.2.4 2Rx TDD FR1 Single PMI with 32Tx Type1 – SinglePanel codebook for both SA and NSA |  | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |
| 6.3.2.2.5 2Rx TDD FR1 Multiple PMI with 16Tx TypeII codebook for both SA and NSA |  | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |
| 6.3.2.2.6 2Rx TDD FR1 Multiple PMI with 16Tx Enhanced TypeII codebook for both SA and NSA |  | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |
| 6.3.2.2.7 2Rx TDD Single PMI with 4TX TypeI-SinglePanel Codebook for RedCap |  | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |
| 6.3.2.2.8 Single PMI with 8 ports TypeI-SinglePanel Codebook for Single-DCI based transmission scheme |  | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |
| 6.3.3.1.1 Single PMI with 4TX TypeI-SinglePanel Codebook– SinglePanel codebook for both SA and NSA | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |  |  |
| 6.3.3.1.2 Single PMI with 8TX TypeI-SinglePanel Codebook– SinglePanel codebook for both SA and NSA | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |  |  |
| 6.3.3.1.3 4Rx FDD FR1 Multiple PMI with 16Tx Type I – SinglePanel Codebook for both SA and NSA | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |  |  |
| 6.3.3.1.4 4Rx FDD FR1 Single PMI with 32Tx Type1 - SinglePanel codebook for both SA and NSA |  | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |
| 6.3.3.1.5 4Rx FDD FR1 Multiple PMI with 16Tx TypeII codebook for both SA and NSA |  | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |
| 6.3.3.1.6 4Rx FDD FR1 Multiple PMI with 16Tx Enhanced TypeII codebook for both SA and NSA |  | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |
| 6.3.3.1.7 Single PMI with 8 ports TypeI-SinglePanel Codebook for Single-DCI based transmission scheme |  | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |
| 6.3.3.1.9 |  | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |
| 6.3.3.2.1 4Rx TDD FR1 Single PMI with 4Tx Type1 - SinglePanel codebook for both SA and NSA | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |  |  |
| 6.3.3.2.2 4Rx TDD FR1 Single PMI with 8Tx Type1 - SinglePanel codebook for both SA and NSA | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |  |  |
| 6.3.3.2.3 4Rx TDD FR1 Single PMI with 16Tx Type1 - SinglePanel codebook for both SA and NSA | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |  |  |
| 6.3.3.2.4 4Rx TDD FR1 Single PMI with 32Tx Type1 - SinglePanel codebook for both SA and NSA | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |  |  |
| 6.3.3.2.5 4Rx TDD FR1 Multiple PMI with 16Tx TypeII codebook for both SA and NSA | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |  |  |
| 6.3.3.2.6 4Rx TDD FR1 Multiple PMI with 16Tx Enhanced TypeII codebook for both SA and NSA |  | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |
| 6.3.3.2.7 Single PMI with 8 ports TypeI-SinglePanel Codebook for Single-DCI based transmission scheme |  | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |
| 6.4.2.1_1 2Rx FDD FR1 RI reporting for both SA and NSA | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |  |  |
| 6.4.2.1.1 2Rx FDD FR1 RI reporting for RedCap | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |  |  |
| 6.4.2.2_1 2Rx TDD FR1 RI reporting for both SA and NSA | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |  |  |
| 6.4.2.2.1 2Rx TDD FR1 RI reporting for RedCap | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |  |  |
| 6.4.3.1_1 4Rx FDD FR1 RI reporting for both SA and NSA | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |  |  |
| 6.4.3.2_1 4Rx TDD FR1 RI reporting for both SA and NSA | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |  |  |

## <<< Skip unchanged sections >>>

### F.1.3.2	Measurement of Demod Performance requirements

The derivation of the test requirements for the test cases in section 5 is defined in Table F.1.3.2-1.

Table F.1.3.2-1: Derivation of Test Requirements (FR1 demodulation performance tests)

| Test | Minimum Requirement in TS 38.101-4 | Test Tolerance (TT) | Test Requirement in TS 38.521-4 |
| --- | --- | --- | --- |
| 5.2.1.1.1 1 Rx FDD FR1 PDSCH performance for RedCap | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.1.1.2 1 Rx FDD FR1 PDSCH performance for eRedCap | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |

<<< Skip unchanged rows >>

| 5.6.3.2.1_1 4Rx TDD FR1 PDSCH performance with link adaptation – 2x4 MIMO for SA | FFS | FFS | FFS |
| --- | --- | --- | --- |
| 9.4B.1.1 Sustained downlink data rate performance for EN-DC within FR1 | Downlink power -112 dBm/Hz | 0 dB | Formula: Downlink power + TT T-put limit unchanged |
| 9.4B.1.1A Sustained downlink data rate performance for EN-DC within FR1 with DL 1024QAM | Downlink power -110.5 dBm/Hz | 0 dB | Formula: Downlink power + TT T-put limit unchanged |
| 9.4B.1.1_1 Sustained downlink data rate performance for EN-DC within FR1 (2 NR CCs) | Same as 9.4B.1.1 | Same as 9.4B.1.1 | Same as 9.4B.1.1 |
| 9.4B.1.1_2 Sustained downlink data rate performance for EN-DC within FR1 (3 NR CCs) | Same as 9.4B.1.1 | Same as 9.4B.1.1 | Same as 9.4B.1.1 |
| 9.4B.1.1_3 Sustained downlink data rate performance for EN-DC within FR1 (4 NR CCs) | Same as 9.4B.1.1 | Same as 9.4B.1.1 | Same as 9.4B.1.1 |

### F.1.3.3	Measurement of Channel State Information reporting

The derivation of the test requirements for the test cases in section 6 is defined in Table F.1.3.3-1.

Table F.1.3.3-1: Derivation of Test Requirements (FR1 channel state information reporting tests)

| Test | Minimum Requirement in TS 38.101-4 | Test Tolerance (TT) | Test Requirement in TS 38.521-4 |
| --- | --- | --- | --- |
| 6.2.1.1.1.1 1Rx FDD FR1 periodic CQI reporting under AWGN conditions for RedCap | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.1.1.1.2 1Rx FDD FR1 periodic CQI reporting under AWGN conditions for eRedCap | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.1.1.2.1 1Rx FDD FR1 periodic wideband CQI reporting under fading conditions for RedCap | SNRs as specified  20% 1.05 BLER 0.02 | SNR 0 dB  0% 0.01 BLER 0 | SNR unchanged  unchanged  1.04 BLER limit unchanged |
| 6.2.1.1.2.2 1Rx FDD FR1 periodic wideband CQI reporting under fading conditions for eRedCap | SNRs as specified | 6.2.1.1.2.2 1Rx FDD FR1 periodic wideband CQI reporting under fading conditions for eRedCap | SNRs as specified |
| 6.2.1.2.1.1 1Rx TDD FR1 periodic CQI reporting under AWGN conditions for RedCap | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.1.2.1.2 1Rx TDD FR1 periodic CQI reporting under AWGN conditions for eRedCap | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.1.2.2.1 1Rx TDD FR1 periodic wideband CQI reporting under fading conditions for RedCap | SNRs as specified  20% 1.05 BLER 0.02 | SNR 0 dB  0% 0.01 BLER 0 | SNR unchanged  unchanged  1.04 BLER limit unchanged |
| 6.2.2.1.1.1 2Rx FDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.2.1.1.2 2Rx FDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.2.1.1.3 2Rx FDD FR1 periodic CQI reporting with Table 4 under AWGN conditions for both SA and NSA | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.2.1.1.4 2 Rx FDD FR1 periodic CQI reporting under AWGN conditions for RedCap | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.2.1.2.1 2Rx FDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA | SNRs as specified  20% 1.05 BLER 0.02 | SNR 0 dB  0% 0.01 BLER 0 | SNR unchanged  unchanged  1.04 BLER limit unchanged |
| 6.2.2.1.2.2 2Rx FDD FR1 periodic subband CQI reporting under fading conditions for both SA and NSA | SNRs as specified  2% 55% 1.05 BLER 0.02 | SNR 0 dB  0% 0% 0.01 BLER 0 | SNR unchanged  limit unchanged limit unchanged  1.04 BLER limit unchanged |
| 6.2.2.1.2.3 2Rx FDD FR1 Wideband CQI reporting with inter-cell interference for both SA and NSA | Noc: -92.23dBm/15kHz INR 2: 10.04dB SINR: -2.00dB 1.9 BLER 0.02 | 0dB +0.3dB +0.03dB 0.01 BLER 0 | Noc: -92.23dBm/15kHz INR 2: 10.34dB SINR: -1.97dB  1.89 BLER limit unchanged The analysis is recorded in 3GPP TR 38.903 [26] |
| 6.2.2.1.2.4 2 Rx FDD FR1 periodic wideband CQI reporting under fading conditions for RedCap | SNRs as specified  20% 1.05 BLER 0.02 | SNR 0 dB  0% 0.01 BLER 0 | SNR unchanged  unchanged  1.04 BLER limit unchanged |
| 6.2.2.2.1.1 2Rx TDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.2.2.1.2 2Rx TDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.2.2.1.4 2Rx TDD FR1 periodic CQI reporting with Table 4 under AWGN conditions for both SA and NSA | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.2.2.1.3 2Rx TDD FR1 CQI reporting for PCell on band with shared spectrum access under AWGN conditions for both SA and NSA | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.2.2.1.5 2Rx TDD FR1 periodic CQI reporting under AWGN conditions for RedCap | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.2.2.2.1 2Rx TDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA | SNRs as specified  20% 1.05 BLER 0.02 | SNR 0 dB  0% 0.01 BLER 0 | SNR unchanged  unchanged  1.04 BLER limit unchanged |
| 6.2.2.2.2.2 2Rx TDD FR1 periodic subband CQI reporting under fading conditions for both SA and NSA | SNRs as specified  2% 55% 1.05 BLER 0.02 | SNR 0 dB  0% 0% 0.01 BLER 0 | SNR unchanged  limit unchanged limit unchanged  1.04 BLER limit unchanged |
| 6.2.2.2.2.3 2Rx TDD FR1 Wideband CQI reporting with inter-cell interference for both SA and NSA | Noc: -89.23dBm/15kHz INR 2: 10.04dB SINR: -2.00dB 1.9 BLER 0.02 | 0dB +0.3dB +0.03dB 0.01 BLER 0 | Noc: -89.23dBm/15kHz INR 2: 10.34dB SINR: -1.97dB  1.89 BLER limit unchanged The analysis is recorded in 3GPP TR 38.903 [26] |
| 6.2.2.2.2.4 2Rx TDD FR1 periodic wideband CQI reporting under fading conditions for RedCap | SNRs as specified  20% 1.05 BLER 0.02 | SNR 0 dB  0% 0.01 BLER 0 | SNR unchanged  unchanged  1.04 BLER limit unchanged |
| 6.2.3.1.1.1 4Rx FDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.3.1.1.2 4Rx FDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.3.1.1.3 4Rx FDD FR1 periodic CQI reporting with Table 4 under AWGN conditions for both SA and NSA | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.3.1.2.1 4Rx FDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA | SNRs as specified  5% 1.05 BLER 0.02 | SNR 0 dB  0% 0.01 BLER 0 | SNR unchanged  unchanged  1.04 BLER limit unchanged |
| 6.2.3.1.2.2 4Rx FDD FR1 aperiodic subband CQI reporting under fading conditions for both SA and NSA | SNRs as specified  2% 55% 1.05 BLER 0.02 | SNR 0 dB  0% 0% 0.01 BLER 0 | SNR unchanged  limit unchanged limit unchanged  1.04 BLER limit unchanged |
| 6.2.3.1.2.3 4Rx FDD FR1 Wideband CQI reporting with inter-cell interference for both SA and NSA | Noc: -92.23dBm/15kHz INR 2: 10.04dB SINR: -2.00dB 2.0 BLER 0.02 | 0dB +0.3dB +0.03dB 0.01 BLER 0 | Noc: -92.23dBm/15kHz INR 2: 10.34dB SINR: -1.97dB  1.99 BLER limit unchanged The analysis is recorded in 3GPP TR 38.903 [26] |
| 6.2.3.2.1.1 4Rx TDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.3.2.1.2 4Rx TDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.3.2.1.4 4Rx TDD FR1 periodic CQI reporting with Table 4 under AWGN conditions for both SA and NSA | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.3.2.1.3 4Rx TDD FR1 CQI reporting for PCell on band with shared spectrum access under AWGN conditions for both SA and NSA | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.3.2.2.1 4Rx TDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA | SNRs as specified  5% 1.05 BLER 0.02 | SNR 0 dB  0% 0.01 BLER 0 | SNR unchanged  unchanged  1.04 BLER limit unchanged |
| 6.2.3.2.2.2 4Rx TDD FR1 aperiodic subband CQI reporting under fading conditions for both SA and NSA | SNRs as specified  2% 55% 1.05 BLER 0.02 | SNR 0 dB  0% 0% 0.01 BLER 0 | SNR unchanged  limit unchanged limit unchanged  1.04 BLER limit unchanged |
| 6.2.3.2.2.3 4Rx TDD FR1 Wideband CQI reporting with inter-cell interference for both SA and NSA | Noc: -89.23dBm/15kHz INR 2: 10.04dB SINR: -2.00dB 2.0 BLER 0.02 | 0dB +0.3dB +0.03dB 0.01 BLER 0 | Noc: -89.23dBm/15kHz INR 2: 10.34dB SINR: -1.97dB  1.99 BLER limit unchanged The analysis is recorded in 3GPP TR 38.903 [26] |
| 6.2A.3.1.1 2Rx CQI reporting accuracy under AWGN conditions for CA (2DL CA) | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | Test requirement unchanged |
| 6.2A.3.1.2 2Rx CQI reporting accuracy under AWGN conditions for CA (3DL CA) | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | Test requirement unchanged |
| 6.2A.3.1.3 2Rx CQI reporting accuracy under AWGN conditions for CA (4DL CA) | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | Test requirement unchanged |
| 6.3.1.1.1 1Rx FDD Single PMI with 4TX TypeI-SinglePanel Codebook for RedCap | SNRs as specified g 1.30 | SNR 0 dB g 0.01 | SNR unchanged g 1.29 |
| 6.3.1.2.1 1Rx TDD Single PMI with 4TX TypeI-SinglePanel Codebook for RedCap | SNRs as specified g 1.30 | SNR 0 dB g 0.01 | SNR unchanged g 1.29 |
| 6.3.2.1.1 2Rx FDD FR1 Single PMI with 4Tx Type I- SinglePanel codebook for both SA and NSA | SNRs as specified 1.30 | SNR 0 dB 0.01 | SNR unchanged  1.29 |
| 6.3.2.1.2 2Rx FDD FR1 Single PMI with 8Tx Type I – SinglePanel codebook for both SA and NSA | SNRs as specified 1.50 | SNR 0 dB 0.01 | SNR unchanged  1.49 |
| 6.3.2.1.3 2Rx FDD FR1 Multiple PMI with 16Tx Type I – SinglePanel Codebook for both SA and NSA | SNRs as specified 2.50 | SNR 0 dB 0.01 | SNR unchanged  2.49 |
| 6.3.2.1.4 2Rx FDD FR1 Single PMI with 32Tx Type1 - SinglePanel codebook for both SA and NSA | SNRs as specified g 5.0 | SNR 0 dB g 0.01 | SNR unchanged g 4.99 |
| 6.3.2.1.5 2Rx FDD FR1 Multiple PMI with 16Tx TypeII codebook for both SA and NSA | SNRs as specified g 1.9 | SNR 0 dB g 0.01 | SNR unchanged g 1.89 |
| 6.3.2.1.6 2Rx FDD FR1 Multiple PMI with 16Tx Enhanced TypeII codebook for both SA and NSA | SNRs as specified 2.2 | SNR 0 dB 0.01 | SNR unchanged  2.19 |
| 6.3.2.1.7 Single PMI with 8 ports TypeI-SinglePanel Codebook for Single-DCI based transmission scheme | SNRs as specified 1.6 | SNR 0 dB 0.01 | SNR unchanged  1.59 |
| 6.3.2.1.9 2Rx FDD FR1 Multiple PMI with 8 ports Enhanced Type II Codebook for CJT for both SA and NSA | SNRs as specified  1.8 | SNR 0 dB  0.01 | SNR unchanged  1.79 |
| 6.3.2.2.1 2Rx TDD FR1 Single PMI with 4Tx TypeI - SinglePanel codebook for both SA and NSA | SNRs as specified 1.30 | SNR 0 dB 0.01 | SNR unchanged  1.29 |
| 6.3.2.2.2 2Rx TDD FR1 Single PMI with 8Tx TypeI - SinglePanel codebook for both SA and NSA | SNRs as specified 1.50 | SNR 0 dB 0.01 | SNR unchanged  1.49 |
| 6.3.2.2.3 2Rx TDD FR1 Single PMI with 16Tx Type1 - SinglePanel codebook for both SA and NSA | SNRs as specified 2.50 | SNR 0 dB 0.01 | SNR unchanged  2.49 |
| 6.3.2.2.4 2Rx TDD FR1 Single PMI with 32Tx Type1 - SinglePanel codebook for both SA and NSA | SNRs as specified 5.0 | SNR 0 dB 0.01 | SNR unchanged  4.99 |
| 6.3.2.2.5 2Rx TDD FR1 Multiple PMI with 16Tx TypeII codebook for both SA and NSA | SNRs as specified g 1.9 | SNR 0 dB g 0.01 | SNR unchanged g 1.89 |
| 6.3.2.2.6 2Rx TDD FR1 Multiple PMI with 16Tx Enhanced TypeII codebook for both SA and NSA | SNRs as specified 2.2 | SNR 0 dB 0.01 | SNR unchanged  2.19 |
| 6.3.2.2.7 2Rx TDD Single PMI with 4TX TypeI-SinglePanel Codebook for RedCap | SNRs as specified 1.30 | SNR 0 dB 0.01 | SNR unchanged  1.29 |
| 6.3.2.2.8 Single PMI with 8 ports TypeI-SinglePanel Codebook for Single-DCI based transmission scheme | SNRs as specified  | SNR 0 dB 0.01 | SNR unchanged  1.59 |
| 6.3.3.1.1 Single PMI with 4TX TypeI-SinglePanel Codebook– SinglePanel codebook for both SA and NSA | SNRs as specified 1.30 | SNR 0 dB 0.01 | SNR unchanged  1.29 |
| 6.3.3.1.2 Single PMI with 8TX TypeI-SinglePanel Codebook– SinglePanel codebook for both SA and NSA | SNRs as specified 1.50 | SNR 0 dB 0.01 | SNR unchanged  1.49 |
| 6.3.3.1.3 4Rx FDD FR1 Multiple PMI with 16Tx Type I – SinglePanel Codebook for both SA and NSA | SNRs as specified 3.00 | SNR 0 dB 0.01 | SNR unchanged  2.99 |
| 6.3.3.1.4 4Rx FDD FR1 Single PMI with 32Tx Type1 - SinglePanel codebook for both SA and NSA | SNRs as specified g 7.0 | SNR 0 dB g 0.01 | SNR unchanged g 6.99 |
| 6.3.3.1.5 4Rx FDD FR1 Multiple PMI with 16Tx TypeII codebook for both SA and NSA | SNRs as specified g 1.9 | SNR 0 dB g 0.01 | SNR unchanged g 1.89 |
| 6.3.3.1.6 4Rx FDD FR1 Multiple PMI with 16Tx Enhanced TypeII codebook for both SA and NSA | SNRs as specified 2.2 | SNR 0 dB 0.01 | SNR unchanged  2.19 |
| 6.3.3.1.7 Single PMI with 8 ports TypeI-SinglePanel Codebook for Single-DCI based transmission scheme | SNRs as specified 1.6 | SNR 0 dB 0.01 | SNR unchanged  1.59 |
| 6.3.3.1.9 4Rx FDD FR1 Multiple PMI with 8 ports Enhanced Type II Codebook for CJT for both SA and NSA | SNRs as specified 1.8 | SNR 0 dB 0.01 | SNR unchanged  1.79 |
| 6.3.3.2.1 4Rx TDD FR1 Single PMI with 4Tx Type1 - SinglePanel codebook for both SA and NSA | SNRs as specified 1.30 | SNR 0 dB 0.01 | SNR unchanged  1.29 |
| 6.3.3.2.2 4Rx TDD FR1 Single PMI with 8Tx Type1 - SinglePanel codebook for both SA and NSA | SNRs as specified 1.50 | SNR 0 dB 0.01 | SNR unchanged  1.49 |
| 6.3.3.2.3 4Rx TDD FR1 Single PMI with 16Tx Type1 - SinglePanel codebook for both SA and NSA | SNRs as specified 3.0 | SNR 0 dB 0.01 | SNR unchanged  2.99 |
| 6.3.3.2.4 4Rx TDD FR1 Single PMI with 32Tx Type1 - SinglePanel codebook for both SA and NSA | SNRs as specified g 7.0 | SNR 0 dB g 0.01 | SNR unchanged g 6.99 |
| 6.3.3.2.5 4Rx TDD FR1 Multiple PMI with 16Tx TypeII codebook for both SA and NSA | SNRs as specified g 1.8 | SNR 0 dB g 0.01 | SNR unchanged g 1.79 |
| 6.3.3.2.6 4Rx TDD FR1 Multiple PMI with 16Tx Enhanced TypeII codebook for both SA and NSA | SNRs as specified 2.2 | SNR 0 dB 0.01 | SNR unchanged  2.19 |
| 6.3.3.2.7 Single PMI with 8 ports TypeI-SinglePanel Codebook for Single-DCI based transmission scheme | SNRs as specified 1.6 | SNR 0 dB 0.01 | SNR unchanged  1.59 |
| 6.4.2.1_1 2Rx FDD FR1 RI reporting for both SA and NSA | SNRs as specified 1.00 for Test 1 1.05 for Test 2 0.90 for Test 3 | SNR 0 dB 0.01 for Test 1 0.01 for Test 2 0.01 for Test 3 | SNR unchanged  0.99 for Test 1  1.04 for Test 2  0.89 for Test 3 |
| 6.4.2.1.1 2Rx FDD FR1 RI reporting for RedCap | SNRs as specified 1.05 | SNR 0 dB 0.01 | SNR unchanged  1.04 |
| 6.4.2.2_1 2Rx TDD FR1 RI reporting for both SA and NSA | SNRs as specified 1.00 for Test 1 1.05 for Test 2 0.90 for Test 3 | SNR 0 dB 0.01 for Test 1 0.01 for Test 2 0.01 for Test 3 | SNR unchanged  0.99 for Test 1  1.04 for Test 2  0.89 for Test 3 |
| 6.4.2.2.1 2Rx TDD FR1 RI reporting for RedCap | SNRs as specified 1.05 | SNR 0 dB 0.01 | SNR unchanged  1.04 |
| 6.4.3.1_1 4Rx FDD FR1 RI reporting for both SA and NSA | SNRs as specified 0.90 for Test 1 1.05 for Test 2 0.90 for Test 3 0.90 for Test 4 | SNR 0 dB 0.01 for Test 1 0.01 for Test 2 0.01 for Test 3 0.01 for Test 4 | SNR unchanged  0.89 for Test 1  1.04 for Test 2  0.89 for Test 3  0.89 for Test 4 |
| 6.4.3.2_1 4Rx TDD FR1 RI reporting for both SA and NSA | SNRs as specified 0.90 for Test 1 1.05 for Test 2 0.90 for Test 3 0.90 for Test 4 | SNR 0 dB 0.01 for Test 1 0.01 for Test 2 0.01 for Test 3 0.01 for Test 4 | SNR unchanged  0.89 for Test 1 as per Table G.3.4  1.04 for Test 2 as per Table G.3.4  0.89 for Test 3 as per Table G.3.4  0.89 for Test 4 as per Table G.3.4 |

## <<< END OF CHANGES >>>