3GPP TSG-RAN5 Meeting #106	R5-250971r1

# Athens, Greece, 17th Feb 2025 – 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3445 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test procedure for 5G ProSe Layer-2 U2N Relay / Remote UE RRC Reestablishment / Remote UE side |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-UEConTest |  |  |  |  |  | Date: |  |  | 2025-01-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test procedure to check 5G ProSe L2 U2N Remote UE in RRC_CONNECTED performs an RRC Reestablishment procedure on a certain L2 U2N Relay UE is needed for Uu RLF & Sidelink RLF U2N Relay testcases and missing from the current spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding test procedure to check 5G ProSe L2 U2N Remote UE in RRC_CONNECTED performs an RRC Reestablishment procedure on a certain L2 U2N Relay UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.45 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: modified the title and Scope. |  |  |  |  |  |  |  |  |

<Start of Changed Section>

4.9.45	Test procedure to check 5G ProSe L2 U2N Remote UE in RRC_CONNECTED performs an RRC Reestablishment procedure on a certain L2 U2N Relay UE

4.9.45.1	Scope

The purpose aims at checking whether 5G ProSe L2 U2N Remote UE in RRC_CONNECTED performs an RRC Reestablishment procedure on a certain L2 U2N Relay UE (as specified in the test case).

4.9.45.2	Procedure description

4.9.45.2.1	Initial conditions

UE is authorised to perform NR sidelink communication.

4.9.45.2.2	Procedure sequence

Table 4.9.45.2.2-1: Procedure for 5G ProSe Layer-2 U2N Relay / Remote UE RRC Reestablishment / Remote UE side

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-6 | Steps 1 to 6 of the procedure for 5G ProSe Layer-2 U2N Relay initial access / Remote UE side in TS 38.508-1 Table 4.9.42.2.2-1 using parameter Cast Type (Unicast) and NR-SS-UE initiated unicast mode ProSe Direct communication is executed. | - | - | - | - |
| 7 | Check: Does the UE transmit an RRCReestablishmentRequest message. | --> | PC5 Relay RLC Channel: RRCReestablishmentRequest | - | P |
| 8 | The NR-SS-UE transmits an RRCReestablishment message. | <-- | PC5 Relay RLC Channel: RRCReestablishment | - | - |
| 9 | The UE transmits an RRCReestablishmentComplete message. | --> | PC5 Relay RLC Channel: RRCReestablishmentComplete | - | - |
| 10 | The NR-SS-UE transmits an RRCReconfiguration message to resume existing radio bearer. | <-- | PC5 Relay RLC Channel: RRCReconfiguration | - | - |
| 11 | The UE transmits an RRCReconfigurationComplete message. | --> | PC5 Relay RLC Channel: RRCReconfigurationComplete | - | - |

4.9.45.3	Specific message contents

Table 4.9.45.3-1: RRCReconfiguration (Step 10, Table 4.9.45.2.2-1)

| Derivation path: Table 4.6.1-13 with condition NR and L2RemoteUE |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig with conditions SRB2 and DRB1 |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig with condition SRB2_DRB1 |  |  |
| dedicatedNAS-MessageList SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message {} | DedicatedNAS-Message |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of Changed Section>