3GPP TSG-RAN WG5 Meeting # 106	R5-251090r1

# Athens, Greece, February 17 – 21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3243 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on clause 4 for applicability and test coverage rules in TS 38.521-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Tejet, SRTC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-01-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To simplify the spurious emission for UE co-existence testing for NSA band combinations among different features, the general descriptions for the applicability and test coverage rules are introduced in RAN5#105. Howevever, the wording for the general descriptions in 4.5 is inaccurate and need to be improved. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Improve the wording for the general descriptions for the applicability and test coverage rules for NSA band combinations among different features. - Remove the editor’s notes in 4.5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The wording for the general descriptions for the applicability and test coverage rules will be inaccurate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-251090r1: Remove the NE-DC related wording in the CR. |  |  |  |  |  |  |  |  |

<< Start of changes >>

4.5	Applicability and test coverage rules

The applicability and test coverage rules for NR/5GC and EN-DC capable devices shall include the following:

If a test case for a FR1 NR band in a device is tested in EN-DC mode inter-band operation for non-exceptional requirement as per TS 38.521-3 [14], it shall fulfil the coverage requirement for that test case for NR/5GC FR1 test requirements for that NR band and need not be retested.

Unless there are exception requirements defined within the NR-DC test cases in clause 6 and 7 of this spec: if a DUT is tested on an FR1 NR-CA configuration, the DUT need not be retested for the FR1 NR-DC configuration involving the same bands within the component carriers/carrier groups (CGs).

If release 16 and forward spurious emission for UE co-existence requirement is tested for an uplink multi-band combination in EN-DC mode as per TS 38.521-3, it shall fulfil the coverage requirement for that test case for NR/5GC FR1 test requirements for that uplink combination in NR CA mode and in NR SUL mode and need not be retested, in which the corresponding SUL bands have same UL frequency range as the bands in EN-DC configuration.

Note:	An uplink multi-band combination in CA mode is CA_nA-nB, the corresponding EN-DC mode is DC_A_nB or DC_B_nA, and the corresponding SUL mode is SUL_nA-nD and SUL_nB-nC, in which the bands A/nA and B/nB are E-UTRA/NR bands, the band nC and nD are the corresponding SUL bands with the same UL frequency range as that of bands A/nA and B/nB, e.g. SUL band n80 has the same UL frequency range as band n3.

4.6	Pass fail decision rule of test case

Unless explicitly specified, a test case is passed only when all the measurements in the test case are passed. A measurement is one execution of the test procedures using a specific combination of various conditions, including test configuration, testing UL/DL power level, frequency location of interference and sweeping frequency location of emission measurement etc. If multiple component carriers are involved in one measurement, the measurement is passed only when all the component carriers are passed.

==============================================================

<< End of changes >>