3GPP TSG-RAN WG5 Meeting # 106	R5-251096r1

# Athens, Greece, February 17 – 21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1918 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on test coverage rules for NSA band combinations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Tejet, SRTC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-01-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To simplify the spurious emission for UE co-existence testing for NSA band combinations among different features, the general descriptions for the applicability and test coverage rules are introduced in RAN5#105. Howevever, the wording for the general descriptions in 4.5.0 is inaccurate and need to be improved. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Improve the wording for the general descriptions for the applicability and test coverage rules for NSA band combinations among different features. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The wording for the general descriptions for the applicability and test coverage rules will be inaccurate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-251096r1: Remove the NE-DC related wording in the CR. |  |  |  |  |  |  |  |  |

<< Start of changes >>

4.5	Applicability and test coverage rules

4.5.0	General

(1)	The applicability and test coverage rules for Non-Standalone (NSA) only capable devices shall include the following:

a)	For each NR band in a device, test all the EN-DC or NE-DC exception test requirements as per test procedures in this specification.

b)	Test all the EN-DC or NE-DC FR2 non-exception test requirements in this specification with test procedures which refer appropriately back to TS 38.521-2 [9] for each NR band. Test only one EN-DC or NE-DC combination per FR2 band for each EN-DC or NE-DC configuration as defined in clause 5.5B of TS 38.101-3 [4] using LTE anchor agnostic approach.

c)	Test all the EN-DC or NE-DC FR1 non-exception test requirements in this specification with test procedures which refer appropriately back to TS 38.521-1 [8] for each NR band. Test only one EN-DC or NE-DC combination per FR1 band for each EN-DC or NE-DC configuration as defined in clause 5.5B of 38.101-3 [4] using LTE anchor agnostic approach.

(2)	The applicability and test coverage rules for Standalone (SA) and NSA capable devices shall include the following:

a)	For each NR band in a device, test all the EN-DC or NE-DC exception test requirements as per test procedures in this specification.

b)	Test all the Standalone FR2 test requirements as per test procedures in TS 38.521-2 [9] for each NR band. This also fulfils coverage for all non-exception EN-DC or NE-DC FR2 test requirements for that NR band and need not be retested. Alternatively test in EN-DC or NE-DC mode following (1) b) above.

c)	Test all the Standalone FR1 test requirements as per test procedures in TS 38.521-1 [8] for each NR band. This also fulfils coverage for all non-exception EN-DC or NE-DC FR1 test requirements for that NR band and need not be retested. Alternatively test in EN-DC or NE-DC mode following (1) c) above.

d)	Test all the Release 16 and forward Standalone FR1 spurious emissions for UE co-existence test requirements for an uplink multi-band combination in NR CA mode or in NR SUL mode as per test procedures in TS 38.521-1 [8]. This also fulfils coverage for corresponding EN-DC exceptional test requirements in this specification for that uplink combination in EN-DC mode and need not be retested.

NOTE:	An uplink multi-band combination in EN-DC mode is DC_A_nB or DC_B_nA, the corresponding CA mode is CA_nA-nB, and the corresponding SUL mode is SUL_nA-nD and SUL_nB-nC, in which the bands A/nA and B/nB are E-UTRA/NR bands, the bands nC and nD are the corresponding SUL bands with the same UL frequency range as that of bands A/nA and B/nB, e.g. SUL band n80 has the same UL frequency range as band n3.

==============================================================

<< End of changes >>