3GPP TSG-RAN5 Meeting #106 	R5-251171r1

# Athens, Greece, 17th Feb 2025 – 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3830 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR-NTN inter-frequency Handover test case 14.2.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Germany |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | NR-NTN inter-frequency Handover test case 14.2.1.2 is missing from the specification. In addition, TT analysis for this test has become available. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Addition of new test case 14.2.1.2 - Implemented TT analysis on test case 14.2.1.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case would be missing in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.2.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: added editor’s note indicating the neighbor cell ephemeris needs to be further analyzed |  |  |  |  |  |  |  |  |

<< Start of Changes >>

#### 14.2.1.2	NR SA FR1-FR1 Handover for Satellite Access

Editor's Note: This test is incomplete in the following aspects:

- Several test parameters and configuration are still in brackets

- AT command related configuration is FFS

- Needs alignment with latest changes introduced in 38.133 (gap, SMTC, SSB configurations)

- Neighbor cell ephemeris needs further analysis

14.2.1.2.1	Test purpose

To verify the requirement for Inter-frequency SAN Handover from FR1 to FR1 specified in clause 14.2.1.0.1.

14.2.1.2.2	Test applicability

This test applies to all types of NR UE release 17 and forward supporting satellite access.

14.2.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 14.2.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.14.2.1.2.

14.2.1.2.4	Test description

14.2.1.2.4.1	Initial conditions

The test scenario comprises of two NR carriers and two satellite access cells as given in tables 14.2.1.2.4.1-3 and 14.2.1.2.5-1. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE may not have any timing information of Cell 2. During T1, the UE is configured to measure inter frequency neighbour cell with Event A3 report and a gap patterns are configured in the test case.

Starting T2, Cell 2 becomes detectable and offset better than Cell 1. The RRC message implying handover to Cell 2 shall be sent to the UE during period T2, after the UE has reported Event A3. The start of T3 is defined as the end of the last TTI containing the RRC message implying handover.

This test shall be tested using any of the test configurations in Table 14.2.1.2.4.1-1.

Table 14.2.1.2.4.1-1: Supported test configurations for NR SA FR1-FR1 Handover for Satellite Access

| Configuration | Description |
| --- | --- |
| 14.2.1.2-1 | GSO, NR FDD, 15kHz SSB SCS, 10 MHz BW |
| 14.2.1.2-2 | NGSO, NR FDD, 15kHz SSB SCS, 10 MHz BW |
| Note: If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. |  |

Configure the test equipment and the DUT according to the parameters in Table 14.2.1.2.4.1-2

Table 14.2.1.2.4.1-2: Initial conditions for NR SA FR1-FR1 Handover for Satellite Access

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.12-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 14.2.1.2.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 14.2.1.2.4.3.

2.	Cell 1 and Cell 2 are NR FR1 cells with the power levels set according to clauses C.1.2 and C.1.3 for this test where Cell 2 is the target cell. Both Cell 1 and Cell 2 are satellite access cells.

3.	The general test parameters are given in Table 14.2.1.2.4.1-3.

4.	The initial test environment conditions are setup according to section 14.0.5.

Table 14.2.1.2.4.1-3: General test parameters for NR SA FR1-FR1 Handover for Satellite Access

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1, 2 | Two NR NTN satellite RF channel |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| Satellite configuration | Config 1 |  | RMC in [A.x] | For GSO satellite configuration |
|  | Config 2 |  | RMC in [A.x] | For NGSO satellite configuration |
| UE position (N,S, H) |  |  | [(0, 0, 0)] | Set by AT command |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

14.2.1.2.4.2	Test procedure

The test scenario comprises of two satellite access NR FDD inter-frequency cells as given in table 14.2.1.2.4.1-3. The handover delay is tested.

1.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Establish SRB2 and DRB in the RRC Reconfiguration message. Cell 1 is the active cell. Set Cell 2 physical cell identity to the initial physical cell identity.

2.	Set the parameters according to T1 in Table 14.2.1.2.5-1. Propagation conditions are set according to Annex C clause C.2.2. T1 starts.

3.	SS shall transmit an RRCReconfiguration message, configuring measurement object.

4.	UE shall transmit RRCReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 14.2.1.2.5-1.

6.	UE shall transmit a MeasurementReport message triggered by Event A3.

7.	SS shall transmit the RRCReconfiguration message implying handover to the UE, at that instant the SS shall switch the power settings from T2 to T3 as specified in Table 14.2.1.2.5-1. T3 starts.

8.	If the UE transmits the uplink PRACH channel to Cell 2 less than 72 ms from the beginning of time period T3 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

9.	The UE shall transmit RRCReconfigurationComplete message on Cell 2.

10.	After T3 expires, the SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

11.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 3) mod 1008) for next iteration of the test procedure loop.

12.	Once the connection is released, the SS shall switch off and then on the UE and then proceed with step 1.

13.	Repeat steps 2-12 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

14.2.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 14.2.1.2.4.3-1: Common Exception messages for NR SA FR1-FR1 Handover for Satellite Access

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTER-FREQ and GAP NEEDED Table H.3.1-3 with Condition INTER-FREQ MO, SSB.1 FR1, SMTC pattern 1 and Synchronous cells Table H.3.1-4 with A3-offset = 0 dB Table H.3.1-5 Table H.3.1-7 with Condition INTER-FREQ Table H.3.2-2 with Condition RBConfig_KeyChange |

14.2.1.2.5	Test requirements

Table 14.2.1.1.5-1 defines the primary level settings including test tolerances for NR SA FR1 Handover for Satellite Access.

Table 14.2.1.2.5-1: Cell specific test parameters for NR SA FR1-FR1 Handover for Satellite Access

| Parameter |  | Test configuration | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| NR RF Channel Number |  | Config 1,2 |  | 1 |  |  | 2 |  |  |
| BWchannel |  |  | MHz | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |  |
| BWP BW |  |  | MHz | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |  |
| TACommon |  | Config 1,2 | s | 0 |  |  | 0 |  |  |
| TACommonDrift |  |  | s | 0 |  |  | 0 |  |  |
| TACommonDriftVariation |  |  | s | 0 |  |  | 0 |  |  |
| Koffset |  | Config 1 | ms | [239] |  |  | [239] |  |  |
|  |  | Config 2 |  | [4] |  |  | [4] |  |  |
| Kmac |  | Config 1,2 | ms | 0 |  |  | 0 |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  |  | SR.1.1 FDD |  |  |  |  |  |
| CORESET Reference Channel |  |  |  | CR.1.1 FDD |  |  |  |  |  |
| TRS configuration |  |  |  | TRS.1.1 FDD |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |  |  |
| SSB Configuration |  |  |  | SSB.1 FR1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | 15 kHz |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  |  | kHz | 15 kHz |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |
| BWP configuration | Initial DL BWP | Config 1,2 |  | DLBWP.0.1 |  |  |  |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |  |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |  |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | Config 1,2 | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 |  | Config 1,2 | dBm/ 15kHz | -98 |  |  |  |  |  |
| Note2 |  |  | dBm/ SCS | -98 |  |  |  |  |  |
|  |  |  | dB | 4 | 4 | 4 | -Infinity | 9 | 9 |
|  |  |  | dB | 4 | 4 | 4 | -Infinity | 9 | 9 |
| SSB_RP |  |  | dBm/ SCS | -94 | -94 | -94 | -Infinity | -89 | -89 |
| IoNote3 |  |  | dBm/ 9.36MHz | -64.59 | -64.59 | -64.59 | -70.05 | -60.53 | -60.53 |
| Propagation condition |  |  | - | AWGN |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |

The UE shall start to transmit the PRACH to Cell 2 less than 72 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 10 ms and is specified in clause 12 in TS 38.331 [13]. Tinterrupt is defined in clause 14.2.1.0.1.2.

Tinterrupt = Tsearch + TIU + Tprocessing  + T∆ + Tmargin ms

Here: Tsearch = 0; TIU = 20ms; Tprocessing = 20ms; T∆ = 20ms; Tmargin = 2ms.

This gives a total of 72 ms.

<< End of Changes >>