3GPP TSG-RAN5 Meeting #106 	R5-251176r1

# Athens, Greece, 17th Feb 2025 – 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3835 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to Annex F for NR-NTN test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Germany |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TT analysis has become available for several NR-NTN RRM test cases. This CR updates Annex F for them, according to the TT analysis. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated Annex F sections F.1.1.2 and F.1.3.2 for several NR-NTN RRM test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases would remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1.1.2, F.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1- Corrected typo in TC reference |  |  |  |  |  |  |  |  |

<< Start of Changes >>

### F.1.1.2	Measurement of RRM requirements

<< Skipped tables >>

Table F.1.1.2-16: Maximum test system uncertainty for RRM requirements for SA FR1 Satellite Access test cases

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 14.1.9 NR SA FR1-FR1 Cell reselection for UE fulfilling low mobility relaxed measurement criterion for Satellite Access | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 14.1.10 NR SA FR1-FR1 Cell reselection for UE fulfilling not-at-cell edge relaxed measurement criterion for Satellite Access | Same as 14.1.9 | Same as 14.1.9 |
| 14.2.1.1 NR SA FR1 Handover for Satellite Access | Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 14.2.1.2 NR SA FR1-FR1 Handover for Satellite Access | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 14.2.1.3 NR SA FR1 Time-based Conditional Handover for NR Satellite Access | Same as 14.2.1.1 | Same as 14.2.1.1 |
| 14.2.1.4 NR SA FR1-FR1 Time-based Conditional Handover for NR Satellite Access | Same as 14.2.1.2 | Same as 14.2.1.2 |
| 14.2.1.5 NR SA FR1 Distance-based Conditional Handover for Satellite Access | Same as 14.2.1.1 | Same as 14.2.1.1 |
| 14.2.1.6 NR SA FR1-FR1 Distance-based Conditional Handover for Satellite Access | Same as 14.2.1.2 | Same as 14.2.1.2 |
| 14.4.1.1 NR SA FR1 Radio Link Monitoring Out-of-sync for PCell configured with SSB-based RLM RS in non-DRX mode for Satellite Access | Average Noc ±1.5 dB Average Ês / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês / Noc ±0.3 dB Fading profile ±0.7 dB | Noc is the AWGN on cell 1 frequency Ês / Noc is the SNR Meas PRB are the measurement PRB for SSB-based RLM #RBJ to RBJ+19 |
| 14.4.1.2 NR SA FR1 Radio Link Monitoring In-sync for PCell configured with SSB-based RLM RS in non-DRX mode for Satellite Access | Same as 14.4.1.1 | Same as 14.4.1.1 |
| 14.4.1.3 NR SA FR1 Radio Link Monitoring Out-of-sync for PCell configured with SSB-based RLM RS in DRX mode for Satellite Access | Same as 14.4.1.1 | Same as 14.4.1.1 |
| 14.4.1.4 NR SA FR1 Radio Link Monitoring In-sync for PCell configured with SSB-based RLM RS in DRX mode for Satellite Access | Same as 14.4.1.1 | Same as 14.4.1.1 |
| 14.4.1.5 NR SA FR1 Radio Link Monitoring Out-of-sync for PCell configured with CSI-RS-based RLM RS in non-DRX mode for Satellite Access | Same as 14.4.1.1 | Same as 14.4.1.1 |
| 14.4.1.6 NR SA FR1 Radio Link Monitoring In-sync for PCell configured with CSI-RS-based RLM RS in non-DRX mode for Satellite Access | Same as 14.4.1.1 | Same as 14.4.1.1 |
| 14.4.1.7 NR SA FR1 Radio Link Monitoring Out-of-sync for PCell configured with CSI-RS-based RLM RS in DRX mode for Satellite Access | Same as 14.4.1.1 | Same as 14.4.1.1 |
| 14.4.1.8 NR SA FR1 Radio Link Monitoring In-sync for PCell configured with CSI-RS-based RLM RS in DRX mode for Satellite Access | Same as 14.4.1.1 | Same as 14.4.1.1 |
| 14.4.2.1 NR SA FR1 Beam Failure Detection and Link Recovery Test for PCell configured with SSB-based BFD and LR in non-DRX mode for Satellite Access | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the SNR for the SSB Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 14.4.2.2 NR SA FR1 Beam Failure Detection and Link Recovery Test for PCell configured with SSB-based BFD and LR in DRX mode for Satellite Access | Same as 14.4.2.1 | Same as 14.4.2.1 |
| 14.4.2.3 NR SA FR1 Beam Failure Detection and Link Recovery Test for PCell configured with CSI-RS-based BFD and LR in non-DRX mode for Satellite Access | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the SNR for the CSI-RS Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |
| 14.4.2.4 NR SA FR1 Beam Failure Detection and Link Recovery Test for PCell configured with CSI-RS-based BFD and LR in DRX mode for Satellite Access | Same as 14.4.2.3 | Same as 14.4.2.3 |
| 14.5.1.1 SA FR1 event triggered reporting tests without gap under non-DRX for NR satellite access | Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 14.5.1.2 SA FR1 Event triggered reporting tests without gap under DRX for NR satellite access | Same as 14.5.1.1 | Same as 14.5.1.1 |
| 14.5.1.3 SA FR1 event triggered reporting tests without gap under non-DRX with FDD PCell with SSB index reading for NR satellite access | Same as 14.5.1.1 | Same as 14.5.1.1 |
| 14.5.1.4 SA FR1 event triggered reporting tests with single measurement gap under non-DRX for satellite access for NR satellite access | Same as 14.5.1.1 | Same as 14.5.1.1 |
| 14.5.1.5 SA FR1 event triggered reporting tests with FNO concurrent gaps under DRX for satellite access for NR satellite access | Same as 14.5.1.1 | Same as 14.5.1.1 |
| 14.5.1.6 SA FR1 event triggered reporting tests with PPO concurrent gaps under non-DRX with SSB index reading for satellite access for NR satellite access | Same as 14.5.1.1 | Same as 14.5.1.1 |
| 14.5.2.1 NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with single gap for satellite access | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 14.5.2.2 NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is used with single gap for satellite access | Same as 14.5.2.1 | Same as 14.5.2.1 |
| 14.5.2.3 NR SA FR1-FR1 Event-triggered reporting with SSB time index detection when DRX is not used with single gap for satellite access | Same as 14.5.2.1 | Same as 14.5.2.1 |
| 14.5.2.4 NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent fully non-overlapped (FNO) gaps for satellite access | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Average Ês3 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB Meas PRB Ês3 / Noc2 ±0.3 dB | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 14.5.2.6 NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent partial overlapping (PPO) gaps for satellite access | Same as 14.5.2.4 | Same as 14.5.2.4 |
| 14.6.3.1 NR SA FR1 SS-SINR Measurement Accuracy for Satellite Access | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB meas PRB Ês1 / Noc ±0.8 dB meas PRB Ês2 / Noc ±0.8 dB | Noc is the AWGN on NR freq1 Ês1 / Noc is the cell 1 SNR Ês2 / Noc is the cell 2 SNR meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 14.6.3.2 NR SA FR1-FR1 SS-SINR Measurement Accuracy for Satellite Access | Same as 14.6.3.1 | Same as 14.6.3.1 |
| 14.6.4.1 NR SA FR1 SSB based L1-RSRP Measurement Accuracy for Satellite Access | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.8 dB | Noc is the AWGN on NR freq1 Ês1 / Noc is the cell 1 SNR Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 14.6.4.2 NR SA FR1 CSI-RS based L1-RSRP Measurement Accuracy for Satellite Access | Same as 14.6.4.1 | Same as 14.6.4.1 |

<< Skipped sections >>

### F.1.3.2	Measurement of RRM requirements

<< Skipped tables >>

Table F.1.3.2-14: Derivation of test requirements for SA FR1 Satellite Access test cases

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 14.1.9 NR SA FR1-FR1 Cell reselection for UE fulfilling low mobility relaxed measurement criterion for Satellite Access | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: +12dB In signalling: Thresh_X_HighP of Cell 1 = 48dB Thresh_X_HighP of Cell 2 = 48dB Thresh_X_LowP of Cell 1 = 50dB Thresh_X_LowP of Cell 2 = 50dB Thresh_Serving_LowP of Cell 1 = 44dB Thresh_Serving_LowP of Cell 2 = 44dB SSearchDeltaP of Cell 1 = 3dB SSearchDeltaP of Cell 2 = 3dB | During T1: 0dB 0dB 0dB 0.3dB During T2: 0dB 0dB 0dB -2.25dB In signalling: 0dB -4dB -2dB 0dB 0dB 2dB 0dB 12dB | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: -3.7dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: +9.75dB In signalling: Thresh_X_HighP of Cell 1 = 48dB Thresh_X_HighP of Cell 2 = 44dB Thresh_X_LowP of Cell 1 = 48dB Thresh_X_LowP of Cell 2 = 50dB Thresh_Serving_LowP of Cell 1 = 44dB Thresh_Serving_LowP of Cell 2 = 46dB SSearchDeltaP of Cell 1 = 3dB SSearchDeltaP of Cell 2 = 15dB |
| 14.1.10 NR SA FR1-FR1 Cell reselection for UE fulfilling not-at-cell edge relaxed measurement criterion for Satellite Access | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: +12dB In signalling: SSearchThresholdP of Cell 1 = 50dB SSearchThresholdP of Cell 2 = 50dB | During T1: 0dB 2dB -2dB 0.35dB During T2: 0dB 0dB -2dB 4dB In signalling: 0dB -16dB | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +16dB Noc2: -100dBm/15kHz Ês2 / Noc2: -3.65dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -100dBm/15kHz Ês2 / Noc2: +16dB In signalling: SSearchThresholdP of Cell 1 = 50dB SSearchThresholdP of Cell 2 = 34dB |
| 14.2.1.1 NR SA FR1 Handover for Satellite Access | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB Signaled A3-Offset: 0dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB -1dB During T3: 0dB 0dB -1dB -1dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +10.00dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +10.00dB Signaled A3-Offset: -1dB |
| 14.2.1.2 NR SA FR1-FR1 Handover for Satellite Access | During T1: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: -infinity During T2: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: +9.00dB During T3: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: +9.00dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: -infinity During T2: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: +9.00dB During T3: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: +9.00dB |
| 14.2.1.3 NR SA FR1 Time-based Conditional Handover for NR Satellite Access | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB Signaled A3-Offset: 0dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB -1dB -1dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +10.00dB Signaled A3-Offset: -1dB |
| 14.2.1.4 NR SA FR1-FR1 Time-based Conditional Handover for NR Satellite Access | During T1: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: -infinity During T2: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: +9.00dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc2: -infinity During T2: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: +9.00dB |
| 14.2.1.5 NR SA FR1 Distance-based Conditional Handover for Satellite Access | Same as 14.2.1.3 | Same as 14.2.1.3 | Same as 14.2.1.3 |
| 14.2.1.6 NR SA FR1-FR1 Distance-based Conditional Handover for Satellite Access | Same as 14.2.1.4 | Same as 14.2.1.4 | Same as 14.2.1.4 |
| 14.4.1.1 NR SA FR1 Radio Link Monitoring Out-of-sync for PCell configured with SSB-based RLM RS in non-DRX mode for Satellite Access | SNR during T1: 1dB T2: -7dB T3: -15dB | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB |
| 14.4.1.2 NR SA FR1 Radio Link Monitoring In-sync for PCell configured with SSB-based RLM RS in non-DRX mode for Satellite Access | SNR during T1: 1dB T2: -7dB T3: -15dB T4: -4.5dB T5: 1dB | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: +0.8dB | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB T4: -5.3dB T5: 1.8dB |
| 14.4.1.3 NR SA FR1 Radio Link Monitoring Out-of-sync for PCell configured with SSB-based RLM RS in DRX mode for Satellite Access | Same as 14.4.1.1 | Same as 14.4.1.1 | Same as 14.4.1.1 |
| 14.4.1.4 NR SA FR1 Radio Link Monitoring In-sync for PCell configured with SSB-based RLM RS in DRX mode for Satellite Access | Same as 14.4.1.2 | Same as 14.4.1.2 | Same as 14.4.1.2 |
| 14.4.1.5 NR SA FR1 Radio Link Monitoring Out-of-sync for PCell configured with CSI-RS-based RLM RS in non-DRX mode for Satellite Access | Same as 14.4.1.1 | Same as 14.4.1.1 | Same as 14.4.1.1 |
| 14.4.1.6 NR SA FR1 Radio Link Monitoring In-sync for PCell configured with CSI-RS-based RLM RS in non-DRX mode for Satellite Access | Same as 14.4.1.2 | Same as 14.4.1.2 | Same as 14.4.1.2 |
| 14.4.1.7 NR SA FR1 Radio Link Monitoring Out-of-sync for PCell configured with CSI-RS-based RLM RS in DRX mode for Satellite Access | Same as 14.4.1.1 | Same as 14.4.1.1 | Same as 14.4.1.1 |
| 14.4.1.8 NR SA FR1 Radio Link Monitoring In-sync for PCell configured with CSI-RS-based RLM RS in DRX mode for Satellite Access | Same as 14.4.1.2 | Same as 14.4.1.2 | Same as 14.4.1.2 |
| 14.4.2.1 NR SA FR1 Beam Failure Detection and Link Recovery Test for PCell configured with SSB-based BFD and LR in non-DRX mode for Satellite Access | q0 SSB SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 SSB during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 SSB during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 SSB during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10dB | Offset during: T1: +0.4dB T2: +0.4dB T3: -0.4dB T4: -0.4dB T5: -0.4dB q1 SSB during T1: 0dB -0.1dB q1 SSB during T2: 0dB -0.1dB q1 SSB during T3, T4 and T5: 0dB +0.1dB | SNR during: T1: +5.4dB T2: -2.6dB T3: -12.4dB T4: -12.4dB T5: -12.4dB For testing of a UE which supports 4RX on all bands, the SNR for RS in set q0 during T3, T4, and T5 is -15dB-TT = -15.4dB (including test tolerances) q1 SSB during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.1dB q1 SSB during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.1dB q1 SSB during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10.1dB |
| 14.4.2.2 NR SA FR1 Beam Failure Detection and Link Recovery Test for PCell configured with SSB-based BFD and LR in DRX mode for Satellite Access | Same as 14.4.2.1 | Same as 14.4.2.1 | Same as 14.4.2.1 |
| 14.4.2.3 NR SA FR1 Beam Failure Detection and Link Recovery Test for PCell configured with CSI-RS-based BFD and LR in non-DRX mode for Satellite Access | q0 CSI-RS SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 CSI-RS during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 CSI-RS during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 CSI-RS during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10dB | Offset during: T1: +0.4dB T2: +0.4dB T3: -0.4dB T4: -0.4dB T5: -0.4dB q1 CSI-RS during T1: 0dB -0.1dB q1 CSI-RS during T2: 0dB -0.1dB q1 CSI-RS during T3, T4 and T5: 0dB +0.1dB | SNR during: T1: +5.4dB T2: -2.6dB T3: -12.4dB T4: -12.4dB T5: -12.4dB For testing of a UE which supports 4RX on all bands, the SNR for RS in set q0 during T3, T4, and T5 is -15dB-TT = -15.4dB (including test tolerances) q1 CSI-RS during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.1dB q1 CSI-RS during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.1dB q1 CSI-RS during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10.1dB |
| 14.4.2.4 NR SA FR1 Beam Failure Detection and Link Recovery Test for PCell configured with CSI-RS-based BFD and LR in DRX mode for Satellite Access | Same as 14.4.2.3 | Same as 14.4.2.3 | Same as 14.4.2.3 |
| 14.5.1.1 SA FR1 event triggered reporting tests without gap under non-DRX for NR satellite access | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |
| 14.5.1.2 SA FR1 Event triggered reporting tests without gap under DRX for NR satellite access | Same as 14.5.1.1 | Same as 14.5.1.1 | Same as 14.5.1.1 |
| 14.5.1.3 SA FR1 event triggered reporting tests without gap under non-DRX with FDD PCell with SSB index reading for NR satellite access | Same as 14.5.1.1 | Same as 14.5.1.1 | Same as 14.5.1.1 |
| 14.5.1.4 SA FR1 event triggered reporting tests with single measurement gap under non-DRX for satellite access for NR satellite access | Same as 14.5.1.1 | Same as 14.5.1.1 | Same as 14.5.1.1 |
| 14.5.1.5 SA FR1 event triggered reporting tests with FNO concurrent gaps under DRX for satellite access for NR satellite access | Same as 14.5.1.1 | Same as 14.5.1.1 | Same as 14.5.1.1 |
| 14.5.1.6 SA FR1 event triggered reporting tests with PPO concurrent gaps under non-DRX with SSB index reading for satellite access for NR satellite access | Same as 14.5.1.1 | Same as 14.5.1.1 | Same as 14.5.1.1 |
| 14.5.2.1 NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with single gap for satellite access | During T1: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: -infinity During T2: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: +7.00dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: -infinity During T2: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: +7.00dB |
| 14.5.2.2 NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is used with single gap for satellite access | Same as 14.5.2.1 | Same as 14.5.2.1 | Same as 14.5.2.1 |
| 14.5.2.3 NR SA FR1-FR1 Event-triggered reporting with SSB time index detection when DRX is not used with single gap for satellite access | Same as 14.5.2.1 | Same as 14.5.2.1 | Same as 14.5.2.1 |
| 14.5.2.4 NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent fully non-overlapped (FNO) gaps for satellite access | During T1: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: -infinity Ês3 / Noc2: -infinity During T2: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: +7.00dB Ês3 / Noc2: +7.00dB | During T1: 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB | During T1: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: -infinity Ês3 / Noc2: -infinity During T2: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: +7.00dB Ês3 / Noc2: +7.00dB |
| 14.5.2.6 NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent partial overlapping (PPO) gaps for satellite access | Same as 14.5.2.4 | Same as 14.5.2.4 | Same as 14.5.2.4 |
| 14.6.3.1 NR SA FR1 SS-SINR Measurement Accuracy for Satellite Access | Test 1: Noc: -93dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +2.66dB Reported SINR values: ±3.5dB Test 2: Noc: -116dBm/15kHz Ês1 / Noc: -4dB Ês2 / Noc: -4dB Reported SINR values: ±3.5dB | Test 1: 0dB 0dB 0dB Via mapping Test 2: 0dB 0.5dB 0.5dB Via mapping | Test 1: Noc: -93dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +2.66dB SINR_31 to SINR_49 Test 2: Noc: -116dBm/15kHz Ês1 / Noc: -3.5dB Ês2 / Noc: -3.5dB SINR_28 to SINR_45 |
| The derivation of the SINR values takes into account the uncertainty in Cell 2 SS- SINR from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 0.5dB wider at each end. |  |  |  |
| 14.6.3.2 NR SA FR1-FR1 SS-SINR Measurement Accuracy for Satellite Access | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Absolute Reported SINR values: ±3dB Relative Reported SINR values: ±3.5dB Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB Absolute Reported SINR values: ±3dB Relative Reported SINR values: ±3.5dB Test 3: Noc1: -119.5dBm/15kHz Noc2: -119.5dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: -4dB Absolute Reported SINR values: ±3.5dB Relative Reported SINR values: ±4dB | Test 1: 0dB 0dB 0dB 0dB Via mapping Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Via mapping Test 3: 0dB 0dB 1.9dB 1.9dB Via mapping Via mapping | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB SINR_35 to SINR_51 SINR_x-10 to SINR_x+10 Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB SINR_79 to SINR_94 SINR_x-10 to SINR_x+10 Test 3: Noc1: -119.5dBm/15kHz Noc2: -119.5dBm/15kHz Ês1 / Noc1: -2.1dB Ês2 / Noc2: -2.1dB SINR_34 to SINR_51 SINR_x-11 to SINR_x+11 |
| The derivation of the SINR values takes into account the uncertainty in Cell 2 SS- SINR from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 1dB wider at each for Tests 1 and 2 and 0.5dB wider at each end for Test 3. |  |  |  |
| 14.6.4.1 NR SA FR1 SSB based L1-RSRP Measurement Accuracy for Satellite Access | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±3dB additionally for extreme conditions) Reported differential L1-RSRP values for normal conditions: -4.6dB to 0dB Reported differential L1-RSRP values for extreme conditions: -5.6dB to 0dB Test 2: Noc1: -117dBm/15kHz Ês1 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±4.5dB additionally for extreme conditions) Reported differential L1-RSRP values for normal conditions: -4.6dB to 0dB Reported differential L1-RSRP values for extreme conditions: -5.6dB to 0dB | Test 1: 0dB 0dB Via mapping Via mapping Via mapping Test 2: 0dB 0.8dB Via mapping Via mapping Via mapping | Test 1: Noc1: -94.65dBm/15kHz Ês1 / Noc1: 10dB RSRP_62 to RSRP_82 DIFFRSRP_0 to DIFFRSRP_2 DIFFRSRP_0 to DIFFRSRP_2 Test 2: Noc1: -117dBm/15kHz Ês1 / Noc1: -2.2dB RSRP_31 to RSRP_44 DIFFRSRP_0 to DIFFRSRP_2 DIFFRSRP_0 to DIFFRSRP_2 |
| 14.6.4.2 NR SA FR1 CSI-RS based L1-RSRP Measurement Accuracy for Satellite Access | Same as 14.6.4.1 | Same as 14.6.4.1 | Same as 14.6.4.1 |

<< End of Changes >>