3GPP TSG-RAN5 Meeting #106	R5-251213r2

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 1142 | rev | - | Current version: | 18.5.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of FR2 RF testability issue list to specification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Trading |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per previous RAN5 agreement the list of known FR2 RF testability issues not related to MU has to be included in this specification |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a summary list of FR2 RF tests with testability issues not related to MU for reference to the industry |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing reference to known FR2 RF tests with testability issues not related to MU |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Follow up to AP#105.21 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1 adds the known testability issues from additional FR2 RF tests r2 adds an Editor’s Note to indicate the list will continue to be updated, removes an TC from the impacted list as it is related to MU and corrects a sequence. |  |  |  |  |  |  |  |  |

<Unchanged sections skipped>

## A.5.2	OCNG Patterns for TDD

### A.5.2.1	OCNG TDD pattern 1: Generic OCNG TDD Pattern for all unused REs

Table A.5.2.1-1: OP.1 TDD: Generic OCNG TDD Pattern for all unused REs

| OCNG Distribution OCNG Parameters | Control Region (Core Set) | Data Region |
| --- | --- | --- |
| Resources allocated | All unused REs (Note 1) | All unused REs (Note 2) |
| Structure | PDCCH | PDSCH |
| Content | Uncorrelated pseudo random QPSK modulated data | Uncorrelated pseudo random QPSK modulated data |
| Transmission scheme for multiple antennas ports transmission | Single Tx port transmission | Spatial multiplexing using any precoding matrix with dimensions same as the precoding matrix for PDSCH |
| Subcarrier Spacing | Same as for RMC PDCCH in the active BWP | Same as for RMC PDSCH in the active BWP |
| Power Level | Same as for RMC PDCCH | Same as for RMC PDSCH |
| Note 1: All unused REs in the active CORESETS appointed by the search spaces in use. Note 2: Unused available REs refer to REs in PRBs not allocated for any physical channels, CORESETs, synchronization signals or reference signals in channel bandwidth. |  |  |

# A.6	Summary of FR2 RF tests with testability issues not related to Measurement Uncertainty (MU)

Editor’s note: The list of FR2 RF testability issues not related to MU and listed in the table below is incomplete and and ongoing updates.

Table A.6-1: Summary of FR2 RF test cases and testability issues

| Clause | Requirement | FR2 RF Testability issue not related to MU |
| --- | --- | --- |
| 6.2.2 | UE maximum output power reduction | How to deal with power classes reusing PC3 MPR requirements, especially those defined from Release 17 and forward, and then the relationship with 6.2.2_1 test is FFS. |
| 6.2D.1.1 | UE maximum output power - EIRP and TRP for UL MIMO | No test points are defined for 2-layer UL MIMO since there is no configuration satisfying MPR=0dB requirements in RAN4. |
| 6.3.2 | Transmit OFF Power | Test Procedure aspects for UE indicating ul-GapFR2-r17 is FFS |
| 6.3.4.2 | Absolute Power Tolerance | The reduction of the impact of DL MU by choosing alpha < 1 is FFS |
| 6.3.3.1 | PRACH Time Mask | Further investigation is essential that how does beamforming affect the initial access procedure |
| 6.3D.3.1 | General ON/OFF time mask for UL MIMO | Testability of OFF power needs further study OTA test procedure for UL-MIMO is still under investigation |
| 6.4.2.3 | In-band emissions | Testing of the general in-band emission requirement and if yes at which UE Tx power level and with which relaxation applied to the requirement is FFS. |
| 6.4A.2.2.1 | Carrier leakage for CA (2UL CA) | This test is incomplete due to lack of RRC framework for LO position retrieval |
| 6.4A.2.2.1 | Carrier leakage for CA (2UL CA) |  |
| 6.4A.2.2.2 | Carrier leakage for CA (3UL CA) |  |
| 6.4A.2.2.3 | Carrier leakage for CA (4UL CA) |  |
| 6.4A.2.2.4 | Carrier leakage for CA (5UL CA) |  |
| 6.4A.2.2.5 | Carrier leakage for CA (6UL CA) |  |
| 6.4A.2.2.6 | Carrier leakage for CA (7UL CA) |  |
| 6.4A.2.2.7 | Carrier leakage for CA (8UL CA) |  |
| 6.4A.2.3.1 | In-band emissions for CA (2UL CA) | Testing of the general in-band emission requirement and if yes at which UE Tx power level and with which relaxation applied to the requirement is FFS. |
| 6.4A.2.3.2 | In-band emissions for CA (3UL CA) | Testing of the general in-band emission requirement and if yes at which UE Tx power level and with which relaxation applied to the requirement is FFS. |
| 6.4A.2.3.3 | In-band emissions for CA (4UL CA) | Testing of the general in-band emission requirement and if yes at which UE Tx power level and with which relaxation applied to the requirement is FFS. |
| 6.4A.2.3.4 | In-band emissions for CA (5UL CA) | Testing of the general in-band emission requirement and if yes at which UE Tx power level and with which relaxation applied to the requirement is FFS. |
| 6.4A.2.3.5 | In-band emissions for CA (6UL CA) | Testing of the general in-band emission requirement and if yes at which UE Tx power level and with which relaxation applied to the requirement is FFS. |
| 6.4A.2.3.6 | In-band emissions for CA (7UL CA) | Testing of the general in-band emission requirement and if yes at which UE Tx power level and with which relaxation applied to the requirement is FFS. |
| 6.4A.2.3.7 | In-band emissions for CA (8UL CA) | Testing of the general in-band emission requirement and if yes at which UE Tx power level and with which relaxation applied to the requirement is FFS. |
| 6.4.2.5 | EVM spectral flatness for pi/2 BPSK modulation | Whether and, if yes, how to test the requirement on shaping filter is FFS. |

<End of changes>