3GPP TSG-RAN5 Meeting #106	R5-251224r1

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.903 | CR | 0977 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of FR2 RF testability issue list to specification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Trading |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per previous RAN5 agreement the list of known FR2 RF testability issues related to MU has to be included in this specification |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a summary list of FR2 RF tests with testability issues related to MU for reference to the industry |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing reference to known FR2 RF tests with testability issues related to MU |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Follow up to AP#105.21. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Reissued from R5-251214 because of wrong spec. r1 adds the known testability issues to table from additional FR2 RF tests |  |  |  |  |  |  |  |  |

<Unchanged sections skipped>

Table B.25.2-16: Uncertainty assessment for TRP measurement (f= 66 GHz to 87 GHz, Quiet Zone size ≤ 30 cm) for PC1 and PC5 UEs

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |
| 1 | Positioning misalignment | 0.02 | Normal | 2.00 | 0.01 |
| 2 | Measure distance uncertainty | 0.00 | Rectangular | 1.73 | 0.00 |
| 3 | Quality of Quiet Zone (NOTE 4) | 0.6 | Actual | 1.00 | 0.6 |
| 4 | Mismatch | 2.30 | Actual | 1.00 | 2.30 |
| 5 | Standing wave between the DUT and measurement antenna | 0.00 | U-shaped | 1.41 | 0.00 |
| 6 | Uncertainty of the RF power measurement equipment | 4.0 | Normal | 2.00 | 2.0 |
| 7 | Phase curvature | 0.00 | U-shaped | 1.41 | 0.00 |
| 8 | Amplifier uncertainties | 3.0 | Normal | 2.00 | 1.50 |
| 9 | Random uncertainty | 0.5 | Normal | 2.00 | 0.25 |
| 10 | Influence of the XPD | 0.09 | U-shaped | 1.41 | 0.064 |
| 11 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | 0.00 |
| 12 | RF leakage (from measurement antenna to the receiver/transmitter) | 0.00 | Actual | 1.00 | 0.00 |
| 13 | Influence of TRP measurement grid (NOTE 1) | 0.25 | Actual | 1 | 0.32 |
| 14 | Influence of beam peak search grid (NOTE 2) | N/A | Actual | 1 | N/A |
| 15 | Multiple measurement antenna uncertainty (NOTE 5) | 0.15 | Actual | 1 | 0.15 |
| 16 | DUT repositioning | 0.00 | Rectangular | 1.73 | 0.00 |
| 17 | Misalignment of DUT due to change of DUT orientation | 0.10 | Actual | 1 | 0.10 |
| Stage 1: Calibration measurement |  |  |  |  |  |
| 18 | Mismatch | 0.00 | U-shaped | 1.41 | 0.00 |
| 19 | Amplifier Uncertainties | 0.00 | Normal | 2.00 | 0.00 |
| 20 | Misalignment of positioning System | 0.00 | Normal | 2.00 | 0.00 |
| 21 | Uncertainty of the Network Analyzer | 1.7 | Normal | 2.00 | 0.85 |
| 22 | Uncertainty of the absolute gain of the calibration antenna | 1.70 | Normal | 2.00 | 0.85 |
| 23 | Positioning and pointing misalignment between the reference antenna and the measurement antenna | 0.05 | Rectangular | 1.73 | 0.03 |
| 24 | Phase centre offset of calibration antenna | 0.00 | Rectangular | 1.73 | 0.00 |
| 25 | Quality of quiet zone for calibration process (NOTE 4) | 0.60 | Actual | 1.00 | 0.60 |
| 26 | Standing wave between reference calibration antenna and measurement antenna | 0.00 | U-shaped | 1.41 | 0.00 |
| 27 | Influence of the calibration antenna feed cable | 0.28 | Normal | 2.00 | 0.14 |
| 28 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | 0.00 |
|  | Expanded uncertainty (1.96σ - confidence interval of 95 %) |  |  |  | Value |
|  | TRP Expanded uncertainty ( 66 GHz < f <= 80 GHz) [dB] (a) |  |  |  | 7.31 |
|  | Systematic uncertainties (NOTE 3) |  |  |  | Value |
| 29 | Systematic error due to TRP calculation/quadrature (NOTE 1) (b) |  |  |  | 0.00 |
| 30 | Influence of noise (66 GHz < f <= 80 GHz) (c1) |  |  |  | 0.64 |
| 30 | Influence of noise (80 GHz < f <= 87 GHz) (c2) (PC5) |  |  |  | 1.00 |
| 31 | Systematic error related to beam peak search (NOTE 2) |  |  |  | N/A |
| Total measurement uncertainty |  |  |  |  | Value |
| Total measurement uncertainty (a)+(b)+(c1) [dB] (66 GHz < f <= 80 GHz) |  |  |  |  | 7.95 |
| Total measurement uncertainty (a)+(b)+(c2) [dB] (80 GHz < f <= 87 GHz) (PC5) |  |  |  |  | 8.31 |
| NOTE 1: This contributor shall only be considered for TRP measurements. NOTE 2: This contributor shall only be considered for EIRP measurements. NOTE 3: In order to obtain the total measurement uncertainty, systematic uncertainties have to be added to the expanded root sum square of the standard deviations of the Stage 1 and Stage 2 contributors. NOTE 4: Value based on procedure defined in clause D.2 of TR 38.810 for Quiet Zone size of less or equal to 30 cm. NOTE 5: Applies to the system which has a structure of mechanical feed antenna positioning. |  |  |  |  |  |

# B.26	Summary of FR2 RF test cases with testability issues related to MU

Table B.26-1: Summary of FR2 RF test cases with known testability issues

| Clause | Requirement | Testability issue | Notes |
| --- | --- | --- | --- |
| 6.3.1 | Minimum output power | Low UL power |  |
| 6.3.2 | Transmit OFF power | Low UL power |  |
| 6.3.4.3 | Relative Power Tolerance | Starting power at ramp up/ramp down/alternating sub-test is TBD (6.3.4.3 MU dependent) |  |
| 6.3A.3.1.1 | General ON/OFF time mask for CA (2UL CA) | Measurement Uncertainties and Test Tolerances and Test limit analysis for intra-band contiguous CA supporting aggregated BW > 400MHz is FFS. |  |
| 6.3A.3.1.2 | General ON/OFF time mask for CA (3UL CA) |  |  |
| 6.3A.3.1.3 | General ON/OFF time mask for CA (4UL CA) |  |  |
| 6.3A.3.1.4 | General ON/OFF time mask for CA (5UL CA) |  |  |
| 6.3A.3.1.5 | General ON/OFF time mask for CA (6UL CA) |  |  |
| 6.3A.3.1.6 | General ON/OFF time mask for CA (7UL CA) |  |  |
| 6.3A.3.1.7 | General ON/OFF time mask for CA (8UL CA) |  |  |
| 6.3D.2 | Transmit OFF power for UL MIMO | The testability of this test case is pending further analysis on relaxation of the requirement for other than Band n257. |  |
| 6.4A.1.1 | Frequency error for CA (2UL CA) | Measurement Uncertainties and Test Tolerances for intra-band contiguous CA supporting aggregated BW > 400MHz is TBD. |  |
| 6.4A.1.2 | Frequency error for CA (3UL CA) |  |  |
| 6.4A.1.3 | Frequency error for CA (4UL CA) |  |  |
| 6.4A.1.4 | Frequency error for CA (5UL CA) |  |  |
| 6.4A.1.5 | Frequency error for CA (6UL CA) |  |  |
| 6.4A.1.6 | Frequency error for CA (7UL CA) |  |  |
| 6.4A.1.7 | Frequency error for CA (8UL CA) |  |  |
| 6.5.2.3 | Adjacent channel leakage ratio | Low UL power | For some MPR values as defined in TS 38.521-2 Table 6.5.2.3.5 |
| 6.5.3.2 | Additional spurious emissions | Low UL power |  |
| 7.4 | Maximum input power | Hidh DL power |  |
| 7.5 | Adjacent channel selectivity (case 1) | High DL power |  |
| 7.5 | Adjacent channel selectivity (case 2) | High DL power |  |
| 7.6.2 | In-band blocking | High DL power |  |
| 7.9 | Receiver spurious emissions | Low UL power |  |

<End of changes>