3GPP TSG-RAN5 Meeting #106	R5-251227r3

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4918 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new LTM Test Case Correct handling of MAC control information / LTM Cell Switch Command / UL transmission without random access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm France |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introducation of new LTM Test Case 7.1.1.3.28 Correct handling of MAC control information / LTM Cell Switch Command / UL transmission without random access as per NR_Mob_enh2-UEConTest |  |  |  |  |  |  |  |  |
| Summary of change: |  | New test case is added as per NR_Mob_enh2-UEConTest WP |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_Mob_enh2-UEConTest WP will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.28 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-2 CR 0548 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-251227r1 Added NR-DC System simulator in preconditions Changed the NR Cell2 to NR Cell3 Added RRC Core Spec Requirements Removed EN-DC from the test as it is NA to LTM R5-251227r2 Added NR Cell 28 instead of NR Cell 3 as PSCell to cover both FR1 and FR2 Moved the TP1 verdict to Step 10 Replaced Intra-Frequency with Inter-Frequency Added Conducted and OTA power levels FR1-FR1 NR-DC and FR1-FR2 NR-DC Fixed Table numbers |  |  |  |  |  |  |  |  |

[Start of modified section]

##### 7.1.1.3.28	Correct handling of MAC control information / LTM Cell Switch Command / UL transmission without random access

Editors Note: Steps 9 and 10 needs to be investigated further in the callflow, /NR-DC to be added to title

7.1.1.3.28.1	Test Purpose (TP)

(1)

with { UE in RRC_Connected state with candidate configuration for RACH-less LTM }

ensure that {

when { UE receives Candidate Cell TCI States Activation/Deactivation MAC CE }

then { UE takes the new TCI state into account at the next RACH-less LTM cell switch }

}

(2)

with { UE in RRC_Connected state with candidate configuration for LTM }

ensure that {

when { UE receives LTM Cell Switch Command MAC CE for a MAC entity associated with SCG }

then { UE skips the Random Access procedure for the LTM cell switch }

}

7.1.1.3.28.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, clauses 5.1.1 and 5.18.36. Unless otherwise stated these are Rel-18 requirements.

[TS 38.321, clause 5.1.1]

The Random Access procedure described in this clause is initiated by a PDCCH order, by the MAC entity itself, or by RRC for the events in accordance with TS 38.300 [2]. There is only one Random Access procedure ongoing at any point in time in a MAC entity. The Random Access procedure on an SCell or an LTM candidate cell shall only be initiated by a PDCCH order with ra-PreambleIndex different from 0b000000.

NOTE 1:	If a new Random Access procedure is triggered while another is already ongoing in the MAC entity, it is up to UE implementation whether to continue with the ongoing procedure or start with the new procedure (e.g. for SI request).

NOTE 2:	If there was an ongoing Random Access procedure that is triggered by a PDCCH order while the UE receives another PDCCH order indicating the same Random Access Preamble, PRACH mask index and uplink carrier, the Random Access procedure is considered as the same Random Access procedure as the ongoing one and not initialized again.

When a Random Access procedure is initiated, UE selects a set of Random Access resources as specified in clause 5.1.1b and initialises the following parameters for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources:

-	prach-ConfigurationIndex: the available set of PRACH occasions for the transmission of the Random Access Preamble for Msg1. These are also applicable to the MSGA PRACH if the PRACH occasions are shared between 2-step and 4-step RA types;

-	prach-ConfigurationPeriodScaling-IAB: the scaling factor defined in TS 38.211 [8] and applicable to IAB-MTs, extending the periodicity of the PRACH occasions baseline configuration indicated by prach-ConfigurationIndex;

-	prach-ConfigurationFrameOffset-IAB: the frame offset defined in TS 38.211 [8] and applicable to IAB-MTs, altering the ROs frame defined in the baseline configuration indicated by prach-ConfigurationIndex;

-	prach-ConfigurationSOffset-IAB: the subframe/slot offset defined in TS 38.211 [8] and applicable to IAB-MTs, altering the ROs subframe or slot defined in the baseline configuration indicated by prach-ConfigurationIndex;

-	msgA-PRACH-ConfigurationIndex: the available set of PRACH occasions for the transmission of the Random Access Preamble for MSGA in 2-step RA type;

-	preambleReceivedTargetPower: initial Random Access Preamble power for 4-step RA type;

-	msgA-PreambleReceivedTargetPower: initial Random Access Preamble power for 2-step RA type;

-	rsrp-ThresholdSSB: an RSRP threshold for the selection of the SSB for 4-step RA type. If the Random Access procedure is initiated for beam failure recovery, rsrp-ThresholdSSB used for the selection of the SSB within candidateBeamRSList refers to rsrp-ThresholdSSB in BeamFailureRecoveryConfig IE;

-	rsrp-ThresholdCSI-RS: an RSRP threshold for the selection of CSI-RS for 4-step RA type. If the Random Access procedure is initiated for beam failure recovery, rsrp-ThresholdCSI-RS is equal to rsrp-ThresholdSSB in BeamFailureRecoveryConfig IE;

-	msgA-RSRP-ThresholdSSB: an RSRP threshold for the selection of the SSB for 2-step RA type;

-	rsrp-ThresholdSSB-SUL: an RSRP threshold for the selection between the NUL carrier and the SUL carrier;

-	msgA-RSRP-Threshold: an RSRP threshold for selection between 2-step RA type and 4-step RA type when both 2-step and 4-step RA type Random Access Resources are configured in the UL BWP;

-	rsrp-ThresholdMsg1-RepetitionNum2: an RSRP threshold for Msg1 repetition with repetition number 2 (see clause 5.1.1b);

-	rsrp-ThresholdMsg1-RepetitionNum4: an RSRP threshold for Msg1 repetition with repetition number 4 (see clause 5.1.1b);

-	rsrp-ThresholdMsg1-RepetitionNum8: an RSRP threshold for Msg1 repetition with repetition number 8 (see clause 5.1.1b);

-	rsrp-ThresholdMsg3: an RSRP threshold for Msg3 repetition (see clause 5.1.1b);

-	FeatureCombination: feature or a combination of features associated with a set of Random Access resources;

-	featurePriorities: priorities for features, such as (e)RedCap, Slicing, etc. (see clause 5.1.1d);

-	msgA-TransMax: The maximum number of MSGA transmissions when both 4-step and 2-step RA type Random Access Resources are configured;

-	candidateBeamRSList: a list of reference signals (CSI-RS and/or SSB) identifying the candidate beams for recovery and the associated Random Access parameters;

-	recoverySearchSpaceId: the search space identity for monitoring the response of the beam failure recovery request;

-	powerRampingStep: the power-ramping factor;

-	msgA-PreamblePowerRampingStep: the power ramping factor for MSGA preamble;

-	powerRampingStepHighPriority: the power-ramping factor in case of prioritized Random Access procedure;

-	scalingFactorBI: a scaling factor for prioritized Random Access procedure;

-	ra-PreambleIndex: Random Access Preamble;

-	ra-ssb-OccasionMaskIndex: defines PRACH occasion(s) associated with an SSB in which the MAC entity may transmit a Random Access Preamble (see clause 7.4);

-	msgA-SSB-SharedRO-MaskIndex: Indicates the subset of 4-step RA type PRACH occasions shared with 2-step RA type PRACH occasions for each SSB. If 2-step RA type PRACH occasions are shared with 4-step RA type PRACH occasions and msgA-SSB-SharedRO-MaskIndex is not configured, then all 4-step RA type PRACH occasions are available for 2-step RA type (see clause 7.4);

-	ssb-SharedRO-MaskIndex: defines PRACH occasions, on which preambles are allocated for a feature or a combination of features, associated with an SSB in which the MAC entity may transmit a Random Access Preamble (see clause 7.4);

-	ra-OccasionList: defines PRACH occasion(s) associated with a CSI-RS in which the MAC entity may transmit a Random Access Preamble;

-	ra-PreambleStartIndex: the starting index of Random Access Preamble(s) for on-demand SI request;

-	startPreambleForThisPartition: the first preamble associated with the set of Random Access Resources applicable to the Random Access procedure;

-	preambleTransMax: the maximum number of Random Access Preamble transmission;

-	preambleTransMax-Msg1-Repetition: the maximum number of Random Access Preamble transmissions with a given Msg1 repetition number before switching to Msg1 repetition with the next available higher Msg1 repetition number;

-	ssb-perRACH-OccasionAndCB-PreamblesPerSSB: defines the number of SSBs mapped to each PRACH occasion for 4-step RA type and the number of contention-based Random Access Preambles mapped to each SSB;

-	msgA-CB-PreamblesPerSSB-PerSharedRO: defines the number of contention-based Random Access Preambles for 2-step RA type mapped to each SSB when the PRACH occasions are shared between 2-step and 4-step RA types;

-	msgA-SSB-PerRACH-OccasionAndCB-PreamblesPerSSB: defines the number of SSBs mapped to each PRACH occasion for 2-step RA type and the number of contention-based Random Access Preambles mapped to each SSB;

-	numberOfPreamblesPerSSB-ForThisPartition: defines the number of consecutive preambles for a feature or a combination of features mapped to each SSB;

-	msgA-PUSCH-ResourceGroupA: defines MSGA PUSCH resources that the UE shall use when performing MSGA transmission using Random Access Preambles group A;

-	msgA-PUSCH-ResourceGroupB: defines MSGA PUSCH resources that the UE shall use when performing MSGA transmission using Random Access Preambles group B;

-	msgA-PUSCH-Resource-Index: identifies the index of the PUSCH resource used for MSGA in case of contention-free Random Access with 2-step RA type;

-	if groupBconfigured is configured, then Random Access Preambles group B is configured for 4-step RA type.

-	Amongst the contention-based Random Access Preambles associated with an SSB (as defined in TS 38.213 [6]), the first numberOfRA-PreamblesGroupA included in groupBconfigured Random Access Preambles belong to Random Access Preambles group A. The remaining Random Access Preambles associated with the SSB belong to Random Access Preambles group B (if configured).

-	if groupB-ConfiguredTwoStepRA is configured, then Random Access Preambles group B is configured for 2-step RA type.

-	Amongst the contention-based Random Access Preambles for 2-step RA type associated with an SSB (as defined in TS 38.213 [6]), the first numberOfRA-PreamblesGroupA included in GroupB-ConfiguredTwoStepRA Random Access Preambles belong to Random Access Preambles group A. The remaining Random Access Preambles associated with the SSB belong to Random Access Preambles group B (if configured).

NOTE 3:	If Random Access Preambles group B is supported by the cell Random Access Preambles group B is included for each SSB.

-	if Random Access Preambles group B is configured for 4-step RA type:

-	ra-Msg3SizeGroupA: the threshold to determine the groups of Random Access Preambles for 4-step RA type;

-	msg3-DeltaPreamble: ∆PREAMBLE_Msg3 in TS 38.213 [6];

-	messagePowerOffsetGroupB: the power offset for preamble selection included in groupBconfigured;

-	numberOfRA-PreamblesGroupA: defines the number of Random Access Preambles in Random Access Preamble group A for each SSB included in groupBconfigured.

-	if Random Access Preambles group B is configured for 2-step RA type:

-	msgA-DeltaPreamble: ∆MsgA_PUSCH in TS 38.213 [6];

-	messagePowerOffsetGroupB: the power offset for preamble selection included in GroupB-ConfiguredTwoStepRA;

-	numberOfRA-PreamblesGroupA: defines the number of Random Access Preambles in Random Access Preamble group A for each SSB included in GroupB-ConfiguredTwoStepRA;

-	ra-MsgA-SizeGroupA: the threshold to determine the groups of Random Access Preambles for 2-step RA type.

-	the set of Random Access Preambles and/or PRACH occasions for SI request, if any;

-	the set of Random Access Preambles and/or PRACH occasions for beam failure recovery request, if any;

-	the set of Random Access Preambles and/or PRACH occasions for reconfiguration with sync, if any;

-	ra-ResponseWindow: the time window to monitor RA response(s) (SpCell only);

-	ra-ContentionResolutionTimer: the Contention Resolution Timer (SpCell only);

-	msgB-ResponseWindow: the time window to monitor RA response(s) for 2-step RA type (SpCell only).

In addition, the following information for related Serving Cell is assumed to be available for UEs:

-	if Random Access Preambles group B is configured:

-	if the Serving Cell for the Random Access procedure is configured with supplementary uplink as specified in TS 38.331 [5], and SUL carrier is selected for performing Random Access Procedure:

-	PCMAX,f,c of the SUL carrier as specified in TS 38.101-1 [14], TS 38.101-2 [15], and TS 38.101-3 [16].

-	else:

-	PCMAX,f,c of the NUL carrier as specified in TS 38.101-1 [14], TS 38.101-2 [15], and TS 38.101-3 [16].

The following UE variables are used for the Random Access procedure:

-	PREAMBLE_INDEX;

-	PREAMBLE_TRANSMISSION_COUNTER;

-	PREAMBLE_POWER_RAMPING_COUNTER;

-	PREAMBLE_POWER_RAMPING_STEP;

-	PREAMBLE_RECEIVED_TARGET_POWER;

-	PREAMBLE_BACKOFF;

-	PCMAX;

-	SCALING_FACTOR_BI;

-	TEMPORARY_C-RNTI;

-	RA_TYPE;

-	POWER_OFFSET_2STEP_RA;

-	MSGA_PREAMBLE_POWER_RAMPING_STEP.

When the Random Access procedure is initiated on a Serving Cell or for an LTM candidate cell, the MAC entity shall:

1>	flush the Msg3 buffer;

1>	flush the MSGA buffer;

1>	set the PREAMBLE_TRANSMISSION_COUNTER to 1;

1>	if the Random Access procedure is initiated on a Serving Cell; or

1>	if the Random Access procedure is initiated by the PDCCH order for an LTM candidate cell and the PDCCH order indicates preamble initial transmission; or

1>	if the Random Access procedure is initiated by the PDCCH order for an LTM candidate cell, which is different from the cell to which the UE performed the last Random Access Preamble transmission, and the PDCCH order indicates preamble re-transmission:

2>	set the PREAMBLE_POWER_RAMPING_COUNTER to 1;

1>	set the PREAMBLE_BACKOFF to 0 ms;

1>	set POWER_OFFSET_2STEP_RA to 0 dB;

1>	if the carrier to use for the Random Access procedure is explicitly signalled:

2>	select the signalled carrier for performing Random Access procedure;

2>	set the PCMAX to PCMAX,f,c of the signalled carrier.

1>	else if the carrier to use for the Random Access procedure is not explicitly signalled; and

1>	if the Serving Cell for the Random Access procedure is configured with supplementary uplink as specified in TS 38.331 [5]; and

1>	if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL:

2>	select the SUL carrier for performing Random Access procedure;

2>	set the PCMAX to PCMAX,f,c of the SUL carrier.

1>	else:

2>	select the NUL carrier for performing Random Access procedure;

2>	set the PCMAX to PCMAX,f,c of the NUL carrier.

NOTE 4:	Void.

1>	perform the BWP operation as specified in clause 5.15, except when the Random Access procedure is initiated by the PDCCH order for an LTM candidate cell;

1>	select the set of Random Access resources applicable to the current Random Access procedure according to clause 5.1.1b;

1>	if the Random Access procedure is initiated by PDCCH order and if the ra-PreambleIndex explicitly provided by PDCCH is not 0b000000; or

1>	if the Random Access procedure was initiated for SI request (as specified in TS 38.331 [5]) and the Random Access Resources for SI request have been explicitly provided by RRC; or

1>	if the Random Access procedure was initiated for SpCell beam failure recovery (as specified in clause 5.17) and if the contention-free Random Access Resources for beam failure recovery request for 4-step RA type have been explicitly provided by RRC for the BWP selected for Random Access procedure; or

1>	if the Random Access procedure was initiated for reconfiguration with sync not initiated for recovery using an LTM candidate configuration as specified in TS 38.331 [5] clause 5.3.7.3 and if the contention-free Random Access Resources for 4-step RA type have been explicitly provided in rach-ConfigDedicated for the BWP selected for Random Access procedure; or

1>	if the contention-free Random Access Resources have been explicitly provided in the LTM Cell Switch Command MAC CE:

2>	set the RA_TYPE to 4-stepRA.

1>	else if the BWP selected for Random Access procedure is configured with both 2-step and 4-step RA type Random Access Resources within the selected set of Random Access resources (as specified in clause 5.1.1b) and the RSRP of the downlink pathloss reference is above msgA-RSRP-Threshold; or

1>	if the BWP selected for Random Access procedure is only configured with 2-step RA type Random Access resources within the selected set of Random Access resources according to clause 5.1.1b; or

1>	if the Random Access procedure was initiated for reconfiguration with sync not initiated for recovery using an LTM candidate configuration as specified in TS 38.331 [5] clause 5.3.7.3 and if the contention-free Random Access Resources for 2-step RA type have been explicitly provided in rach-ConfigDedicated for the BWP selected for Random Access procedure:

2>	set the RA_TYPE to 2-stepRA.

1>	else:

2>	set the RA_TYPE to 4-stepRA.

1>	perform initialization of variables specific to Random Access type as specified in clause 5.1.1a;

1>	if RA_TYPE is set to 2-stepRA:

2>	perform the Random Access Resource selection procedure for 2-step RA type (see clause 5.1.2a).

1>	else:

2>	perform the Random Access Resource selection procedure (see clause 5.1.2).

[TS 38.321, clause 5.18.35]

The network may instruct the UE to perform LTM cell switch procedure by sending the LTM Cell Switch Command MAC CE described in clause 6.1.3.75.

The MAC entity shall:

1>	if the MAC entity receives an LTM Cell Switch Command MAC CE on a Serving Cell:

2>	indicate to upper layers that the LTM cell switch procedure is triggered and the Target Configuration ID included in the LTM Cell Switch Command MAC CE;

2>	if the MAC reset operation as specified in clause 5.12 is performed, as requested by upper layers:

3>	if Timing Advance Command value (hexa-decimal) is not set as FFF:

4>	process the received Timing Advance Command (see clause 5.2);

4>	consider the RACH-less LTM cell switch to be ongoing;

4>	if the MAC entity is associated with the SCG:

5>	indicate to upper layers to skip the Random Access procedure for this LTM cell switch.

3>	else if the UE is configured with UE-based Timing Advance measurement as specified in TS 38.331 [5] and the UE has successfully measured the Timing Advance for the SpCell of the indicated LTM target configuration:

4>	process the measured Timing Advance (see clause 5.2);

4>	consider the RACH-less LTM cell switch to be ongoing.

4>	if the MAC entity is associated with the SCG:

5>	indicate to upper layers to skip the Random Access procedure for this LTM cell switch.

3>	indicate to lower layers the information regarding the TCI state information included in the LTM Cell Switch Command MAC CE.

[TS 38.321, clause 6.1.3.75]

The LTM Cell Switch Command MAC CE is identified by MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a variable size with following fields (Figure 6.1.3.75-1):

-	R: Reserved bit, set to 0;

-	Target Configuration ID: This field indicates the index of candidate target configuration to apply for LTM cell switch, corresponding to ltm-CandidateId minus 1 as specified in TS 38.331 [5]. The length of the field is 3 bits;

-	Timing Advance Command: This field indicates whether the TA is valid for the LTM target cell (i.e. the SpCell corresponding to the target configuration indicated by Target Configuration ID field). If the value of this field is set to FFF, this field indicates that no valid timing adjustment is available for the PTAG of the LTM target cell; otherwise, this field indicates the index value TA used to control the amount of timing adjustment that the MAC entity has to apply in TS 38.213 [6], and that the UE can skip the Random Access procedure for this LTM cell switch. If tag-Id-ptr is configured for the TCI state indicated by the UL TCI state ID field, if present, or by the TCI state ID field otherwise, in the LTM target cell and tag-Id-ptr is set to value n1, this field indicates the TA for the TAG indicated by the tag2-Id of the LTM target cell; otherwise, this field indicates the TA for the TAG indicated by the tag-id of the LTM target cell. The length of the field is 12 bits;

-	TCI state ID: This field indicates and activates the TCI state for the LTM target cell (i.e. the SpCell of the target configuration indicated by the Target Configuration ID field). The TCI state is identified by TCI-StateId in ltm-DL-OrJointTCI-StateToAddModList as specified in TS 38.331 [5]. If the value of unifiedTCI-StateType in the ltm-TCI-Info of the configuration indicated by Target Configuration ID field is joint, this field is for joint TCI state, otherwise, this field is for downlink TCI state. The length of the field is 7 bits;

-	UL TCI state ID: This field indicates and activates the uplink TCI state for the LTM target cell (i.e. the SpCell of the target configuration indicated by the Target Configuration ID field). The UL TCI state is identified by TCI-UL-StateId in ltm-UL-TCI-StateToAddModList as specified in TS 38.331 [5]. The octet containing this field (i.e. this field and the two reserved bits in the same octet) is included if the value of unifiedTCI-StateType in the ltm-TCI-Info of the configuration indicated by Target Configuration ID field is separate. The length of the field is 6 bits;

-	C: This field indicates the presence of the contention-free Random Access Resources fields. If the value of this field is set to 1, the following fields are present: Random Access Preamble index field, S/U field, SS/PBCH index field, PRACH Mask index field, Repetition number field and the reserved bits in the same octet. If the value of this field is set to 0, these fields are absent.

-	S/U: This field indicates which UL carrier to transmit the PRACH of the contention-free Random Access Resources. If the value of this field is set to 1, SUL is used; otherwise, NUL is used. The length of the field is 1 bit;

-	Random Access Preamble index: This field indicates the Random Access Preamble index of the contention-free Random Access Resources. This field should not be set to 0b000000. The length of the field is 6 bits;

-	SS/PBCH index: This field indicates the SS/PBCH that shall be used to determine the RACH occasion for the PRACH transmission of the contention-free Random Access Resources. The length of the field is 6 bits;

-	PRACH Mask index: This field indicates the RACH occasion(s) associated with the SS/PBCH indicated by 'SS/PBCH index' for the PRACH transmission of the contention-free Random Access Resources. It indicates a subset of RACH occasion(s) from the rach-ConfigDedicated for the UL carrier (indicated by S/U field), (if provided, otherwise it indicates a subset of RACH occasion(s) from the rach-ConfigCommon for the UL carrier (indicated by S/U field) in the UL BWP configuration of firstActiveUplinkBWP-Id as specified in TS 38.331 [5]. When the repetition number field is not set to 0, the UE ignores this field. The length of the field is 4 bits;

-	Repetition number: This field indicates the Msg1 repetition number to be applied to the contention-free Random Access. If this field is set to 0, Msg1 repetition number does not apply. If this field is set to 1, the Msg1 repetition number is 2. If this field is set to 2, the Msg1 repetition number is 4. If this field is set to 3, the Msg1 repetition number is 8. The length of the field is 2 bits.

NOTE 1:	Void

Figure 6.1.3.75-1: LTM Cell Switch Command MAC CE

NOTE 2:	If UE receives the LTM Cell Switch Command MAC CE with a Target Configuration ID value not matching any configured ltm-CandidateId minus 1, as specified in TS 38.331 [5], the procedure of handling LTM Cell Switch Command MAC CE in clause 5.18.35 does not apply.

[TS 38.321, clause 5.18.36]

The network may activate and deactivate the TCI states of LTM candidate cell(s) configured in CandidateTCI-State and CandidateTCI-UL-State by sending the Candidate Cell TCI States Activation/Deactivation MAC CE described in clause 6.1.3.76. The network deactivates the TCI state(s) for one LTM candidate cell by not including the corresponding TCI state ID field(s) in the Candidate Cell TCI States Activation/Deactivation MAC CE. The configured candidate cell TCI states are initially deactivated upon (re-)configuration by upper layer and after reconfiguration with sync that is not triggered by LTM.

The MAC entity shall:

1>	if the MAC entity receives a Candidate Cell TCI States Activation/Deactivation MAC CE on a Serving Cell:

2>	indicate to lower layers the information regarding the Candidate Cell TCI States Activation/Deactivation MAC CE.

[TS 38.331, clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration, upon execution of the conditional reconfiguration (CHO, CPA, CPC, or subsequent CPAC), or upon execution of an LTM cell switch:

…

1>  if the RRCReconfiguration message includes the ltm-Config:

2> if the ltm-Config is set to setup:

3>  perform the LTM configuration procedure as specified in 5.3.5.18.1;

…

1>  set the content of the RRCReconfigurationComplete message as follows:

…

2> if this RRCReconfiguration message is applied due to an LTM cell switch execution procedure according to clause 5.3.5.18.6:

3>  include in the appliedLTM-CandidateId the LTM-CandidateId of the applied LTM candidate configuration;

…

1>  else if the RRCReconfiguration message was received via SRB1 within the nr-SCG within mrdc-SecondaryCellGroup (UE in NR-DC, mrdc-SecondaryCellGroup was received in RRCReconfiguration or RRCResume via SRB1):

2> if the RRCReconfiguration is applied due to a conditional reconfiguration execution for CPC or subsequent CPAC which is configured via conditionalReconfiguration contained in nr-SCG within mrdc-SecondaryCellGroup; or

2> if the RRCReconfiguration is applied due to an LTM cell switch execution:

3>  submit the RRCReconfigurationComplete message via SRB1 embedded in NR RRC message ULInformationTransferMRDC as specified in clause 5.7.2a.3.

…

1>  else if the RRCReconfiguration message was received via SRB3 (UE in NR-DC):

2> if the RRCReconfiguration message was received within DLInformationTransferMRDC:

3>  if the RRCReconfiguration message was received within the nr-SCG within mrdc-SecondaryCellGroup (NR SCG RRC Reconfiguration):

…

4>  submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration;

2> else:

…

3>  submit the RRCReconfigurationComplete message via SRB3 to lower layers for transmission using the new configuration;

1>  else (RRCReconfiguration was received via SRB1):

2> if the UE is in NR-DC and;

…

2> submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration;

…

7.1.1.3.28.3	Test description

7.1.1.3.28.3.1	Pre-test conditions

7.1.1.3.28.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that System information combination NR-2 and in addition NR Cell 10 is configured as NR PSCell.

System Simulator:

-	NR Cell 1 is the PCell, NR Cell 10 is the source PSCell and NR Cell 28 is Neighbour Cell of the PSCell.

7.1.1.3.28.3.2	Test procedure sequence

Table 7.1.1.3.28.3.2-1 and 7.1.1.3.28.3.2-2 illustrates the downlink power levels to be applied for NR Cell 1 and NR Cell 28 at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while the configuration marked "T1" and "T2", are applied at the point indicated in the Main behaviour description in Table 7.1.1.3.28.3.2-3.

Table 7.1.1.3.28.3.2-1: Time instances of cell power level and parameter changes for conducted test environment

|  | Parameter | Unit | NR Cell 1 | NR Cell 10 | NR Cell 28 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -82 | -85 | -91 | Power levels are such that entry condition for event A3 is not satisfied for the neighbour NR cell |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | -82 | -85 | -79 | Power levels are such that entry condition for event A3 is satisfied for inter-frequency neighbour NR cell |

Table 7.1.1.3.28.3.2-2: Time instances of cell power level and parameter changes for OTA test environment

|  | Parameter | Unit | NR Cell 1 | NR Cell 10 | NR Cell 28 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -82 | -91 | -100 | Power levels are such that entry condition for event A3 is not satisfied for the neighbour NR cell |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | -82 | -91 | -82 | Power levels are such that entry condition for event A3 is satisfied for inter-frequency neighbour NR cell |

Table 7.1.1.3.28.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message including MeasConfig to setup NR measurement and reporting for inter-frequency event A3 on NR Cell 1. | <-- | RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message. | --> | RRCReconfigurationComplete | - | - |
| 3 | SS re-adjusts the cell-specific reference signal level according to row "T1" in table 7.1.1.3.28.3.2-1/2. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message to report event A3 with the measured RSRP value for NR Cell 28 in NR Cell 1. | --> | MeasurementReport | - | - |
| 5 | The SS transmits an RRCReconfiguration message with LTM-Candidate-r18 for NR Cell 28 on NR Cell 1. | <-- | RRCReconfiguration | - | - |
| 6 | The UE transmits RRCReconfigurationComplete message. | --> | RRCReconfigurationComplete | - | - |
| 7 | The SS transmits the candidate cell TCI activation MAC CE with Candidate Cell ID set to 0 for NR Cell 28 on NR Cell 1. | <-- | MAC PDU (Candidate Cell TCI Activation/Deactivation MAC CE) |  |  |
| 8 | The SS transmits LTM cell switch command MAC CE with Timing Advance Command value set to 0 and SCG of Target Configuration ID value set to 0 on NR Cell1 to the UE. | <-- | MAC PDU (LTM Cell Switch Command MAC CE) | - | - |
| 9 | The SS transmits an UL Grant, allowing the UE to transmit the RRCReconfigurationComplete message on NR Cell 28. | <-- | UL grant (C-RNTI) | - | - |
| 10 | Check: Does the UE transmit RRCReconfigurationComplete message in NR Cell 28? | --> | RRCReconfigurationComplete | 1 | P |
| 11 | The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR Cell 28 | <-- | PDCCH (C-RNTI) | - | - |
| 12 | The SS transmits in the indicated downlink assignment a MAC PDU including a RLC PDU with poll bit not set on NR Cell 28. | <-- | MAC PDU | - | - |
| 13 | Check: Does the UE transmit an HARQ ACK on PUCCH? | --> | HARQ ACK | 2 | P |

7.1.1.1.3.28.3.3	Specific message contents

Table 7.1.1.3.28.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.3.28.3.2-3)

Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 with condition NR_MEAS

Table 7.1.1.3.28.3.3-2: MeasConfig (step 1, Table 7.1.1.3.28.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR | Table 7.1.1.3.28.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-EventA3 | Table 7.1.1.3.28.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.28.3.3-3: MeasObjectNR (Table 7.1.1.3.28.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 1 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |

Table 7.1.1.3.28.3.3-4: ReportConfigNR-EventA3 (Table 7.1.1.3.28.3.3-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-142 with condition EVENT_A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | 2 | 1 dB(2*0.5 dB) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportAmount | infinity |  |  |
| maxNrofRS-IndexesToReport | 1 |  |  |
| includeBeamMeasurements | true |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.28.3.3-5: MeasurementReport (step 4, Table 7.1.1.3.28.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-5A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| measurementReport SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServingMOList SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResultServMO { | 1 entry |  |  |
| MeasResultServMO[1] SEQUENCE { |  | entry 1 |  |
| servCellId | ServCellIndex of NR Cell 1 |  | Step 4 |
| measResultServingCell SEQUENCE { |  |  |  |
| physCellId | PCI of NR Cell 1 |  | Step 4 |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not present |  |  |
|  | Not checked |  | pc_ss_SINR_Meas |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListNR SEQUENCE (SIZE (1.. maxCellReport)) OF MeasResultNR { | 1 entry |  |  |
| MeasResultNR[1] SEQUENCE { |  | entry 1 |  |
| physCellId | Physical layer cell identity of NR Cell 28 |  | Step 4 |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | Not present |  |  |
| sinr | Not present |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell | Not present |  |  |
| } |  |  |  |
| rsIndexResults | Not present |  |  |
| } |  |  |  |
| cgi-Info | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.28.3.3-6: RRCReconfiguration (step 5, Table 7.1.1.3.28.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 with condition NR-DC and LTM |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ltm-Config-r18 CHOICE { |  |  |  |
| setup | LTM-Config-r18 | Table 7.1.1.3.28.3.3-7 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.28.3.3-7: LTM-Config-r18 (Table 7.1.1.3.28.3.3-6)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-67D |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-Config-r18 ::= SEQUENCE { |  |  |  |
| ltm-CandidateToAddModList-r18 SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-Candidate-r18 { | 1 entry |  |  |
| LTM-Candidate-r18[1] | LTM-Candidate-r18 | Table 7.1.1.3.28.3.3-8 |  |
| } |  |  |  |
| ltm-CSI-ResourceConfig-r18 | Not present |  |  |
| } |  |  |  |

Table 7.1.1.3.28.3.3-8: LTM-Candidate-r18 (Table 7.1.1.3.28.3.3-7)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-67C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-Candidate-r18 ::= SEQUENCE { |  |  |  |
| ltm-CandidateId-r18 | 1 |  |  |
| ltm-CandidatePCI-r18 | Physical layer cell identity of NR Cell 28 |  | Step 5 |
| ltm-CandidateConfig-r18 OCTET STRING (CONTAINING RRCReconfiguration) { |  |  |  |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig specified in TS 38.508-1 [4] Table 4.6.3-19 with condition NR-DC_SCG |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ltm-TCI-Info-r18 | LTM-TCI-Info specified in TS 38.508-1 [4] Table 4.6.3-67I with condition Joint |  |  |
| } |  |  |  |

[End of modified section]