3GPP TSG-RAN5 Meeting #106	R5-251249r1

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5437 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE RACS Test case 8.5.5.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu EMEA Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, RACS-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Total grant size at step 5 for 17 UL grants would be 8241 bytes but total 17 UL grant size required is 8260. BSR timer is 20ms and PHR timer is 500ms. MAX PDCP SDU size 8188 bytes MAC header for last element is 1 byte RLC heder is 2 bytes PDCP header is 5 bytes incuding MACI 5 BSR are generated for 16 Grant with 5ms periodicity – 5x2 = 10bytes 1 PHR is sent – 2 bytes RLC STATUS PDU for UE capability enquiry 8260 bytes total size needed for 17 MAC PDUs = 8188 + 17B(MAC Headers) + 34 bytes(RLC headers) + 5 bytes(PDCP header+MACI) + 10(5xBSR)+2(PHR)+4(RLC STATUS including MAC header) Currently, total grant size for 17 PDUs is 16 * 501 + 225 = 8241 bytes. Therefore, it is propose to use 8260. But, to achieve 8260, TB size of 244 is needed as 17th Grant but next available TB is 249B(1992bits) as per 36.213 Table 7.1.7.2.1 . Hence, total grant size allocated is 8265 bytes. TBS size for MCS = 19 and Nprb = 5 is 249 bytes. - As per TS36.331, 80ms RRC procedure delay is allowed for response UECapabiltiyEnquiry<>ULDedicatedMessageSegment. Currently, prose provides the grant after 15ms for FDD and 20ms for TDD, but, upto 80ms should be allowed as per cl11.2 in TS36.331. - Since BSR and PHR report affect overall grant size, its better to define periodicity so that number of BSR reports(BSR timer 20ms) can be calculated accurately as it will affect grant size |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modified Note1 to set the grant size for 249 bytes since 244 bytes is not allowed as per TS36.523-3 TB size allocation sheet. - Modifed step 5 to 80ms as starting point for grant allocation - Periodicity of 5ms applied |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE would fail the tests case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Associated TTCN CR: R5s250059 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: MCS/nPRB (19,5) is TBS 249 bytes as per TS36.213. Updated the reason for change and NOTE. |  |  |  |  |  |  |  |  |

<Start of Change>

#### 8.5.5.1	RACS / UL Message Segment transfer / UECapabilityInformation / Success

8.5.5.1.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives a UECapabilityEnquiry message with rrc-SegAllowed-r16 enabled }

then { UE successfully segments UECapabilityInformation message at RRC layer and sends ULDedicatedMessageSegment messages with rrc-MessageSegmentType set accordingly for the first and last segment and rrc-MessageSegmentContainer containing the segments of UECapabilityInformation message }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and having received UECapabilityEnquiry message with rrc-segAllowed-r16 enabled and having transmitted the firstSegment of UECapabilityInformation message within the ULDedicatedMessageSegment message }

ensure that {

when { UE detects radio link failure on expiry of timer T310 }

then { UE successfully completes RRC re-establishment procedure and discards the remaining segments of UECapabilityInformation message }

}

8.5.5.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331 clause 5.6.3, 5.6.22.2, 5.6.22.3 and 5.3.7.2. Unless otherwise stated these are Rel-16 requirements.

[TS 36.331, clause 5.6.3]

The UE shall:

1>	if the RRC message segmentation is enabled based on the field rrc-SegAllowed received, and the encoded RRC message is larger than the maximum supported size of a PDCP SDU specified in TS 36.323 [8]:

2>	initiate the UL message segment transfer procedure as specified in clause 5.6.22;

1>	else:

2>	submit the UECapabilityInformation message to lower layers for transmission, upon which the procedure ends;

[TS 36.331, clause 5.6.22.2]

A UE capable of UL RRC message segmentation in RRC_CONNECTED will initiate the procedure when the following conditions are met:

1>	if the RRC message segmentation is enabled based on the field rrc-SegAllowed received, and

1>	if the encoded RRC message is larger than the maximum supported size of a PDCP SDU specified in TS 36.323 [8];

Upon initiating the procedure, the UE shall:

- initiate transmission of the ULDedicatedMessageSegment message as specified in 5.6.22.3;

[TS 36.331, clause 5.6.22.3]

The UE shall segment the encoded RRC PDU based on the maximum supported size of a PDCP SDU specified in TS 36.323 [8]. UE shall minimize the number of segments and set the contents of the ULDedicatedMessageSegment messages as follows:

1>	For each new UL DCCH message, set the segmentNumber to 0 for the first message segment and increment the segmentNumber for each subsequent RRC message segment;

1>	set rrc-MessageSegmentContainer to include the segment of the UL DCCH message corresponding to the segmentNumber;

1>	if the segment included in the rrc-MessageSegmentContainer is the last segment of the UL DCCH message:

2>	set the rrc-MessageSegmentType to lastSegment;

1>	else:

2>	set the rrc-MessageSegmentType to notLastSegment;

1>	submit all the ULDedicatedMessageSegment messages generated for the segmented RRC message to lower layers for transmission in ascending order based on the segmentNumber, upon which the procedure ends.

[TS 36.331, clause 5.3.7.2]

Upon initiation of the procedure, the UE shall:

…

1>	if UE is not configured with conditionalReconfiguration:

2>	release uplinkDataCompression, if configured;

2>	suspend all RBs, including RBs configured with NR PDCP, except SRB0;

2>	reset MAC;

8.5.5.1.3	Test description

8.5.5.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) according to [18].

8.5.5.1.3.2	Test procedure sequence

Table 8.5.5.1.3.2-1: Power levels

|  | Parameter | Unit | Cell 1 |
| --- | --- | --- | --- |
| T1 | Cell-specific RS RPRE | dBm/15kHz | “Off” |
| T2 | Cell-specific RS RPRE | dBm/15kHz | -85 |

Table 8.5.5.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a UECapabilityEnquiry message including eutra to enquire max capability and with rrc-segAllowed-r16 set to enabled. | <-- | UECapabilityEnquiry | - | - |
| - | EXCEPTION: Steps 2a1 to 2b7 describe behaviour that depends on the size of UEs capability container; the "lower case letter" identifies a step sequence that takes place depending on the size of UE capability message. IF UE’s encoded UECapabilityInformation message >= Max PDCP SDU size then step 2a1 will be performed else steps 2b1 to 2b7 will be performed. | - | - | - | - |
| - | EXCEPTION: Step 2a1 is repeated a maximum of 16 times or till rrc-MessageSegmentType-16 IE within the ULDedicatedMessageSegment message indicate rrc-MessageSegmentType-r16 = lastSegment. | - | - | - | - |
| 2a1 | Check: Does the UE send segments of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message? NOTE: The SS shall ensure that the concatenated segments decode into a valid UECapabilityInformation message (specific capability content is not checked). | --> | ULDedicatedMessageSegment | 1 | P |
| 2b1 | The UE sends UECapabilityInformation Message. | --> | UECapabilityInformation | - | - |
| 3a1-4a11 | Void | - | - | - | - |
| 2b2 | The SS transmits a SET UL MESSAGE REQUEST message. | <-- | SET UL MESSAGE REQUEST | - | - |
| 2b3 | The UE transmits a SET UL MESSAGE RESPONSE message. | --> | SET UL MESSAGE RESPONSE | - | - |
| 2b4 | The SS transmits a UECapabilityEnquiry message with rrc-segAllowed-r16 set to enabled. | <-- | UECapabilityEnquiry | - | - |
| - | EXCEPTION: Step 2b5 is repeated a maximum of 2 times. | - | - | - | - |
| 2b5 | Check: Does the UE send segments of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message? NOTE: The SS shall ensure that the concatenated segments decode into a valid UECapabilityInformation message (specific capability content is not checked). | --> | ULDedicatedMessageSegment | 1 | P |
| 2b6 | The SS transmits a SET UL MESSAGE REQUEST message. | <-- | SET UL MESSAGE REQUEST | - | - |
| 2b7 | The UE transmits a SET UL MESSAGE RESPONSE message. | --> | SET UL MESSAGE RESPONSE | - | - |
| 3 | SS stops regular grant transmission upon scheduling request. | - | - | - | - |
| 4 | The SS transmits a UECapabilityEnquiry message to enquire max capability and with rrc-segAllowed-r16 set to enabled. | <-- | UECapabilityEnquiry | - | - |
| 5 | The SS allocates 17 UL grants periodically(periodicity of 5ms) from 80 subframes after successful completion of step 4 to schedule PUSCH till the ULDedicatedMessageSegment at step 6 is received. (Note 1) | - | - | - | - |
| 6 | The UE sends 1st segment of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message. | --> | ULDedicatedMessageSegment | - | - |
| 7 | The SS changes Cell 1 parameters according to the row "T1" in table 8.5.5.1.3.2-1 in order that the radio link quality of Cell 1 is degraded. | - | - | - | - |
| 8 | Wait for T310 to expire | - | - | - | - |
| 9 | The SS changes Cell 1 parameters according to the row "T2" in table 8.5.5.1.3.2-1. | - | - | - | - |
| 10 | The UE sends an RRCConnectionReestablishmentRequest message on Cell 1. | --> | RRCConnectionReestablishmentRequest | - | - |
| 11 | The SS transmits an RRCConnectionReestablishment message to resume SRB1 operation on Cell 1. | <-- | RRCConnectionReestablishment | - | - |
| 12 | The UE transmits an RRCConnectionReestablishmentComplete message on Cell 1? | --> | RRCConnectionReestablishmentComplete | - | - |
| 12A | The SS transmits an RRCConnectionReconfiguration message to resume existing radio bearer. | <-- | RRCConnectionReconfiguration | - | - |
| 12B | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationtComplete | - | - |
| 13 | SS resumes regular UL grant transmission. | - | - | - | - |
| 14 | Check: Upon successful completion of RRC re-establishment procedure, does the UE discards remaining segments of UECapabilityInformation message and does not send any ULDedicatedMessageSegment message in the next 5 seconds? | - | - | 2 | P |
| NOTE 1: 16 UL grants of default size (501 bytes, as per 36.523-3[20] Table 7.2.0-1) and an additional UL grant of size 249 bytes (NPRB=5, IMCS=19, as per 36.213 Table 7.1.7.2.1) are chosen to enable UE to transmit only one ulDedicatedMessageSegment as maximum PDCP SDU size 8188 bytes. |  |  |  |  |  |

8.5.5.1.3.3	Specific message contents

Table 8.5.5.1.3.3-1: UECapabilityEnquiry (steps 1, 2b4, 4, Table 8.5.5.1.3.2-2)

| Derivation Path: TS 36.508 [7], Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityEnquiry ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueCapabilityEnquiry-r8 SEQUENCE { |  |  |  |
| ue-CapabilityRequest SEQUENCE (SIZE (1..maxRAT-Capabilities)) OF RAT TYPE { |  |  |  |
| RAT-Type [1] | eutra |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| requestedFrequencyBands-r11 SEQUENCE { | n entries, where n is equal to UE supported bands or 16 if number of UE supported bands exceeds 16. |  |  |
| FreqBandIndicator-r11 [1] | px_ePrimaryFrequencyBand | E-UTRA primary frequency band under test. |  |
| FreqBandIndicator-r11 [k] | For k=2 to n set to value different from values set in preceding entries FreqBandIndicator-r11 [1] to FreqBandIndicatorr11 [k-1] | Frequency bands supported by UE. Any value β such that pc_eBandβ_Supp is TRUE |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rrc-SegAllowed-r16 | enabled |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.5.1.3.3-2: ULDedicatedMessageSegment (steps 2a1, 2b5 and step 6, Table 8.5.5.1.3.2-2)

| Derivation Path: 36.508 [18], Table 4.6.1-22A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ULDedicatedMessageSegment-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ulDedicatedMessageSegment-r16 SEQUENCE { |  |  |  |
| segmentNumber-r16 | 0 for first segment, and incremented by 1 for each subsequent segment |  |  |
| rrc-MessageSegmentContainer-r16 | Not Checked | OCTET STRING including a segment of the encoded DL DCCH message |  |
| rrc-MessageSegmentType-r16 | notLastSegment for all segments except the last segment. lastSegment for the last segment |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.5.1.3.3-3: RRCConnectionReestablishmentRequest (step 11, Table 8.5.5.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | otherFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.5.1.3.3-4: UECapabilityInformation (step 2b1, Table 8.5.5.1.3.2-2)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| ueCapabilityInformation-r8 SEQUENCE { |  |  |  |
| ue-CapabilityRAT-ContainerList SEQUENCE (SIZE (1..maxRAT-Capabilities)) OF SEQUENCE { | 1 entry |  |  |
| rat-Type | eutra | E-UTRA only |  |
| ueCapabilityRAT-Container | UE-EUTRA-Capability | Encoded as per TS 36.331 [17] clause 5.6.3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of Change>