| 3GPP RAN5 PRD 20 v1.a.0 (2025-02) |  |
| --- | --- |
| Permanent Reference Document |  |
| 3rd Generation Partnership Project; Technical Specification Group RAN WG5; Permanent Reference Document (PRD); E-UTRA bands and CA configuration handling in RAN5 (Release 10 and later releases) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	5

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	7

3.3	Abbreviations	7

4	Overview of the RAN5 PRD20 E-UTRA CA list	7

4.1	General	7

4.2	"Introduction" worksheet	7

4.2a	"E-UTRA bands" worksheet	8

4.2b	"IoT NTN bands" worksheet	8

4.3	"E-UTRA CA Configurations" worksheet	9

4.4	"CA Config Status Report " worksheet	10

5	User guidelines to get E-UTRA CA configurations into RAN5 test specifications	13

5.1	Assign and create WP for one or more E-UTRA CA configurations	13

5.2	Using the E-UTRA CA configuration WP	16

5.2.1	General	16

5.2.2	Maintaining the WP	17

5.2.3	Splitting a WP between completed and not completed E-UTRA CA configurations	18

5.3	WI code to be used for CRs for E-UTRA CA configurations	19

5.3	Action to report a E-UTRA CA configuration as completed	19

6	PRD rapporteur guidelines	19

6.1	Handling assignment requests	19

6.2	Update the PRD20 E-UTRA bands, IoT NTN bands and E-UTRA CA list when new version of TS 36.101 or 36.102 is published	20

6.2.1	Update of the "E-UTRA bands", "IoT NTN bands" and "E-UTRA CA Configurations" work sheet	20

6.2.2	Update of the "Support data" work sheet	20

6.3	Update the PRD20 after end of RAN5 meetings	21

6.3.1	Update status of E-UTRA bands, IoT NTN bands and E-UTRA CA Configurations	21

6.3.2	Update when a RAN5 E-UTRA bands, IoT NTN bands or E-UTRA CA basket WI is closed	21

6.3.3	Update when all E-UTRA bands, IoT NTN bands or E-UTRA CA configurations in a RAN5 E-UTRA WI have been completed	21

6.3.4	Update of Annex B in TS 36.521-2 when status in PRD20 CA list is changed	21

6.4	Update the WP template	22

6.5	Update when PRD20 rapporteur is changed	22

Annex A (informative): Change history	23

Foreword

This Permanent Reference Document (PRD) has been produced by the 3rd Generation Partnership Project (3GPP) TSG RAN Working Group 5 (RAN WG5 = RAN5).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

Introduction

The handling of E-UTRA CA configurations in RAN5 is based on the following principles endorsed by RAN5 at RAN5#92-e in [1]:

-	RAN5 E-UTRA CA configuration work items should focus on updating existing test cases and/or adding new test cases for the new type of CA configurations introduced by the work item. RAN5 E-UTRA CA configuration work items do not need to track status of the new E-UTRA CA configurations introduced by the work item.

-	The minimum criteria for closing a RAN5 E-UTRA CA work item is that the associated RAN4 core work item(s) are completed; that all required new or existing test cases have been completed for at least one representative CA configuration; and that PRD20 includes all the CA configurations introduced by the associated RAN4 core work items.

-	A Permanent Reference Document PRD20 is used to keep track of TS 36.101 [2] E-UTRA CA configurations and the status of the configurations in RAN5 conformance test specifications.

-	The scope of PRD20 is limited to the RAN5 ongoing and completed E-UTRA CA work items.

-	Completed E-UTRA CA configurations available to industry for testing are specified in TS 36.521-2 [3] clause A.4.6. For exceptional cases RAN5 may allow not fully completed CA configuration to be added to clause A.4.6 together with a note stating not completed parts.

The handling of E-UTRA bands and IoT NTN bands is extended for the scope of PRD20 by RAN5 at RAN5#103 in [5].

The PRD20 rapporteur is responsible together with the RAN5 E-UTRA bands, IoT NTN bands and CA work item rapporteurs for keeping the E-UTRA bands, IoT NTN bands and CA configuration list up to date with latest version of TS 36.101 [2] within the scope of RAN5 E-UTRA work items.

The PRD20 rapporteur is responsible for handling assignment of E-UTRA bands, IoT NTN bands and CA configuration to volunteering companies and to update status based on completed E-UTRA bands, IoT NTN bands and CA configurations in RAN5 conformance test specifications.

1	Scope

The scope of present document is RAN5 status tracking of for E-UTRA bands, IoT NTN bands and CA configurations covered by RAN5 LTE work items and to provide a tool and guideline for companies volunteering to take responsibility to introduce new E-UTRA bands, IoT NTN bands and CA configurations to 3GPP RAN5 test specifications.

The RAN5 work items covered by RAN5 PRD20 are:

| Release | 3GPP Work Item Name | 3GPP Work Item Acronym | Status |
| --- | --- | --- | --- |
| Rel-10 | Test part: Carrier Aggregation for LTE | LTE_CA-UEConTest | Completed |
| Rel-11 | UE Conformance Test Aspects – LTE Advanced Carrier Aggregation of Intra-band contiguous and Inter-band configurations | LTE_CA_intraC_inter_UEConTest | Completed |
| Rel-12 | UE Conformance Test Aspects - Further Rel-12 configurations for LTE Advanced Carrier Aggregation with 2UL | LTE_CA_Rel12-UEConTest | Completed |
|  | UE Conformance Test Aspects – Further Rel-12 configurations for LTE Advanced Carrier Aggregation | LTE_CA_Rel12_2UL-UEConTest | Completed |
|  | UE Conformance Test Aspects – Further Rel-12 configurations for LTE Advanced Carrier Aggregation with 3DL | LTE_CA_Rel12_3DL-UEConTest | WI closed but not all configurations specified |
| Rel-13 | UE Conformance Test Aspects - Rel-13 LTE CA configurations | LTE_CA_Rel13-UEConTest | WI closed but not all configurations specified |
| Rel-14 | UE Conformance Test Aspects - Rel-14 LTE CA configurations | LTE_CA_R14-UEConTest | WI closed but not all configurations specified |
| Rel-15 | UE Conformance Test Aspects - Rel-15 LTE CA configurations | LTE_CA_R15-UEConTest | Completed |
| Rel-16 | UE Conformance Test Aspects - Rel-16 LTE CA configurations | LTE_CA_R16-UEConTest | WI closed but not all configurations specified |
| Rel-17 | UE Conformance Test Aspects – Rel-17 LTE CA configurations | LTE_CA_R17-UEConTest | Completed |
| Rel-18 | NB-IoT (Narrowband IoT)/eMTC (enhanced Machine Type Communication) core & performance requirements for Non-Terrestrial Networks (NTN) | LTE_NBIOT_eMTC_NTN_req-UEConTest | Completed |
| Rel-18 | UE Conformance - High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands in Rel-18 | LTE_NR_HPUE_FWVM_R18-UEConTest | Completed |
| Rel-18 | UE Conformance - Introduction of FDD LTE band (L+S band) for IoT NTN operation | IoT_NTN_FDD_LS_band-UEConTest | Completed |
| Rel-18 | UE Conformance - Introduction of the Extended L-band (UL 1688-1675MHz, DL 1518-1525MHz) for IoT NTN | IoT_NTN_extLband-UEConTest | Ongoing |
| Rel-18 | UE Conformance - New bands and bandwidth allocation for LTE based 5G terrestrial broadcast | LTE_terr_bcast_bands_part2-UEConTest | Ongoing |

2	References

[1]	R5-215708: "Discussion paper on handling of work items on LTE CA configurations ".

[2]	3GPP TS 36.101: "E-UTRA UE radio transmission and reception".

[3]	3GPP TS 36.521-2: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Implementation Conformance Statement (ICS)".

[4]	3GPP TS 36.102: "E-UTRA UE radio transmission and reception for satellite access".

[5]	R5-243069: "Disc on handling of PRD21 and PRD20".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

Assigned band: An E-UTRA band is assigned to a company volunteering to take responsibility to make sure that the necessary contributions to include the band into RAN5 conformance test specifications are prepared, submitted and agreed in RAN5.

Assigned configuration: An E-UTRA CA Ongoing Configuration is assigned to a company volunteering to take responsibility to make sure that the necessary contributions to include the configuration into RAN5 conformance test specifications are prepared, submitted and agreed in RAN5.

Completed band: All CRs required to make the E-UTRA band completed have been agreed by RAN5 for inclusion in next version of impacted RAN5 conformance test specifications.

Completed configuration: 	All CRs required to make the E-UTRA CA configuration completed have been agreed by RAN5 for inclusion in next version of impacted RAN5 conformance test specifications.

Interested Deployer: An organization (e.g. railway union) interested in deploying an E-UTRA band or E-UTRA CADC configuration (e.g. E-UTRA railway band or E-UTRA railway CADC configuration), or a V2X company interested in deploying an E-UTRA V2X band or E-UTRA V2X CADC configuration, or an industry company (e.g. agriculture company, power company) interested in deploying an E-UTRA band or E-UTRA CADC configuration. For the configurations tagged with “Legacy” as the “Interested Deployer” in PRD20 E-UTRA bands and CADC configurations list, they were completed before PRD20 v1.7.0 was introduced.

Interested Operator: A mobile network operator interested in deploying an E-UTRA band or E-UTRA CADC configuration.

IoT NTN band: An IoT NTN band as specified in TS 36.102 [4].

Ongoing band: An E-UTRA band that has been interested by at least one "Interested Operator" or "Interested Deployer" and is open for assignment or contributions in RAN5. As long as an E-UTRA band has been interested by at least one "Interested Operator" or "Interested Deployer", it can be regarded as an Ongoing band no matter it has been assigned to a volunteering company or not.

Ongoing configuration: An E-UTRA CA configuration that is assigned to a company and is open for contributions in RAN5.

Pending band: An E-UTRA band that has not been interested by any operator or deployer yet in RAN5.

Pending configuration: An E-UTRA CA configuration that has not yet been assigned to any company.

Responsible company: A company that volunteers to take responsibility to make sure that the necessary contributions to include an E-UTRA band or configuration into RAN5 conformance test specifications are prepared, submitted and agreed in RAN5.

3.2	Symbols

None

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BCS	Bandwidth Combination Set

CA	Carrier Aggregation

CDS	Completion Declaration Statement

CBW	Channel Bandwidth

N/A	Not Applicable

WI	Work Item

WP	Work Plan

4	Overview of the RAN5 PRD20 E-UTRA CA list

4.1	General

The RAN5 PRD20 E-UTRA CA list is attached as an macro enabled Excel file named "PRD20 E-UTRA CA list vx.y.z " where vx.y.z is the same version number as the version number of this PRD.

The PRD20 E-UTRA CA list includes three worksheets named:

1. 		Introduction

2. 	E-UTRA bands

3. 	IoT NTN bands

4.	E-UTRA CA Configurations

5. 	CA Config Status Report

4.2	"Introduction" worksheet

The worksheet "Introduction" contains a short description of the PRD20 E-UTRA CA list and includes a table showing the overall status of the RAN5 E-UTRA CA configurations vs TS 36.101 [2] CA configuration tables. By selecting a specific release or "All Releases" current RAN5 status of completed, assigned and pending E-UTRA CA configurations.

Picture 4.2-1 shows a snapshot of the table with overall status of RAN5 E-UTRA CA configurations for the case "Rel-12" has been selected. The picture shows that all except for two of the Inter-band CA (two bands) E-UTRA CA configurations introduced in Rel-12 of TS 36.101 [2] have been completed.

Picture 4.2-2 shows a snapshot of the table with overall status of RAN5 E-UTRA CA configurations for the case "All Releases" has been selected.

The examples in Picture 4.2-1 and 4.2-2 are snapshots reflecting the status after RAN5#92-e (August 2021). For the actual status see worksheet "Introduction" in the attached version of the attached PRD20 E-UTRA CA list.

Picture 4.2-1: Overall status of RAN5 E-UTRA CA configurations for the case "Rel-12" selected.

Picture 4.2-2: Overall status of RAN5 E-UTRA CA configurations for the case "All Releases" selected.

4.2a	"E-UTRA bands" worksheet

The worksheet "E-UTRA bands" lists all E-UTRA bands in TS 36.101 [2] up to the version indicated in the top left corner of the worksheet "Based on: TS 36.101 vx.y.z". The list is structured by E-UTRA band tables in TS 36.101 [2], clause 5.5.

4.2b	"IoT NTN bands" worksheet

The worksheet "IoT NTN bands" lists all IoT NTN bands in TS 36.102 [4] up to the version indicated in the top left corner of the worksheet "Based on: TS 36.102 vx.y.z". The list is structured by IoT NTN band tables in TS 36.102 [4], clause 5.2.

4.3	"E-UTRA CA Configurations" worksheet

The worksheet "E-UTRA CA Configurations" lists all E-UTRA CA configurations in TS 36.101 [2] up to the version indicated in the top left corner of the worksheet "Based on: TS 36.101 vx.y.z", see Picture 4.3-1. The list is structured by E-UTRA CA configuration tables in TS 36.101 [2], clause 5.6A.

The list has two outline levels (see Picture 4.3-1):

- Outline Level 1: Shows list of E-UTRA CA configuration tables in TS 36.101 [2]. To show all the configurations within a TS 36.101 table press the expansion button "+", see Picture 4.3-2.

- Outline Level 2: Shows the complete list of E-UTRA CA configurations.

The examples in Picture 4.3-2 and 4.3-3 are snapshots reflecting the status after RAN5#92-e (August 2021). For the actual status see the attached version of the PRD20 E-UTRA CA list.

The colour labelling of E-UTRA C configuration rows is:

Green row indicates that the LTE CA configuration is complete in the RAN5 test specifications. Available for testing.

Yellow row indicates that the LTE CA configuration is assigned to a company but not yet completed in the RAN5 test specifications.

White row with red text indicates that the LTE CA configuration is not yet assigned to any company and not included in the RAN5 test specifications.

The column "Applicable RAN5 WI code(s) for CRs" shows the RAN5 WI code(s) to be specified on the CR coversheet for CRs to the E-UTRA CA configuration. See picture 4.3-3.

By double-click on a E-UTRA CA configuration in the list a E-UTRA CA configuration status report is created in the worksheet "CA Config Status Report". See clause 4.3 for further details.

Picture 4.3-1: Selecting outline level and indication of source of TS 36.101.

Select Outline Level 1   
                  Select Outline Level 2     
                                           TS 36.101 version the list of E-UTRA CA configurations is based on

Picture 4.3-2: Outline Level 1 selected showing list of TS 36.101 E-UTRA CA configuration tables.

Press the expansion button "+" for a table to show the list of configurations of the table

Picture 4.3-3: Outline Level 2 selected showing all configurations for all TS 36.101 tables.

WI code to be used for CRs to E-UTRA CA configurations

A E-UTRA CA configurations in TS 36.101 [2] clause 5.6A having UL CA and/or more than one BCS will in the PRD20 E-UTRA CA list be split into multiple rows where each row is limited to without UL CA or with one UL CA configuration and one BCS.

The purpose of the splitting of the TS 36.101 [2] E-UTRA CA configurations into multiple configurations is to allow progress of individual configurations dependent on industry interest. I.e. in the example below if the initial interest is for DL without UL for BCS0 and BCS1 then can those configurations be progressed and completed in RAN5 specifications while the configurations with UL CA can be deferred until later.

Example:

CA_1A-3A-7A is in TS 36.101 Table 5.6A.1-2a defined as:

The E-UTRA CA configuration has three UL CA configurations and two BCSs (BCS0 and BCS1). In the PRD20 E-UTRA CA list this configuration will appear as:

4.4	"CA Config Status Report " worksheet

The worksheet "CA Config Status Report" shows the latest created E-UTRA CA configuration status report.

A status report is created by double-click on an E-UTRA CA configuration row in the in the "E-UTRA CA Configurations" worksheet, see clause 4.2.

When double-click the E-UTRA CA Configuration a pop-up window appears asking for the UL CA scope of the selected E-UTRA CA configuration. Picture 4.4-1 shows the pop-up window for the case the double-click on the row with E-UTRA CA configuration CA_1A-3A-7A BCS1 with UL CA_3A-7A and UL CA scope "UL_3A-7A only" has been selected.

Picture 4.4-1: Pop-up window for selection of UL CA scope of the status report.

Pressing the continue button creates the status report of the selected configurations and its fallbacks. See Picture 4.4-2 for the created report for CA_1A_3A_7A BCS1 with UL CA_3A-7A.

In the status report the result is split into four sections:

1.	Status of the selected E-UTRA CA configuration(s)

2.	Unassigned not completed CA fallback configuration(s)

3.	Assigned but not completed CA fallback configuration(s)

4.	Completed CA fallback configuration(s)

As can be seen in Picture 4.4-2 and under section 1 the selected configuration CA_1A_3A_7A BCS1 with UL CA_3A-7A is not yet assigned. Section 2, 3 and 4 list the CA fallbacks to CA_1A_3A_7A BCS1 with UL CA_3A-7A. Section 2 lists the unassigned CA fallbacks, section 3 lists the assigned but not yet completed CA fallbacks and section 4 lists the completed CA fallbacks.

Picture 4.4-2: Status report for CA_1A-3A-7A BCS1 with UL CA_3A-7A and associated fallback configurations (status at RAN5#92-e, August 2021) .

5	User guidelines to get E-UTRA CA configurations into RAN5 test specifications

5.1	Assign and create WP for one or more E-UTRA CA configurations

To get one or more E-UTRA CA configurations into RAN5 conformance test specifications do:

1.	Locate the E-UTRA CA configuration in the PRD20 E-UTRA CA list in worksheet "E-UTRA CA Configurations" and double click on the row it is listed to create a status report. See clause 4.3 for further information about the PRD20 E-UTRA CA list.

2.	In the created status report in worksheet "CA Config Status Report" select the E-UTRA CA configuration(s) You want to request Your company to be assigned for. Maximum 10 E-UTRA CA configurations can be selected for one assignment and one WP. 

See picture 5.1-1 for E-UTRA CA configuration selection pane of the status report (using the CA_1A-3A-7A BCS1 as an example). The white empty rows indicate that the configurations at those rows can be selected. See picture 5.1-2 for the case when all have been selected.

Picture 5.1-1: Selection pane of status report before any configurations have been selected.

Picture 5.1-2: Selection pane of status report after all have been selected.

3.	Press "Request Assignment and Create Work Plan" button to create and email request form and a WP.

4.	Confirm by press "ok in the pop-up confirmation window (see Picture 5.1-3).

Picture 5.1-3: Assignment request confirmation.

5.	In the created email fill in Your company and Your contact details and press the send button. See Picture 5.1-4 for an example.

Picture 5.1-4: Example of a PRD20 created assignment request email.

6.	In the pop-up window confirm that You want to create a WP for the requested E-UTRA CA configurations (see Picture 5.1-5).

Picture 5.1-5: Assignment request confirmation.

7.	Press OK in the pop-up window (Picture 5.1-6) to select a folder to store the workplan(s).

Picture 5.1-6: Select folder to store workplans.

8.	Press Ok in the pop-up window (Picture 5.1-7) to start creating the work plans. Note the warning that Excel shall not be used until after the workplan(s) have been confirmed created.

Picture 5.1-7: Confirmation of strage location of the selected folder.

9.	A pop-up window confirms that a WP have been created and stored in the selected storage location. Press OK to close the window (see Picture 5.1-6). 

For the example E-UTRA CA configuration and its BCS and UL CA configuration impact Rel-13, Rel-14 and Rel-15 RAN5 WIs and separate workplans for each release have been created. Each WP include a separate worksheet for each type of DL/UL CC combination (2DL/1UL, 2DL/2UL etc).

NOTE:	A copy of the completed work plan shall be attached to the CR to TS 36.521-2 adding the E-UTRA CA configuration(s) to clause A.4.6. See clause 5.3 for further details on action to report a E-UTRA CA configuration as completed.

Picture 5.1-6: Confirmation that the WP has been created.

5.2	Using the E-UTRA CA configuration WP

5.2.1	General

The E-UTRA CA configuration work plans are used as a guideline and checklist for the assigned company to introduce and document the introduction of the E-UTRA CA configurations into the relevant RAN5 technical specification and technical reports.

PRD20 CA list creates separate workplan Excel workbooks for each impacted release of the selected E-UTRA CA configurations. Each work plan Excel workbook group the E-UTRA CA configurations in one worksheet per type of  DL/UL CC combinations, i.e. one for 2DL/1UL configurations, one for 2DL/2UL configurations etc.

The E-UTRA CA configuration WP worksheets is also used in the final step to confirm completion of the E-UTRA Configurations by attaching the WP to the CR to TS 36.521-2 [3], clause A.4.6, see clause 5.3.

The created WP to introduce E-UTRA CA configurations in to is has three outline levels:

Outline Level 1: Showing overview of recommended work flow steps to introduce the E-UTRA CA configurations

Outline Level 2: Showing all WP items under each work flow step.

Outline Level 3: Showing all details of the WP (as Outline Level 2 + details of WP scope and overall status).

The outline level is selected in the upper left corner of the WP by selecting 1, 2 or 3:

The WP is structured in the recommended work flow steps to introduce the new E-UTRA CA configurations in RAN5 technical specifications and technical reports. The overall completion and the completion of each work flow step is shown in section 2 of the WP, see Picture 5.2-1. The completion status is calculated based on the reported status for each WP item in section 3 of the WP.

Picture 5.2-1: WP work flow steps and WP overall status information.

For each work flow step the WP include a number of WP item rows with status columns to indicate progress and completion of the WP item tasks, see Picture 5.2-2 for an example of WP item rows for work flow step 1.

Picture 5.2-2: Example of a work flow step and its WP item rows.

The applicable WP Item rows is dependent on the type of E-UTRA CA configurations covered by the WP. Not applicable WP item rows are marked as grey strikethrough.

5.2.2	Maintaining the WP

The columns marked as "WP item columns to be filled in" shall be filled in showing what have been done to complete the WP items. The purpose of the different columns are:

-	The "Company" column is used to track company responsibility for the WP Item in the case more companies than the assigned company of the E-UTRA CA configurations is contributing.

-	The "Target" column is by default linked to the values of the target completion date specified in the WP header. If needed specific target for a WP item can be added by replace the current formula in the target column with specific target information for the WP item.

-	The "TDOC(s)" column is used to track RAN5 contributions (CRs) progressing and completing the WP items.

- 	The "Comments" column can be used to add additional comments as needed for the WP item. E.g. for the case no changes were needed for the WP item then it is useful to add a comment "No changes needed" as justification for setting the WP item as completed (100%).

- 	WP item status columns is used to state the current status (0% to 100%) for the different areas covered by the status columns.

5.2.3	Splitting a WP between completed and not completed E-UTRA CA configurations

In case a WP covers multiple E-UTRA CA configurations and there is a wish to split into multiple WPs, e.g. due to a subset of them have been completed then do:

-	Copy the existing WP into the number of WPs You want. Rename each individual WP Excel filed reflecting the scope of the WP.

-	For each copy of the WP update the "Workplan Name" and WP scope table in section 1 of the WP to reflect the scope of the updated scope of the WPs. See the example below for splitting a WP in to two WPs.

Example: Splitting a WP into two WPs

Picture 5.2.3-1 shows the WP scope table for a set of E-UTRA CA configurations. To split the WP into two WPs one for the DL only configurations and one for the UL CA WPs the WP scope table for the two WPs need to be updated as shown in Pictures 5.2.3-2 (changes indicated in yellow) and Picture 5.2.3-1 (changes indicated in orange).

Picture 5.2.3-1: Example of a work plan scope table.

Picture 5.2.3-2: Work plan scope split#1 for DL CA only configurations.

Picture 5.2.3-3: Work plan scope split for UL CA configurations.

5.3	WI code to be used for CRs for E-UTRA CA configurations

Any CR submitted to RAN5 to introduce details for a E_UTRA CA configuration or to maintain existing text in RAN5 technical specification and technical reports shall use the WI code as indicated in the PRD20 E-UTRA CA list in worksheet "E-UTRA CA Configurations".

5.3	Action to report a E-UTRA CA configuration as completed

The final work flow step (step 4) in the WP covers the actions needed to get E-UTRA CA configurations to be considered completed by RAN5.

The confirmation of completion of E-UTRA CA configurations is done by a CR to TS 36.521-2 [3] adding the  CA Physical Layer Baseline Implementation Capabilities for the CA configurations in Annex A.4.6.

The CR shall add the E-UTRA CA configurations in the relevant Physical Layer Baseline Implementation Capabilities for the CA configurations in Annex A.4.6 using the a table format where each row covers one DL and UL E-UTRA CA configuration. The covered BCSs and the introduction release of the initial BCS0 shall also be specified in teh CR. See example below for the format to use in TS 36.521-2 [3].

Any exceptions for not completed parts need to be stated in the CR cover sheet and as a note in the CR body text for the entry of the CA configuration in the impacted CA Physical Layer Baseline Implementation Capability table.

A copy of the work plan shall be attached to the CR to TS 36.521-2 in the TDOC zip-file.

Example: CA_1A-3A status in TS 36.521-2 [3], clause A.4.6.

The completed E-UTRA CA configuration CA_1A-3A is listed in PRD20 E-UTRA CA as (status 2021-09):

DL CA_1A-3A for BCS0 and BCS1; and UL CA_1A-3A BCS0 have been completed in RAN5 specifications while UL CA_1A-3A BCS1 has not.

In TS 36.521-2 [3] this shall be reflected in the Table A.4.6.3-3 as:

When UL CA_1A-3A BCS1 will be completed the change to Table A.4.6.3-3 will be to change the BCS column of the second row from "0" to "0,1"

6	PRD rapporteur guidelines

6.1	Handling assignment requests

When receiving an assignment request email do:

-	Add the responsible company, and contact person email and the interested deployer to the requested E-UTRA CA configurations in worksheet "E-UTRA CA Configurations"; and to the requested E-UTRA bands in worksheet "E-UTRA bands"; and to the requested IoT NTN bands in worksheet "IoT NTN bands".

-	Add the RAN5 meeting the request was received in column "RAN5 Assignment [RAN5 meeting]".

-	Confirm the assignment by responding to the request email.

6.2	Update the PRD20 E-UTRA bands, IoT NTN bands and E-UTRA CA list when new version of TS 36.101 or 36.102 is published

6.2.1	Update of the "E-UTRA bands", "IoT NTN bands" and "E-UTRA CA Configurations" work sheet

Add any new or changed E-UTRA bands, IoT NTN bands and CA configuration and its CBW combinations in accordance to the latest version of TS 36.101 [2] up to the release covered by the RAN5 E-UTRA bands, IoT NTN bands and E-UTRA CA work items listed in clause 1.

The column "Applicable RAN5 WI code(s) for CRs" is set in accordance to the current applicable WI codes as listed in clause 1 depending on the current status of the RAN5 WI the configuration belongs to.

Example: 4D/2UL E-UTRA CA configuration CA_1A-3A-5A-7A

TS 36.101 [2] v16.11.0, Table 5.6A.1-2b specifies the E_UTRA CA configuration CA_1A-3A-5A-7A as:

BCS0 was introduced in Rel-14 and BCS1 in Rel-16 of TS 36.101 [2].

For PRD20 CA list this configuration and its UL CA configurations split into separate rows for each BCS and for each UL CA configuration as well as one row each for each BCS for the case without UL CA:

6.2.2	Update of the "Support data" work sheet

The support data Excel worksheet is hidden in the published version of PRD20 E-UTRA CA list. Unhide the worksheet and do:

- Unhide the worksheet "Support data" in the Excel file attached to PRD20.

- Update the version of TS 36.101 [1] in the cell next to "Source of PRD20 support data"

- Check if any changes are needed in the support data tables due to the new version of TS 36.101 [2].

- Hide the worksheet "Support data"

6.3	Update the PRD20 after end of RAN5 meetings

6.3.1	Update status of E-UTRA bands, IoT NTN bands and E-UTRA CA Configurations

Update the status of completed E-UTRA bands, IoT NTN bands and E-UTRA CA Configurations by adding the RAN5 meeting to column "RAN5 Completion" and add the reference to the agreed CR to TS 36.521-2 [3] confirming the completion of the E-UTRA CA configuration(s). A CDS (Completion Declaration Statement) document should be submitted to RAN5 meeting to declare completion of E-UTRA bands, IoT NTN bands and E-UTRA CA configurations. The reference to the agreed CDS document in column "Completion Reference" should be added to the corresponding worksheet.

6.3.2	Update when a RAN5 E-UTRA bands, IoT NTN bands or E-UTRA CA basket WI is closed

When a RAN5 E-UTRA bands, IoT NTN bands or E-UTRA CA basket work item has been closed or when all E-UTRA CA configurations within a WI has been completed the following need to be updated:

- 	Update the "Status" column in the scope table in clause 1 of this document for the WI.

-	If all the E-UTRA bands, IoT NTN bands or E-UTRA CA configurations have been completed set the status to "Completed"; else

-	If the WI is closed but not all the E-UTRA bands, IoT NTN bands or E-UTRA CA configurations have been completed set the status to "WI closed but not all configurations specified".

- 	Make the same changes to the "Status" column in worksheet "Introduction" of the PRD20 CA list as done above for clause 1.

6.3.3	Update when all E-UTRA bands, IoT NTN bands or E-UTRA CA configurations in a RAN5 E-UTRA WI have been completed

When all E-UTRA bands, IoT NTN bands or E-UTRA CA Configurations for a closed RAN5 WI have been completed do:

-	In the "E-UTRA bands", "IoT NTN bands" or "E-UTRA CA configurations" worksheet of the PRD20 CA list:

-	For each E-UTRA band, IoT NTN band or E-UTRA CA configuration of the closed WI add "TEIx_Test, before the existing WI code in column Applicable RAN5 WI code(s) for CRs where x is the number of the release, e.g. 15 for Rel-15 etc.

6.3.4	Update of Annex B in TS 36.521-2 when status in PRD20 CA list is changed

TS 36.521-2 [3], Annex B shall include a reference to the latest version of an attached macro-free version of the PRD20 CA list as a zip-file in the TS 36.521-2 [3] zip file.

When the version of the PRD20 CA list is updated at a RAN5 meeting the PRD20 rapporteur shall do:

-	Create a macro-free version of the PRD20 CA list by

-	save the “PRD20 E-UTRA CA list vx.y.z.xlsm” as a “xlsx” file where vx.y.z is the version of the PRD20 CA list reflecting the status after the RAN5 meeting,

- 	remove worksheet “CA Config Status Report”, and

-	zip the “PRD20 E-UTRA CA list vx.y.z.xlsx” file as “PRD20 E-UTRA CA list vx.y.z.zip”

-	Create a CR to TS 36.521-2 [3] including

-	updating Annex B to reference the new version of the PRD20 E-UTRA CA list, and

-	attach the macro-free copy of the “PRD20 E-UTRA CA list vx.y.z.zip” to the CR.

6.4	Update the WP template

To update information in the WP template used to create WPs do:

- Unhide the worksheet "WP template" in the Excel file attached to PRD20.

- Update the WP template (pay attention to not change cells with formulas). Add new WP items by copy an existing WP item row and insert above the relevant black row under each WP step area and modify as needed.

- Hide the worksheet "WP template" in the Excel file attached to PRD20.

6.5	Update when PRD20 rapporteur is changed

When a new rapporteur is assigned to PRD20 do:

- Unhide the worksheet "Support data" in the Excel file attached to PRD20.

- In the "Support data" worksheet update the PRD20 contact person details Name, Company and email address. It is important that the email address is correct as it will be used in the assignment request emails.

- Hide the worksheet "Support data"

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | Tdoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-11 | RAN5#93-e | R5-217088 | - | - | - | First version | 1.0.0 |
| 2022-05 | RAN5#95-e | R5-222244 | - | - | - | Update of PRD20 CA list adding new configurations in the PRD20 CA list. | 1.1.0 |
| 2023-03 | RAN5#98 | R5-231193 | - | - | - | Change of rapporteur from Leif Mattisson, Ericsson to Zhifeng Ma, ZTE. PRD20 list updated Applicable WI codes for Rel-15 and Rel-16 configurations. Corrected duplicated clause 6.2.2. Added clause 6.2.4 including the rapporteur guideline for updating TS 36.521-2 [3], Annex B when the PRD20 CA list is updated at RAN5 meetings. | 1.2.0 |
| 2023-05 | RAN5#99 | R5-233204 | - | - | - | (1) Correct the subclause number in Section 6.2 and 6.3. (2) Typo corrections in Picture 4.3-3. (3) Typo corrections in Section 6.1 and 6.2. | 1.3.0 |
| 2023-08 | RAN5#100 | R5-235123 | - | - | - | Version raised to v1.4.0 to enable alignment with the version number of the PRD20 Excel file. | 1.4.0 |
| 2023-11 | RAN5#101 | R5-236923 | - | - | - | Update applicable WI code for LTE_CA_R17-UEConTest and add new Rel-17 configurations in the PRD20 CA list. | 1.5.0 |
| 2024-03 | RAN5#102 | R5-240832 | - | - | - | Version raised to v1.6.0 to enable alignment with the version number of the PRD20 Excel file. | 1.6.0 |
| 2024-06 | RAN5#103 | R5-243282 | - | - | - | Update the title of PRD20 to “PRD; E-UTRA bands and CA configuration handling in RAN5 (Release 10 and later releases)”. | 1.7.0 |
| 2024-09 | RAN5#104 | R5-244910 | - | - | - | Add the WID “LTE_NR_HPUE_FWVM_R18-UEConTest” and “LTE_NBIOT_eMTC_NTN_req-UEConTest” to be covered by PRD20. | 1.8.0 |
| 2024-11 | RAN5#105 | R5-247358 | - | - | - | Update PRD20 handling for E-UTRA bands, IoT NTN bands and CA configurations. | 1.9.0 |
| 2025-02 | RAN5#106 | R5-251063 | - | - | - | Update the status of RAN5 work items covered by RAN5 PRD20. Add the general description for “E-UTRA bands” and “IoT NTN bands” worksheets in new clauses 4.2a and 4.2b. | 1.a.0 |