3GPP TSG-RAN5 Meeting #107			R5-252414

# St. Julian's, Malta, 19th May 2025 – 23rd May 2025

Agenda item:	5.3.5.9

Source: 	EchoStar

Title: 	Support of Variable Tx-Rx spacing for NR NTN bands

Document for:	Discussion and Endorsement

## 1.	Introduction

During RAN plenary meeting TSGR#105, CRs [1] related to support for variable Tx-Rx separation for NR NTN bands was approved and subsequently incorporated in the core specifications [2].

This paper summarizes updates made to the core specifications related to variable Tx-Rx frequency separation requirements, assesses the impact to various conformance specifications and proposes updates to the affected clauses of the affected specifications.

## 2.	Overview of Variable Tx-Rx spacing related updates to core specifications

Based on the approved CRs, Tables 5.4.4-1 in [2] was updated as follows:

Table 5.4.4-1: UE TX-RX frequency separation (FR1-NTN)

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n256 | 190 MHz1 165 to 215 MHz2 |
| n255 | -101.5 MHz1 -72.5 to -130.5 MHz2 |
| n254 | 862 – 885 MHz |
| NOTE 1: Default Tx-Rx separation NOTE 2: The verification of flexible Tx-Rx frequency separation within this range is limited to reference sensitivity. Further details are specified in clause 7.3.2 |  |

Observation 1: Support for variable Tx-Rx spacing for NR NTN is mandatory from Rel-17 onwards.

Additionally, to limit the scope of RF conformance testing resulting from wide range of Tx-Rx separation, Clauses 5.4.4-1, 7.1 and 7.3 in [2] specify that only reference sensitivity requirements need to be tested for conformance with Tx-Rx separation other than the default value. The minimum reference sensitivity requirements for the NTN band remain unchanged from that determined for the default Tx-Rx separation. There are no new demodulation performance and RRM requirements resulting from introduction of variable Tx-Rx spacing for NR NTN bands.

Observation 2: Except for reference sensitivity requirements, all other requirements are to be tested with the default Tx-Rx separation.

Observation 3: The minimum reference sensitivity requirements for all Tx-Rx spacings is same as that for the default Tx-Rx spacing.

Observation 4: The demodulation and RRM requirements are unaffected by the introduction of variable Tx-Rx spacing.

Finally, Table 7.3.2-3 in [2] identifies the additional test points for reference sensitivity tests to verify UE’s support of variable Tx-Rx separation.

Table 7.3.2-3: TX – RX carrier centre frequency separation for REFSENS verification

| Operating Band | Channel bandwidth | TX – RX carrier centre frequency separation for REFSENS verification |
| --- | --- | --- |
| n256 | 5 MHz | 165 MHz, 215 MHz |
|  | 20 MHz | 180 MHz, 200 MHz |
| n255 | 5 MHz | -72.5 MHz, -130.5 MHz |
|  | 20 MHz | -87.5 MHz, -115.5 MHz |

Observation 5: The number of additional test points for reference sensitivity tests to verify support of variable Tx-Rx separation to four (4):

- smallest Tx-Rx separation, smallest channel bandwidth

- smallest Tx-Rx separation, largest channel bandwidth

- largest Tx-Rx separation, smallest channel bandwidth

- largest Tx-Rx separation, largest channel bandwidth

## 3.	Conformance Specification Impact Assessment and Proposals

| TS & Clause | Assessment | Proposals |
| --- | --- | --- |
| TS 38.508-1 |  |  |
| Clauses 4.3.1.9 | Except for Band n24, legacy TN NR bands have used the default Tx-Rx separation to determine the test frequencies. Band 24 used two Tx-Rx separation values with Low & Med Range based on one value and High Range based on another value. This was possible since the two values were defined as mandatory for across RF test cases in a release independent way for Band n24. For NR NTN bands, except for reference sensitivity test cases, all other test cases are to be performed using the default Tx-Rx spacing using the low/mid/high test frequencies currently specified for the NTN bands. | Add a second table with suffix “a” for each band and SCS for variable Tx-Rx spacing under clauses 4.3.1.9.1.1/2/3. |
| Clauses 4.6.1, 4.6.3, | UplinkConfigCommonSIB IE is included in SIB1. Therefore, no further updates are necessary to SIB1. However, RRCConnectionConfiguration will have to be updated to include CellGroupConfig with condition ntn_variable_txrx_separation. Similarly, ServingCellConfig may have to be updated to include uplinkConfig with condition ntn_variation_txrx_separation. | No updates are required since UplinkConfigCommonSIB IE is already included in SIB1 for all test cases. Update ServingCellConfig, RRCConnectionConfiguration. |
| Clause 5.2.2 | Since there are no impact on the Demod performance and test, default Tx-Rx spacing is adequate and no updates are necessary for variable Tx-Rx spacing | No updates are required since RF testing with variable Tx-Rx spacing is limited to reference sensitivity test case. |
| Clauses 6.2.3.8 | Since there are two test frequency tables, one for default Tx-Rx spacing and another variable Tx-Rx spacing, new signalling test frequencies tables will be required to verify protocol related conformance. | Add a second table 6.2.3.8-1 for each band for variable Tx-Rx spacing. |
| TS 38.508-2 |  |  |
| Clause A.4.3 | A new item needs to be added to an appropriate ICS proforma table for variable Tx-Rx spacing capability | Add a new PICS in Table A.4.3-7 (CR has been submitted at this meeting for agreement) |
| TS 38.521-5 |  |  |
| Clause 5 | Clause 5.4.4 has been updated in TS 38.101-5. Clause 5 needs to be updated to reflect this change | Update Clause 5.4.4 (CR has been submitted at this meeting for agreement) |
| Clauses 7 | Existing clause 7.3.2 for reference sensitivity test cases only support test frequencies specified based on default Tx-Rx spacing. Variable Tx-Rx spacing is mandatory a mandatory capability from Rel-17. | Update Clauses 7.1, 7.3.2.3 to align with updates to the core specifications in 38.101-5 Add new test cases 7.3.3 for conformance of reference sensitivity performance with Tx-Rx frequency separation other than default (CR has been submitted at this meeting for agreement) |
| TS 38.522 |  |  |
| Clause 4 | Condition needs to be updated to include a new condition for the new test case | Update Table 4.0-1 (CR has been submitted at this meeting for agreement) |
| Clause 4.3.1 | Applicability of test cases related to variable Tx-Rx spacing for NTN bands need to be updated | Update Table 4.3.1-1 with the new test case specified in TS 38.521-5 (CR has been submitted at this meeting for agreement) |
| TS 38.533: No changes are required as R4 RRM specifications remain unimpacted as result of the introduction of support for variable Tx-Rx spacing |  |  |
| TS 38.523-1 |  |  |
| Clauses 8.1.4.4.6, 8.1.4.4.7 | NR NTN UE’s ability to receive UplinkConfigCommon in RRCConnectionReconfiguration during the CHO procedure for NR NTN UE needs to be verified. The current definition of RRCConnectionReconfiguration (Table 8.1.4.4.6.3.3-1) does not include UplinkConfigCommon IE of the target cells. | Update the Tables 8.1.4.4.6.3.3-5, 8.1.4.4.7.3.3-5 to include the uplinkConfigCommon IE and associated condition (CR has been submitted at this meeting) |
| TS 38.523-2 |  |  |
| Clause 4.1 | Applicability of test cases related to variable Tx-Rx spacing for NTN bands have to be updated to include specific PICS, PIXIT and number of TC executions | Update applicability tables Tables 4-1-1b, 4-1-3b The equivalent of agreement reached for eMTC/NB-IoT NTN will be applied for NR NTN as well at the next meeting. |
| Clause 4.2 | Condition of test cases may have to be updated | Update Applicability Conditions Table 4.2-1 The equivalent of agreement reached for eMTC/NB-IoT NTN will be applied for NR NTN as well at the next meeting. |
| TS 38.523-3 | No updates are required as the PIXIT, if agreed to, carries over from 36.523-3. |  |
| TR 38.903 | No impact since MU and TT remain unchanged. |  |
| TR 38.905 | Since core specification has already limited the number of test points to four, no additional assessment is required |  |

Proposal 1: Endorse the changes proposed as part of the above assessment and encourage contributions for the next meeting.

## 4.	Conclusion

Observation 1: Support for variable Tx-Rx spacing for NR NTN is mandatory from Rel-17 onwards.

Observation 2: Except for reference sensitivity requirements, all other requirements are to be tested with the default Tx-Rx separation.

Observation 3: The minimum reference sensitivity requirements for all Tx-Rx spacings is same as that for the default Tx-Rx spacing.

Observation 4: The demodulation and RRM requirements are unaffected by the introduction of variable Tx-Rx spacing.

Observation 5: The number of additional test points for reference sensitivity tests to verify support of variable Tx-Rx separation to four (4):

- smallest Tx-Rx separation, smallest channel bandwidth

- smallest Tx-Rx separation, largest channel bandwidth

- largest Tx-Rx separation, smallest channel bandwidth

- largest Tx-Rx separation, largest channel bandwidth

Proposal 1: Endorse the changes proposed as part of the above assessment and encourage contributions for the next meeting.

## 5.		References

[1]	RP-242181, RAN4 CRs to Maintenance for Closed WIs/SIs for REL-18 and earlier – Batch 33

[2]	TS 38.101-5 v18.8.0