3GPP TSG-RAN5 Meeting #107	R5-252600

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3503 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to clause 4.6.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the table 6.3.4.1-1 in 38.508-1, for NR NTN GSO scenario, the cellSpecificKoffset is 550ms. But the default values of parameters IE periodicityAndOffset and t300 in TS 38.508-1 do not consider the NR NTN RTT delays. IE periodicityAndOffset in SchedulingRequest : The current default value for this is sl10. For NR NTN GSO scenario, it takes about 550ms (RTT) for the whole procedure, but for the UE sending SR for UL grant, the default value is about 10ms (SCS=kHZ15). This results in the UE transmitting extra SRs before getting the UL grant for the first SR. Hence it is proposed that for NR NTN GSO scenario, taking the RTT (550ms) into account, change the periodicityAndOffset (SCS=kHZ15) from sl10 to sl640. IE T300: The current value of T300 is set to ms1000. In UE’s side, T300 starts when UE prepares to transmit RRCSetupRequest in RRC layer and stops when receive the RRCSetup message For NTN GSO scenario, the whole procedure (sending Prach, receiving RAR, sending RRSetupRequest and receiving RRCSetup) takes about 550ms *2 =1100ms. This is more than default value of ms1000. So the UE’s T300 expires before it receives the RRCSetup message from SS. Hence it is proposed to increase the T300 to ms2000 for NR NTN GSO scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In Table 4.6.3-157, set the periodicityAndOffset= sl640 for a new condition NR NTN GSO. - In Table 4.6.3-200, set t300=ms2000 for a new condition NR NTN GSO |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TTCN based on current Prose will fail a conformant UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 4.6.3	Radio resource control information elements

#### –	SchedulingRequestResourceConfig

Table 4.6.3-157: SchedulingRequestResourceConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequestResourceConfig ::= SEQUENCE { |  |  |  |
| schedulingRequestResourceId | SchedulingRequestResourceId |  |  |
| schedulingRequestID | SchedulingRequestId |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sl10 | 9 | With SCS = kHz15 results in repetition every 10 ms | SCS15 |
| sl640 | 639 | With SCS = kHz15 results in repetition every 640 ms | SCS15, NR NTN GSO |
| sl20 | 9 | With SCS = kHz30 results in repetition every 10 ms | SCS30 |
| sl40 | 19 | With SCS = kHz60 results in repetition every 10 ms | FR1 AND SCS60 |
|  | 9 | With SCS = kHz60 results in repetition every 10 ms | FR2 AND SCS60 |
| sl80 | 9 | With SCS = kHz120 results in repetition every 10 ms | SCS120 |
| } |  |  |  |
| resource | 0 | ID of the PUCCH resource as configured by PUCCH-Config (Table 4.6.3-112) |  |
|  | 6 | Same as the PUCCH resource for HARQ feedback indicated in Table 4.3.6.1.2.1-1 and Table 4.3.6.1.2.1-2 | SIG AND (NOT pc_twoPUCCH_AnyOthersInSlot) |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| NR NTN GSO | Applicable for NR NTN GSO scenario |

#### –	UE-TimersAndConstants

Table 4.6.3-200: UE-TimersAndConstants

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE-TimersAndConstants ::= SEQUENCE { |  |  |  |
| t300 | ms1000 |  |  |
|  | ms2000 |  | NR NTN GSO |
| t301 | ms1000 |  |  |
| t310 | ms1000 |  |  |
| n310 | n1 |  |  |
| t311 | ms30000 |  |  |
| n311 | n1 |  |  |
| t319 | ms1000 |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| NR NTN GSO | Applicable for NR NTN GSO scenario |