3GPP TSG-RAN5 Meeting #107	R5-252985

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | 0098 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | correction to minimum test time table for NB-NTN npdsch and npdcch test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM Europe Inc. - Italy |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is inconsistency in MNAS field value in the min test time table for NPDSCH and NPDCCH NB-NTN test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | Renamed the MNAS field name to accurately reflect the output from simulation results as per previously agreed proposals in R5-245748 and R5-247915 for NPDSCH and R5-250546 for NPDCCH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The calculations for deriving MNSF will remain ambiguous |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex G.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table G.3.5-1: Minimum Test time for NPDSCH for UE Category NB1

| Test No | Demodulation scenario (info only) | Minimum number of TB (Simulation) | Min Gap between consecutive NPDCCH taking into account RTT (ms) | Actual Gap between consecutive NPDCCH taking into account RTT (ms) | MNSF (Min No Sub frames) HD-FDD (Note2) |
| --- | --- | --- | --- | --- | --- |
| 8.3.1.1.1 Test 1 | R.NB.1 FDD (200kHz, QPSK,1/2) (1x1) NTN TDLC5-200 | 845 (Note 1) | 241 | 288 | 243360 |
| 8.3.1.1.1 Test 2 | R.NB.2 FDD (200kHz, QPSK,1/3) (1x1) NTN TDLA100-10 | 523 (Note 1) | 881 | 1152 | 602496 |
| 8.3.1.1.2 Test 1 | R.NB.6 FDD (200kHz, QPSK,1/2) (1x1) EPA5 | 683 (Note 1) | 489 | 576 | 393408 |
| 8.3.1.1.2 Test 2 | R.NB.6-1 FDD (200kHz, QPSK,1/2) (1x1) ETU1 | 440 (Note 1) | 1982 | 2304 | 1013760 |
| Note1: Minimum number of transport block (TB) has been derived by simulating the test scenario taking into account the minimum scheduling gap needed between successive NPDCCH/NPDSCH without considering RTT. Note2: For MNSF calculations, with the current assumption of SIB2/SIB31 scheduling every 640ms with max 8 subframes and max 2 repetition per SI window, 16 additional subframes are needed every 640ms to avoid collisions with NPDSCH. |  |  |  |  |  |

Table G.3.5-2: Minimum Test time for NPDSCH for UE Category NB2

| Test No | Demodulation scenario (info only) | Minimum number of TB (Simulation) | Min Gap between consecutive NPDCCH taking into account RTT (ms) | Actual Gap between consecutive NPDCCH taking into account RTT (ms) | MNSF (Min No Sub frames) HD-FDD (Note2) |
| --- | --- | --- | --- | --- | --- |
| 8.3.1.1.3 Test 1 | R.NB.7 FDD (200kHz, QPSK,1/2) (1x1) EPA5 | 2083 (Note 1) | 283 | 288 | 599904 |
| Note1: Minimum number of transport block (TB) has been derived by simulating the test scenario taking into account the minimum scheduling gap needed between successive NPDCCH/NPDSCH without considering RTT. Note2: For MNSF calculations, with the current assumption of SIB2/SIB31 scheduling every 640ms with max 8 subframes and max 2 repetition per SI window, 16 additional subframes are needed every 640ms to avoid collisions with NPDSCH. |  |  |  |  |  |

<<several sections skipped>>

## G.4.6	Minimum Test time

Table G.4.6-1: Minimum Test time for NPDCCH for UE Category NB1

| Test No | Demodulation scenario (info only) | Minimum number of TB (Simulation) | Min Gap between consecutive NPDCCH taking into account RTT (ms) | Actual Gap between consecutive NPDCCH taking into account RTT (ms) | MNSF (Min No Sub frames) HD-FDD (Note2) |
| --- | --- | --- | --- | --- | --- |
| 8.3.1.2.1 Test 1 | R.NB.3 FDD (200kHz, QPSK,1/2) (1x1) EPA5 | 489 (Note 1) | 1129 | 1280 | 625920 |
| 8.3.1.2.1 Test 2 | R.NB.3 FDD (200kHz, QPSK,1/2) (1x1) ETU1 | 85 (Note 1) | 7102 | 7680 | 652800 |
| Note1: Minimum number of transport block (TB) has been derived by simulating the test scenario taking into account the minimum scheduling gap needed between successive NPDCCH/NPDSCH without considering RTT. Note2: For MNSF calculations, with the current assumption of SIB2/SIB31 scheduling every 640ms with max 8 subframes and max 2 repetition per SI window, 16 additional subframes are needed every 640ms to avoid collisions with NPDSCH. |  |  |  |  |  |

<<end of changes>>