3GPP TSG-RAN5 Meeting #107	R5-253004

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 1163 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to 2AoA spherical coverage test |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR2_multiRX_DL-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Missing test case details for the 2AoA spherical coverage test case |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates made to the 2AoA spherical coverage test |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing test case details for the 2AoA spherical coverage test |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3K |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged sections skipped>

#### 7.3K.1.1	UE spherical coverage for simultaneous reception from multiple directions (2 AoAs)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

-	Measurement Uncertainties and Test Tolerances are FFS for all power classes.

-	Test Methodology is FFS for Power Class 6

7.3K.1.1.1	Test purpose

[To verify that the UE meets the spherical coverage requirements when receiving downlink from two directions or angle of arrivals].

7.3K.1.1.2	Test applicability

This test case applies to all types of release 18 and forward NR UEs that support simultaneousReceptionDiffTypeD-r16 and at least one of At least one of:

a.	singleDCI-SDM-scheme-r16 or

b.	multiDCI-MultiTRP-r16 and either of:

i.	overlapPDSCHsFullyFreqTime-r16.

ii.	overlapPDSCHsInTimePartiallyFreq-r16

7.3K.1.1.3	Minimum conformance requirements

7.3K.1.1.3.1	2AoA spherical coverage for power class 3

The requirements apply to the UE when tested in a test system as described in Annex L. The requirement is verified with the test metric of throughput (Link= 2AoA spherical coverage grid, Meas=Link Angle).

The spherical coverage requirement for simultaneous reception from multiple directions is defined in terms of the probability to support simultaneous reception of rank 2 PDSCH defined in sub-clause 7.3K.0. The probability (see Annex L) is defined as the spatial average over the full sphere around the UE of the probability of any one direction to support 2 AoA reception. In the applicable test system (see Annex L), the probability of any one direction of the UE to support 2 AoA reception for any specific AoA separation is the ratio of the number of unique AoA pairs that include that direction and can support 2 AoA reception to the total number of verified unique AoA pairs that include that direction.

The requirement applies only for the UE’s declared orientation in the positioner of the test system. The requirement for each AoA separation condition applies only for the UE’s declared orientation in the positioner of the test system for that AoA separation. The minimum required overall probability to support 2 AoA reception for power class 3 UEs for any channel bandwidth is specified by AoA separation in table 7.3K.3-1. The UE is only required to fulfil the requirement at any one of AoA separations declared from Table 7.3K.3-1.

Table 7.3K.3-1: Requirement for power class 3

| AoA separation (degrees) | Probability (%) |
| --- | --- |
| 30 | 18.5 |
| 60 | 13.5 |
| 90 | 12.5 |
| 120 | 20.5 |
| 150 | 28.5 |

7.3K.1.1.3.2	2AoA spherical coverage for power class 6

The requirements for a power class 6 UE are applicable with network signalling highSpeedDeploymentTypeFR2-r17 configured as bidirectional. UE spherical coverage evaluation areas are found in Table 6.2.1.6-3a in clause 6.2.1.6, by consisting of Area-1 and Area-2, in the reference coordinate system in Annex L.1. If one AoA is within Area-1 and another AoA is within Area-2, the 2AoA spherical coverage requirements apply with DL power specified in Table 7.3K.6-1 for the PDSCH of each AoA. For any AoA pair selected from Area-1 and Area-2, respectively, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels. The requirement is verified with a 150° angular separation between 2AoAs. The requirement is verified with the test metric of Throughput (Link=2AoA Spherical coverage grid, Meas=Link angle).

Table 7.3K.1.1.3.2-1: DL power for 2AoA spherical coverage requirement for power class 6

| Operating band | PDSCH DL power over UE spherical coverage evaluation areas (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -82.6 | -79.6 | -76.6 | -73.6 |
| n258 | -82.8 | -79.8 | -76.8 | -73.8 |
| n261 | -82.6 | -79.6 | -76.6 | -73.6 |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: The 2AoA spherical coverage requirements are verified only under normal thermal conditions as defined in TS 38.508-1 [10] subclause 4.1.1. NOTE 3: The requirements in this table are applicable with the network signalling highSpeedMeasFlagFR2-r17 configured as set2. |  |  |  |  |

The requirement shall be met for an uplink transmission using QPSK DFT-s-OFDM waveforms and for uplink transmission bandwidth less than or equal to that specified in Table 7.3.2.1-2.

Unless given by Table 7.3.2.1-3, the minimum requirements for 2AoA spherical coverage shall be verified with the network signalling value NS_200 (Table 6.2.3.1-1) configured.

The normative reference for this requirement is TS 38.101-2 [3] clause 7.3K.6.

7.3K.1.1.4	Test description

7.3K.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table [TBD]. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in Table [TBD]. The details of the uplink reference measurement channels (RMCs) are specified in Annexes [TBD]. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

The test configuration table is same as Table 7.3.2.4.1-1

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure [TBD] for TE diagram and Figure [TBD] for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause [TBD].

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	The UL Reference Measurement channels are set according to Table [TBD].

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.2.1.1.4.3

7.3K.1.1.4.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.3.2.4.1-1. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Tables 7.3.2.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

3.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200msec for the UE to reach PUMAX.

4.	Set the UE in the Rx beam peak direction found with a 3D EIS scan as performed in Annex K.1.2. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Rx beam selection to complete.

5.	Accurately introduce the 2 downlink signals from AoA1 and AoA2 on the selected polarization combination at each grid point as defined in Annex [TBD].

6.	FFS several steps on the throughput measurements on AoA1 and AoA2 for both mDCI and sDCI scenarios. The measurements have to be performed at each grid point.

7.	Calculate and record the multi-Rx reception RF performance result for the polarization combinations [TBD] including area weights [TBD]

8.	Calculate and perform the final multi-Rx reception RF performance metric as defined in test requirement. FFS - Details of metric need to be added here.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.1.2.

7.3K.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6 with TRANSFORM_PRECODER_ENABLED condition in Table 4.6.3-118 PUSCH-Config.

7.3K.1.1.5	Test requirement

If one AoA is within Area-1 and another AoA is within Area-2, the 2AoA spherical coverage test requirements in Table 7.3K.1.1.5-1 (for PC3) and Table 7.3K.1.1.5-2 (for PC6) apply with DL power specified in Table 7.3K.6-1 for the PDSCH of each AoA. For any AoA pair selected from Area-1 and Area-2, respectively, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels. The requirement is verified with a 150° angular separation between 2AoAs. The requirement is verified with the test metric of Throughput (Link=2AoA Spherical coverage grid, Meas=Link angle).

Table 7.3K.1.1.4.3-1: Test requirement for power class 3

| AoA separation (degrees) | Probability (%) |
| --- | --- |
| 30 | 18.5 |
| 60 | 13.5 |
| 90 | 12.5 |
| 120 | 20.5 |
| 150 | 28.5 |

Table 7.3K.1.1.4.3-1: DL power for 2AoA spherical coverage test requirement for power class 6

| Operating band | PDSCH DL power over UE spherical coverage evaluation areas (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -82.6 | -79.6 | -76.6 | -73.6 |
| n258 | -82.8 | -79.8 | -76.8 | -73.8 |
| n261 | -82.6 | -79.6 | -76.6 | -73.6 |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: The 2AoA spherical coverage requirements are verified only under normal thermal conditions as defined in TS 38.508-1 [10] subclause 4.1.1. NOTE 3: The requirements in this table are applicable with the network signalling highSpeedMeasFlagFR2-r17 configured as set2. |  |  |  |  |

<End of changes>