3GPP TSG-RAN5 Meeting #107	R5-253018

# St. Julian's, Malta, 19th May 2025 – 23rd May 2025

Agenda item:	6.5.6

Source: 	EchoStar

Title: 	Support of Variable Tx-Rx spacing for IoT NTN bands

Document for:	Discussion and Endorsement

## 1.	Introduction

During RAN plenary meeting TSGR#107, CRs [1] related to support for variable Tx-Rx separation for eMTC and NB-IoT NTN bands were approved and subsequently incorporated in the core specifications [2] and [3].

This paper summarizes updates made to the core specifications related to variable Tx-Rx frequency separation requirements, assesses the impact to various conformance specifications and proposes updates to the affected clauses of the affected specifications.

## 2.	Overview of Variable Tx-Rx spacing related updates to core specifications

Based on the approved CRs, Tables 5.4A.3-1, 5.4B.3-1 in [2] were updated as follows (updates are highlighted in yellow):

Table 5.4A.3-1: Default UE TX-RX frequency separation

| E-UTRA Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| 256 | 190 MHz1 161.4 to 218.6 MHz2 |
| 255 | -101.5 MHz1 -68.9 to -134.1 MHz2 |
| 254 | 873.5 MHz1 858.4 to 888.6 MHz2 |
| 253 | -150 MHz1 -144.4 to -155.6 MHz2 |
| NOTE 1: Default TX-RX separation. NOTE 2: The verification of flexible TX-RX frequency separation within this range is limited to reference sensitivity. Further details are specified in clause 7.3A. |  |

Table 5.4B.3-1: Default UE TX-RX frequency separation

| E-UTRA Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| 256 | 190 MHz1 160.2 to 219.8 MHz2 |
| 255 | -101.5 MHz1 -67.7 to -135.3 MHz2 |
| 254 | 873.5 MHz1 857.2 to 889.8 MHz2 |
| 253 | -150 MHz1 -143.2 to -156.8 MHz2 |
| NOTE 1: Default Tx-Rx separation. NOTE 2: The verification of flexible TX-RX frequency separation within this range is limited to reference sensitivity. Further details are specified in clause 7.3B. |  |

Furthermore, Tables 3A.4-1 and F.1-1 in [3] also note that the while the support for variable Tx-Rx separation is mandatory from Rel-18 onwards, Rel-17 NTN UEs are not required to support the capability.

Observation 1: Support for variable Tx-Rx spacing for eMTC/NTN is mandatory for Rel-18 but not required for Rel-17.

Observation 2: Support for variable Tx-Rx spacing for NB-IoT/NTN is mandatory for Rel-18 but not required for Rel-17.

Additionally, to limit the scope of RF conformance testing resulting from wide range of Tx-Rx separation, Clauses 7.1, 7.3A and 7.3B in [2] specify that only reference sensitivity requirements need to be tested for conformance with Tx-Rx separation values other than the default value. The minimum reference sensitivity requirements for the NTN band remain unchanged from that determined for the default Tx-Rx separation. There are no new demodulation performance and RRM requirements and existing ones remain unaffected resulting from introduction of variable Tx-Rx spacing.

Observation 3: Except for reference sensitivity requirements, all other RF requirements are to be tested with the default Tx-Rx separation.

Observation 4: The minimum reference sensitivity requirements for all Tx-Rx spacings is same as that for the default Tx-Rx spacing.

Observation 5: The demodulation and RRM performance requirements are unaffected by the introduction of variable Tx-Rx spacing.

Finally, Tables 7.3A-4 and 7.3B-4 in [2] identify the additional test points for RF reference sensitivity tests to verify UE’s support of variable Tx-Rx separation.

Table 7.3A-4: TX – RX carrier centre frequency separation for REFSENS verification

| E-UTRA Operating Band | Channel bandwidth | TX – RX carrier centre frequency separation for REFSENS verification |
| --- | --- | --- |
| 256 | 1.4 MHz | 161.4 MHz, 218.6 MHz |
| 255 | 1.4 MHz | -68.9 MHz, -134.1 MHz |
| 254 | 1.4 MHz | 858.4 MHz, 888.6 MHz |
| 253 | 1.4 MHz | -144.4 MHz, -155.6 MHz |

Table 7.3B-2: TX – RX carrier centre frequency separation for REFSENS verification

| E-UTRA Operating Band | Channel bandwidth | TX – RX carrier centre frequency separation for REFSENS verification |
| --- | --- | --- |
| 256 | 0.2 MHz | 160.2 MHz, 219.8 MHz |
| 255 | 0.2 MHz | -67.7 MHz, -135.3 MHz |
| 254 | 0.2 MHz | 857.2 MHz, 889.8 MHz |
| 253 | 0.2 MHz | -143.2 MHz, -156.8 MHz |

Observation 6: The number of additional test points for reference sensitivity tests to verify support of variable Tx-Rx separation is limited to two (2):

1) smallest Tx-Rx separation and

2) largest Tx-Rx separation

So far, since test frequencies were always specified with default Tx-Rx spacing, none of the test cases include ul-CarrierInfo IE in SIB or mobility signalling messages.

Observation 7: Relevant signalling aspects such as reception and implementation of ul-CarrierInfo IE received in signalling during NTN cell selection and mobility test cases need to be verified.

## 3.	SIG Conformance Specifications Impact Assessment and Proposals

| TS & Clause | Assessment | Proposals |
| --- | --- | --- |
| TS 36.508 |  |  |
| Clauses 4.4.3.3, 8.1.4.3.3 | A condition needs to be added to ensure that the SIB2 includes the ul-CarrierFreq IE so that UE can tune and transmit using the correct UL frequency | Update the SIB2 and SIB2-NB clauses to include a new condition (CR has been submitted at this meeting for agreement) |
| Clause 4.6.5 | A condition needs to be added to ensure that ul-CarrierFreq IE is included as part of the eMTC / NTN handover test procedures | Update MobilityControlInfo-HO IE (CR has been submitted at this meeting for agreement) |
| Clauses 6.2.3.1, 8.3.2.3.1 | Since there are two test frequency tables, one for default Tx-Rx spacing and another variable Tx-Rx spacing, new signalling test frequencies tables will be required | Add new test frequency table for signalling tests with variable Tx-Rx separation (CR has been submitted at this meeting for agreement) |
| TS 36.523-1: No new test cases are introduced. Existing test cases with appropriate enabling of relevant IEs (e.g., ul-CarrierFreq IE in SIB2, SIB2-NB, MobilityControlInfo) are sufficient to verify the signalling protocol associated with variable Tx-Rx frequency separation |  |  |
| TS 36.523-2 |  |  |
| Clause 4 | It is important that the proper execution of the signalling protocol is verified when variable Tx-Rx frequency separation is configured and signalled. Accordingly, a new note for relevant test cases needs to be specified | Update Table 4-1 with a new Note that indicates whether the TC needs to be executed for only default Tx-Rx separation or for both default and variable Tx-Rx separation configuration to ensure protocol implementation conforms to the protocol specs or not. (CR has been submitted at this meeting for agreement) |
| Clause A.4.4 | A new item needs to be added to an appropriate ICS table to indicate whether UE supports variable Tx-Rx spacing or not. | Add a new PICS in Table A.4.4-1 (CR has been submitted at this meeting for agreement) |
| TS 36.523-3 |  |  |
| Clauses 9.1, 9.3 | Further discussion how to indicate whether an existing test case in TS 523-1 needs to executed both using signalling test frequencies specified for default Tx-Rx separation and variable Tx-Rx separation and whether a PIXIT needs to be specified. | Discuss and reach consensus on whether a new PIXIT needs to be specified or not or just the note introduced in Clause 4 is adequate. |

Observation 8: Handover protocol test cases for eMTC NTN using default Tx-Rx spacing have not yet been specified.

Proposal 1: Endorse the changes proposed as part of the above assessment.

Proposal 2: Defer handover related protocol conformance testing (clauses 7 and 8 of TS 36.523-1) for variable Tx-Rx spacing for eMTC / NTN until tests for default Tx-Rx spacing are specified.

Proposal 3: Reach consensus on whether PIXIT needs to be specified or not to enable protocol conformance testing for default and non-default Tx-Rx spacings.

## 4.	Conclusion

Observation 1: Support for variable Tx-Rx spacing for eMTC/NTN is mandatory for Rel-18 but not required for Rel-17.

Observation 2: Support for variable Tx-Rx spacing for NB-IoT/NTN is mandatory for Rel-18 but not required for Rel-17.

Observation 3: Except for reference sensitivity requirements, all other requirements are to be tested with the default Tx-Rx separation.

Observation 4: The minimum reference sensitivity requirements for all Tx-Rx spacings is same as that for the default Tx-Rx spacing.

Observation 5: The demodulation and RRM performance requirements are unaffected by the introduction of variable Tx-Rx spacing.

Observation 6: The number of additional test points for reference sensitivity tests to verify support of variable Tx-Rx separation is limited to two (2):

1) smallest Tx-Rx separation and

2) largest Tx-Rx separation

Observation 7: Relevant signalling aspects such as reception and implementation of ul-CarrierInfo IE received in signalling during NTN cell selection and mobility test cases need to be verified.

Observation 8: Handover protocol test cases for eMTC NTN using default Tx-Rx spacing have not yet been specified.

Proposal 1: Endorse the changes proposed as part of the above assessment.

Proposal 2: Defer handover related protocol conformance testing (clauses 7 and 8 of TS 36.523-1) for variable Tx-Rx spacing for eMTC / NTN until tests for default Tx-Rx spacing are specified.

Proposal 3: Reach consensus on whether PIXIT needs to be specified or not to enable protocol conformance testing for default and non-default Tx-Rx spacings.

## 5.		References

[1]	RP-250600, RAN4 CRs to Maintenance for Closed WIs/SIs for REL-18 and earlier – Batch 23

[2]	TS 36.102 v18.8.0

[3]	TS 36.307 v18.8.0