3GPP TSG-RAN WG5 Meeting #107	R5-253087

# St. Julian’s, Malta, 19th May 2025 – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4989 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to message content for handover tests cases to verify protocol aspects of variable Tx-Rx spacing for NR NTN bands |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | EchoStar |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN2 specifications allow configuration UL and DL center frequency independently. Normally RAN4 defines minimum requirements with a single default Tx-Rx separation. Satellite industry has a strong need to use different Tx-Rx spacing in different deployments. Therefore, RAN4 specified variable Tx-Rx spacing for NTN bands and added a requirement to validate those different Tx-Rx spacing for reference sensitivity test case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates to message contents to include uplink frequency info IE to CellGroupConfig IE to verify proper operation of the NTN UE when variable Tx-Rx spacing is configured |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Protocol conformance of variable Tx-Rx spacing during handover cannot be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.4.6.3.3, 8.1.4.4.7.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: i) Updated based on comment received from TF160 expert |  |  |  |  |  |  |  |  |

8.1.4.4.6.3.3	Specific message contents

<<Unchanged text skipped>>

Table 8.1.4.4.6.3.3-5: CellGroupConfig (Table 8.1.4.4.6.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with Condition PCell_change and CFRA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| spCellConfigCommon SEQUENCE { |  |  |  |
| physCellId | Physical Cell Identity of NR Cell 2 |  | PCI_NR cell 2 |
|  | Physical Cell Identity of NR Cell 4 |  | PCI_NR Cell 4 |
| } |  |  |  |
| uplinkConfigCommon SEQUENCE { |  |  |  |
| frequencyInfoUL | Uplink Frequency Info of NR Cell 2 | When test performed using variable Tx-Rx freq separation test frequenices | NTN_Variable_TxTx_Separation |
|  | Uplink Frequency Info of NR Cell 4 | When test performed using variable Tx-Rx freq separation test frequenices | NTN_Variable_TxTx_Separation |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| NTN_Variable_TxTx_Separation | Applicable for NTN bands test with signalling test frequencies specified for variable Tx-Rx frequency separation |

<< NEXT CHANGE >>

8.1.4.4.7.3.3	Specific message contents

Table 8.1.4.4.7.3.3-5: CellGroupConfig (Table 8.1.4.4.7.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with Condition PCell_change and CFRA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| spCellConfigCommon SEQUENCE { |  |  |  |
| physCellId | Physical Cell Identity of NR Cell 2 |  | PCI_NR cell 2 |
|  | Physical Cell Identity of NR Cell 4 |  | PCI_NR Cell 4 |
| } |  |  |  |
| uplinkConfigCommon { |  |  |  |
| frequencyInfoUL | Uplink Frequency Info of NR Cell 2 | When test performed using variable Tx-Rx freq separation test frequenices | NTN_Variable_TxTx_Separation |
|  | Uplink Frequency Info of NR Cell 4 | When test performed using variable Tx-Rx freq separation test frequenices | NTN_Variable_TxTx_Separation |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| NTN_Variable_TxTx_Separation | Applicable for NTN bands test with signalling test frequencies specified for variable Tx-Rx frequency separation |

<< END OF CHANGES >>