3GPP TSG-RAN5 Meeting #107 	R5-253235

# St Julian's, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-5 | CR | 0097 | rev | 2 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of active UL slots and RMC for RF tests of FR1 NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Due to lack of HARQ processes for PUSCH/PDSCH and considering CellSpecificKoffset, all Uplink/Downlink slots cannot be activated for NTN, even in case of FDD. - There are no RMC tables for TRx tests with valid scheduling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added tables in A.2.1 to represents active Uplink slots for NTN testing. - Added tables in A.3.4 to represents RMCs for NTN TRx testing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Invalid scheduling will be used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2.1, A.3.3 (new), A.3.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Depending on RAN4 CR R4-2508913 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-252514 and the outcome of 1 revision to this document. r1: A.3.3 was changed from void to blank. This is an update of R5-253616 and the outcome of 1 revision to this document. r1: Updates by replacing the corresponding RAN4 CR with R4-2508913. |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

A.2.1	General

The measurement channels in the following subclauses are defined to derive the requirements in clause 6 (Transmitter Characteristics) and clause 7 (Receiver Characteristics). The measurement channels represent example configurations of physical channels for different data rates.

The measurement channels in the following clauses are applicable only to FDD.

The active uplink slots for FR1-NTN FDD configurations for NGSO are specified in Table A.2.1-1. FR1-NTN FDD slot patterns defined for reference sensitivity tests will be used for FR1-NTN FDD UL RMCs, unless otherwise stated.

Table A.2.1-1: FR1-NTN FDD active uplink slots for NGSO

| SCS | Active Uplink slots |
| --- | --- |
| 15 kHz | 0,1,2,3,4,5,8,9 in every radio frame |
| 30 kHz | FFS |
| 60 kHz | FFS |
| NOTE 1: Due to lack of HARQ processes for PUSCH and considering CellSpecificKoffset, all Uplink slots cannot be activated for NTN. NOTE 2: Assuming K2 is 2, CellSpecificKoffset is 14 |  |

The active uplink slots for FR1-NTN FDD configurations for GSO are specified in Table A.2.1-2. FR1-NTN FDD slot patterns defined for reference sensitivity tests will be used for FR1-NTN FDD UL RMCs, unless otherwise stated.

Table A.2.1-2: FR1-NTN FDD active uplink slots for GSO

| SCS | Active Uplink slots |
| --- | --- |
| 15 kHz | 262,263,264,265,266,267,268,269, 272,273,274, 275, 276, 277, 278, 279 in every 32 radio frame |
| 30 kHz | FFS |
| 60 kHz | FFS |
| NOTE 1: The active slots are counted from the 0th slot in the 1st radio frame of the periodicity. NOTE 2: Due to lack of HARQ processes for PUSCH and considering CellSpecificKoffset, all Uplink slots cannot be activated for NTN. NOTE 3: Assuming K2 is 2, CellSpecificKoffset is 258 |  |

<<Unchanged sections skipped>>

Table A.3.2.1.3-5: PDSCH Reference Channel for FDD (64QAM)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 50 | 100 | 200 | 400 |
| Subcarrier spacing configuration |  | 3 | 3 | 3 | 3 |
| Allocated resource blocks |  | 32 | 66 | 132 | 264 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 |
| Allocated slots per Frame (NOTE 6) |  | 47/48 | 47/48 | 47/48 | 47/48 |
| MCS index |  | 19 | 19 | 19 | 19 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 1/2 | 1/2 | 1/2 | 1/2 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | Bits | 9992 | 20496 | 40976 | 81976 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | CBs | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | CBs | 2 | 3 | 5 | 10 |
| Binary Channel Bits Per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | Bits | 19872 | 40986 | 81972 | 163944 |
| Max. Throughput averaged over 1 frame (NOTE 7) | Mbps | 47.962 | 98.381 | 196.685 | 393.485 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 of each frame. NOTE 4: Slot i is slot index per frame. NOTE 5: PTRS is configured on symbols containing PDSCH with 1 port, per 2PRB in frequency domain, per symbol in time domain. Overhead for TBS calculation is assumed to be 6. NOTE 6: First number corresponds to the number slots allocated in the first frame of the RMC; second number corresponds to the number slots allocated in the second frame of the RMC. NOTE 7: Throughput is averaged over 2nd frame of RMC. |  |  |  |  |  |

A.3.3

A.3.4	Reference measurement channels for receiver requirements

A.3.4.1	FDD

A.3.4.1.1	Fixed reference channels for SCS 15kHz FR1-NTN

In addition to general description, no user data is scheduled on slot #1 within 20 ms in order to avoid SIB and PDSCH transmissions in one slot and simplify test configuration.

Table A.3.4.1.1-1: Fixed reference channel for receiver requirements (SCS 15kHz, FDD, QPSK 1/3, NGSO)

| Parameter | Unit | Value |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 |  |  |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 |  |  |  |  |
| Subcarrier spacing configuration |  | 0 | 0 | 0 | 0 |  |  |  |  |
| Allocated resource blocks |  | 25 | 52 | 79 | 106 |  |  |  |  |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 |  |  |  |  |
| Allocated slots per 2 Radio Frame |  | 16 | 16 | 16 | 16 |  |  |  |  |
| MCS Index |  | 4 | 4 | 4 | 4 |  |  |  |  |
| MCS Table for TBS determination |  | 64QAM |  |  |  |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK |  |  |  |  |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 1/3 |  |  |  |  |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0, 1, 10, 11 | Bits | N/A | N/A | N/A | N/A |  |  |  |  |
| For Slots 2, …, 9, 12, …, 19 | Bits | 1672 | 3368 | 5120 | 6912 |  |  |  |  |
| Transport block CRC | Bits | 16 | 16 | 24 | 24 |  |  |  |  |
| LDPC base graph |  | 2 | 2 | 1 | 1 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0, 1, 10, 11 | CBs | N/A | N/A | N/A | N/A |  |  |  |  |
| For Slots 2, …, 9, 12, …, 19 | CBs | 1 | 1 | 1 | 1 |  |  |  |  |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |
| For Slots0, 1, 10, 11 | Bits | N/A | N/A | N/A | N/A |  |  |  |  |
| For Slots 2, …, 9, 12, …, 19 | Bits | 5400 | 11232 | 17064 | 22896 |  |  |  |  |
| Max. Throughput averaged over 2 frame | Mbps | TBD | TBD | TBD | TBD |  |  |  |  |
| Note 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1 from TS 38.101-1 [5]. Note 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 3: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms. Note 4: Slot i is slot index per 2 frames. Note 5: PDSCHs are scheduled from 1st frame of the periodicity. |  |  |  |  |  |  |  |  |  |

Table A.3.4.1.1-2: Fixed reference channel for receiver requirements (SCS 15kHz, FDD, 64QAM, NGSO)

| Parameter | Unit | Value |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 |  |  |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 |  |  |  |  |
| Subcarrier spacing configuration |  | 0 | 0 | 0 | 0 |  |  |  |  |
| Allocated resource blocks |  | 25 | 52 | 79 | 106 |  |  |  |  |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 |  |  |  |  |
| Allocated slots per 2 Radio Frame |  | 16 | 16 | 16 | 16 |  |  |  |  |
| MCS Index |  | 24 | 24 | 24 | 24 |  |  |  |  |
| MCS Table for TBS determination |  | 64QAM |  |  |  |  |  |  |  |
| Modulation |  | 64 QAM | 64 QAM | 64 QAM | 64 QAM |  |  |  |  |
| Target Coding Rate |  | 3/4 | 3/4 | 3/4 | 3/4 |  |  |  |  |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0, 1, 10, 11 | Bits | N/A | N/A | N/A | N/A |  |  |  |  |
| For Slots 2, …, 9, 12, …, 19 | Bits | 12296 | 25608 | 38936 | 52224 |  |  |  |  |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 |  |  |  |  |
| LDPC base graph |  | 1 | 1 | 1 | 1 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0, 1, 10, 11 | CBs | N/A | N/A | N/A | N/A |  |  |  |  |
| For Slots 2, …, 9, 12, …, 19 | CBs | 2 | 4 | 5 | 7 |  |  |  |  |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |
| For Slots0, 1, 10, 11 | Bits | N/A | N/A | N/A | N/A |  |  |  |  |
| For Slots 2, …, 9, 12, …, 19 | Bits | 16200 | 33696 | 51192 | 68688 |  |  |  |  |
| Max. Throughput averaged over 2 frame | Mbps | TBD | TBD | TBD | TBD |  |  |  |  |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1 from TS 38.101-1 [5]. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms. NOTE 4: Slot i is slot index per 2 frames. NOTE 5: PDSCHs are scheduled from 1st frame of the periodicity. |  |  |  |  |  |  |  |  |  |

Table A.3.4.1.1-3: Fixed reference channel for receiver requirements (SCS 15kHz, FDD, QPSK 1/3, GSO)

| Parameter | Unit | Value |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 |  |  |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 |  |  |  |  |
| Subcarrier spacing configuration |  | 0 | 0 | 0 | 0 |  |  |  |  |
| Allocated resource blocks |  | 25 | 52 | 79 | 106 |  |  |  |  |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 |  |  |  |  |
| Allocated slots per 32 Radio Frame |  | 16 | 16 | 16 | 16 |  |  |  |  |
| MCS Index |  | 4 | 4 | 4 | 4 |  |  |  |  |
| MCS Table for TBS determination |  | 64QAM |  |  |  |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK |  |  |  |  |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 1/3 |  |  |  |  |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0, 1, 10, 11, 20, …, 319 | Bits | N/A | N/A | N/A | N/A |  |  |  |  |
| For Slots 2, …, 9, 12, …, 19 | Bits | 1672 | 3368 | 5120 | 6912 |  |  |  |  |
| Transport block CRC | Bits | 16 | 16 | 24 | 24 |  |  |  |  |
| LDPC base graph |  | 2 | 2 | 1 | 1 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0, 1, 10, 11, 20, …, 319 | CBs | N/A | N/A | N/A | N/A |  |  |  |  |
| For Slots 2, …, 9, 12, …, 19 | CBs | 1 | 1 | 1 | 1 |  |  |  |  |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0, 1, 10, 11, 20, …, 319 | Bits | N/A | N/A | N/A | N/A |  |  |  |  |
| For Slots 2, …, 9, 12, …, 19 | Bits | 5400 | 11232 | 17064 | 22896 |  |  |  |  |
| Max. Throughput averaged over 2 frame | Mbps | TBD | TBD | TBD | TBD |  |  |  |  |
| Note 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1 from TS 38.101-1 [5]. Note 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 3: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms. Note 4: Slot i is slot index per 32 frames. Note 5: PDSCHs are scheduled from 1st frame of the periodicity. |  |  |  |  |  |  |  |  |  |

Table A.3.4.1.1-4: Fixed reference channel for receiver requirements (SCS 15kHz, FDD, 64QAM, GSO)

| Parameter | Unit | Value |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 |  |  |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 |  |  |  |  |
| Subcarrier spacing configuration |  | 0 | 0 | 0 | 0 |  |  |  |  |
| Allocated resource blocks |  | 25 | 52 | 79 | 106 |  |  |  |  |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 |  |  |  |  |
| Allocated slots per 32 Radio Frame |  | 16 | 16 | 16 | 16 |  |  |  |  |
| MCS Index |  | 24 | 24 | 24 | 24 |  |  |  |  |
| MCS Table for TBS determination |  | 64QAM |  |  |  |  |  |  |  |
| Modulation |  | 64 QAM | 64 QAM | 64 QAM | 64 QAM |  |  |  |  |
| Target Coding Rate |  | 3/4 | 3/4 | 3/4 | 3/4 |  |  |  |  |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0, 1, 10, 11, 20, …, 319 | Bits | N/A | N/A | N/A | N/A |  |  |  |  |
| For Slots 2, …, 9, 12, …, 19 | Bits | 12296 | 25608 | 38936 | 52224 |  |  |  |  |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 |  |  |  |  |
| LDPC base graph |  | 1 | 1 | 1 | 1 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0, 1, 10, 11, 20, …, 319 | CBs | N/A | N/A | N/A | N/A |  |  |  |  |
| For Slots 2, …, 9, 12, …, 19 | CBs | 2 | 4 | 5 | 7 |  |  |  |  |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0, 1, 10, 11, 20, …, 319 | Bits | N/A | N/A | N/A | N/A |  |  |  |  |
| For Slots 2, …, 9, 12, …, 19 | Bits | 16200 | 33696 | 51192 | 68688 |  |  |  |  |
| Max. Throughput averaged over 2 frame | Mbps | TBD | TBD | TBD | TBD |  |  |  |  |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1 from TS 38.101-1 [5]. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 with 20ms periodicity. NOTE 4: Slot i is slot index per 32 frames. NOTE 5: PDSCHs are scheduled from 1st frame of the periodicity. |  |  |  |  |  |  |  |  |  |

<<End of change>>