3GPP TSG-RAN5 Meeting #107	R5-251844r1

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.522 | CR | 0580 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to R18 LTM test applicability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Germany GmbH |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The subtest of LTM in Table 4.0-4 dosent consider “Uniqueness” compared to conformance conditions in Table 4.0-1. This Tdoc submitted for updating/adding the LTM subtests and conformance conditions. - The test applicabilities of FR2 LTM are still FSS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the subtest in Table 4.0-1: F029~F042 - Add the subtest in Table 4.0-1: Fxx1 - Update the conformance condition in Table 4.0-4: C357, C358, C362, C363, C381, C382, C383 - Update FR1 LTM test applicability in Table 4.2-3: TC_6.3.4.1, 6.3.4.2, 6.3.4.3, 6.3.4.4, 6.6.28.1 - Update FR2 LTM test cases in Table 4.2-4: TC_7.3.4.1, 7.3.4.2, 7.3.4.3, 7.6.20.1, 7.6.21.1, 7.7.15.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test applicability of LTM test cases are incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.0, 4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.533 CR 3869 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: Updated the title of LTM test cases with the new name - FR1 TCs in Table 4.0-4: 6.3.2.4.2, 6.3.4.1, 6.3.4.2, 6.3.4.3, 6.3.4.4, 6.6.26.1, 6.6.27.1, 6.6.28.1, 6.7.17.1.1 - FR2 TCs in Table 4.2-4: 7.3.4.1, 7.3.4.2, 7.3.4.3 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

Table 4.0-1: Applicability of conformance test cases conditions

| C001 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 THEN R ELSE N/A |  |
| --- | --- |
| … |  |
| C356 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/33 AND A.4.3.2-1/212 AND (NOT A.4.3.9-1/2 AND A.4.3.1-7a/3) THEN R ELSE N/A |
| C357 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/191 AND A.4.3.2-1/181 AND A.4.3.6-1/102 AND A.4.3.2-1/196 THEN R ELSE N/A |
| C358 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/191 AND A.4.3.2-1/181 AND A.4.3.6-1/102 THEN R ELSE N/A |
| C359 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/196 THEN R ELSE N/A |
| C360 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.6/97 THEN R ELSE N/A |
| C361 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/195 THEN R ELSE N/A |
| C361a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/8 AND A.4.1-3/1 AND A.4.3.2-1/195 THEN R ELSE N/A |
| C362 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/191 AND A.4.3.1-1/196 THEN R ELSE N/A |
| C363 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.6-1/100 AND A.4.3.6-1/97 THEN R ELSE N/A |
| C364 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/191 AND A.4.3.2-1/178 AND A.4.3.2-1/183 AND A.4.3.2-1/186 AND A.4.3.6-1/102 AND A.4.3.2-1/196 THEN R ELSE N/A |
| C365 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/183 AND A.4.3.2-1/186 AND A.4.3.2-1/191 AND A.4.3.2-1/178 AND A.4.3.6-1/102 THEN R ELSE N/A |
| C366 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/178 AND A.4.3.2-1/183 AND A.4.3.2-1/184 AND A.4.3.2-1/185 AND A.4.3.2-1/186 AND A.4.3.6-1/102 AND A.4.3.2-1/194 THEN R ELSE N/A |
| C367 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.13-1/6 THEN R ELSE N/A |
| C368 IF A.4.1-1/1 AND A.4.1-2/7 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.1-7a/1 AND (NOT A.4.3.1-7a/2) AND A.4.3.2-1/208 THEN R ELSE N/A |
| C369 IF A.4.1-1/1 AND A.4.1-2/7 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.1-7a/2 AND A.4.3.2-1/208 THEN R ELSE N/A |
| C370 IF A.4.1-1/2 AND A.4.1-2/7 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.1-7a/1 AND (NOT A.4.3.1-7a/2) AND A.4.3.2-1/172 THEN R ELSE N/A |
| C371 IF A.4.1-1/1 AND A.4.1-2/7 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.1-7a/2 AND A.4.3.2-1/172 THEN R ELSE N/A |
| C372 IF (A.4.1-1/1 OR A.4.1-1/2 OR A.4.3.12-1/5) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.12A-1/1 THEN R ELSE N/A |
| C372a IF A.4.1-1/1 AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.1-7a/4 AND A.4.3.12A-1/1 THEN R ELSE N/A |
| C372b IF A.4.1-1/1 AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.1-7a/4 AND A.4.3.12A-1/1 AND A.4.3.12A-1/2 THEN R ELSE N/A |
| C372c IF A.4.1-1/1 AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.1-7a/1 AND A.4.3.12A-1/1 THEN R ELSE N/A |
| C372d IF A.4.1-1/1 AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.1-7a/1 AND A.4.3.12A-1/1 AND A.4.3.12A-1/2 THEN R ELSE N/A |
| C372e IF A.4.1-1/2 AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.1-7a/4 AND A.4.3.12A-1/1 THEN R ELSE N/A |
| C372f IF A.4.1-1/2 AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.1-7a/4 AND A.4.3.12A-1/1 AND A.4.3.12A-1/2 THEN R ELSE N/A |
| C372g IF A.4.1-1/2 AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.1-7a/1 AND A.4.3.12A-1/1 THEN R ELSE N/A |
| C372h IF A.4.1-1/2 AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.1-7a/1 AND A.4.3.12A-1/1 AND A.4.3.12A-1/2 THEN R ELSE N/A |
| C373 IF A.4.1-1/1 AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.9-1/6 AND (A.4.3.1-7a/1 AND NOT A.4.3.1-7a/2) THEN R ELSE N/A |
| C374 IF A.4.1-1/2 AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.9-1/6 AND (A.4.3.1-7a/1 AND NOT A.4.3.1-7a/3) THEN R ELSE N/A |
| C375 IF A.4.1-1/1 AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.9-1/6 AND (NOT A.4.3.9-1/2 AND A.4.3.1-7a/2) THEN R ELSE N/A |
| C376 IF A.4.1-1/2 AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.9-1/6 AND (NOT A.4.3.9-1/2 AND A.4.3.1-7a/3) THEN R ELSE N/A |
| C377 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND A.4.3.9-1/>6 THEN R ELSE N/A |
| C378 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND A.4.3.2-1/227 THEN R ELSE N/A |
| C379 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND A.4.3.2-1/53 AND A.4.3.2-1/56 AND A.4.3.2-1/78 AND A.4.3.2-1/226 AND A.4.3.2-1/227 THEN R ELSE N/A |
| C380 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND A.4.3.2-1/56 AND A.4.3.2-1/78 AND A.4.3.2-1/226 AND A.4.3.2-1/227 AND A.4.3.2-1/41 THEN R ELSE N/A |
| C381 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/8 AND A.4.1-3/1 AND A.4.3.2-1/191 AND A.4.3.2-1/196 THEN R ELSE N/A |
| C382 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/8 AND A.4.1-3/1 AND A.4.3.2-1/191 AND A.4.3.2-1/181 AND A.4.3.6-1/102 AND A.4.3.2-1/196 THEN R ELSE N/A |
| C383 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/8 AND A.4.1-3/1 AND A.4.3.6-1/100 AND A.4.3.6-1/97 THEN R ELSE N/A |
| C384 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND A.4.3.2-1/196 THEN R ELSE N/A |
| C385 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND A.4.3.6/97 THEN R ELSE N/A |
| C386 IF A.4.1-1/1 AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/151 AND A.4.3.5-1/1 THEN R ELSE N/A |
| C387 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/3 AND A.4.3.2B.2.0-1A /2 AND A.4.3.2B.2.3.1-1/1 AND A.4.3.2-1/231 THEN R ELSE N/A |
| C388 IF (A.4.1-4/4 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.2-1/174 THEN R ELSE N/A |
| C389 IF A.4.1-1/2 AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.1-4A/5 AND A.4.3.2A.1-2/3 AND A.4.3.2A.4.3-2/1 AND A.4.3.2-1/127a AND A.4.3.6-1/80 AND A.4.3.2-1/142 THEN R ELSE N/A |
| NOTE 1: Cxxxx applicability is defined for enhanced type 1 receiver for NR related tests (A.4.3.9-1/1). NOTE 2: Cxxxy applicability is defined for alternative additional DMRS position for co-existence with LTE CRS related tests (A.4.3.2-1/20). NOTE 3: Cxxxz applicability is defined for modified MPR behaviour related test (A.4.3.2-1/25). NOTE 4: Cxxxw applicability is defined for mpr Power Boost related test (A.4.3.2-1/38). |  |

<End of modified section 1>

<Start of modified section 2>

Table 4.0-4: Subtest Selection Criteria for performance and RRM test cases

| Code | Tested Bands Selection Criteria | Comment |
| --- | --- | --- |
| F001 | A.4.3.2-1/9 | UEs supporting CSI-RS based PRACH |
| F002 | A.4.3.5-1/1 | UEs supporting long DRX cycle |
| F003 to F013 | Void |  |
| F014 | A.4.3.6-1/2 AND A.4.3.6-1/92 | UEs supporting per-FR gap and Gap Pattern 13 |
| F015 | A.4.3.5-1/1 AND A.4.3.6-1/2 AND A.4.3.6-1/92 | UEs supporting per-FR gap and long DRX cycle and Gap Pattern 13 |
| F016 | A.4.3.6-1/2 AND A.4.3.6-1/21 | UEs supporting per-FR gap and Gap Pattern 4 |
| F017 | Void |  |
| F018 | Void |  |
| F019 | A.4.3.5-1/1 AND A.4.3.6-1/2 AND A.4.3.6-1/21 | UEs supporting per-FR gap and long DRX cycle and Gap Pattern 4 |
| F020 | Void |  |
| F021 | Void |  |
| F022 | A.4.3.2-1/20 | UE supporting alternative additional DMRS position for co-existence with LTE CRS |
| F023 | A.4.3.2-1/2 | UE supporting 256QAM for PDSCH for FR1 |
| F024 | A.4.3.6-1/70 | UE supporting per-FR NCSG |
| F025 | A.4.3.2-1/213 | UE supporting crs-IM-nonDSS-15kHzSCS-r17 |
| F026 | A.4.3.2-1/214 | UE supporting crs-IM-nonDSS-NWA-15kHzSCS-r17 |
| F027 | A.4.3.2-1/213 OR A.4.3.2-1/215 | UE supporting crs-IM-nonDSS-15kHzSCS-r17 OR crs-IM-nonDSS-30kHzSCS-r17 |
| F028 | A.4.3.2-1/214 OR A.4.3.2-1/216 | UE supporting crs-IM-nonDSS-NWA-15kHzSCS-r17 OR crs-IM-nonDSS-NWA-30kHzSCS-r17 |
| F029 | A.4.3.2-1/203 | UEs supporting simultaneous L1-RSRP measurements for more than one cell when the max RTD among the cells is larger than CP length |
| F030 | Void |  |
| F031 | A.4.3.2-1/179 AND NOT (A.4.3.2-1/178) | UEs supporting MAC-CE activated separate LTM TCI states and do not support MAC-CE activated joint LTM TCI states |
| F032 | A.4.3.2-1/175 AND NOT (A.4.3.2-1/178 OR A.4.3.2-1/179) | UEs supporting unified TCI with joint DL/UL LTM TCI-state indication for LTM procedure and do not support MAC-CE activated joint or separate LTM TCI states. |
| F033 | A.4.3.2-1/176 AND NOT (A.4.3.2-1/175 OR A.4.3.2-1/178 OR A.4.3.2-1/179) | UEs supporting unified TCI with separate DL/UL LTM TCI-state indication for LTM procedure and do not support MAC-CE activated joint or separate LTM TCI states and do not support unified TCI with joint DL/UL LTM TCI-state indication for LTM procedure |
| F034 | Void |  |
| F035 | Void |  |
| F036 | Void |  |
| F037 | Void |  |
| F038 | Void |  |
| F039 | Void |  |
| F040 | Void |  |
| F041 | Void |  |
| F042 | Void |  |
| F043 | IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/191 AND A.4.3.2-1/179 AND A.4.3.2-1/183 AND A.4.3.2-1/186 AND NOT (A.4.3.2-1/178) AND A.4.3.6-1/102 AND A.4.3.2-1/196 THEN R ELSE N/A | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, MAC-CE activated separate LTM TCI states, RACH-less LTM with dynamic grant, intra-frequency L1-RSRP measurement, reporting based on SSB(s) of candidate cell(s) and do not support MAC-CE activated joint LTM TCI states, interruptions caused by PDCCH order |
| F044 | IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/183 AND A.4.3.2-1/186 AND A.4.3.2-1/191 AND A.4.3.6-1/102 AND A.4.3.2-1/196 THEN R ELSE N/A | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, RACH-less LTM with dynamic grant, intra-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s) , interruptions caused by PDCCH order |
| F045 | IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/183 AND A.4.3.2-1/186 AND A.4.3.2-1/191 AND A.4.3.2-1/179 AND NOT (A.4.3.2-1/178) AND A.4.3.6-1/102 THEN R ELSE N/A | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, MAC-CE activated separate LTM TCI states, RACH-less LTM with dynamic grant and do not support MAC-CE activated joint LTM TCI states, interruptions caused by PDCCH order |
| F046 | IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/183 AND A.4.3.2-1/186 AND A.4.3.2-1/191 AND A.4.3.6-1/102 THEN R ELSE N/A | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, RACH-less LTM with dynamic grant, interruptions caused by PDCCH order |
| F047 | IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/183 AND A.4.3.2-1/184 AND A.4.3.2-1/185 AND A.4.3.2-1/186 AND A.4.3.6-1/102 AND A.4.3.2-1/194 THEN R ELSE N/A | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, RACH-less LTM with dynamic grant and intra-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s) , interruptions caused by PDCCH order and Support simultaneous transmission to handle the overlap between UL transmission on serving cell(s) and PRACH on candidate cell(s) |
| F048 | IF A.4.3.2-1/224 THEN R ELSE N/A | UE supporting max-HARQ-ProcessNumber-r17 with 32 HARQ processes |
| F049 | IF A.4.3.5-1/18 THEN R ELSE N/A | UE supporting harq-FeedbackDisabled-r17 |
| F050 | A.4.3.2-1/78 AND A.4.3.2-1/56 | UE supporting maxNumberActiveTCI-PerBWP other than n1 and maxSimultaneousResourceSetsPerCC atleast 2 |
| Fxx1 | A.4.3.2-1/178 | UEs supporting MAC-CE activated joint LTM TCI states |

<End of modified section 2>

<Start of modified section 3>

Table 4.2-3: Applicability of RRM NR SA FR1 conformance test cases, ref. TS 38.533 [5]

| Clause | TC Title | Release |  | Applicability |  |  |  | Additional Information |  | Branch |  | Subtest Selection Criteria |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Condition |  | Comment |  |  |  |  |  |  |  |  |
| 6.1 | RRC_IDLE state mobility |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.1.1 | NR cell re-selection |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.1.1.1 | NR SA FR1 cell re-selection | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  |  | 2Rx 4Rx 8Rx |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.3.2.4.2 | NR SA FR1-FR1 PDCCH-ordered RACH to an candidate cell for LTM | Rel-18 |  | C366 |  | UEs supporting 5GS NR SA FR1, inter-frequency LTM for MCG with RACH, MAC-CE activated joint LTM TCI states, RACH-less LTM with dynamic grant, inter-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s), interruptions caused by PDCCH order and Support simultaneous transmission to handle the overlap between UL transmission on serving cell(s) and PRACH on candidate cell(s) |  |  |  | 2Rx 4Rx 8Rx |  |  |  | Test 2: F047 |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.3.4 | LTM PCell Switch |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.3.4.1 | NR SA FR1 RACH-based PCell switch from FR1 to FR1 | Rel-18 |  | C362 |  | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH and intra-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s) |  |  |  | 2Rx 4Rx 8Rx |  | Test 1A: Fxx1 Test 1B: F031 Test 2A: F032 Test 2B: F033 |  |  |
| 6.3.4.2 | NR SA FR1-FR1 RACH based LTM PCell switch from FR1 to FR1 | Rel-18 |  | C363 |  | UEs supporting 5GS NR SA FR1, inter-frequency LTM for MCG with RACH without NR-DC configuration and inter-frequency L1-RSRP measurement and reporting with measurement gaps based on SSB(s) of candidate cell(s) |  |  |  | 2Rx 4Rx 8Rx |  | Test 1A: Fxx1 Test 1B: F031 Test 2A: F032 Test 2B: F033 |  |  |
| 6.3.4.3 | NR SA FR1 RACH-less PCell switch from FR1 to FR1 | Rel-18 |  | C357 |  | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, RACH-less LTM with dynamic grant, TA indication in cell switch command and intra-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s) |  |  |  | 2Rx 4Rx 8Rx |  | Test 1A: Fxx1 Test 1B: F031 Test 2A: F032 Test 2B: F033 |  |  |
| 6.3.4.4 | NR SA FR1 RACH-less PCell switch from FR1 to FR1 without L1-RSRP measurement | Rel-18 |  | C358 |  | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, RACH-less LTM with dynamic grant and TA indication in cell switch command |  |  |  | 2Rx 4Rx 8Rx |  | Test 1A: Fxx1 Test 1B: F031 Test 2A: F032 Test 2B: F033 |  |  |
| 6.3.5 | LTM PSCell Switch |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.3.5.1 | NR SA FR1 RACH-based Intra-frequency LTM PSCell switch | Rel-18 |  | FFS |  |  |  | NOTE 1 |  | 2Rx 4Rx 8Rx |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.6.26 | LTM Intra-frequency L1-RSRP measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.6.26.1 | NR SA FR1 SSB based L1-RSRP measurement | Rel-18 |  | C359 |  | UEs supporting 5GS NR SA FR1, intra-frequency L1- RSRP measurement and reporting based on SSB(s) of candidate cell(s) |  |  |  | 2Rx 4Rx 8Rx |  | Subtest 2: F029 |  |  |
| 6.6.27 | LTM Inter-frequency L1-RSRP measurement with measurement gap |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.6.27.1 | NR SA FR1-FR1 SSB based L1-RSRP measurement with measurement gap | Rel-18 |  | C360 |  | UEs supporting 5GS NR SA FR1, inter-frequency L1- RSRP measurement and reporting based on SSB(s) of candidate cell(s) |  |  |  | 2Rx 4Rx 8Rx |  |  |  |  |
| 6.6.28 | LTM Inter-frequency L1-RSRP measurement without measurement gap |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.6.28.1 | NR SA FR1-FR1 SSB based L1-RSRP measurement without measurement gap | Rel-18 |  | C361 |  | UEs supporting 5GS NR SA FR1, SSB based inter-frequency L1-RSRP measurements on SSBs within active DL BWP without measurement gaps for LTM |  |  |  | 2Rx 4Rx 8Rx |  | Subtest 2: F029 |  |  |
| 6.7 | Measurement performance requirements |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.1 | SS-RSRP |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.1.1 | Intra-frequency measurements |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.1.1.1 | NR SA FR1 SS-RSRP absolute measurement accuracy | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  | Test execution not necessary if test 4.7.1.1.1 has been executed. |  | 2Rx 4Rx 8Rx |  |  |  |  |
| 6.7.1.1.2 | NR SA FR1 SS-RSRP relative measurement accuracy | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  | Test execution not necessary if test 4.7.1.1.2 has been executed. |  | 2Rx 4Rx 8Rx |  |  |  |  |
| 6.7.1.2 | Inter-frequency measurements |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.1.2.1 | NR SA FR1-FR1 SS-RSRP absolute measurement accuracy | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  | Test execution not necessary if test 4.7.1.2.1 has been executed. |  | 2Rx 4Rx 8Rx |  |  |  |  |
| 6.7.1.2.2 | NR SA FR1-FR1 SS-RSRP relative measurement accuracy | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  | Test execution not necessary if test 4.7.1.2.2 has been executed. |  | 2Rx 4Rx 8Rx |  |  |  |  |
| 6.7.2 | SS-RSRQ |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.2.1 | NR SA FR1 SS-RSRQ measurement accuracy | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  | Test execution not necessary if test 4.7.2.1 has been executed. |  | 2Rx 4Rx 8Rx |  |  |  |  |
| 6.7.2.2.1 | NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  | Test execution not necessary if test 4.7.2.2.1 has been executed. |  | 2Rx 4Rx 8Rx |  |  |  |  |
| 6.7.2.2.2 | NR SA FR1-FR1 SS-RSRQ relative measurement accuracy | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  | Test execution not necessary if test 4.7.2.2.2 has been executed. |  | 2Rx 4Rx 8Rx |  |  |  |  |
| 6.7.3 | SS-SINR |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.3.1 | NR SA FR1 SS-SINR measurement accuracy | Rel-15 |  | C034 |  | UEs supporting 5GS NR SA FR1 and SS-SINR-meas |  | Test execution not necessary if test 4.7.3.1 has been executed. |  | 2Rx 4Rx 8Rx |  |  |  |  |
| 6.7.3.2.1 | NR SA FR1-FR1 SS-SINR absolute measurement accuracy | Rel-15 |  | C034 |  | UEs supporting 5GS NR SA FR1 and SS-SINR-meas |  | Test execution not necessary if test 4.7.3.2.1 has been executed. |  | 2Rx 4Rx 8Rx |  |  |  |  |
| 6.7.3.2.2 | NR SA FR1-FR1 SS-SINR relative measurement accuracy | Rel-15 |  | C034 |  | UEs supporting 5GS NR SA FR1 and SS-SINR-meas |  | Test execution not necessary if test 4.7.3.2.2 has been executed. |  | 2Rx 4Rx 8Rx |  |  |  |  |
| 6.7.4 | L1-RSRP for beam reporting |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.4.1.1 | NR SA FR1 SSB based L1-RSRP absolute measurement accuracy | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  | Test execution not necessary if test 4.7.4.1.1 has been executed. |  | 2Rx 4Rx 8Rx |  |  |  |  |
| 6.7.4.1.2 | NR SA FR1 SSB based L1-RSRP relative measurement accuracy | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  | Test execution not necessary if test 4.7.4.1.2 has been executed. |  | 2Rx 4Rx 8Rx |  |  |  |  |
| 6.7.4.2.1 | NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  | Test execution not necessary if test 4.7.4.2.1 has been executed. |  | 2Rx 4Rx 8Rx |  |  |  |  |
| 6.7.4.2.2 | NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  | Test execution not necessary if test 4.7.4.2.2 has been executed. |  | 2Rx 4Rx 8Rx |  |  |  |  |
| 6.7.5 | E-UTRAN RSRP |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.5.1 | NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  |  | 2Rx 4Rx 8Rx |  |  |  |  |
| 6.7.6 | E-UTRAN RSRQ |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.6.1 | NR SA FR1 – E-UTRAN RSRQ absolute measurement accuracy | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  |  | 2Rx 4Rx 8Rx |  |  |  |  |
| 6.7.7 | E-UTRAN RS-SINR |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.7.1 | NR SA FR1 – E-UTRAN RS-SINR absolute measurement accuracy | Rel-15 |  | C168 |  | UEs supporting 5GS NR SA FR1 and E-UTRA RS-SINR measurements |  |  |  | 2Rx 4Rx 8Rx |  |  |  |  |
| 6.7.9 | L1-SINR |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.9.1.1 | NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR absolute measurement accuracy | Rel-16 |  | C132 |  | UEs supporting 5GS NR SA FR1 and L1-SINR-measurement based on CSI-RS as CMR without dedicated IMR configured |  | Test execution not necessary if test 4.7.7.1.1 has been executed. |  | 2Rx 4Rx 8Rx |  |  |  |  |
| 6.7.9.1.2 | NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy | Rel-16 |  | C132 |  | UEs supporting 5GS NR SA FR1 and L1-SINR-measurement based on CSI-RS as CMR without dedicated IMR configured |  | Test execution not necessary if test 4.7.7.1.2 has been executed. |  | 2Rx 4Rx 8Rx |  |  |  |  |
| 6.7.9.2 | NR SA FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy | Rel-16 |  | C133 |  | UEs supporting 5GS NR SA FR1 and L1-SINR-measurement based on SSB as CMR and dedicated CSI-IM as IMR |  | Test execution not necessary if test 4.7.7.2 has been executed. |  | 2Rx 4Rx 8Rx |  |  |  |  |
| 6.7.9.3.1 | NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR absolute measurement accuracy | Rel-16 |  | C134 |  | UEs supporting 5GS NR SA FR1 and L1-SINR-measurement based on CSI-RS as CMR and dedicated CSI-IM as IMR |  | Test execution not necessary if test 4.7.7.3.1 has been executed. |  | 2Rx 4Rx 8Rx |  |  |  |  |
| 6.7.9.3.2 | NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy | Rel-16 |  | C134 |  | UEs supporting 5GS NR SA FR1 and L1-SINR-measurement based on CSI-RS as CMR and dedicated CSI-IM as IMR |  | Test execution not necessary if test 4.7.7.3.2 has been executed. |  | 2Rx 4Rx 8Rx |  |  |  |  |
| 6.7.17 | LTM L1-RSRP measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.17.1.1 | NR SA FR1-FR1 L1-RSRP absolute accuracy requirements for neighbour cell | Rel-18 |  | C360 |  | UEs supporting 5GS NR SA FR1, inter-frequency L1- RSRP measurement and reporting based on SSB(s) of candidate cell(s) |  |  |  | 2Rx 4Rx 8Rx |  |  |  |  |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.533. NOTE 2: Test X refers to the corresponding Sub-Test as defined in TS 38.533 [5]. NOTE 3: Test cases in TS 38.533 [5] clause 6 only apply to FR1 non-RedCap UEs. For FR1 RedCap UEs, Test cases in TS 38.533 [5] clause 16 apply. |  |  |  |  |  |  |  |

<End of modified section 3>

<Start of modified section 4>

Table 4.2-4: Applicability of RRM NR SA FR2 conformance test cases, ref. TS 38.533 [5]

| Clause | TC Title | Release | Applicability |  | Additional Information | Branch | Test Selection Criteria |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |  |
| 7.1 | RRC_IDLE state mobility |  |  |  |  |  |  |
| 7.1.1 | NR cell re-selection |  |  |  |  |  |  |
| 7.1.1.1 | NR SA FR2 cell re-selection | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | PC1 PC3 |  |
| 7.1.1.2 | NR SA FR2-FR2 cell re-selection | FFS | FFS | FFS |  | PC1(NOTE 1) PC3(NOTE 1) |  |
| 7.1.1.3 | NR SA FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | Rel-16 | C095 | UEs supporting 5GS NR SA FR2 and relaxed RRM measurement |  | PC1(NOTE 1) PC3 |  |
| 7.1.1.4 | NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | Rel-16 | C095 | UEs supporting 5GS NR SA FR2 and relaxed RRM measurement |  | PC1(NOTE 1) PC3 |  |
| 7.1.1.5 | NR SA FR2-FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | Rel-16 | C095 | UEs supporting 5GS NR SA FR2 and relaxed RRM measurement |  | PC1(NOTE 1) PC3 |  |
| 7.1.1.6 | NR SA FR2-FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | Rel-16 | C095 | UEs supporting 5GS NR SA FR2 and relaxed RRM measurement |  | PC1(NOTE 1) PC3 |  |
| 7.1.1.7 | NR SA FR2 cell re-selection for power class 6 UE configured with highSpeedMeasFlagFR2-r17 | Rel-17 | C240 | UEs supporting 5GS NR SA FR2 and intra-frequency RRC_IDLE measurements in HST |  | PC1(NOTE 1) PC3(NOTE 1) |  |
| 7.1.1.8 | NR SA FR2 inter-frequency cell re-selection for power class 6 UE configured with highSpeedMeasFlagFR2-r17 | Rel-18 | C345 | UEs supporting 5GS NR SA FR2 and inter-frequency RRC_IDLE measurements in HST |  | PC1(NOTE 1) PC3(NOTE 1) |  |
| 7.2 | RRC_INACTIVE state mobility |  |  |  |  |  |  |
| 7.2.1 | Small Data Transmission |  |  |  |  |  |  |
| 7.2.1.1 | TA Validation for CG-SDT in FR2 | Rel-17 | C095a | UEs supporting 5GS NR SA FR2 and TA Validation for CG-SDT in FR2 |  | PC1(NOTE 1) PC3(NOTE 1) |  |
| 7.3 | RRC_CONNECTED state mobility |  |  |  |  |  |  |
| 7.3.1 | Handover |  |  |  |  |  |  |
| 7.3.1.4 | NR SA FR1-FR2 synchronous DAPS handover | Rel-16 | N/A | Not recommended due to FR1 – FR2 testability issue |  | PC1(NOTE 1) PC3(NOTE 1) |  |
| 7.3.1.5 | NR SA FR1-FR2 asynchronous DAPS handover | Rel-16 | N/A | Not recommended due to FR1 – FR2 testability issue |  | PC1(NOTE 1) PC3(NOTE 1) |  |
| 7.3.2 | RRC connection mobility control |  |  |  |  |  |  |
| 7.3.2.1 | RRC re-establishment |  |  |  |  |  |  |
| 7.3.2.1.1 | NR SA FR2 RRC re-establishment | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | PC1(NOTE 1) PC3 |  |
| 7.3.2.1.2 | NR SA FR2 - FR2 RRC re-establishment | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | PC1(NOTE 1) PC3 |  |
| 7.3.2.1.3 | NR SA FR2 RRC re-establishment without serving cell timing | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | PC1(NOTE 1) PC3 |  |
| 7.3.2.2 | Random access |  |  |  |  |  |  |
| 7.3.2.2.1 | NR SA FR2 contention based random access | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | PC1 PC3 |  |
| 7.3.2.2.2 | NR SA FR2 non-contention based random access | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | PC1 PC3 | Subtest 2: F007 |
| 7.3.2.2.4 | NR SA FR2 2-step non-contention based random access | Rel-16 | C160 | UEs supporting 5GS NR SA FR2 and 2-step RACH |  | PC1(NOTE 1) PC3(NOTE 1) |  |
| 7.3.2.3 | RRC connection release with redirection |  |  |  |  |  |  |
| 7.3.2.3.1 | NR SA FR2-FR2 RRC connection release with redirection | FFS | FFS | FFS |  | PC1(NOTE 1) PC3(NOTE 1) |  |
| 7.3.3 | Conditional Handover |  |  |  |  |  |  |
| 7.3.3.1 | NR SA FR2 conditional handover | Rel-16 | C106 | UEs supporting 5GS NR SA FR2 and conditional handover |  | PC1(NOTE 1) PC3 |  |
| 7.3.3.2 | NR SA FR2-FR2 conditional handover | Rel-16 | C106 | UEs supporting 5GS NR SA FR2 and conditional handover |  | PC1(NOTE 1) PC3(NOTE 1) |  |
| 7.3.3.3 | NR SA FR2 intra-frequency NES triggering conditional handover | Rel-18 | C106a | UEs supporting 5GS NR SA FR2 and NES triggered conditional handover | NOTE 1 |  |  |
| 7.3.3.4 | NR SA FR2-FR1 inter-frequency NES triggering conditional handover | Rel-18 | N/A | Not recommended due to FR1 – FR2 testability issue | NOTE 1 |  |  |
| 7.3.4 | LTM PCell Switch |  |  |  |  |  |  |
| 7.3.4.1 | NR SA FR2 RACH based PCell switch from FR2 to FR2 | Rel-18 | C381 | UEs supporting 5GS NR SA FR2, intra-frequency LTM for MCG with RACH, intra-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s) |  | PC1(NOTE 1) PC3 | Test 1A: Fxx1 Test 1B: F031 Test 2A: F032 Test 2B: F033 |
| 7.3.4.2 | NR SA FR2 RACH-less PCell switch from FR2 to FR2 | Rel-18 | C382 | UEs supporting 5GS NR SA FR2, intra-frequency LTM for MCG with RACH, RACH-less LTM with dynamic grant, TA indication in cell switch command and intra-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s) |  | PC1(NOTE 1) PC3 | Test 1A: Fxx1 Test 1B: F031 Test 2A: F032 Test 2B: F033 |
| 7.3.4.3 | NR SA FR2-FR2 RACH-based LTM PCell switch from FR2 to FR2 | Rel-18 | C383 | UEs supporting 5GS NR SA FR2, inter-frequency LTM for MCG with RACH without NR-DC configuration, inter-frequency L1-RSRP measurement and reporting with measurement gaps based on SSB(s) of candidate cell(s) |  | PC1(NOTE 1) PC3 | Test 1A: Fxx1 Test 1B: F031 Test 2A: F032 Test 2B: F033 |
| 7.3.5 | LTM PSCell Switch |  |  |  |  |  |  |
| 7.3.5.1 | NR SA FR2 RACH-based Intra-frequency LTM PSCell switch | Rel-18 | FFS |  | NOTE 1 |  |  |
| … |  |  |  |  |  |  |  |
| 7.6.20 | LTM Intra-frequency L1-RSRP measurement |  |  |  |  |  |  |
| 7.6.20.1 | NR SA FR2 SSB based L1-RSRP measurement | Rel-18 | C384 | UEs supporting 5GS NR SA FR2, intra-frequency L1- RSRP measurement and reporting based on SSB(s) of candidate cell(s) |  | PC1(NOTE 1) PC3 | Subtest 2: F029 |
| 7.6.21 | LTM Inter-frequency L1-RSRP measurement with measurement gap |  |  |  |  |  |  |
| 7.6.21.1 | NR SA FR2-FR2 SSB-based L1-RSRP measurement with measurement gap for LTM | Rel-18 | C385 | UEs supporting 5GS NR SA FR2, inter-frequency L1- RSRP measurement and reporting based on SSB(s) of candidate cell(s) |  | PC1(NOTE 1) PC3 |  |
| 7.6.22 | LTM Inter-frequency L1-RSRP measurement without measurement gap |  |  |  |  |  |  |
| 7.6.22.1 | NR SA FR2 event triggered reporting tests with per-UE NCSG under non-DRX | Rel-18 | C361a | UEs supporting 5GS NR SA FR2, SSB based inter-frequency L1-RSRP measurements on SSBs within active DL BWP without measurement gaps for LTM | NOTE 1 |  |  |
| 7.7 | Measurement performance requirements |  |  |  |  |  |  |
| 7.7.1 | SS-RSRP |  |  |  |  |  |  |
| 7.7.1.1 | NR SA FR2 SS-RSRP measurement accuracy | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | PC1 PC3 |  |
| 7.7.1.2 | NR SA FR2-FR2 SS-RSRP measurement accuracy | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | PC1(NOTE 1) PC3 |  |
| 7.7.1.3 | Inter-frequency measurements between FR1 and FR2 |  |  |  |  |  |  |
| 7.7.1.3.1 | NR SA FR1-FR2 SS-RSRP measurement accuracy | Rel-15 | N/A | Not recommended due to FR1 – FR2 testability issue |  | PC1(NOTE 1) PC3(NOTE 1) |  |
| 7.7.2 | SS-RSRQ |  |  |  |  |  |  |
| 7.7.2.1 | NR SA FR2 SS-RSRQ measurement accuracy | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | PC1(NOTE 1) PC3 |  |
| 7.7.2.2 | NR SA FR2-FR2 SS-RSRQ measurement accuracy | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | PC1(NOTE 1) PC3 |  |
| 7.7.3 | SS-SINR |  |  |  |  |  |  |
| 7.7.3.1 | NR SA FR2 SS-SINR measurement accuracy | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | PC1(NOTE 1) PC3 |  |
| 7.7.3.2 | NR SA FR2-FR2 SS-SINR measurement accuracy | Rel-16 | C006 | UEs supporting 5GS NR SA FR2 |  | PC1(NOTE 1) PC3 |  |
| 7.7.4 | L1-RSRP for beam reporting |  |  |  |  |  |  |
| 7.7.4.1 | NR SA FR2 SSB based L1-RSRP measurement accuracy | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | PC1(NOTE 1) PC3 |  |
| 7.7.4.2 | NR SA FR2 CSI-RS based L1-RSRP measurement accuracy | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | PC1(NOTE 1) PC3 |  |
| 7.7.5 |  |  |  |  |  |  |  |
| 7.7.5.1 | NR SA FR2 SRS-RSRP measurement accuracy | Rel-16 | C006l | UEs supporting 5GS NR SA FR2 and SRS-RSRP measurements |  | PC1(NOTE 1) PC3 |  |
| 7.7.6 | L1-SINR |  |  |  |  |  |  |
| 7.7.6.1 | NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | Rel-16 | C138 | UEs supporting 5GS NR SA FR2 and L1-SINR-measurement based on CSI-RS as CMR without dedicated IMR configured |  | PC1(NOTE 1) PC3 |  |
| 7.7.6.2 | NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy | Rel-16 | C139 | UEs supporting 5GS NR SA FR2 and L1-SINR-measurement based on SSB as CMR and dedicated CSI-IM as IMR |  | PC1(NOTE 1) PC3 |  |
| 7.7.6.3 | NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | Rel-16 | C140 | UEs supporting 5GS NR SA FR2 and L1-SINR-measurement based on CSI-RS as CMR and dedicated CSI-IM as IMR |  | PC1(NOTE 1) PC3 |  |
| 7.7.14 | L1-RSRP measurement for group-based beam reporting |  |  |  |  |  |  |
| 7.7.14.1 | NR SA FR2 SSB based L1-RSRP measurement for group-based beam reporting | Rel-18 | C378 | UEs supporting 5GS NR SA FR2 and perform group based L1-RSRP measurement based on SSB resources. | NOTE 1 |  |  |
| 7.7.14.2 | NR SA FR2 CSI-RS based L1-RSRP measurement on resource set with repetition off | Rel-18 | C378 | UEs supporting 5GS NR SA FR2 and perform group based L1-RSRP measurement based on CSI-RS resources. | NOTE 1 |  |  |
| 7.7.15 | LTM L1-RSRP measurement |  |  |  |  |  |  |
| 7.7.15.1 | NR SA FR2-FR2 SSB based L1-RSRP measurement | Rel-18 | C385 | UEs supporting 5GS NR SA FR2, inter-frequency L1- RSRP measurement and reporting based on SSB(s) of candidate cell(s) |  | PC1(NOTE 1) PC3 |  |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.533. NOTE 2: Void. NOTE 3: Void. NOTE 4: Test cases in TS 38.533 [5] clause 7 only apply to FR2 non-RedCap UEs. For FR2 RedCap UEs, Test cases in TS 38.533 [5] clause 17 apply. |  |  |  |  |  |  |  |

<End of modified section 4>