3GPP TSG-RAN5 Meeting #107	R5-252031r1

# Malta, MT, 19th May 2025 – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4941 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new NR NTN enh Sateliite Hard Switch with Resync Test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test case 8.1.5.14.2 needs to be added as per NTN Enh WP |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case 8.1.5.14.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This aspect of WP will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.14.2 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.523-2 CR0573 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1:Unnecessary references are removed from Conformance requirement, Main behaviour Step2: Cell corrected, Table reference corrected in Specific message contents |  |  |  |  |  |  |  |  |

<Unchanged Sections Omitted>

##### 8.1.5.14.2	NR NTN Enhancements / SIB19 / Sateliite Hard Switch with Resync

8.1.5.14.2.1	Test Purpose (TP)

(1)

with {UE in NR RRC_CONNECTED state and t-Service and SatSwitchWithReSync are broadcasted in SIB19 }

ensure that {

when{ upon the time indicated by t-Service }

then{ UE acquire target satellite indicated by ntn-Config in SatSwitchWithReSync }

}

8.1.5.14.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331 clauses 5.7.19 and TS 38.321 clause 5.2a. Unless otherwise stated these are Rel-18 requirements.

[TS 38.331, clause 5.7.19]

A UE capable of hard satellite switch with resynchronization in RRC_CONNECTED initiates the procedure when SatSwitchWithReSync and t-Service are included in SIB19.

Upon initiating the procedure, the UE shall:

1>	if t-ServiceStart is included in SIB19 and the UE supports soft satellite switch with resynchronization:

2>	start acquiring DL synchronization with the SpCell served by the satellite indicated by ntn-Config in SatSwitchWithReSync between the time indicated by t-ServiceStart and the time indicated by t-Service for the serving cell;

1>	upon the time indicated by t-Service:

2>	stop timer T430 if running;

2>	inform lower layers that UL synchronisation is lost due to satellite switch with resynchronization;

2>	synchronise to the DL of the SpCell served by the satellite indicated by ntn-Config in SatSwitchWithReSync, if the UE has not previously synchronized to the DL of the SpCell;

2>	start timer T430 with the timer value set to ntn-UlSyncValidityDuration from the subframe indicated by epochTime in ntn-Config in SatSwitchWithReSync;

2>	inform lower layers when UL synchronisation is obtained.

NOTE:	UE should attempt to re-acquire SIB19 after satellite switch with resynchronization. The exact time is left to UE implementation.

[TS 38.321, clause 5.2a]

The MAC entity shall for each Serving Cell:

1>	if an indication of uplink synchronization has been received from upper layers (see clauses 5.2.2.6 and 5.7.19 of TS 38.331 [5]):

2>	if indication of uplink synchronization is received after indication of uplink synchronization loss due to satellite switch with re-synchronization (see clause 5.7.19 of TS 38.331 [5]):

3>	set NTA value (as defined in TS 38.211 [8]) to zero for PTAG;

3>	indicate to lower layers a Differential Koffset with value zero.

2>	allow uplink transmission on the Serving Cell.

1>	if an indication of uplink synchronization loss is received from upper layers (see clause 5.2.2.6 and 5.7.19 of TS 38.331 [5]):

2>	if uplink synchronization loss is due to satellite switch with re-synchronization (see clause 5.7.19 of TS 38.331 [5]):

3>	not perform any uplink transmission on the Serving Cell.

2>	else:

3>	flush all HARQ buffers;

3>	not perform any uplink transmission on the Serving Cell.

NOTE:	The MAC entity suspends all UL operations (e.g. stop RACH, SR, and UL HARQ operation) after receiving the indication of an uplink synchronization loss and resumes the operation when receiving an indication of uplink synchronization.

8.1.5.14.2.3	Test description

8.1.5.14.2.3.1	Pre-test conditions

System Simulator:

NR Cell 1 is serving cell connected to the source satellite and NR Cell 2 is an intra-freq cell with same PCI connected to the target satellite

System information combination NR-29 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	The pre-configured UE location is defined in TS 38.508-1 [4] Clause 4.5C.

Preamble:

-	The UE is in 3N-A on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3 and Test Mode Activated according to 38.508-1 [4] Table 4.4A.2-1 with UE test loop mode B is established IP PDU delay set to FFS seconds.

8.1.5.14.2.3.2	Test procedure sequence

Table 8.1.5.14.2.3.2-1 for FR1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions for preamble, while columns marked "T1" to be applied subsequently in the Main behaviour. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.5.14.2.3.2-1: Cell configuration changes over time for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remarks |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | "Off" | Power level “Off” is defined in TS 38.508-1 Table 6.2.2.1-3 |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -88 |  |

Table 8.1.5.14.2.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a downlink assignment including the C-RNTI assigned to the UE | <-- | (PDCCH (C-RNTI)) |  |  |
| 2 | SS transmits in the indicated downlink assignment a RLC PDU in a MAC PDU. on NR Cell 1 | <-- | MAC PDU |  |  |
| 3 | The SS re-adjusts the power level to row "T1" in table 8.1.5.14.2.3.2-1 | - | - |  |  |
| 4 | Wait for FFS ms (t-Service+) to allow the UE to switch to Target Satellite | - | - | - | - |
| 5 | Check: Does the UE transmit a MAC PDU containing Loop backed PDU on expiry of IP PDU delay on NR Cell 2? | --> | MAC PDU (containing 1 MAC sub PDU containing RLC SDU) | 1 | P |
| 6 | The SS transmits an RRCRelease message | <-- | NR RRC: RRCRelease | - | - |

8.1.5.14.2.3.3	Specific message contents

Table 8.1.5.14.2.3.3-1: PhysCellId for NR Cell 2 (preamble and all steps, Table 8.1.5.14.2.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-105 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysCellId | Set according to table 4.4.2-2 for the NR Cell 1 | NR Cell1 and NR Cell2 to have same PCI |  |

Table 8.1.5.14.2.3.3-2: SIB1 for NR Cell 2 (preamble and all steps, Table 8.1.5.14.2.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| servingCellConfigCommon | ServingCellConfigCommonSIB | Table 8.1.5.14.2.3.3-3 |  |
| } |  |  |  |

Table 8.1.5.14.2.3.3-3: ServingCellConfigCommonSIB (Table 8.1.5.14.2.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| uplinkConfigCommon | UplinkConfigCommonSIB | Table 8.1.5.14.2.3.3-4 |  |
| } |  |  |  |

Table 8.1.5.14.2.3.3-4: UplinkConfigCommonSIB (Table 8.1.5.14.2.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-202 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkConfigCommonSIB SEQUENCE { |  |  |  |
| timeAlignmentTimerCommon | TimeAlignmentTimer |  |  |
| } |  |  |  |

Table 8.1.5.14.2.3.3-6: SIB19 for NR Cell 1 (Preamble and all steps, Table 8.1.5.14.2.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-18C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| distanceThresh-r17 | FFS |  |  |
| movingReferenceLocation-r18 | FFS |  |  |
| t-Service | FFS |  |  |
| satSwitchWithReSync-r18 SEQUENCE { |  |  |  |
| ntn-Config-r18 | NTN-Config | Table 8.1.5.14.2.3.3-8 for NR Cell 2 |  |
| t-ServiceStart-r18 | Not Present |  |  |
| ssb-TimeOffset-r18 | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.14.2.3.3-7: SIB19 for NR Cell 2 (Preamble and all steps, Table 8.1.5.14.2.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-18C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r18 | NTN-Config | Table 8.1.5.14.2.3.3-8 |  |
| distanceThresh-r17 | FFS |  |  |
| movingReferenceLocation-r18 | FFS |  |  |
| } |  |  |  |

Table 8.1.5.14.2.3.3-8: NTN-Config (Table 8.1.5.14.2.3.3-6,7)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-84C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NTN-Config-r17 | FFS |  |  |

Table 8.1.5.14.2.3.3-9: CLOSE UE TEST LOOP (Preamble)

| Derivation path: 36.508-1 [7] table 4.7A-3 condition UE test loop mode B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE test loop mode B LB setup |  |  |  |
| IP PDU delay | FFS | XX seconds |  |  |

<Unchanged Sections Omitted>