3GPP TSG-RAN5 Meeting #107	R5-252032r1

# Malta, MT, 19th May 2025 – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4942 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new NR NTN enh Sateliite Soft Switch with Resync Test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test case 8.1.5.14.3 needs to be added as per NTN Enh WP |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case 8.1.5.14.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This aspect of WP will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.14.3 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.523-2 CR0573 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Table number corrected for Test Procedure sequence. Step2 of Main behaviour is updated with the correct cell. IEs are being put in the correct position in table 8.1.5.14.3.3.3-1. Missing NTN-Config-r17 resolved by adding new table. Subsequent table numbers are updated. |  |  |  |  |  |  |  |  |

<Unchanged Sections Omitted>

##### 8.1.5.14.3	NR NTN Enhancements / SIB19 / Sateliite Soft Switch with Resync

8.1.5.14.3.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state  and t-ServiceStart, t-Service, ssb-TimeOffset, SatSwitchWithReSync are broadcasted in SIB19 }

ensure that {

when{ t-ServiceStart starts }

then{ UE acquires target satellite indicated by ntn-Config in SatSwitchWithReSync before t-Service expiry}

}

8.1.5.14.3.2	Conformance requirements

Same as 8.1.5.14.2,2

8.1.5.14.3.3	Test description

8.1.5.14.3.3.1	Pre-test conditions

System Simulator:

NR Cell 1 is serving cell connected to the source satellite and NR Cell 2 is an intra-freq cell with same PCI connected to the target satellite

System information combination NR-29 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	The pre-configured UE location is defined in TS 38.508-1 [4] Clause 4.5C.

Preamble:

-	The UE is in 3N-A on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3 and Test Mode Activated according to 38.508-1 [4] Table 4.4A.2-1 with UE test loop mode B is established IP PDU delay set to FFS seconds.

8.1.5.14.3.3.2	Test procedure sequence

Table 8.1.5.14.3.3.3-1 for FR1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions for preamble, while columns marked "T1" to be applied subsequently in the Main behaviour. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.5.14.3.3.3-1: Cell configuration changes over time for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remarks |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | "Off" | Power level “Off” is defined in TS 38.508-1 Table 6.2.2.1-3 |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -88 |  |

Table 8.1.5.14.3.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a downlink assignment including the C-RNTI assigned to the UE | <-- | (PDCCH (C-RNTI)) |  |  |
| 2 | SS transmits in the indicated downlink assignment a RLC PDU in a MAC PDU. on NR Cell 1 | <-- | MAC PDU |  |  |
| 3 | The SS re-adjusts the power level to row "T1" in table 8.1.5.14.2.3.2-1 | - | - |  |  |
| 4 | Wait for FFS ms (After t-ServiceStart and before t-Service) to allow the UE to switch to Target Satellite | - | - | - | - |
| 5 | Check: Does the UE transmit a MAC PDU containing Loop backed PDU on expiry of IP PDU delay on NR Cell 2? | --> | MAC PDU (containing 1 MAC sub PDU containing RLC SDU) | 1 | P |
| 6 | The SS transmits an RRCRelease message | <-- | NR RRC: RRCRelease | - | - |

8.1.5.14.3.3.3	Specific message contents

Same as 8.1.5.14.2.3.3 except

Table 8.1.5.14.3.3.3-1: SIB19 for NR Cell 1 (Preamble and all steps, Table 8.1.5.14.2.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-18C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| t-Service | FFS |  |  |
| distanceThresh-r17 | FFS |  |  |
| movingReferenceLocation-r18 | FFS |  |  |
| satSwitchWithReSync-r18 SEQUENCE { |  |  |  |
| ntn-Config-r18 | NTN-Config | Table 8.1.5.14.3.3.3-2 for NR Cell 2 |  |
| t-ServiceStart-r18 | FFS |  |  |
| ssb-TimeOffset-r18 | 10 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.14.3.3.3-2: NTN-Config (Table 8.1.5.14.3.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-84C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NTN-Config-r17 | FFS |  |  |

Table 8.1.5.14.3.3.3-3: CLOSE UE TEST LOOP (Preamble)

| Derivation path: 36.508-1 [7] table 4.7A-3 condition UE test loop mode B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE test loop mode B LB setup |  |  |  |
| IP PDU delay | FFS | XX seconds |  |  |

<Unchanged Sections Omitted>