3GPP TSG-RAN5 Meeting #107	R5-252058r1

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | 0088 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NB-IoT TC 6.5B test configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek (Chengdu) Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Align with legacy LTE A-Spur test environment to test NB-IoT A-spur in normal condition only |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1: Editorial correction to Clause 6.5A.4.4.3.4 2: Removed extreme test environuments from test configuration Table 6.5B.4.4.4.1-1 and 6.5B.4.4.4.1-2 3: Editorial correction to Annex F.3.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test configuration is incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5A.4.4, 6.5B.4.4, F.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: Removed the modification for TC 6.5B..3.3 to resolve the overlap issue with R&S CR R5-252359 and updated the coversheet accordingly |  |  |  |  |  |  |  |  |

<Start of Change1>

6.5A.4.4.3.4	Minimum requirement (network signalled value "NS_04N, NS_05N")

When "NS_04N or NS_05N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.3.4-1 where BWchannel equals to 1.4MHz. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5A.4.2.3-1 from the edge of the channel bandwidth.

Table 6.5A.4.4.3.4-1: Additional out-of-band requirements for "NS_04N, NS_05N"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | BWchannel |  |  |
| 1559 ≤ f ≤ 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605 ≤ f ≤ 1610 | -40 + 60/5 (f-1605) | 1MHz |  |
| 1628.5 ≤ f ≤ 1631.5 | -30 | 30kHz |  |
| 1631.5 ≤ f ≤ 1636.5 | -30 | 100kHz |  |
| 1636.5 ≤ f ≤ 1646.5 | -30 | 300kHz |  |
| 1646.5 ≤ f ≤ 1666.5 | -30 | 1MHz |  |
| 1666.5 ≤ f ≤ 2200 | -30 | 3MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

Table 6.5A.4.4.3.4-2: Void

The normative reference for this requirement is TS 36.102[11] subclause 6.5A.4.4.5.

< End of Change1 >

< Skip unchanged >

<Start of Change2>

6.5B.4.4.4	Test description

6.5B.4.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NB-IOT NTN operating bands specified in sub-clause 5.2B. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 6.5B.4.4.4.1-1 to Table 6.5B.4.4.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of NPDSCH and NPDCCH before measurement are specified in Annex C.2.

Table 6.5B.4.4.4.1-1: Test Configuration Table (network signalled value " NS_02N, NS_03N, NS_04N, NS_05N")

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 8.1.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 8.1.3.1 |  | Low range, Mid range, High range |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A for Maximum Power Reduction (MPR) test case | Modulation | Ntones | Sub-carrier spacing (kHz) |
| 1 (Note 1) |  | QPSK | 3@0 | 15 |
| 2 (Note 1) |  | QPSK | 3@3 | 15 |
| 3 (Note 1) |  | QPSK | 3@9 | 15 |
| 4 (Note 1) |  | QPSK | 6@0 | 15 |
| 5 (Note 1) |  | QPSK | 6@6 | 15 |
| 6 (Note 1) |  | QPSK | 12@0 | 15 |
| Note 1: Applicable to UE supporting UL multi-tone transmissions |  |  |  |  |

Table 6.5B.4.4.4.1-2 Test Configuration Table (network signalled value "NS_24")

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 8.1.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 8.1.3.1 |  | Low range, Mid range, High range |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A for Maximum Power Reduction (MPR) test case | Modulation | Ntones | Sub-carrier spacing (kHz) |
| 1 (Note 1) |  | QPSK | 3@0 | 15 |
| 2 (Note 1) |  | QPSK | 3@3 | 15 |
| 3 (Note 1) |  | QPSK | 3@9 | 15 |
| 4 (Note 1) |  | QPSK | 6@0 | 15 |
| 5 (Note 1) |  | QPSK | 6@6 | 15 |
| 6 (Note 1) |  | QPSK | 12@0 | 15 |
| Note 1: Applicable to UE supporting UL multi-tone transmissions |  |  |  |  |

< End of Change2 >

< Skip unchanged >

<Start of Change3>

F.3.2	Measurement of transmitter

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Test | Minimum Requirement in TS 36.102 | Test Tolerance (TT) | Test Requirement in TS 36.521-1 |
| --- | --- | --- | --- |
| 6.2A.1 UE maximum output power for category M1 | Same as clause 6.2.2EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz, Power class 3” and “f ≤ 3.0GHz, Power class 5”. | Same as clause 6.2.2EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz, Power class 3” and “f ≤ 3.0GHz, Power class 5”. | Same as clause 6.2.2EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz, Power class 3” and “f ≤ 3.0GHz, Power class 5”. |
| 6.2A.2 UE maximum output power reduction for category M1 | Same as clause 6.2.3EA in TS 36.521-1 [14] for FDD band with “Power class 3, f ≤ 3.0GHz” and “Power class 5, f ≤ 3.0GHz”. | Same as clause 6.2.3EA in TS 36.521-1 [14] for FDD band with “Power class 3, f ≤ 3.0GHz” and “Power class 5, f ≤ 3.0GHz”. | Same as clause 6.2.3EA in TS 36.521-1 [14] for FDD band with “Power class 3, f ≤ 3.0GHz” and “Power class 5, f ≤ 3.0GHz”. |
| 6.2A.3 UE additional maximum output power reduction for category M1 UE | Same as clause 6.2.4EA in TS 36.521-1 [14] | Same as clause 6.2.4EA in TS 36.521-1 [14]] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.2.4EA in TS 36.521-1 [14] |
| 6.2A.4 Configured transmitted Power for category M1 | Same as clause 6.2.5EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.2.5EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.2.5EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |
| 6.2B.1 UE maximum output power for category NB1 and NB2 | Same as clause 6.2.2F in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz, Power class 3” and “f ≤ 3.0GHz, Power class 5” | Same as clause 6.2.2F in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.2.2F in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |
| 6.2B.2 UE maximum output power reduction for category NB1 and NB2 | Same as clause 6.2.3F in TS 36.521-1 [14] for FDD band with “Power class 3, f ≤ 3.0GHz” and “Power class 5, f ≤ 3.0GHz”. | Same as clause 6.2.3F in TS 36.521-1 [14] for FDD band with “Power class 3, f ≤ 3.0GHz” and “Power class 5, f ≤ 3.0GHz”. | Same as clause 6.2.3F in TS 36.521-1 [14] for FDD band with “Power class 3, f ≤ 3.0GHz” and “Power class 5, f ≤ 3.0GHz”. |
| 6.2B.3 UE additional maximum output power reduction for category NB1 and NB2 UE | Same as clause 6.2.3F in TS 36.521-1[14] for FDD band with “f ≤ 3.0GHz” | Same as clause 6.2.3F in TS 36.521-1[14] for FDD band with “f ≤ 3.0GHz” | Same as clause 6.2.3F in TS 36.521-1[14] for FDD band with “f ≤ 3.0GHz” |
| 6.2B.4 Configured transmitted Power for category NB1 and NB2 | Same as clause 6.2.5F in TS 36.521-1 [14] | Same as clause 6.2.5F in TS 36.521-1 [14] | Same as clause 6.2.5F in TS 36.521-1 [14] |
| 6.3A.1 UE Minimum output power for category M1 | Same as clause 6.3.2EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.2EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.2EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |
| 6.3A.2 Transmit OFF power for category M1 | Same as clause 6.3.3EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.3EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.3EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |
| 6.3A.3.1 General ON/OFF time mask for category M1 | Same as clause 6.3.4EA.1 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.4EA.1 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.4EA.1 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |
| 6.3A.3.2.1 PRACH time mask for UE category M1 | Same as clause 6.3.4EA.2 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.4EA.2 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.4EA.2 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |
| 6.3A.3.2.2 SRS time mask for UE category M1 | Same as clause 6.3.4EA.2 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.4EA.2 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.4EA.2 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |
| 6.3A.4.1 Power Control Absolute power tolerance for UE category M1 | Same as clause 6.3.5EA.1 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.5EA.1 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.5EA.1 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |
| 6.3A.4.2 Power Control Relative power tolerance for UE category M1 | Same as clause 6.3.5EA.2 in TS 36.521-1 [14]. | Same as clause 6.3.5EA.2 in TS 36.521-1 [14]. | Same as clause 6.3.5EA.2 in TS 36.521-1 [14]. |
| 6.3A.4.3 Aggregate power control tolerance for UE category M1 | Same as clause 6.3.5EA.3 in TS 36.521-1 [14]. | Same as clause 6.3.5EA.3 in TS 36.521-1 [14]. | Same as clause 6.3.5EA.3 in TS 36.521-1 [14]. |
| 6.3B.1 UE Minimum output power for category NB1 and NB2 | Same as clause 6.3.2F in TS 36.521-1 [14] | Same as clause 6.3.2F in TS 36.521-1 [14] | Same as clause 6.3.2F in TS 36.521-1 [14] |
| 6.3B.2 Transmit OFF power for category NB1 and NB2 | Same as clause 6.3.3F in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.3F in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.3F in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |
| 6.3B.3.1 General ON/OFF time mask for category NB1 and NB2 | Same as clause 6.3.4F.1 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.4F.1 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.4F.1 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |
| 6.3B.3.2 NPRACH time mask for category NB1 and NB2 | Same as clause 6.3.4F.2 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.4F.2 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.4F.2 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |
| 6.3B.4.1 Power Control Absolute power tolerance for category NB1 and NB2 | Same as clause 6.3.5F.1 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.5F.1 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.5F.1 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |
| 6.3B.4.2 Power Control Relative power tolerance for category NB1 and NB2 | Same as clause 6.3.5F.2 in TS 36.521-1 [14] | Same as clause 6.3.5F.2 in TS 36.521-1 [14] | Same as clause 6.3.5F.2 in TS 36.521-1 [14] |
| 6.3B.4.3 Aggregate power control tolerance for category NB1 and NB2 | Same as clause 6.3.5F.3 in TS 36.521-1 [14] | Same as clause 6.3.5F.3 in TS 36.521-1 [14] | Same as clause 6.3.5F.3 in TS 36.521-1 [14] |
| 6.4A.1_1 Frequency error with GSO ephemeris for UE category M1 | Same as clause 6.5.1EA in TS 36.521-1 [14] | Same as clause 6.5.1EA in TS 36.521-1 [14] | Same as clause 6.5.1EA in TS 36.521-1 [14] |
| 6.4A.1_2 Frequency error with NGSO ephemeris for UE category M1 | Same as clause 6.5.1EA in TS 36.521-1 [14] | Same as clause 6.5.1EA in TS 36.521-1 [14] | Same as clause 6.5.1EA in TS 36.521-1 [14] |
| 6.4A.2.1 Error Vector Magnitude (EVM) for category M1 | Same as clause 6.5.2.1EA.1 in TS 36.521-1 [14] | Same as clause 6.5.2.1EA.1 in TS 36.521-1 [14] | Same as clause 6.5.2.1EA.1 in TS 36.521-1 [14] |
| 6.4A.2.2 Carrier leakage for category M1 | Same as clause 6.5.2.2EA in TS 36.521-1 [14] | Same as clause 6.5.2.2EA in TS 36.521-1 [14] | Same as clause 6.5.2.2EA in TS 36.521-1 [14] |
| 6.4A.2.3 In-band emissions for non allocated RB for category M1 | Same as clause 6.5.2.3EA in TS 36.521-1 [14] | Same as clause 6.5.2.3EA in TS 36.521-1 [14] | Same as clause 6.5.2.3EA in TS 36.521-1 [14] |
| 6.4A.2.4 EVM equalizer spectrum flatness for category M1 | Same as clause 6.5.2.4EA in TS 36.521-1 [14] | Same as clause 6.5.2.4EA in TS 36.521-1 [14] | Same as clause 6.5.2.4EA in TS 36.521-1 [14] |
| 6.4B.1_1 Frequency error with GSO ephemeris for UE category NB1 and NB2 | Same as clause 6.5.1F in TS 36.521-1 [14] | Same as clause 6.5.1F in TS 36.521-1 [14] | Same as clause 6.5.1F in TS 36.521-1 [14] |
| 6.4B.1_2 Frequency error with NGSO ephemeris for UE category NB1 and NB2 | Same as clause 6.5.1F in TS 36.521-1 [14] | Same as clause 6.5.1F in TS 36.521-1 [14] | Same as clause 6.5.1F in TS 36.521-1 [14] |
| 6.4B.2.1 Error Vector Magnitude (EVM) for Category NB1 and NB2 | Same as clause 6.5.2.1F.1 in TS 36.521-1 [14] | Same as clause 6.5.2.1F.1 in TS 36.521-1 [14] | Same as clause 6.5.2.1F.1 in TS 36.521-1 [14] |
| 6.4B.2.2 Carrier leakage for Category NB1 and NB2 | Same as clause 6.5.2.2F in TS 36.521-1 [14] | Same as clause 6.5.2.2F in TS 36.521-1 [14] | Same as clause 6.5.2.2F in TS 36.521-1 [14] |
| 6.4B.2.3 In-band emissions for Category NB1 and NB2 | Same as clause 6.5.2.3F in TS 36.521-1 [14] | Same as clause 6.5.2.3F in TS 36.521-1 [14] | Same as clause 6.5.2.3F in TS 36.521-1 [14] |
| 6.5A.2 Occupied bandwidth for category M1 | Same as clause 6.6.1EA in TS 36.521-1 [14] for FDD band with “channel bandwidth = 1.4MHz”. | Same as clause 6.6.1EA in TS 36.521-1 [14] | Same as clause 6.6.1EA in TS 36.521-1 [14] |
| 6.5A.3.2 Spectrum emission mask for category M1 | Same as clause 6.6.2.1EA in TS 36.521-1 [14] for FDD band with “channel bandwidth = 1.4MHz”. | Same as clause 6.6.2.1EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.6.2.1EA in TS 36.521-1 [14] |
| 6.5A.3.3 Additional Spectrum Emission Mask for category M1 | NS_02N and NS_03N -2 dBm / 4kHz -5 dBm / 4kHz -12 dBm / 4kHz -13 dBm / 4kHz -15 dBm / 4kHz NS_04N -2 dBm / 30kHz -26 dBm / 30kHz NS_05N -5 dBm / 30kHz -8.5 dBm / 30kHz -15 dBm / 30kHz -23 dBm / 30kHz -26 dBm / 30kHz | Same as clause 6.6.2.2EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.6.2.2EA in TS 36.521-1 [14] |
| 6.5A.3.4 Adjacent Channel Leakage Ratio for category M1 | Same as clause 6.6.2.3EA in TS 36.521-1 [14] | Same as clause 6.6.2.3EA in TS 36.521-1 [14] | Same as clause 6.6.2.3EA in TS 36.521-1 [14] |
| 6.5A.4.2 Transmitter Spurious emissions for category M1 | Same as clause 6.6.3EA.1 in TS 36.521-1 [14] for FDD band for spurious frequencies up to 12.75 GHz. | Same as clause 6.6.3EA.1 in TS 36.521-1 [14] | Same as clause 6.6.3EA.1 in TS 36.521-1 [14] |
| 6.5A.4.3 Spurious emission band UE co-existence for category M1 | Same as clause 6.6.3EA.2 in TS 36.521-1 [14] | Same as clause 6.6.3EA.2 in TS 36.521-1 [14] | Same as clause 6.6.3EA.2 in TS 36.521-1 [14] |
| 6.5A.4.4 Additional spurious emissions for category M1 | -26 dBm / 700kHz -50 dBm / 700kHz -16 dBm / 1MHz -40 dBm / 1MHz Frequencies as detailed in core requirement | 0 dB | Formula: Minimum Requirement + TT |
| 6.5B.2 Occupied bandwidth for category NB1 and NB2 | Same as clause 6.6.1F in TS 36.521-1 [14] | Same as clause 6.6.1F in TS 36.521-1 [14] | Same as clause 6.6.1F in TS 36.521-1 [14] |
| 6.5B.3.2 Spectrum emission mask for category NB1 and NB2 | Same as clause 6.6.2.1F in TS 36.521-1 [14] | Same as clause 6.6.2.1F in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.6.2.1F in TS 36.521-1 [14] |
| 6.5B.3.3 Additional Spectrum Emission Mask for category NB1 and NB2 | NS_02N and NS_03N -2 dBm / 4kHz -5 dBm / 4kHz -12 dBm / 4kHz -13 dBm / 4kHz -15 dBm / 4kHz NS_04N -2 dBm / 30kHz -26 dBm / 30kHz NS_05N -5 dBm / 30kHz -8.5 dBm / 30kHz -15 dBm / 30kHz -23 dBm / 30kHz -26 dBm / 30kHz | Same as clause 6.6.2.2 in TS 36.521-1 [14] | Same as clause 6.6.2.2 in TS 36.521-1 [14] |
| 6.5B.3.4 Adjacent Channel Leakage Ratio for category NB1 and NB2 | Same as clause 6.6.2.3F in TS 36.521-1 [14] | Same as clause 6.6.2.3F in TS 36.521-1 [14] | Same as clause 6.6.2.3F in TS 36.521-1 [14] |
| 6.5B.4.2 Transmitter Spurious emissions for category NB1 and NB2 | Same as clause 6.6.3F.1 in TS 36.521-1[14] for f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | Same as clause 6.6.3F in TS 36.521-1[14] for f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | Same as clause 6.6.3F in TS 36.521-1[14] for f < 5th harmonic of the upper frequency edge of the UL operating band in GHz |
| 6.5B.4.3 Spurious emission band UE co-existence for category NB1 and NB2 | Same as 6.6.3F.2 in TS 36.521-1 [14] | Same as 6.6.3F.2 in TS 36.521-1 [14] | Same as 6.6.3F.2 in TS 36.521-1 [14] |
| 6.5B.4.4 Additional spurious emissions for category NB1 and NB2 | -26 dBm / 700kHz -50 dBm / 700kHz -16 dBm / 1MHz -40 dBm / 1MHz Frequencies as detailed in core requirement | 0 dB | Formula: Minimum Requirement + TT |
| 6.6B Transmit intermodulation for category NB1 and NB2 | Same as clause 6.7F in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz” | Same as clause 6.7F in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz” | Same as clause 6.7F in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz” |

< End of Change3 >