3GPP TSG-RAN WG5 Meeting #107	R5-252087r1

# Malta, MT, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3457 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test frequency for protocol testing for n104 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_lic_bands_BW_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test frequencies for protocol testing for n104 are missing and need to be added. SSB SCS for n38/n39/n40/n48/n50/n41/n77/n78/n79 to 30Khz are incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test frequencies for protocol testing for n104 has been added. Correct SSB SCS for n38/n39/n40/n48/n50/n41/n77/n78/n79 to 30Khz. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The band can not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.508-1 CR 3502 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is related to R5-252585. Test frequency for RF testing for band n104 is introduced in R5-252585 (CR number 3502). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Add the test frequency of n104 for (e)Redcap test in Table 6.2.3.1-5and for configuring a BWP without CORESET#0 in Table 6.2.3.1-8. Correct SSB SCS for n38/n39/n40/n48/n50/n41/n77/n78/n79 to 30Khz. Also add Huawei, Hisilicon as co-source company. |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

Table 6.2.3.1-5: Test frequencies for NR TDD FR1 bands using 100 MHz channel bandwidth for non-(e)RedCap UE

| NR Band | SCS [kHz] | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA[ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n77 | 30 | 100 | 273 | Downlink | Low, High | Same values as for Low and High range in clause 4.3.1.1.1.77 for bandwidth=100 MHz and SCS=30 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | & | Mid-Low | 3616.68 | 641112 | 3563.22 | 637548 | 12 | 30 | 7896 | 638112 | 12 | 0 | 1 (1) | 26 |
|  |  |  |  | Uplink | Mid-High | 3883.32 | 658888 | 3825.54 | 655036 | 24 |  | 8081 | 655872 | 20 | 0 | 0 (0) | 48 |
| n78 | 30 | 100 | 273 | Downlink | Low, High | Same values as for Low and High range in clause 4.3.1.1.1.78 for bandwidth=100 MHz and SCS=30 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | & | Mid-Low | 3483.33 | 632222 | 3429.87 | 628658 | 12 | 30 | 7804 | 629280 | 22 | 0 | 3 (3) | 30 |
|  |  |  |  | Uplink | Mid-High | 3616.68 | 641112 | 3558.9 | 637260 | 24 |  | 7896 | 638112 | 12 | 0 | 1 (1) | 50 |
| n79 | 30 | 100 | 273 | Downlink | Low, High | Same values as for Low and High range in clause 4.3.1.1.1.79 for bandwidth=100 MHz and SCS=30 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | & | Mid-Low | 4616.67 | 707778 | 4563.21 | 704214 | 12 | 30 | 8592 | 704928 | 18 | 6 | 1 (4) | 38 |
|  |  |  |  | Uplink | Mid-High | 4783.35 | 718890 | 4725.57 | 715038 | 24 |  | 8720 | 717216 | 18 | 54 | 1 (4) | 160 |
| n104 | 30 | 100 | 273 | Downlink | Low, High | Same values as for Low and High range in clause 4.3.1.1.1.104 for bandwidth=100 MHz and SCS=30 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | & | Mid-Low | 6675.00 | 845000 | 6621.54 | 841436 | 12 | 30 | 10022 | 842208 | 4 | 6 | 1 (4) | 44 |
|  |  |  |  | Uplink | Mid-High | 6875.01 | 858334 | 6817.23 | 854482 | 24 |  | 10162 | 855648 | 14 | 10 | 1 (4) | 76 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22] for all bands in the table except for band n79 and n104 where Table 13-6 apply. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.1-5A: Test frequencies for NR TDD FR1 bands using 20 MHz channel bandwidth for (e)RedCap UE

| NR Band | SCS [kHz] | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA[ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n77 | 30 | 20 | 51 | Downlink | Low, High | Same values as for Low and High range in clause 4.3.1.1.1.77 for bandwidth=20 MHz and SCS=30 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | & | Mid-Low | 3603.33 | 640222 | 3589.83 | 639322 | 12 | 30 | 7915 | 639936 | 14 | 0 | 3 (3) | 30 |
|  |  |  |  | Uplink | Mid-High | 3896.67 | 659778 | 3878.85 | 658590 | 24 |  | 8118 | 659424 | 18 | 0 | 0 (0) | 48 |
| n78 | 30 | 20 | 51 | Downlink | Low, High | Same values as for Low and High range in clause 4.3.1.1.1.78 for bandwidth=20 MHz and SCS=30 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | & | Mid-Low | 3470.01 | 631334 | 3456.51 | 630434 | 12 | 30 | 7822 | 631008 | 22 | 0 | 1 (1) | 26 |
|  |  |  |  | Uplink | Mid-High | 3630 | 642000 | 3612.18 | 640812 | 24 |  | 7933 | 641664 | 12 | 0 | 1 (1) | 50 |
| n79 | 30 | 20 | 51 | Downlink | Low, High | Same values as for Low and High range in clause 4.3.1.1.1.79 for bandwidth=20 MHz and SCS=30 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | & | Mid-Low | 4603.35 | 706890 | 4589.85 | 705990 | 12 | 30 | 8609 | 706560 | 18 | 1 | 0 (0) | 26 |
|  |  |  |  | Uplink | Mid-High | 4796.67 | 719778 | 4778.85 | 718590 | 24 |  | 8743 | 719424 | 18 | 0 | 0 (0) | 48 |
| n104 | 30 | 20 | 51 | Downlink | Low, High | Same values as for Low and High range in clause 4.3.1.1.1.104 for bandwidth=20 MHz and SCS=30 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | & | Mid-Low | 6661.65 | 844110 | 6648.15 | 843210 | 12 | 30 | 10043 | 844224 | 6 | 16 | 4 (4) | 64 |
|  |  |  |  | Uplink | Mid-High | 6888.33 | 859222 | 6870.51 | 858034 | 24 |  | 10197 | 859008 | 14 | 2 | 4 (4) | 60 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22] for all bands in the table except for band n79 where Table 13-6 apply. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<< Unchanged Parts Skipped >>

Table 6.2.3.1-8: Test frequencies for NR FR1 bands configuring a BWP without CORESET#0

| NR Band | SCS [kHz] Note2 | CBW [MHz] Note3 | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | point A [MHz] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] Note 4 | absoluteFrequencySSB [ARFCN] Note 5 |  | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 | absoluteFrequencySSB for NCD-SSB [ARFCN] | Rbstart of BWP | LRB of BWP | locationAndBandwidth of BWP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n5 | 15 | 20 | 106 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.5 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 177312 | 80 | 25 | 6680 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.5 for bandwidth=20 MHz and SCS=15 kHz |  |  |  |  |  |  |  | - | 80 | 25 | 6680 |
| n8 | 15 | 20 | 106 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.8 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 188512 | 80 | 25 | 6680 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.8 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 80 | 25 | 6680 |
| n12 | 15 | 15 | 79 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.12 for bandwidth=15 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 148362 | 54 | 25 | 6654 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.12 for bandwidth=15 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 54 | 25 | 6654 |
| n13 | 15 | 10 | 52 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.13 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 151668 | 24 | 25 | 6624 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.13 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 24 | 25 | 6624 |
| n14 | 15 | 10 | 52 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.14 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 153068 | 24 | 25 | 6624 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.14 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 24 | 25 | 6624 |
| n20 | 15 | 20 | 106 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.20 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 161712 | 80 | 25 | 6680 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.20 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 80 | 25 | 6680 |
| n24 | 15 | 10 | 52 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.24 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 306668 | 24 | 25 | 6624 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.24 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 24 | 25 | 6624 |
| n26 | 15 | 20 | 106 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.26 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 175312 | 80 | 25 | 6680 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.26 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 80 | 25 | 6680 |
| n29 | NA | NA | NA | NA | NA | NA |  |  |  |  |  |  |  | NA | NA | NA | NA |
| n30 | 15 | 10 | 52 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.30 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 471468 | 24 | 25 | 6624 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.30 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 24 | 25 | 6624 |
| n70 | 15 | 15 | 79 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.70 for bandwidth=15 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 401562 | 54 | 25 | 6654 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.70 for bandwidth=15 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 54 | 25 | 6654 |
| n71 | 15 | 20 | 106 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.71 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 126912 | 80 | 25 | 6680 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.71 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 80 | 25 | 6680 |
| n76 | NA | NA | NA | NA | NA | NA |  |  |  |  |  |  |  | NA | NA | NA | NA |
| n91 | NA | NA | NA | NA | NA | NA |  |  |  |  |  |  |  | NA | NA | NA | NA |
| n92 | 15 | 20 | 106 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.92 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 289912 | 80 | 25 | 6680 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.92 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 80 | 25 | 6680 |
| n93 | NA | NA | NA | NA | NA | NA |  |  |  |  |  |  |  | NA | NA | NA | NA |
| n100 | NA | NA | NA | NA | NA | NA |  |  |  |  |  |  |  | NA | NA | NA | NA |
| n1 | 15 | 50 | 270 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.1 for bandwidth=50 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 431500 | 218 | 52 | 14243 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.1 for bandwidth=50 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 218 | 52 | 14243 |
| n2 | 15 | 40 | 216 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.2 for bandwidth=40 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 393456 | 162 | 52 | 14187 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.2 for bandwidth=40 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 162 | 52 | 14187 |
| n3 | 15 | 50 | 270 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.3 for bandwidth=50 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 370500 | 218 | 52 | 14243 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.3 for bandwidth=50 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 218 | 52 | 14243 |
| n7 | 15 | 50 | 270 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.7 for bandwidth=50 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 533500 | 218 | 52 | 14243 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.7 for bandwidth=50 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 218 | 52 | 14243 |
| n25 | 15 | 40 | 216 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.25 for bandwidth=40 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 393456 | 162 | 52 | 14187 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.25 for bandwidth=40 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 162 | 52 | 14187 |
| n28 | 15 | 30 | 160 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.28 for bandwidth=30 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 157120 | 108 | 52 | 14133 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.28 for bandwidth=30 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 108 | 52 | 14133 |
| n65 | 15 | 20 | 106 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.65 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 425548 | 54 | 52 | 14079 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.65 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 54 | 52 | 14079 |
| n66 | 15 | 40 | 216 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.66 for bandwidth=40 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 429456 | 162 | 52 | 14187 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.66 for bandwidth=40 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 162 | 52 | 14187 |
| n74 | 15 | 20 | 106 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.74 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 298548 | 54 | 52 | 14079 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.74 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 54 | 52 | 14079 |
| n75 | NA | NA | NA | NA | NA | NA |  |  |  |  |  |  |  | NA | NA | NA | NA |
| n94 | 15 | 20 | 106 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.94 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 289948 | 54 | 52 | 14079 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.94 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 54 | 52 | 14079 |
| n109 | 15 | 30 | 160 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.109 for bandwidth=30 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 288050 | 108 | 52 | 14133 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.109 for bandwidth=30 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 108 | 52 | 14133 |
| n34 | 15 | 15 | 79 | Downlink& Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.34 for bandwidth=15 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 404562 | 54 | 25 | 6654 |
| n51 | NA | NA | NA | NA | NA | NA |  |  |  |  |  |  |  | NA | NA | NA | NA |
| n53 | 15 | 10 | 52 | Downlink& Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.53 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 498168 | 24 | 25 | 6624 |
| n54 | NA | NA | NA | NA | NA | NA |  |  |  |  |  |  |  | NA | NA | NA | NA |
| n101 | NA | NA | NA | NA | NA | NA |  |  |  |  |  |  |  | NA | NA | NA | NA |
| n38 | 30 | 40 | 106 | Downlink& Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.38 for bandwidth=40 MHz and SCS=30 kHz. |  |  |  |  |  |  |  | 520952 | 80 | 24 | 6405 |
| n39 | 30 | 40 | 106 | Downlink& Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.39 for bandwidth=40 MHz and SCS=30 kHz. |  |  |  |  |  |  |  | 382952 | 80 | 24 | 6405 |
| n40 | 30 | 80 | 217 | Downlink& Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.40 for bandwidth=80 MHz and SCS=30 kHz. |  |  |  |  |  |  |  | 475020 | 192 | 24 | 6517 |
| n48 | 30 | 40 | 106 | Downlink& Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.48 for bandwidth=40 MHz and SCS=30 kHz. |  |  |  |  |  |  |  | 638984 | 80 | 24 | 6405 |
| n50 | 30 | 60 | 162 | Downlink& Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.50 for bandwidth=60 MHz and SCS=30 kHz. |  |  |  |  |  |  |  | 297512 | 138 | 24 | 6463 |
| n41 | 30 | 100 | 273 | Downlink& Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.41 for bandwidth=100 MHz and SCS=30 kHz. |  |  |  |  |  |  |  | 518310 | 222 | 51 | 13972 |
| n77 | 30 | 100 | 273 | Downlink& Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.77 for bandwidth=100 MHz and SCS=30 kHz. |  |  |  |  |  |  |  | 626370 | 222 | 51 | 13972 |
| n78 | 30 | 100 | 273 | Downlink& Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.78 for bandwidth=100 MHz and SCS=30 kHz. |  |  |  |  |  |  |  | 626370 | 222 | 51 | 13972 |
| n79 | 30 | 100 | 273 | Downlink& Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.79 for bandwidth=100 MHz and SCS=30 kHz. |  |  |  |  |  |  |  | 699704 | 222 | 51 | 13972 |
| n104 | 30 | 100 | 273 | Downlink& Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.104 for bandwidth=100 MHz and SCS=30 kHz. |  |  |  |  |  |  |  | 834704 | 222 | 51 | 13972 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: Subcarrier spacing is configured according to Table 6.2.3.1-1 to 6.2.3.1-5. Note 3: Channel bandwidth is configured according to highest mandatory UL and DL channel bandwidth in clause 4.3.1. Note 4: SS block SCS for both CD-SSB and NCD-SSB. Note 5: AbsoluteFrequencySSB for CD-SSB Note 6: n29,n75,n76 are SDL band Note 7: The maximum carrierBandwidth for n91,n93,n100,n51 and n101 with signalling test SCS in 6.2.3.1-1 to 6.2.3.1-5 are no more than 25RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<< END OF CHANGES >>