3GPP TSG-RAN5 Meeting #107	R5-252222r1

# St Julian’s, Malta, 19th - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0664 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of eCall test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCallCEN-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - At RAN5#106 new eCall test cases 11.7, 11.8 and 11.9 have been introduced with "eCall capable" instead of "eCall over IMS" for clarification in the title. The same needs to be applied for test cases 11.1 .. 11.6 - Test cases 11.10, 11.11 and 11.12 need condition A1 (Response sent by SS for INVITE for eCall over IMS) for 486 Busy Here (11.10), 600 Busy Everywhere (11.11) and 603 Decline (11.12) and condition A4 (ACK for non-2xx response) for the SIP ACK. - In test cases 11.10, 11.11 and 11.12 it shall be checked in the last step that the UE stays at the NR cell i.e. that it does not select another RAT (CEN requirement #3: the UE shall wait for a potential PSAP callback but not "immediately retry the eCall in the CS domain if available or over another PS RAT or PLMN"). It is assumed that even though there are no other cells in the test cases, a UE which does not fulfil CEN requirement #3 will be busy seaching for other cells on other RAT(s) and therefore not respond to the paging at the NR cell. - At steps 33 and 53 in test case 11.14 it needs to be specified that the SIP INFO as specified for step 6 of A.23 (specifying condition A3) shall be used. - The terms "eCall capable" and "eCall only" are not well defined for the eCall test cases. - Editorial issues. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Title changed for eCall test cases 11.1, 11.2, 11.4, 11.5 and 11.6. - Specific message contents added to test cases 11.10, 11.11 and 11.12. - Note added for clarification to the last steps of 11.10, 11.11 and 11.12. - Specific message content added for the MO SIP INFO at steps 33 and 53 in test case 11.14. - Clause 11.0 added for definitions. - Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent and incorrect test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision1: Further editorial corrections, SIP ACK in test cases 11.10, 11.11 and 11.12 needs condition A4 instead of A1. |  |  |  |  |  |  |  |  |

<Start of modification>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 34.229-1: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 1: Protocol conformance specification".

[3]	3GPP TS 34.229-2: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 2: Implementation Conformance Statement (ICS) proforma specification".

[4]	3GPP TS 34.229-3: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 3: Abstract Test Suites (ATS)".

[5]	3GPP TS 34.229-4: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 4: Enabler for IP multimedia applications testing".

[6]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[7]	3GPP TS 24.229: "IP Multimedia Call Control Protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[8]	ETSI ES 201 873-1: "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 1: TTCN-3 Core Language".

[9]	ETSI ES 201 873-2: "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 2: TTCN-3 Tabular Presentation Format (TFT)".

[10]	ETSI TR 201 873-3: "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 3: TTCN-3 Graphical Presentation Format (GFT)".

[11]	ETSI ETS 300 406: "Methods for testing and Specification (MTS); Protocol and profile conformance testing specifications; Standardization methodology".

[12]	ISO/IEC 9646-1: "Information technology - Open systems interconnection - Conformance testing methodology and framework - Part 1: General concepts".

[13]	ISO/IEC 9646-7: "Information technology - Open systems interconnection - Conformance testing methodology and framework - Part 7: Implementation Conformance Statements".

[14]	3GPP TS 24.341: "Support of SMS over IP networks; Stage 3".

[15]	IETF RFC 3310: "Hypertext Transfer Protocol (HTTP) Digest Authentication Using Authentication and Key Agreement (AKA)".

[16]	3GPP TS 33.203: "3G security;Access security for IP-based services".

[17]	IETF RFC 3329: "Security Mechanism Agreement for the Session Initiation Protocol (SIP)".

[18]	IETF RFC 3680: "A Session Initiation Protocol (SIP) Event Package for Registrations".

[19]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[20]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[21]	3GPP TS 38.508-1: "5GS; User Equipment (UE) conformance specification; Part 1: Common test environment".

[22]	3GPP TS 27.007: "AT command set for User Equipment (UE)".

[23]	IETF RFC 2617: "HTTP Authentication: Basic and Digest Access Authentication".

[24]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[25]	3GPP TS 24.011: "Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface".

[26]	3GPP TS 24.237: "IP Multimedia (IM) Core Network (CN) subsystem IP Multimedia Subsystem (IMS) Service Continuity".

[27]	3GPP TS 23.003: "Numbering, addressing and identification".

[28]	IETF RFC 6665: “SIP-Specific Event Notification”.

[29]	IETF RFC 3312: “Integration of Resource Management and SIP”.

[30]	IETF RFC 3262: “Reliability of Provisional Responses in the Session Initiation Protocol (SIP)”.

[31]	GSMA PRD NG.114: "IMS Profile for Voice, Video and Messaging over 5GS".

[32]	3GPP TS 24.610: "Communication HOLD (HOLD) using IP Multimedia (IM) Core Network (CN) subsystem".

[33]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[34]	3GPP TS 24.606: "Message Waiting Indication (MWI) using IP Multimedia (IM) Core Network (CN) subsystem".

[35]	3GPP TS 24.147: "Conferencing using the IP Multimedia (IM) Core Network (CN) subsystem".

[36]	3GPP TS 24.629: "Explicit Communication Transfer (ECT) using IP Multimedia (IM) Core Network (CN) subsystem".

[37]	IETF RFC 4028: "Session Timers in the Session Initiation Protocol (SIP)".

[38]	IETF RFC 4566: "SDP: Session Description Protocol".

[39]	IETF RFC 7462: "URNs for the Alert-Info Header Field of the Session Initiation Protocol (SIP) ".

[40]	IETF RFC 3891: "The Session Initiation Protocol (SIP) "Replaces" Header".

[41]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax".

[42]	IETF RFC 3326: "The Reason Header Field for the Session Initiation Protocol (SIP)".

[43]	3GPP TS 24.109: "Bootstrapping interface (Ub) and network application function interface (Ua); Protocol details".

[44]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture".

[45]	IETF RFC 4825: "The Extensible Markup Language (XML) Configuration Access Protocol (XCAP)".

[46]	IETF RFC 2616: "Hypertext Transfer Protocol -- HTTP/1.1".

[47]	IETF RFC 3310: "Hypertext Transfer Protocol (HTTP) Digest Authentication Using Authentication and Key Agreement (AKA)".

[48]	3GPP TS 38.509: "5GS; Special conformance testing functions for User Equipment (UE)".

[49]	IETF RFC 6228: "Session Initiation Protocol (SIP) Response Code for Indication of Terminated Dialog".

[50]	IETF RFC 3323: "A Privacy Mechanism for the Session Initiation Protocol (SIP)".

[51]	IETF RFC 3325: "Private Extensions to the Session Initiation Protocol (SIP) for Asserted Identity within Trusted Networks".

[52]	3GPP TS 38.508-2: "5GS; User Equipment (UE) conformance specification; Part 2: Common
Implementation Conformance Statement (ICS) proforma".

[53]	3GPP TS 36.523-3: "LTE; Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification; Part 3: Test suites".

[xyz]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

<End of modification>

<Start of modification>

# 11	eCall over IMS

## 11.0	General

The following terminology is used for eCall test cases in clause 11:

eCall capable: A UE which is able to do an eCall is considered as "eCall capable". After switch-on such a UE does normal IMS registration.

eCall only mode: A UE configured for eCall only mode as defined in clause 3.1 of TS 23.401 [xyz].

## 11.1	eCall capable / Manual initiation / Emergency registration / Success / 200 OK with ACK / 5GS

11.1.1	Test Purpose (TP)

(1)

with { UE being registered to IMS }

ensure that {

when { UE is being made to initiate a manual eCall over IMS }

then { UE sends a correctly composed initial REGISTER request for IMS emergency registration }

}

(2)

with { UE having sent an unprotected REGISTER request }

ensure that {

when { UE receives a valid 401 (Unauthorized) response for the initial REGISTER request sent }

then { UE correctly authenticates itself by sending another REGISTER request with a correctly composed Authorization header using the AKAv1-MD5 algorithm }

}

(3)

with { UE having sent unprotected and then protected REGISTER request }

ensure that {

when { UE receives a valid 200 OK response for the REGISTER sent for authentication }

then { UE sends a correctly composed INVITE request with MSD }

}

(4)

with { UE having sent INVITE with MSD for manual eCall over IMS }

ensure that {

when { UE receives 200 OK containing acknowledgement of reception of MSD }

then { UE sends ACK }

}

(5)

with { eCall over IMS being established }

ensure that {

when { Network sends BYE }

then { UE sends 200 OK for BYE }

}

11.1.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-16 requirements.

[TS 24.229 clause 5.1.6.11.1]:

If the upper layers request establishment of an IMS emergency call of the manually initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.manual" as specified in RFC 8147 [244].

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.automatic" as specified in RFC 8147 [244].

NOTE 1:	The manually initiated eCall type of emergency service is used when the eCall IMS emergency session is invoked with user input. The automatically initiated eCall type of emergency service is used if the eCall IMS emergency session is invoked without user input.

[TS 24.229 clause 5.1.6.11.2]:

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service or of the manually initiated eCall type of emergency service and if allowed by IP-CAN specific annex, the UE shall send an INVITE request as specified in the procedures in subclause 5.1.6.8 with the following additions:

1)	the UE shall set the Request-URI to "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual"; and

2)	if the IP-CAN indicates the eCall support indication, the UE shall:

a)	insert a multipart/mixed body containing an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244], containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 PER as specified in CEN EN 15722:2015 [245] and include a Content-Disposition header field with a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

b)	insert an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244]; and

c)	insert a Recv-Info header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244].

NOTE:	Further content for the INVITE is as defined in RFC 8147 [244].

Then the UE shall proceed as follows:

…

2)	if the UE receives a 200 (OK) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "true"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as successful;

11.1.3	Test description

11.1.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC with eCall subscription according to TS 38.508-1 [21] table 6.4.1-23.

-	UE is configured to register for IMS after switch on.

Preamble:

-	UE is in state 1N-A (TS 38.508-1 [21]) and registered to IMS.

11.1.3.2	Test procedure sequence

Table 11.1.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is triggered to start a manual eCall | - | - | - | - |
| 2-11 | Steps 1-10 of generic procedure specified in Table 4.9.11.2.2-1 of TS 38.508-1 [21] with condition ‘eCall’ are performed. | - | - | - | - |
| 12 | Step 1 of Annex A.3 happens Check: Does UE send a correctly composed initial REGISTER request for IMS emergency registration? | --> | REGISTER | 1 | P |
| 13 | Step 2 of Annex A.3 happens | <-- | 401 Unauthorized | - | - |
| 14 | Step 3 of Annex A.3 happens Check: Does UE send another REGISTER with AKAv1-MD5 credentials? | --> | REGISTER | 2 | P |
| 15 | Step 4 of Annex A.3 happens | <-- | 200 OK | - | - |
| - | EXCEPTION: In parallel to the events described in steps 16 and 17, steps 11 to 13 specified in 38.508-1 [21] Table 4.9.11.2.2-1 takes place. | - | - | - | - |
| 16 | Step 1 of Annex A.23 happens Check: Does the UE include MSD in the message body? | --> | INVITE | 3 | P |
| 17 | Step 2 of Annex A.23 happens | <-- | 200 OK | - | - |
| 18 | Step 3 of Annex A.23 happens Check: Does the UE send ACK? | --> | ACK | 4 | P |
| 19 | Step 1 of Annex A.8 happens | <-- | BYE | - | - |
| 20 | Step 2 of Annex A.8 happens Check: Does the UE send 200 OK for the BYE request and ends the call? | --> | 200 OK | 5 | P |

11.1.3.3	Specific message contents

Table 11.1.3.3-1: INVITE (step 16, table 11.1.3.2-1)

Derivation path: Step 1 in Annex A.23 with Condition A20

Table 11.1.3.3-2: Void

Table 11.1.3.3-3: BYE (step 19, table 11.1.3.2-1)

Derivation Path: TS 34.229-1 [2], Annex A.2.8 with condition A9.

## 11.2	eCall capable / Automatic initiation / Emergency registration / Success / 200 OK with ACK / 5GS

11.2.1	Test Purpose (TP)

(1)

with { UE being registered to IMS }

ensure that {

when { UE is being made to initiate an automatic eCall over IMS }

then { UE sends a correctly composed initial REGISTER request for IMS emergency registration }

}

(2)

with { UE having sent an unprotected REGISTER request }

ensure that {

when { UE receives a valid 401 (Unauthorized) response for the initial REGISTER request sent }

then { UE correctly authenticates itself by sending another REGISTER request with a correctly composed Authorization header using the AKAv1-MD5 algorithm }

}

(3)

with { UE having sent unprotected and then protected REGISTER request }

ensure that {

when { UE receives a valid 200 OK response for the REGISTER sent for authentication }

then { UE sends a correctly composed INVITE request with MSD }

}

(4)

with { UE having sent INVITE with MSD for automatic eCall over IMS }

ensure that {

when { UE receives 200 OK containing acknowledgement of reception of MSD }

then { UE sends ACK }

}

(5)

with { eCall over IMS being established }

ensure that {

when { Network sends BYE }

then { UE sends 200 OK for BYE }

}

11.2.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-16 requirements.

[TS 24.229 clause 5.1.6.11.1]:

If the upper layers request establishment of an IMS emergency call of the manually initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.manual" as specified in RFC 8147 [244].

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.automatic" as specified in RFC 8147 [244].

NOTE 1:	The manually initiated eCall type of emergency service is used when the eCall IMS emergency session is invoked with user input. The automatically initiated eCall type of emergency service is used if the eCall IMS emergency session is invoked without user input.

[TS 24.229 clause 5.1.6.11.2]:

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service or of the manually initiated eCall type of emergency service and if allowed by IP-CAN specific annex, the UE shall send an INVITE request as specified in the procedures in subclause 5.1.6.8 with the following additions:

1)	the UE shall set the Request-URI to "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual"; and

2)	if the IP-CAN indicates the eCall support indication, the UE shall:

a)	insert a multipart/mixed body containing an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244], containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 PER as specified in CEN EN 15722:2015 [245] and include a Content-Disposition header field with a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

b)	insert an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244]; and

c)	insert a Recv-Info header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244].

NOTE:	Further content for the INVITE is as defined in RFC 8147 [244].

Then the UE shall proceed as follows:

…

2)	if the UE receives a 200 (OK) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "true"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as successful;

11.2.3	Test description

11.2.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC with eCall subscription according to TS 38.508-1 [21] table 6.4.1-23.

-	UE is configured to register for IMS after switch on.

Preamble:

-	UE is in state 1N-A (TS 38.508-1 [21]) and registered to IMS.

11.2.3.2	Test procedure sequence

Table 11.2.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is triggered to start an automatic eCall | - | - | - | - |
| 2-9 | Steps 1-8 of generic procedure specified in Table 4.9.11.2.2-1 of TS 38.508-1 [21] with condition ‘eCall’ are performed. | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in steps 10-11 below the events specified in Table 11.2.3.2-2 take place. | - | - | - | - |
| 10-11 | Steps 9-10 of Table 4.9.11.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in steps 12 and 13, steps 11 to 13 specified in 38.508-1 [21] Table 4.9.11.2.2-1 take place. | - | - | - | - |
| 12 | Step 1 of Annex A.23 happens Check: Does the UE include MSD in the message body? | --> | INVITE | 3 | P |
| 13 | Step 2 of Annex A.23 happens | <-- | 200 OK | - | - |
| 14 | Step 3 of Annex A.23 happens Check: Does the UE send ACK? | --> | ACK | 4 | P |
| 15 | Step 1 of Annex A.8 happens | <-- | BYE | - | - |
| 16 | Step 2 of Annex A.8 happens Check: Does the UE send 200 OK for the BYE request and end the call? | --> | 200 OK | 5 | P |

Table 11.2.3.2-2: Parallel Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour depending UE implementation; the "lower case letter" identifies a step sequence that takes place if the UE performs a specific action. | - | - | - | - |
| 1a1 | The generic procedure for IP address allocation in the user plane specified in subclause 4.5A.3 takes place. | - | - | - | - |
| 2 | Step 1 of Annex A.3 happens Check: Does UE send a correctly composed initial REGISTER request for IMS emergency registration? | --> | REGISTER | 1 | P |
| 3 | Step 2 of Annex A.3 happens | <-- | 401 Unauthorized | - | - |
| 4 | Step 3 of Annex A.3 happens Check: Does UE send another REGISTER with AKAv1-MD5 credentials? | --> | REGISTER | 2 | P |
| 5 | Step 4 of Annex A.3 happens | <-- | 200 OK | - | - |

11.2.3.3	Specific message contents

Table 11.2.3.3-1: INVITE (step 12, table 11.2.3.2-1)

Derivation path: Step 1 in Annex A.23 with Condition A21

Table 11.2.3.3-2: Void

Table 11.2.3.3-3: BYE (step 15, table 11.2.3.2-1)

Derivation Path: TS 34.229-1 [2], Annex A.2.8 with condition A9.

## 11.3	Void

## 11.4	eCall capable / Manual initiation / MSD transfer and 200 OK with ACK / SIP INFO for MSD Update / Success / 5GS

11.4.1	Test Purpose (TP)

(1)

with { UE being registered to IMS }

ensure that {

when { UE is being made to initiate a manual eCall over IMS }

then { UE performs IMS emergency registration and sends correctly composed INVITE with MSD }

}

(2)

with { UE having sent an  INVITE }

ensure that {

when { UE receives 200 OK containing acknowledgement of reception of MSD }

then { UE sends ACK }

}

(3)

with { eCall over IMS being established }

ensure that {

when { UE receives SIP INFO requesting MSD update }

then { UE sends 200 OK acknowledging SIP INFO }

}

(4)

with { UE having sent 200 OK for SIP INFO }

ensure that {

when { UE is able to provide the updated MSD }

then { UE sends a correctly composed SIP INFO with updated MSD information }

}

(5)

with { UE having received 200 OK acknowledging the reception of updated MSD in SIP INFO }

ensure that {

when { Network sends BYE }

then { UE sends 200 OK for BYE }

}

11.4.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-16 requirements.

[TS 24.229 clause 5.1.6.11.3]:

During an emergency session established for eCall type of emergency service as described in subclause 5.1.6.11.2, if the UE receives an INFO request with:

1)	an Info-Package header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244];

2)	a multipart/mixed body including:

a)	an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] containing a "request" element with an "action" attribute set to "send-data" and a "datatype" attribute set to "eCall.MSD"; and

b)	a Content-Disposition header field set to "By-Reference" associated with the "application/EmergencyCallData.Control+xml" MIME body part; and

3)	a Content-Disposition header field set to "Info-Package" associated with the multipart/mixed body;

the UE shall proceed as follows:

1)	if the UE is able to provide an updated MSD, the UE shall send an INFO request containing:

a)	an Info-Package header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244];

b)	a multipart/mixed body including:

i)	an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244] containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 as specified in CEN EN 15722:2015 [245]; and

ii)	a Content-Disposition header field set to "By-Reference" associated with the "application/EmergencyCallData.eCall.MSD" MIME body part; and

c)	a Content-Disposition header field set to "Info-Package" associated with the multipart/mixed body; and

2)	if the UE is not able to provide an updated MSD, the UE shall send an INFO request containing:

a)	an Info-Package header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244];

b)	a multipart/mixed body including:

i)	an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

-	a "ref" attribute set to the Content-ID of the "application/EmergencyCallData.Control+xml" MIME body part in the INFO request received by the UE; and

-	an "actionResult" child element containing:

A)	an "action" attribute set to "send-data";

B)	a "success" attribute set to "false"; and

C)	a "reason" attribute set to an appropriate value as defined in RFC 8147 [244]; and

ii)	a Content-Disposition header field set to "By-Reference" associated with the "application/EmergencyCallData.Control+xml" MIME body part; and

c)	a Content-Disposition header field set to "Info-Package" associated with the multipart/mixed body.

NOTE:	Further content for the INFO request is as defined in RFC 8147 [244].

11.4.3	Test description

11.4.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC with eCall subscription according to TS 38.508-1 [21] table 6.4.1-23.

-	UE is configured to register for IMS after switch on.

Preamble:

-	UE is in state 1N-A (TS 38.508-1 [21]) and registered to IMS.

11.4.3.2	Test procedure sequence

Table 11.4.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is triggered to start a manual eCall. | - | - | - | - |
| 2-11 | Steps 1-10 of generic procedure specified in Table 4.9.11.2.2-1 of TS 38.508-1 [21] with condition ‘eCall’ are performed. | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in steps 12 and 13, steps 11 to 13 specified in 38.508-1 [21] Table 4.9.11.2.2-1 takes place. | - | - | - | - |
| 12 | Step 1 of Annex A.23 happens Check: Does the UE include MSD in the message body? | --> | INVITE | 1 | P |
| 13 | Step 2 of Annex A.23 happens | <-- | 200 OK | - | - |
| 14 | Step 3 of Annex A.23 happens Check: Does the UE send ACK? | --> | ACK | 2 | P |
| 15 | Step 4 of Annex A.23 happens | <-- | SIP INFO | - | - |
| 16 | Step 5 of Annex A.23 happens Check: Does the UE send 200 OK acknowledging reception of SIP INFO? | --> | 200 OK | 3 | P |
| 17 | Step 6 of Annex A.23 happens Check: Does UE send a correctly composed SIP INFO with updated MSD information? | --> | SIP INFO | 4 | P |
| 18 | Step 7 of Annex A.23 happens | <-- | 200 OK | - | - |
| 19 | Step 1 of Annex A.8 happens | <-- | BYE | - | - |
| 20 | Step 2 of Annex A.8 happens Check: Does the UE send 200 OK for the BYE request and end the call? | --> | 200 OK | 5 | P |

11.4.3.3	Specific message contents

Table 11.4.3.3-1: INVITE (step 12, table 11.4.3.2-1)

Derivation path: Step 1 in Annex A.23 with Condition A20

Table 11.4.3.3-2: Void

Table 11.4.3.3-3: Void

Table 11.4.3.3-4: BYE (step 19, table 11.4.3.2-1)

Derivation Path: TS 34.229-1 [2], Annex A.2.8 with condition A9.

## 11.5	eCall capable / Automatic initiation / MSD transfer and 200 OK with ACK / SIP INFO request for MSD update / Success / 5GS

11.5.1	Test Purpose (TP)

(1)

with { UE being registered to IMS }

ensure that {

when { UE is being made to initiate an automatic eCall over IMS }

then { UE performs IMS emergency registration and sends correctly composed INVITE with MSD }

}

(2)

with { UE having sent an  INVITE }

ensure that {

when { UE receives 200 OK containing acknowledgement of reception of MSD }

then { UE sends ACK }

}

(3)

with { eCall over IMS being established }

ensure that {

when { UE receives SIP INFO requesting MSD update }

then { UE sends 200 OK acknowledging SIP INFO }

}

(4)

with { UE having sent 200 OK for SIP INFO }

ensure that {

when { UE is able to provide the updated MSD  }

then { UE sends a correctly composed SIP INFO with updated MSD information }

}

(5)

with { UE having received 200 OK acknowledging the reception of updated MSD in SIP INFO }

ensure that {

when { Network sends BYE }

then { UE sends 200 OK for BYE }

}

11.5.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-16 requirements.

[TS 24.229 clause 5.1.6.11.3]:

During an emergency session established for eCall type of emergency service as described in subclause 5.1.6.11.2, if the UE receives an INFO request with:

1)	an Info-Package header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244];

2)	a multipart/mixed body including:

a)	an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] containing a "request" element with an "action" attribute set to "send-data" and a "datatype" attribute set to "eCall.MSD"; and

b)	a Content-Disposition header field set to "By-Reference" associated with the "application/EmergencyCallData.Control+xml" MIME body part; and

3)	a Content-Disposition header field set to "Info-Package" associated with the multipart/mixed body;

the UE shall proceed as follows:

1)	if the UE is able to provide an updated MSD, the UE shall send an INFO request containing:

a)	an Info-Package header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244];

b)	a multipart/mixed body including:

i)	an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244] containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 as specified in CEN EN 15722:2015 [245]; and

ii)	a Content-Disposition header field set to "By-Reference" associated with the "application/EmergencyCallData.eCall.MSD" MIME body part; and

c)	a Content-Disposition header field set to "Info-Package" associated with the multipart/mixed body; and

2)	if the UE is not able to provide an updated MSD, the UE shall send an INFO request containing:

a)	an Info-Package header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244];

b)	a multipart/mixed body including:

i)	an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

-	a "ref" attribute set to the Content-ID of the "application/EmergencyCallData.Control+xml" MIME body part in the INFO request received by the UE; and

-	an "actionResult" child element containing:

A)	an "action" attribute set to "send-data";

B)	a "success" attribute set to "false"; and

C)	a "reason" attribute set to an appropriate value as defined in RFC 8147 [244]; and

ii)	a Content-Disposition header field set to "By-Reference" associated with the "application/EmergencyCallData.Control+xml" MIME body part; and

c)	a Content-Disposition header field set to "Info-Package" associated with the multipart/mixed body.

NOTE:	Further content for the INFO request is as defined in RFC 8147 [244].

11.5.3	Test description

11.5.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC with eCall subscription according to TS 38.508-1 [21] table 6.4.1-23.

-	UE is configured to register for IMS after switch on.

Preamble:

-	UE is in state 1N-A (TS 38.508-1 [21]) and registered to IMS.

11.5.3.2	Test procedure sequence

Table 11.5.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is triggered to start an automatic eCall | - | - | - | - |
| 2-9 | Steps 1-8 of generic procedure specified in Table 4.9.11.2.2-1 of TS 38.508-1 [21] with condition ‘eCall’ are performed. | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in steps 10-11 below the events specified in Table 11.5.3.2-2 take place | - | - | - | - |
| 10-11 | Steps 9-10 of Table 4.9.11.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in steps 12 and 13, steps 11 to 13 specified in 38.508-1 [21] Table 4.9.11.2.2-1 takes place. | - | - | - | - |
| 12 | Step 1 of Annex A.23 happens Check: Does the UE include MSD in the message body? | --> | INVITE | 1 | P |
| 13 | Step 2 of Annex A.23 happens | <-- | 200 OK | - | - |
| 14 | Step 3 of Annex A.23 happens Check: Does the UE send ACK? | --> | ACK | 2 | P |
| 15 | Step 4 of Annex A.23 happens | <-- | SIP INFO | - | - |
| 16 | Step 5 of Annex A.23 happens Check: Does the UE send 200 OK acknowledging reception of SIP INFO? | --> | 200 OK | 3 | P |
| 17 | Step 6 of Annex A.23 happens Check: Does UE send a correctly composed SIP INFO with updated MSD information? | --> | SIP INFO | 4 | P |
| 18 | Step 7 of Annex A.23 happens | <-- | 200 OK | - | - |
| 19 | Step 1 of Annex A.8 happens | <-- | BYE | - | - |
| 20 | Step 2 of Annex A.8 happens Check: Does the UE send 200 OK for the BYE request and end the call? | --> | 200 OK | 5 | P |

Table 11.5.3.2-2: Parallel Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour depending UE implementation; the "lower case letter" identifies a step sequence that takes place if the UE performs a specific action. | - | - | - | - |
| 1a1 | The generic procedure for IP address allocation in the user plane specified in subclause 4.5A.3 takes place. | - | - | - | - |
| 2 | Step 1 of Annex A.3 happens Check: Does UE send a correctly composed initial REGISTER request for IMS emergency registration? | --> | REGISTER | - | - |
| 3 | Step 2 of Annex A.3 happens | <-- | 401 Unauthorized | - | - |
| 4 | Step 3 of Annex A.3 happens Check: Does UE send another REGISTER with AKAv1-MD5 credentials? | --> | REGISTER | - | - |
| 5 | Step 4 of Annex A.3 happens | <-- | 200 OK | - | - |

11.5.3.3	Specific message contents

Table 11.5.3.3-1: INVITE (step 12, table 11.5.3.2-1)

Derivation path: Step 1 in Annex A.23 with Condition A21

Table 11.5.3.3-2: Void

Table 11.5.3.3-3: Void

Table 11.5.3.3-4: BYE (step 19, table 11.5.3.2-1)

Derivation Path: TS 34.229-1 [2], Annex A.2.8 with condition A9.

## 11.6	eCall capable / Automatic initiation / MSD transfer and 200 OK with ACK / SIP INFO request for unsupported MSD / UE indicates unsuccessful in SIP INFO / 5GS

11.6.1	Test Purpose (TP)

(1)

with { UE being registered to IMS }

ensure that {

when { UE is being made to initiate an automatic eCall over IMS }

then { UE performs IMS emergency registration and sends correctly composed INVITE with MSD }

}

(2)

with { UE having sent an  INVITE }

ensure that {

when { UE receives 200 OK containing acknowledgement of reception of MSD }

then { UE sends ACK }

}

(3)

with { automatic eCall over IMS being established }

ensure that {

when { UE receives SIP INFO requesting MSD update with unsupported datatype }

then { UE sends 200 OK acknowledging SIP INFO }

}

(4)

with { UE having sent 200 OK for SIP INFO }

ensure that {

when { UE is unable to provide the MSD information }

then { UE sends a correctly composed SIP INFO with “success” attribute set to false in ack element }

}

11.6.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-16 requirements.

[TS 24.229 clause 5.1.6.11.3]:

During an emergency session established for eCall type of emergency service as described in subclause 5.1.6.11.2, if the UE receives an INFO request with:

1)	an Info-Package header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244];

2)	a multipart/mixed body including:

a)	an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] containing a "request" element with an "action" attribute set to "send-data" and a "datatype" attribute set to "eCall.MSD"; and

b)	a Content-Disposition header field set to "By-Reference" associated with the "application/EmergencyCallData.Control+xml" MIME body part; and

3)	a Content-Disposition header field set to "Info-Package" associated with the multipart/mixed body;

the UE shall proceed as follows:

1)	if the UE is able to provide an updated MSD, the UE shall send an INFO request containing:

a)	an Info-Package header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244];

b)	a multipart/mixed body including:

i)	an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244] containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 as specified in CEN EN 15722:2015 [245]; and

ii)	a Content-Disposition header field set to "By-Reference" associated with the "application/EmergencyCallData.eCall.MSD" MIME body part; and

c)	a Content-Disposition header field set to "Info-Package" associated with the multipart/mixed body; and

2)	if the UE is not able to provide an updated MSD, the UE shall send an INFO request containing:

a)	an Info-Package header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244];

b)	a multipart/mixed body including:

i)	an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

-	a "ref" attribute set to the Content-ID of the "application/EmergencyCallData.Control+xml" MIME body part in the INFO request received by the UE; and

-	an "actionResult" child element containing:

A)	an "action" attribute set to "send-data";

B)	a "success" attribute set to "false"; and

C)	a "reason" attribute set to an appropriate value as defined in RFC 8147 [244]; and

ii)	a Content-Disposition header field set to "By-Reference" associated with the "application/EmergencyCallData.Control+xml" MIME body part; and

c)	a Content-Disposition header field set to "Info-Package" associated with the multipart/mixed body.

NOTE:	Further content for the INFO request is as defined in RFC 8147 [244].

11.6.3	Test description

11.6.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC with eCall subscription according to TS 38.508-1 [21] table 6.4.1-23.

-	UE is configured to register for IMS after switch on.

Preamble:

-	UE is in state 1N-A (TS 38.508-1 [21]) and registered to IMS.

11.6.3.2	Test procedure sequence

Table 11.6.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is triggered to start an automatic eCall | - | - | - | - |
| 2-9 | Steps 1-8 of generic procedure specified in Table 4.9.11.2.2-1 of TS 38.508-1 [21] with condition ‘eCall’ are performed. | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in steps 10-11 below the events specified in Table 11.6.3.2-2 take place | - | - | - | - |
| 10-11 | Steps 9-10 of Table 4.9.11.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in steps 12 and 13, steps 11 to 13 specified in 38.508-1 [21] Table 4.9.11.2.2-1 takes place. | - | - | - | - |
| 12 | Step 1 of Annex A.23 happens | --> | INVITE | 1 | P |
| 13 | Step 2 of Annex A.23 happens | <-- | 200 OK | - | - |
| 14 | Step 3 of Annex A.23 happens | --> | ACK | 2 | P |
| 15 | Step 4 of Annex A.23 happens | <-- | SIP INFO | - | - |
| 16 | Step 5 of Annex A.23 happens Check: Does the UE send 200 OK acknowledging reception of SIP INFO? | --> | 200 OK | 3 | P |
| 17 | Step 6 of Annex A.23 happens Check: Does UE send a correctly composed SIP INFO with ack element containing “success” attribute set to “false”? | --> | SIP INFO | 4 | P |
| 18 | Step 7 of Annex A.23 happens | <-- | 200 OK | - | - |
| 19 | Step 1 of Annex A.8 happens | <-- | BYE | - | - |
| 20 | Step 2 of Annex A.8 happens | --> | 200 OK | - | - |

Table 11.6.3.2-2: Parallel Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour depending UE implementation; the "lower case letter" identifies a step sequence that takes place if the UE performs a specific action. | - | - | - | - |
| 1a1 | The generic procedure for IP address allocation in the user plane specified in subclause 4.5A.3 takes place. | - | - | - | - |
| 2 | Step 1 of Annex A.3 happens Check: Does UE send a correctly composed initial REGISTER request for IMS emergency registration? | --> | REGISTER | - | - |
| 3 | Step 2 of Annex A.3 happens | <-- | 401 Unauthorized | - | - |
| 4 | Step 3 of Annex A.3 happens Check: Does UE send another REGISTER with AKAv1-MD5 credentials? | --> | REGISTER | - | - |
| 5 | Step 4 of Annex A.3 happens | <-- | 200 OK | - | - |

11.6.3.3	Specific message contents

Table 11.6.3.3-1: INVITE (step 12, table 11.6.3.2-1)

Derivation path: Step 1 in Annex A.23 with Condition A21

Table 11.6.3.3-2: Void

Table 11.6.3.3-3: SIP INFO (step 15, table 11.6.3.2-1)

| Derivation path: Step 4 in Annex A.23 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Message-body |  | --boundaryXXX Content-Type: application/EmergencyCallData.Control+xml Content-ID: <test-info@3gpp.org> Content-Disposition: by-reference <?xml version="1.0" encoding="UTF-8"?> <EmergencyCallData.Control xmlns="urn:ietf:params:xml:ns:EmergencyCallData:control"> <request action="send-data" datatype="eCall.invalidMSD"/> </EmergencyCallData.Control> --boundaryXXX |  |  |

Table 11.6.3.3-4: SIP INFO (step 17, table 11.6.3.2-1)

Derivation path: Step 6 in Annex A.23 with Condition A4 instead of A3

Table 11.6.3.3-5: BYE (step 19, table 11.6.3.2-1)

Derivation Path: TS 34.229-1 [2], Annex A.2.8 with condition A9.

## 11.7	eCall capable / Manual initiation / Emergency registration / 200 OK with NACK

11.7.1	Test Purpose (TP)

(1)

with { UE having sent an INVITE for manual initiated ecall over IMS }

ensure that {

when { UE receives 200 OK containing NACK for MSD transmission }

then { UE continue with the call establishment }

}

11.7.2	Conformance requirement

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-19 requirements.

[TS 24.229, clause 5.1.6.11.1]:

If the upper layers request establishment of an IMS emergency call of the manually initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.manual" as specified in RFC 8147 [244].

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.automatic" as specified in RFC 8147 [244].

NOTE 1:	The manually initiated eCall type of emergency service is used when the eCall IMS emergency session is invoked with user input. The automatically initiated eCall type of emergency service is used if the eCall IMS emergency session is invoked without user input.

NOTE 2:	To ensure the identification and the related routing (e.g. to a test centre) of non-emergency calls initiated for eCall testing or eCall terminal reconfiguration purposes, "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual" are not used for such calls. Additionally the usage of any other service URN to establish non-emergency calls initiated for eCall testing or eCall terminal reconfiguration purposes needs to be in accordance with local operator policy.

[TS 24.229, clause 5.1.6.11.2]:

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service or of the manually initiated eCall type of emergency service and if allowed by IP-CAN specific annex, the UE shall send an INVITE request as specified in the procedures in subclause 5.1.6.8 with the following additions:

1)	the UE shall set the Request-URI to "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual"; and

2)	if the IP-CAN indicates the eCall support indication, the UE shall:

a)	insert a multipart/mixed body containing an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244], containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 PER as specified in CEN EN 15722:2020 [245] and include a Content-Disposition header field with a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

b)	insert an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244]; and

c)	insert a Recv-Info header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244].

NOTE 1:	Further content for the INVITE is as defined in RFC 8147 [244].

Then the UE shall proceed as follows:

…

2a)	if the UE receives a 200 (OK) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "false"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as unsuccessful;

…

4)	in all other cases, the UE shall perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

NOTE 2:	For case 3-a-iv). upper layers can wait for an incoming call from the PSAP as defined in CEN EN 17184:2024 [300] if incoming IMS calls are possible or if the upper layers can instigate CS fallback to receive incoming calls over CS. If incoming calls cannot be received by the UE, upper layers can perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

Reference(s)

3GPP TS 24.229 [10], clauses 5.1.6.11.1 and 5.1.6.11.2.

11.7.3	Test description

11.7.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC with eCall subscription according to TS 38.508-1 [21] table 6.4.1-23.

-	UE is configured to register for IMS after switch on

Preamble:

-	UE is in state 1N-A (TS 38.508-1 [21]) and registered to IMS.

11.7.3.2	Test procedure sequence

Table 11.7.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is triggered to start a manual eCall. | - | - | - | - |
| 2-11 | Steps 1-10 of generic procedure specified in Table 4.9.11.2.2-1 of TS 38.508-1 [21] with condition ‘eCall’ are performed. | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in steps 12-14, steps 11 to 13 specified in 38.508-1 [21] Table 4.9.11.2.2-1 takes place. | - | - | - | - |
| 12 | Step 1 of Annex A.23 happens | --> | INVITE | - | - |
| 13 | SS sends 200 OK with NACK | <-- | 200 OK | - | - |
| 14 | Step 3 of Annex A.23 happens Check: Does the UE send ACK? | --> | ACK | 1 | P |
| 15 | SS waits for 5 seconds | - | - | - | - |
| 16 | Generic test procedure for IMS MT call release in 5GS described in TS 38.508-1 [21] subclause 4.9.18 takes place. | - | - | - | - |

11.7.3.3	Specific message contents

Table 11.7.3.3-1: INVITE (step 12, table 11.7.3.2-1)

Derivation path: Step 1 in Annex A.23 with Condition A20

Table 11.7.3.3-2: 200 OK for INVITE (step 13, table 11.7.3.2-1)

Derivation path: Step 2 in Annex A.23 with Conditions A14 instead of A13

Table 11.7.3.3-3: BYE (step 16, table 11.7.3.2-1)

Derivation Path: TS 34.229-1 [2], Annex A.2.8 with condition A9.

## 11.8	eCall capable / Automatic initiation / Emergency registration / 200 OK with NACK

11.8.1	Test Purpose (TP)

(1)

with { UE having sent an INVITE for automatic initiated ecall over IMS }

ensure that {

when { UE receives 200 OK containing NACK for MSD transmission }

then { UE continue with the call establishment }

}

11.8.2	Conformance requirement

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-19 requirements.

[TS 24.229, clause 5.1.6.11.1]:

If the upper layers request establishment of an IMS emergency call of the manually initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.manual" as specified in RFC 8147 [244].

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.automatic" as specified in RFC 8147 [244].

NOTE 1:	The manually initiated eCall type of emergency service is used when the eCall IMS emergency session is invoked with user input. The automatically initiated eCall type of emergency service is used if the eCall IMS emergency session is invoked without user input.

NOTE 2:	To ensure the identification and the related routing (e.g. to a test centre) of non-emergency calls initiated for eCall testing or eCall terminal reconfiguration purposes, "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual" are not used for such calls. Additionally the usage of any other service URN to establish non-emergency calls initiated for eCall testing or eCall terminal reconfiguration purposes needs to be in accordance with local operator policy.

[TS 24.229, clause 5.1.6.11.2]:

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service or of the manually initiated eCall type of emergency service and if allowed by IP-CAN specific annex, the UE shall send an INVITE request as specified in the procedures in subclause 5.1.6.8 with the following additions:

1)	the UE shall set the Request-URI to "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual"; and

2)	if the IP-CAN indicates the eCall support indication, the UE shall:

a)	insert a multipart/mixed body containing an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244], containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 PER as specified in CEN EN 15722:2020 [245] and include a Content-Disposition header field with a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

b)	insert an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244]; and

c)	insert a Recv-Info header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244].

NOTE 1:	Further content for the INVITE is as defined in RFC 8147 [244].

Then the UE shall proceed as follows:

…

2a)	if the UE receives a 200 (OK) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "false"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as unsuccessful;

…

4)	in all other cases, the UE shall perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

NOTE 2:	For case 3-a-iv). upper layers can wait for an incoming call from the PSAP as defined in CEN EN 17184:2024 [300] if incoming IMS calls are possible or if the upper layers can instigate CS fallback to receive incoming calls over CS. If incoming calls cannot be received by the UE, upper layers can perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

Reference(s)

3GPP TS 24.229 [10], clauses 5.1.6.11.1 and 5.1.6.11.2.

11.8.3	Test description

11.8.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC with eCall subscription according to TS 38.508-1 [21] table 6.4.1-23.

-	UE is configured to register for IMS after switch on

Preamble:

-	UE is in state 1N-A (TS 38.508-1 [21]) and registered to IMS.

11.8.3.2	Test procedure sequence

Table 11.8.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is triggered to start an automatic eCall. | - | - | - | - |
| 2-11 | Steps 1-10 of generic procedure specified in Table 4.9.11.2.2-1 of TS 38.508-1 [21] with condition ‘eCall’ are performed. | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in steps 12 and 14, steps 11 to 13 specified in 38.508-1 [21] Table 4.9.11.2.2-1 takes place. | - | - | - | - |
| 12 | Step 1 of Annex A.23 happens | --> | INVITE | - | - |
| 13 | The SS sends 200 OK with NACK | <-- | 200 OK |  |  |
| 14 | Step 3 of Annex A.23 happens Check: Does the UE send ACK? | --> | ACK | 1 | P |
| 15 | The SS waits for 5 seconds | - | - | - | - |
| 16 | Generic test procedure for IMS MT call release in 5GS described in TS 38.508-1 [21] subclause 4.9.18 takes place. | - | - | - | - |

11.8.3.3	Specific message contents

Table 11.8.3.3-1: INVITE (step 12, table 11.8.3.2-1)

Derivation path: Step 1 in Annex A.23 with Condition A21

Table 11.8.3.3-2: 200 OK for INVITE (step 13, table 11.8.3.2-1)

Derivation path: Step 2 in Annex A.23 with Conditions A14 instead of A13

Table 11.8.3.3-3: BYE (step 16, table 11.8.3.2-1)

Derivation Path: TS 34.229-1 [2], Annex A.2.8 with condition A9.

## 11.9	eCall capable / eCall to URI for test service / 200 OK with NACK

11.9.1	Test Purpose (TP)

(1)

with { UE having sent an INVITE for eCall to URI for test service initiated }

ensure that {

when { UE receives 200 OK containing NACK for MSD transmission }

then { UE continue with the call establishment }

}

11.9.2	Conformance requirement

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-19 requirements.

[TS 24.229, clause 5.1.6.11.1]:

If the upper layers request establishment of an IMS emergency call of the manually initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.manual" as specified in RFC 8147 [244].

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.automatic" as specified in RFC 8147 [244].

NOTE 1:	The manually initiated eCall type of emergency service is used when the eCall IMS emergency session is invoked with user input. The automatically initiated eCall type of emergency service is used if the eCall IMS emergency session is invoked without user input.

NOTE 2:	To ensure the identification and the related routing (e.g. to a test centre) of non-emergency calls initiated for eCall testing or eCall terminal reconfiguration purposes, "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual" are not used for such calls. Additionally the usage of any other service URN to establish non-emergency calls initiated for eCall testing or eCall terminal reconfiguration purposes needs to be in accordance with local operator policy.

[TS 24.229, clause 5.1.6A]:

This clause specifies a feature which applies to UEs subject to national regulations in CEN EN 17184:2024 [300]. If the upper layers request establishment of an IMS call of the test eCall type while in limited service state, the request shall be rejected. Otherwise, the UE shall follow normal voice call setup to attempt the test eCall. If the CS domain is selected, the UE shall attempt a test eCall setup as a non-emergency services call using appropriate access technology specific procedures.

If the IM CN subsystem is selected and the UE is not currently registered, the UE shall perform an initial registration, as described in subclause 5.1.1.2. The UE shall then apply voice domain selection as specified in 3GPP TS 23.221 [6] and 3GPP TS 23.501 [257] and the procedures as specified in subclause 5.1.2A and 5.1.3 to send an INVITE with the following additions:

1)	the UE shall set the Request-URI to a non-emergency service URN (e.g. SIP, Service, Tel URIs) that is defined by a national regulator, PSAP or eCall test provider for testing purposes; and

NOTE 1:	The Request-URI can be set to "urn:service:test.sos.ecall" as specified in RFC 8147 [244] or the URI can be configured in USIM files; see 3GPP TS 31.102 [15C].

2)	the UE shall insert in the INVITE request:

a)	a To header field with the same service URN as in the Request-URI;

b)	a From header field that includes the public user identity or the tel URI associated with the public user identity, as described in subclause 4.2;

c)	a multipart/mixed body containing an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244], containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 PER as specified in CEN EN 15722:2020 [245] and include a Content-Disposition header field with a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

d)	an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244]; and

e)	 a Recv-Info header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244].

NOTE 2:	Further content for the INVITE such as set the test bit in the MSD is defined in CEN EN 15722:2020 [245] and others is defined in RFC 8147 [244].

Then the UE shall proceed as follows:

…

3)	if the UE receives a 200 (OK) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "false"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as unsuccessful;

…

After a test eCall has been established and released, successfully or unsuccessfully, a UE shall support the procedures specified in subclause 5.1.6.11.3 for transfer of an updated MSD during an incoming IMS call when the following conditions are fulfilled:

1)	the incoming IMS call happens within an implementation dependent time period according to the national requirements within the region of eCall service (i.e. for EU region the timer value is 1 hour as defined by the T9 timer in CEN EN 17184:2024 [300]);

2)	the INVITE for the incoming IMS call contains a P-Asserted-Identity header field containing the same non-emergency service URN as included in the Request-URI for the test eCall or as included in a P-Asserted-Identity header field received in a response to the INVITE for the test eCall;

3)	the INVITE for the incoming IMS call contains an Accept header field that includes the value "application/EmergencyCallData.Control+xml"; and

4)	the INVITE for the incoming IMS call contains a Recv-Info header field with header field value EmergencyCallData.eCall.MSD.

When conditions 1) to 4) are fulfilled, the UE shall include in the 200 OK:

a)	a Recv-Info header field with header field value EmergencyCallData.eCall.MSD; and

b)	an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244].

The UE shall also support the procedures specified in subclause 5.1.6.11.3 for transfer of an updated MSD.

NOTE 4:	As defined in CEN EN 17184:2024 [300], the recipient of a test eCall can instigate a callback to the UE to verify UE callback capability and ability to transfer updated MSD.

NOTE 5:	The recipient of a test eCall is encouraged to call back the UE after a shorter period in order to complete the test eCall without a long wait by a test eCall user.

When conditions 1) and 2) are fulfilled, the UE shall support request and transfer of an updated MSD using audio media stream as described in 3GPP TS 26.267 [9C].

NOTE 6:	A test eCall recipient can request transfer of an updated MSD using audio media stream if the procedures specified in subclause 5.1.6.11.3 cannot be supported by the recipient, the serving PLMN, the HPLMN or by some other intermediate network.

NOTE 7:	For a trustable and secure identification of an eCall callback and the related MSD transfer between UE and the PSAP (or test centre), the PSAP (or test centre) is part of the trust domain of the IMS home operator network where the UE under test belongs to.

Reference(s)

3GPP TS 24.229 [10], clauses 5.1.6.11.1 and 5.1.6A.

11.9.3	Test description

11.9.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC with eCall subscription according to TS 38.508-1 [21] table 6.4.1-23.

-	UE is configured to register for IMS after switch on

Preamble:

-	UE is in state 1N-A (TS 38.508-1 [21]) and registered to IMS.

11.9.3.2	Test procedure sequence

Table 11.9.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is triggered to initiate test eCall to uri for test services | - | - | - | - |
| 2-13 | Steps 2-13 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| 14 | Step 1 of Annex A.23 happens | --> | INVITE | - | - |
| 15 | The SS sends 200 OK with NACK | <-- | 200 OK | - | - |
| 16 | Step 3 of Annex A.23 happens Check: Does the UE send ACK? | --> | ACK | 1 | P |
| 17 | The SS waits for 5 seconds | - | - | - | - |
| 18 | Generic test procedure for IMS MT call release in 5GS described in TS 38.508-1 [21] subclause 4.9.18 takes place. | - | - | - | - |

11.9.3.3	Specific message contents

Table 11.9.3.3-1: INVITE (step 14, table 11.9.3.2-1)

Derivation path: Step 1 in annex A.23 with condition A25 instead of A20/A21 and without condition A7.

Table 11.9.3.3-2: 200 OK for INVITE (step 15, table 11.9.3.2-1)

Derivation path: Step 2 in annex A.23 with condition A14 instead of A13 and without condition A6

## 11.10	eCall only mode / Manual initiation / Emergency registration / Abnormal case / IM CN sends a 486 (Busy Here) with positive ACK / Successful MSD

11.10.1	Test Purpose (TP)

(1)

with { UE is requested to make a manual eCall over IMS }

ensure that {

when { UE receives 486(Busy Here) with positive ACK }

then { UE considers MSD transmission successful }

}

11.10.2	Conformance requirement

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-19 requirements.

[TS 24.229, clause 5.1.6.11.1]:

If the upper layers request establishment of an IMS emergency call of the manually initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.manual" as specified in RFC 8147 [244].

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.automatic" as specified in RFC 8147 [244].

NOTE 1:	The manually initiated eCall type of emergency service is used when the eCall IMS emergency session is invoked with user input. The automatically initiated eCall type of emergency service is used if the eCall IMS emergency session is invoked without user input.

NOTE 2:	To ensure the identification and the related routing (e.g. to a test centre) of non-emergency calls initiated for eCall testing or eCall terminal reconfiguration purposes, "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual" are not used for such calls. Additionally the usage of any other service URN to establish non-emergency calls initiated for eCall testing or eCall terminal reconfiguration purposes needs to be in accordance with local operator policy.

[TS 24.229, clause 5.1.6.11.2]:

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service or of the manually initiated eCall type of emergency service and if allowed by IP-CAN specific annex, the UE shall send an INVITE request as specified in the procedures in subclause 5.1.6.8 with the following additions:

1)	the UE shall set the Request-URI to "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual"; and

2)	if the IP-CAN indicates the eCall support indication, the UE shall:

a)	insert a multipart/mixed body containing an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244], containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 PER as specified in CEN EN 15722:2020 [245] and include a Content-Disposition header field with a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

b)	insert an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244]; and

c)	insert a Recv-Info header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244].

NOTE 1:	Further content for the INVITE is as defined in RFC 8147 [244].

Then the UE shall proceed as follows:

…

3)	if the UE receives a 486 (Busy Here), 600 (Busy Everywhere) or 603 (Decline) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "true"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as successful and shall act as follows:

iii)	if the UE is in limited service state, perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B]; and

iv)	if the UE is not in limited service state, report the response and successful MSD transfer to upper layers; and

4)	in all other cases, the UE shall perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

NOTE 2:	For case 3-a-iv). upper layers can wait for an incoming call from the PSAP as defined in CEN EN 17184:2024 [300] if incoming IMS calls are possible or if the upper layers can instigate CS fallback to receive incoming calls over CS. If incoming calls cannot be received by the UE, upper layers can perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

Reference(s)

3GPP TS 24.229 [10], clauses 5.1.6.11.1 and 5.1.6.11.2.

11.10.3	Test description

11.10.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC with eCall only subscription according to TS 38.508-1 [21] table 6.4.1-24.

Preamble:

-	The UE is in test state 0-A (Switched Off) as defined in TS 38.508-1 [21], subclause 4.4A.2.

11.10.3.2	Test procedure sequence

Table 11.10.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is triggered to start a manual eCall. | - | - | - | - |
| 2-22 | Steps 3-23 of generic procedure specified in Table 4.9.29.2.2-1 of TS 38.508-1 [21] is performed. | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in steps 23-25 below steps 11-13 of TS 38.508-1 [21] Table 4.9.11.2.2-2 take place. | - | - | - | - |
| 23 | Step 1 of Annex A.23 happens | --> | INVITE | - | - |
| 24 | The SS sends 486 Busy Here response | <-- | 486 Busy Here | - | - |
| 25 | Check: Does UE send ACK ? | --> | ACK | 1 | P |
| 26 | The SS releases the RRC connection. | - | - | - | - |
| 27 | The SS waits for 10 seconds | - | - | - | - |
| 28 | Check: Does the generic test procedure in TS 38.508-1 [21] subclause 4.9.4 indicate that the UE is camped on same NR cell takes place? (NOTE 1) | - | - | 1 | P |
| NOTE 1: Even though there is no cell of any other RAT configured at the SS, it assumed that a UE which does not comply to requirement TS 24.229 [7] clause 5.1.6.11.2 case 3-a-iv) of staying at the NR cell, will be busy searching for other cells and therefore not respond to the paging at the NR cell. |  |  |  |  |  |

11.10.3.3	Specific message contents

Table 11.10.3.3-1: INVITE (step 23, table 11.10.3.2-1)

Derivation path: Step 1 in Annex A.23 with Condition A20

Table 11.10.3.3-2: 486 Busy Here (step 24, table 11.10.3.2-1)

Derivation path: TS 34.229-1 [2], Annex A.2.21 with condition A1.

Table 11.10.3.3-3: ACK (step 25, table 11.10.3.2-1)

Derivation path: TS 34.229-1 [2], Annex A.2.7 with condition A4.

## 11.11	eCall only mode / Automatic initiation / Emergency registration / Abnormal case / IM CN sends a 600 (Busy Everywhere) with positive ACK / Successful MSD

11.11.1	Test Purpose (TP)

(1)

with { UE is requested to make an automatic eCall over IMS }

ensure that {

when { UE receives 600 (Busy Everywhere) with positive ACK }

then { UE considers MSD transmission successful }

}

11.11.2	Conformance requirement

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-19 requirements.

[TS 24.229, clause 5.1.6.11.1]:

If the upper layers request establishment of an IMS emergency call of the manually initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.manual" as specified in RFC 8147 [244].

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.automatic" as specified in RFC 8147 [244].

NOTE 1:	The manually initiated eCall type of emergency service is used when the eCall IMS emergency session is invoked with user input. The automatically initiated eCall type of emergency service is used if the eCall IMS emergency session is invoked without user input.

NOTE 2:	To ensure the identification and the related routing (e.g. to a test centre) of non-emergency calls initiated for eCall testing or eCall terminal reconfiguration purposes, "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual" are not used for such calls. Additionally the usage of any other service URN to establish non-emergency calls initiated for eCall testing or eCall terminal reconfiguration purposes needs to be in accordance with local operator policy.

[TS 24.229, clause 5.1.6.11.2]:

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service or of the manually initiated eCall type of emergency service and if allowed by IP-CAN specific annex, the UE shall send an INVITE request as specified in the procedures in subclause 5.1.6.8 with the following additions:

1)	the UE shall set the Request-URI to "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual"; and

2)	if the IP-CAN indicates the eCall support indication, the UE shall:

a)	insert a multipart/mixed body containing an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244], containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 PER as specified in CEN EN 15722:2020 [245] and include a Content-Disposition header field with a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

b)	insert an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244]; and

c)	insert a Recv-Info header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244].

NOTE 1:	Further content for the INVITE is as defined in RFC 8147 [244].

Then the UE shall proceed as follows:

…

3)	if the UE receives a 486 (Busy Here), 600 (Busy Everywhere) or 603 (Decline) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "true"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as successful and shall act as follows:

iii)	if the UE is in limited service state, perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B]; and

iv)	if the UE is not in limited service state, report the response and successful MSD transfer to upper layers; and

4)	in all other cases, the UE shall perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

NOTE 2:	For case 3-a-iv). upper layers can wait for an incoming call from the PSAP as defined in CEN EN 17184:2024 [300] if incoming IMS calls are possible or if the upper layers can instigate CS fallback to receive incoming calls over CS. If incoming calls cannot be received by the UE, upper layers can perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

Reference(s)

3GPP TS 24.229 [10], clauses 5.1.6.11.1 and 5.1.6.11.2.

11.11.3	Test description

11.11.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC with eCall only subscription according to TS 38.508-1 [21] table 6.4.1-24.

Preamble:

-	The UE is in test state 0-A (Switched Off) as defined in TS 38.508-1 [4], subclause 4.4A.2.

11.11.3.2	Test procedure sequence

Table 11.11.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is triggered to start an automatic eCall. | - | - | - | - |
| 2-22 | Steps 3-23 of generic procedure specified in Table 4.9.29.2.2-1 of TS 38.508-1 [21] is performed. | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in steps 23-25 below steps 11-13 of TS 38.508-1 [21] Table 4.9.11.2.2-2 take place. | - | - | - | - |
| 23 | Step 1 of Annex A.23 happens | --> | INVITE | - | - |
| 24 | The SS sends 600 Busy Everywhere response | <-- | 600 Busy Everywhere | - | - |
| 25 | Check: Does UE send ACK ? | --> | ACK | 1 | P |
| 26 | The SS releases the RRC connection. |  |  | - | - |
| 27 | SS waits for 10 seconds | - | - | - | - |
| 28 | Check: Does the generic test procedure in TS 38.508-1 [21] subclause 4.9.4 indicate that the UE is camped on same NR cell takes place? (NOTE 1) | - | - | 1 | P |
| NOTE 1: Even though there is no cell of any other RAT configured at the SS, it assumed that a UE which does not comply to requirement TS 24.229 [7] clause 5.1.6.11.2 case 3-a-iv) of staying at the NR cell, will be busy searching for other cells and therefore not respond to the paging at the NR cell. |  |  |  |  |  |

11.11.3.3	Specific message contents

Table 11.11.3.3-1: INVITE (step 23, table 11.11.3.2-1)

Derivation path: Step 1 in Annex A.23 with Condition A21

Table 11.11.3.3-2: 600 Busy Everywhere (step 24, table 11.11.3.2-1)

Derivation path: TS 34.229-1 [2], Annex A.2.22 with condition A1.

Table 11.11.3.3-3: ACK (step 25, table 11.11.3.2-1)

Derivation path: TS 34.229-1 [2], Annex A.2.7 with condition A4.

## 11.12	eCall only mode / Manual initiation / Emergency registration / Abnormal case / IM CN sends a 603 (Decline) with positive ACK / Successful MSD

11.12.1	Test Purpose (TP)

(1)

with { UE is requested to make a manual eCall over IMS }

ensure that {

when { UE receives 603 (Decline) with positive ACK }

then { UE considers MSD transmission successful }

}

11.12.2	Conformance requirement

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-19 requirements.

[TS 24.229, clause 5.1.6.11.1]:

If the upper layers request establishment of an IMS emergency call of the manually initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.manual" as specified in RFC 8147 [244].

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.automatic" as specified in RFC 8147 [244].

NOTE 1:	The manually initiated eCall type of emergency service is used when the eCall IMS emergency session is invoked with user input. The automatically initiated eCall type of emergency service is used if the eCall IMS emergency session is invoked without user input.

NOTE 2:	To ensure the identification and the related routing (e.g. to a test centre) of non-emergency calls initiated for eCall testing or eCall terminal reconfiguration purposes, "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual" are not used for such calls. Additionally the usage of any other service URN to establish non-emergency calls initiated for eCall testing or eCall terminal reconfiguration purposes needs to be in accordance with local operator policy.

[TS 24.229, clause 5.1.6.11.2]:

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service or of the manually initiated eCall type of emergency service and if allowed by IP-CAN specific annex, the UE shall send an INVITE request as specified in the procedures in subclause 5.1.6.8 with the following additions:

1)	the UE shall set the Request-URI to "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual"; and

2)	if the IP-CAN indicates the eCall support indication, the UE shall:

a)	insert a multipart/mixed body containing an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244], containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 PER as specified in CEN EN 15722:2020 [245] and include a Content-Disposition header field with a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

b)	insert an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244]; and

c)	insert a Recv-Info header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244].

NOTE 1:	Further content for the INVITE is as defined in RFC 8147 [244].

Then the UE shall proceed as follows:

…

3)	if the UE receives a 486 (Busy Here), 600 (Busy Everywhere) or 603 (Decline) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "true"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as successful and shall act as follows:

iii)	if the UE is in limited service state, perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B]; and

iv)	if the UE is not in limited service state, report the response and successful MSD transfer to upper layers; and

4)	in all other cases, the UE shall perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

NOTE 2:	For case 3-a-iv). upper layers can wait for an incoming call from the PSAP as defined in CEN EN 17184:2024 [300] if incoming IMS calls are possible or if the upper layers can instigate CS fallback to receive incoming calls over CS. If incoming calls cannot be received by the UE, upper layers can perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

Reference(s)

3GPP TS 24.229 [10], clauses 5.1.6.11.1 and 5.1.6.11.2.

11.12.3	Test description

11.12.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC with eCall only subscription according to TS 38.508-1 [21] table 6.4.1-24.

Preamble:

-	The UE is in test state 0-A (Switched Off) as defined in TS 38.508-1 [4], subclause 4.4A.2.

11.12.3.2	Test procedure sequence

Table 12.1.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is triggered to start a manual eCall. | - | - | - | - |
| 2-22 | Steps 3-23 of generic procedure specified in Table 4.9.29.2.2-1 of TS 38.508-1 [21] is performed. | - | - | - | - |
|  | EXCEPTION: In parallel to the events described in steps 23-25 below steps 11-13 of TS 38.508-1 [21] Table 4.9.11.2.2-2 take place. | - | - | - | - |
| 23 | Step 1 of Annex A.23 happens | --> | INVITE | - | - |
| 24 | The SS sends 603 Decline response | <-- | 603 Decline | - | - |
| 25 | Check: Does UE send ACK ? | --> | ACK | 1 | P |
| 26 | The SS releases the RRC connection. | - | - | - | - |
| 27 | The SS waits for 10 seconds | - | - | - | - |
| 28 | Check: Does the generic test procedure in TS 38.508-1 [21] subclause 4.9.4 indicate that the UE is camped on same NR cell takes place? (NOTE 1) | - | - | 1 | P |
| NOTE 1: Even though there is no cell of any other RAT configured at the SS, it assumed that a UE which does not comply to requirement TS 24.229 [7] clause 5.1.6.11.2 case 3-a-iv) of staying at the NR cell, will be busy searching for other cells and therefore not respond to the paging at the NR cell. |  |  |  |  |  |

11.12.3.3	Specific message contents

Table 11.12.3.3-1: INVITE (step 23, table 11.12.3.2-1)

Derivation path: Step 1 in Annex A.23 with Condition A20

Table 11.12.3.3-2: 603 Decline (step 24, table 11.12.3.2-1)

Derivation path: TS 34.229-1 [2], Annex A.2.23 with condition A1.

Table 11.12.3.3-3: ACK (step 25, table 11.12.3.2-1)

Derivation path: TS 34.229-1 [2], Annex A.2.7 with condition A4.

## 11.13	eCall only mode / Automatic initiation / MSD transfer and 200 OK with ACK / SIP INFO request for MSD Update / PSAP Callback / Success

11.13.1	Test Purpose (TP)

(1)

with { UE subjected to national regulations in CEN EN 17184 having previously established eCall type of emergency service }

ensure that {

when { UE receives INVITE for an incoming IMS call for PSAP callback }

then { UE responds with 200 OK including correctly composed header fields }

}

(2)

with { UE subjected to national regulations in CEN EN 17184 having previously established eCall type of emergency service }

ensure that {

when { SS requests for transfer of MSD during an incoming IMS call }

then { UE responds with 200 OK and performs MSD transfer }

}

11.13.2	Conformance requirement

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-19 requirements.

[TS 24.229, clause 5.1.6.11.3]:

During an emergency session established for eCall type of emergency service as described in subclause 5.1.6.11.2, if the UE receives an INFO request with:

1)	an Info-Package header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244];

2)	a multipart/mixed body including:

a)	an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] containing a "request" element with an "action" attribute set to "send-data" and a "datatype" attribute set to "eCall.MSD"; and

b)	a Content-Disposition header field set to "By-Reference" associated with the "application/EmergencyCallData.Control+xml" MIME body part; and

3)	a Content-Disposition header field set to "Info-Package" associated with the multipart/mixed body;

the UE shall proceed as follows:

1)	if the UE is able to provide an updated MSD, the UE shall send an INFO request containing:

a)	an Info-Package header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244];

b)	a multipart/mixed body including:

i)	an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244] containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 as specified in CEN EN 15722:2015 [245]; and

ii)	a Content-Disposition header field set to "By-Reference" associated with the "application/EmergencyCallData.eCall.MSD" MIME body part; and

c)	a Content-Disposition header field set to "Info-Package" associated with the multipart/mixed body; and

…

[Rel-19, TS 24.229, clause 5.1.6B]:

This clause specifies a feature which applies to UEs subject to national regulations in CEN EN 17184:2024 [300]. The procedures specified in subclause 5.1.6.11.3 for transfer of an updated MSD shall be supported by a UE during an incoming IMS call when the following conditions are fulfilled:

1)	the UE previously established or attempted to establish an emergency call either over IMS or in the CS domain of the automatically initiated eCall type of emergency service or of the manually initiated eCall type of emergency service;

2)	the incoming IMS call occurred within an implementation dependent time period of at least one hour;

3)	the INVITE for the incoming IMS call contains a Priority header field with header field value "psap-callback";

4)	the INVITE for the incoming IMS call contains an Accept header field that includes the value “application/EmergencyCallData.Control+xml”; and

5)	the INVITE for the incoming IMS call contains a Recv-Info header field with header field value EmergencyCallData.eCall.MSD.

NOTE 1:	When there are multiple attempts to establish the emergency call, the emergency call in condition 2) refers to the last attempt.

When conditions 1) to 5) are fulfilled, the UE shall include in the 200 OK:

a)	a Recv-Info header field with header field value EmergencyCallData.eCall.MSD; and

b)	an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244].

The UE shall also support the procedures specified in subclause 5.1.6.11.3 for transfer of an updated MSD.

When conditions 1) and 2) and, as an implementation option, condition 3) are fulfilled, the UE shall support request and transfer of an updated MSD using audio media stream as described in 3GPP TS 26.267 [9C].

NOTE 2:	A PSAP can request transfer of an updated MSD using audio media stream if the procedures specified in subclause 5.1.6.11.3 cannot be supported by the PSAP, the serving PLMN, the HPLMN or by some other intermediate network.

Reference(s)

3GPP TS 24.229 [10], clauses 5.1.6.11.3 and 5.1.6B.

11.13.3	Test description

11.13.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC with eCall only subscription according to TS 38.508-1 [21] table 6.4.1-24.

Preamble:

-	The UE is in test state 0-A (Switched Off) as defined in TS 38.508-1 [21], subclause 4.4A.2.

11.13.3.2	Test procedure sequence

Table 11.13.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is triggered to start an automatic eCall | - | - | - | - |
| 2-25 | Steps 3-26 of generic procedure specified in Table 4.9.29.2.2-1 of TS 38.508-1 [21] with condition ‘eCall’ are performed. | - | - | - | - |
| 26 | The Test procedure for IMS MT Emergency call release as specified in TS 38.508-1 [21], subclause 4.9.12B takes place | - |  | - | - |
| 27-38 | Steps 1 to 12 of the generic test procedure for IMS MT speech call establishment in 5GC specified in TS 38.508-1 [21] subclause 4.9.16.2.2.1 are performed. | - | - | - | - |
| - | EXCEPTION: Steps 39a1 to 39b6 describe behaviour that depends on UE configuration; the “lower case letter” identifies a step sequence that takes place if such configuration was conducted. | - | - | - | - |
| 39a1-39a6 | IF the UE is configured to use preconditions THEN steps 6-10A of Annex A.5.1a take place. | - | - | - | - |
| 39a7 | Check: Does the UE send a correctly composed 200 OK for INVITE request? (Step 11 of annex A.5.1a) | --> | 200 OK | 1 | P |
| 39a8 | The SS sends ACK | <-- | ACK | - | - |
| 39b1-39b4 | ELSE steps 6-8A of Annex A.5.2a of take place | - | - | - | - |
| 39b5 | Check: Does the UE send a correctly composed 200 OK for INVITE request? (Step 9 of annex A.5.2a) | --> | 200 OK | 1 | P |
| 39b6 | SS send ACK | <-- | ACK | - | - |
| 40-41 | Steps 4-5, as defined in Annex A.23 of TS 34.229-5 [47] is performed. | - | - | - | - |
| 42 | Check: Does UE send SIP INFO message with MSD update? (Step 6 of annex A.23) | --> | SIP INFO | 2 | P |
| 43 | The SS responds SIP INFO with 200 OK. | <-- | 200 OK | - | - |
| 44 | Generic test procedure for IMS MT call release in 5GS described in TS 38.508-1 [21] subclause 4.9.18 takes place. | - | - | - | - |

11.13.3.3	Specific message contents

Table 11.13.3.3-1: INVITE (Step 1 of annex A.23, in parallel to steps 23-25, Table 11.13.3.2-1)

Derivation path: Step 1 in Annex A.23 with Condition A21

Table 11.13.3.3-2: INVITE (Step 35, Table 11.13.3.2-1)

Derivation path: Step 1 in Annex C.11 of TS 34.229-1[2] with condition A12

Table 11.13.3.3-3: 200 OK (Steps 39a7, 39b5, Table 11.13.3.2-1)

Derivation path: Step 12 in Annex C.11 of TS 34.229-1[2] with condition A24

Table 11.13.3.3-4: BYE (Steps 26, 44, Table 11.13.3.2-1)

Derivation path: Step 1 in Annex A.8 with Condition A9

## 11.14	eCall only mode / eCall to URI for test service / MSD transfer and 200 OK with ACK / SIP INFO request for MSD Update / PSAP Callback / Success

11.14.1	Test Purpose (TP)

(1)

with { UE subjected to national regulations in CEN EN 17184 has an ongoing ecall to uri for test services }

ensure that {

when { SS sends SIP INFO message for MSD }

then { the UE sends SIP INFO message including MSD transfer }

}

(2)

with { UE subjected to national regulations in CEN EN 17184 having previously established eCall to URI for test services }

ensure that {

when { UE receives INVITE for an incoming IMS call for PSAP callback }

then { UE responds with 200 OK including correctly composed header fields }

}

(3)

with { UE subjected to national regulations in CEN EN 17184 having previously established eCall to URI for test services }

ensure that {

when { SS requests for transfer of MSD during an incoming IMS call }

then { UE responds with 200 OK and performs MSD transfer }

}

11.14.2.	Conformance requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-19 requirements.

[Rel-19, TS 24.229, clause 5.1.6A]:

This clause specifies a feature which applies to UEs subject to national regulations in CEN EN 17184:2024 [300]. If the upper layers request establishment of an IMS call of the test eCall type while in limited service state, the request shall be rejected. Otherwise, the UE shall follow normal voice call setup to attempt the test eCall. If the CS domain is selected, the UE shall attempt a test eCall setup as a non-emergency services call using appropriate access technology specific procedures.

If the IM CN subsystem is selected and the UE is not currently registered, the UE shall perform an initial registration, as described in subclause 5.1.1.2. The UE shall then apply voice domain selection as specified in 3GPP TS 23.221 [6] and 3GPP TS 23.501 [257] and the procedures as specified in subclause 5.1.2A and 5.1.3 to send an INVITE with the following additions:

1)	the UE shall set the Request-URI to a non-emergency service URN (e.g. SIP, Service, Tel URIs) that is defined by a national regulator, PSAP or eCall test provider for testing purposes; and

NOTE 1:	The Request-URI can be set to "urn:service:test.sos.ecall" as specified in RFC 8147 [244] or the URI can be configured in USIM files; see 3GPP TS 31.102 [15C].

2)	the UE shall insert in the INVITE request:

a)	a To header field with the same service URN as in the Request-URI;

b)	a From header field that includes the public user identity or the tel URI associated with the public user identity, as described in subclause 4.2;

c)	a multipart/mixed body containing an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244], containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 PER as specified in CEN EN 15722:2020 [245] and include a Content-Disposition header field with a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

d)	an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244]; and

e)	 a Recv-Info header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244].

NOTE 2:	Further content for the INVITE such as set the test bit in the MSD is defined in CEN EN 15722:2020 [245] and others is defined in RFC 8147 [244].

Then the UE shall proceed as follows:

1)	if the UE receives a 200 (OK) response to the INVITE request not containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to either "true" or "false"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall send the MSD using audio media stream encoded as described in 3GPP TS 26.267 [9C];

2)	if the UE receives a 200 (OK) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "true"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as successful;

3)	if the UE receives a 200 (OK) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "false"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as unsuccessful;

4)	if the UE receives a 486 (Busy Here), 600 (Busy Everywhere) or 603 (Decline) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "true"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as successful and shall report the response and successful MSD transfer to upper layers; and

5)	in all other cases, the UE may perform a normal voice call redial to reattempt the test eCall as described above

While a test eCall remains established a UE shall support the procedures specified in subclause 5.1.6.11.3 for transfer of an updated MSD and the procedures specified in 3GPP TS 26.267 [9C] for transfer of an updated MSD using the audio media stream.

NOTE 3:	For the updated MSD the test bit is set in the MSD according to CEN EN 15722:2020 [245].

After a test eCall has been established and released, successfully or unsuccessfully, a UE shall support the procedures specified in subclause 5.1.6.11.3 for transfer of an updated MSD during an incoming IMS call when the following conditions are fulfilled:

1)	the incoming IMS call happens within an implementation dependent time period according to the national requirements within the region of eCall service (i.e. for EU region the timer value is 1 hour as defined by the T9 timer in CEN EN 17184:2024 [300]);

2)	the INVITE for the incoming IMS call contains a P-Asserted-Identity header field containing the same non-emergency service URN as included in the Request-URI for the test eCall or as included in a P-Asserted-Identity header field received in a response to the INVITE for the test eCall;

3)	the INVITE for the incoming IMS call contains an Accept header field that includes the value "application/EmergencyCallData.Control+xml"; and

4)	the INVITE for the incoming IMS call contains a Recv-Info header field with header field value EmergencyCallData.eCall.MSD.

When conditions 1) to 4) are fulfilled, the UE shall include in the 200 OK:

a)	a Recv-Info header field with header field value EmergencyCallData.eCall.MSD; and

b)	an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244].

The UE shall also support the procedures specified in subclause 5.1.6.11.3 for transfer of an updated MSD.

NOTE 4:	As defined in CEN EN 17184:2024 [300], the recipient of a test eCall can instigate a callback to the UE to verify UE callback capability and ability to transfer updated MSD.

NOTE 5:	The recipient of a test eCall is encouraged to call back the UE after a shorter period in order to complete the test eCall without a long wait by a test eCall user.

When conditions 1) and 2) are fulfilled, the UE shall support request and transfer of an updated MSD using audio media stream as described in 3GPP TS 26.267 [9C].

NOTE 6:	A test eCall recipient can request transfer of an updated MSD using audio media stream if the procedures specified in subclause 5.1.6.11.3 cannot be supported by the recipient, the serving PLMN, the HPLMN or by some other intermediate network.

NOTE 7:	For a trustable and secure identification of an eCall callback and the related MSD transfer between UE and the PSAP (or test centre), the PSAP (or test centre) is part of the trust domain of the IMS home operator network where the UE under test belongs to.

[TS 24.229, clause 5.1.6.11.3]:

During an emergency session established for eCall type of emergency service as described in subclause 5.1.6.11.2, if the UE receives an INFO request with:

1)	an Info-Package header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244];

2)	a multipart/mixed body including:

a)	an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] containing a "request" element with an "action" attribute set to "send-data" and a "datatype" attribute set to "eCall.MSD"; and

b)	a Content-Disposition header field set to "By-Reference" associated with the "application/EmergencyCallData.Control+xml" MIME body part; and

3)	a Content-Disposition header field set to "Info-Package" associated with the multipart/mixed body;

the UE shall proceed as follows:

1)	if the UE is able to provide an updated MSD, the UE shall send an INFO request containing:

a)	an Info-Package header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244];

b)	a multipart/mixed body including:

i)	an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244] containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 as specified in CEN EN 15722:2015 [245]; and

ii)	a Content-Disposition header field set to "By-Reference" associated with the "application/EmergencyCallData.eCall.MSD" MIME body part; and

c)	a Content-Disposition header field set to "Info-Package" associated with the multipart/mixed body; and

…

Reference(s)

3GPP TS 24.229 [10], clauses 5.1.6.11.3 and 5.1.6A.

11.14.3	Test description

11.14.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC with eCall only subscription according to TS 38.508-1 [21] table 6.4.1-24.

Preamble:

-	The UE is in test state 0-A (Switched Off) as defined in TS 38.508-1 [21], subclause 4.4A.2.

11.14.3.2	Test procedure sequence

Table 11.14.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is triggered to initiate a test eCall to uri for test services. | - | - | - | - |
| 2-16 | Steps 1-15 of generic procedure specified in Table 4.5.2.2-2 of TS 38.508-1 [21] are performed to register the UE to 5GC. | - | - | - | - |
| 17 | Step 2a1 of generic procedure specified in Table 4.5A.2.2.2-2 of TS 38.508-1 [21] is performed. | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT REQUEST | - | - |
| 18-20 | Steps 3-5a1 of generic procedure specified in Table 4.5A.2.2.2-1 of TS 38.508-1 [21] are performed to establish the IMS PDU session. | - | - | - | - |
| 21-24 | Steps 1-4 as defined in Annex A.2 are performed (IMS registration). | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in steps 25-33, steps 5-8 as defined in Annex A.2 are performed and the steps specified in Annex A.10 on PUBLISH may happen. | - | - | - | - |
| 25 | Step 1 as defined in A.23 is performed | --> | INVITE | - | - |
| 26-28 | Steps 11-13 of generic procedure specified in Table 4.9.11.2.2-1 of TS 38.508-1 [21] are performed to establish the QoS flow for the test call. |  |  |  |  |
| 29-30 | Steps 2-3 as defined in Annex A.23 are performed to complete the establishment of the test eCall. |  |  |  |  |
| 31-32 | Steps 4-5 as defined in Annex A.23 are performed to request MSD update. | - | - | - | - |
| 33 | Check: Does UE send SIP INFO message with MSD update? | --> | SIP INFO | 1 | P |
| 34 | SS responds SIP INFO with 200 OK. | <-- | 200 OK | - | - |
| 35 | Release eCall for test service using the generic procedure described in TS 38.508-1 [21] subclause 4.9.18. | - |  | - | - |
| 36 | The SS waits for 10 seconds | - | - | - | - |
| 37-49 | Steps 1-12 of the generic test procedure for IMS MT speech call establishment in 5GC specified in TS 38.508-1 [21] subclause 4.9.16.2.2 are performed. | - | - | - | - |
| - | EXCEPTION: Steps 50a1-50b6 describe behaviour that depends on UE configuration; the “lower case letter” identifies a step sequence that takes place if such configuration was conducted. | - | - | - | - |
| 50a1-50a6 | IF the UE is configured to use preconditions THEN steps 6-10A of Annex A.5.1a take place. | - | - | - | - |
| 50a7 | Check: Does the UE send a correctly composed 200 OK for INVITE request? (Step 11 of annex A.5.1a) | --> | 200 OK | 2 | P |
| 50a8 | The SS sends ACK | <-- | ACK | - | - |
| 50b1-50b4 | ELSE steps 6-8A of Annex A.5.2a of take place | - | - | - | - |
| 50b5 | Check: Does the UE send a correctly composed 200 OK for INVITE request? (Step 9 of annex A.5.2a) | --> | 200 OK | 2 | P |
| 50b6 | The SS send ACK | <-- | ACK | - | - |
| 51-52 | Steps 4-5, as defined in Annex A.23 of TS 34.229-5 [47] are performed to request MSD update. | - | - | - | - |
| 53 | Check: Does UE send SIP INFO message with MSD update? | --> | SIP INFO | 3 | P |
| 54 | The SS responds SIP INFO with 200 OK. | <-- | 200 OK | - | - |
| 55 | Generic test procedure for IMS MT call release in 5GS described in TS 38.508-1 [21] subclause 4.9.18 takes place. | - | - | - | - |

11.14.3.3	Specific message contents

Table 11.14.3.3-1: INVITE (Step 25, table 11.14.3.2-1)

Derivation path: Step 1 in Annex A.23 with condition A25 instead of A20/A21 and without condition A7

Table 11.14.3.3-2: 200 OK for INVITE (Step 29, Table 11.14.3.2-1)

Derivation path: Step 2 in Annex A.23 with Conditions A12 and A13 and without condition A6

Table 11.14.3.3-3: INVITE (Step 46, Table 11.14.3.2-1; step 9a1 or 9b1 of TS 38.508-1 [21] Table 4.9.16.2.2-1)

Derivation path: Step 1 in Annex A.5.1a or A.5.2a with condition A13

Table 11.14.3.3-4: 200 OK (Steps 50a7,50b5, Table 11.14.3.2-1)

Derivation path: Step 2 in Annex A.23 with condition A24

Table 11.14.3.3-5: INFO (Steps 31,51, Table 11.14.3.2-1)

Derivation path: Step 4 in Annex A.23 with condition A1

Table 11.14.3.3-6: INFO (Steps 33,53, Table 11.14.3.2-1)

Derivation path: Step 6 in Annex A.23

<End of modification>