3GPP TSG-RAN5 Meeting #107	R5-252265r1

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 1150 | rev |  | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to TC 6.6.3 Beam Correspondence in RRC_INACTIVE and initial access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple AB Denmark |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_Ph3-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on the discussion paper R5-251738, the test procedure requires updates using the UE beamlock approach. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated test procedure with UE bem lock function, message contents |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The FR2 Beam Correspondence in RRC_INACTIVE and initial access test case will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Associated discussion paper R5-251738 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

### 6.6.3	Beam Correspondence in RRC_INACTIVE and initial access

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- MU/TT Analysis is pending.

- The UE Beamlock Function in RRC_Inactive and Initial Access’ (UBF_IA) framework is FFS in TS 38.509.

6.6.3.1	Test purpose

The purpose of this test is to verify the UE RACH MSG1 performance and uplink spatial coverage of the UE in expected directions is acceptable.

Transmission of the wrong power increases interference or transmission errors in the uplink channel.

6.6.3.2	Test applicability

This test case applies to all types of NR UE release 18 and forward.

6.6.3.3	Minimum conformance requirements

The minimum conformance requirements for beam correspondence in RRC_INACTIVE and initial access are same as specified in section 6.2.1.1.3.3.

For the beam correspondence requirement for UEs in initial access and in RRC_INACTIVE, the following applicability rules apply:

-	If a UE meets UE beam correspondence requirements in initial access, it is considered to have met the beam correspondence requirements in RRC_INACTIVE.

Table 6.6.3.3-1: Conditions for SSB based L1-RSRP measurements for beam correspondence

| Angle of arrival | NR operating bands | Minimum SSB_RP Note 2 | SSB Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm / SCSSSB | dB |
|  |  | SCSSSB = 120 kHz |  |
| All angles Note 1 | n257 | -96.2 | ≥6 |
|  | n258 | -96.2 |  |
|  | n259 | -90.7 |  |
|  | n260 | -91.9 |  |
|  | n261 | -96.2 |  |
|  | n262 | -88.5 |  |
|  | n263 | -88.0 |  |
| NOTE 1: For UEs that support multiple FR2 bands, the Minimum SSB_RP values for all angles are increased by MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1. NOTE 2: Values specified at the radiated requirements reference point to give minimum SSB Ês/Iot, with no applied noise. |  |  |  |

The normative reference for this requirement is TS 38.101-2 [3] clause 6.6.3.

6.6.3.4	Test description

6.6.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in Table 6.6.3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.3.4.1-1: Test Configuration Table

| Default Conditions |  |
| --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 | Normal |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 | Low range, Mid Range, High range |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 | 100 MHz |
| Test SCS as specified in Table 5.3.5-1 | 120 kHz |
| PRACH Parameters |  |
| PRACH Configuration Index | 143 [Format B4] |
| preambleReceivedTargetPower | -60 |
| preambleTransMax | n200 |
| ra-ResponseWindow | sl8 |
| SSB configuration |  |
| SSB index | 0 |
| SSB_RP | As per Table 6.6.3.3-1 depending on the band |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.    The UL Reference Measurement channels are set according to Table 6.6.3.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.6.3.4.3.

        - Test procedure

With the ‘UE Beamlock Function in RRC_Inactive and Initial Access’ (UBF_IA)

- Select any of the three Alignment Options (1, 2, or 3) from Tables N.2-1 through N.2-7 [3] to mount the DUT inside the QZ.

- Position the DUT in DUT Orientation 1 from Tables N.2-1 through N.2-7 [3].

- Connect the SS (System Simulator) with the DUT through the measurement antenna with PolLink= polarization to form the TX beam towards the measurement antenna.

- The SS activates the ‘UE Beamlock Function in RRC_Inactive and Initial Access’ (UBF_IA) using condition Tx only. The UE memorizes this UBF_IA activation.

- The SS shall initiate the RRCRelease procedure to transit a UE from RRC_CONNECTED to RRC_IDLE state.

- The SS Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble from the UE. The network initiates the paging procedure by transmitting the Paging message.

- The UE shall send the signalled preamble to the SS. The UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power.

- The UE shall activate the UBF_IA prior to the measurement period at n188 of the PreambleTransMax counter.

- The SS uses 10 UL slots of PRACH for a 1ms measurement period from n189 to n199 of the PreambleTransMax counter.

- EIRP is calculated considering both polarizations, theta and phi. Calculate total EIRP(PolLink=) = EIRP(PolMeas= PolLink=)  + EIRP(PolMeas= PolLink=).

- The SS shall transmit a Random Access Response containing a Random Access Preamble identifier matching the transmitted Random Access Preamble at the n200 PreambleTransMax counter.

- The SS shall ensure the UE is in RRC_CONNECTED state.

- Connect the SS with the DUT through the measurement antenna with PolLink= polarization to form the TX beam towards the measurement antenna.

- Repeat steps 4 through 12 and get the result of total EIRP(PolLink=) = EIRP (PolMeas= PolLink=) + EIRP (PolMeas= PolLink=)

- Advance to the next grid point and repeat steps 3 through 14 until measurements within zenith range 0o≤θ≤90o have been completed.

- After the measurements within zenith range 0o≤θ≤90o have been completed and

- if the re-positioning concept is applied, position the device in DUT Orientation 2 (either Options 1 or 2) from Tables N.2-1 through N.2-7 [3] for the Alignment Option selected in Step 1and perform steps 3 through 14 for the range of zenith angles 0o≤θ≤90o

- if the re-positioning concept is not applied, continue steps 3 through 14 for the range of zenith angles 90o≤θ≤180o

- Measure the UE EIRP value for each grid point and obtain a cumulative distribution function (CDF) of all EIRP dBm values according to the EIRP spherical coverage procedure defined in Annex K.1.5.

- Identify the EIRP dBm value corresponding to %-tile (UE power class dependent) value in the applicable test requirement table in section 6.6.3.5.

6.6.3.4.3	Message contents

Message contents are according to TS 38.508-1 subclause 4.6 with following exceptions:

Table 6.6.3.4.3-1: RACH-ConfigGeneric

| Derivation Path: TS 38.508-1 [14], table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 143 |  | FR2 |
| msg1-FDM | one |  | FR2 |
| zeroCorrelationZoneConfig | 11 |  |  |
| preambleReceivedTargetPower | -60 |  |  |
| preambleTransMax | n200 |  |  |
| powerRampingStep | dB2 |  |  |
| ra-ResponseWindow | sl8 |  |  |
| } |  |  |  |

      - Test requirement

The requirement for the power measured in the test procedure shall not exceed the values specified in Table 6.6.3.5-1.

Table 6.6.3.5-1: UE spherical coverage for power class 3

| Operating band | Min EIRP at 50 %-tile CDF (dBm) |
| --- | --- |
| n257 | 11.5 |
| n258 | 11.5 |
| n259 | 5.8 |
| n260 | 8 |
| n261 | 11.5 |
| n262 | 2.9 |
| n263 | 2.3 |
| NOTE 1: Minimum EIRP at 50 %-tile CDF is defined as the lower limit without tolerance in RRC_CONNECTED. NOTE 2: Void NOTE 3: The requirements in this table are verified only under normal temperature conditions as defined in Annex E.2.1. NOTE 4: Minimum EIRP at 50%-tile CDF is defined as the lower limit minus 2 dB in initial access and RRC_INACTIVE |  |

## <<< END OF CHANGES >>>