3GPP TSG-RAN5 Meeting #107	R5-252398r1

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3332 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to Spectrum emission mask for ATG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Update to Spectrum emission mask for ATG |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update 6.5J.2.2, 6.5J.2.1.5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_ATG-UEConTest work item cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5J.2.2, 6.5J.2.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision (Source to TSG) |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

#### 6.5J.2.2	Spectrum emission mask for ATG

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Test procedure and even minimum requirements are FFS in case of devices with more than one antenna/TAB connector."

- MU and TT are FFS

6.5J.2.2.1	Test purpose

Same test purpose as in 6.5.2.1.1.

6.5J.2.2.2	Test applicability

This test case applies to all types of NR UE release 18 and forward that support NR standalone ATG.

6.5J.2.2.3	Minimum conformance requirements

If the actual transmission power of ATG UE is less than or equal to 31dBm, the requirements for spectrum emission mask in clause 6.5.2.2 apply; if the actual transmission power of ATG UE is larger than 31dBm, the requirements of spectrum emission mask in clause 6.5.2.2 shall be relaxed with scaling factor equal to (the actual transmission power minus 31) dB. For ATG UE, the requirements for spectrum emission mask are defined as the sum of the emissions from all UE transmit antenna connectors or all TAB connectors.

NOTE:	This scaling factor is only applicable to ATG airborne UE.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5J.2.2.

#### 6.5J.2.2.4	Test description

Same test purpose as in 6.5.2.1.4

#### 6.5J.2.2.4.1	Initial conditions

Same initial conditions as in 6.5.2.1.4.1

6.5J.2.2.4.2	Test procedure

FFS

6.5J.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6.

6.5J.2.1.5	Test requirement

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Tables 6.2.2.5-1 to 6.2.2.5-9 as appropriate, and the power of any UE emission shall fulfil requirements in Table 6.5.2.2.5-1.

Table 6.5.2.2.5-1: General NR spectrum emission mask

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- |
|  | 3 | 5 | 10, 15, 20, 25, 30, 35, 40, 45 | 50, 60, 70, 80, 90, 100 |  |
| ± 0-1 | -13 + TT | -13 + TT | -13 + TT |  | 1 % of channel BW |
| ± 0-1 |  |  |  | -24 + TT | 30 kHz |
| ± 1-5 | -10 + TT | -10 + TT | -10 + TT |  | 1 MHz |
| ± 5-6 | -25 + TT | -13 + TT |  |  |  |
| ± 6-10 |  | -25 + TT |  |  |  |
| ± 5-BWChannel |  |  | -13 + TT |  |  |
| ± BWChannel-(BWChannel+5) |  |  | -25 + TT |  |  |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel. Note 4: TT for each frequency and channel bandwidth is specified in Table 6.5.2.2.5-2. |  |  |  |  |  |

Table 6.5.2.2.5-2: Test Tolerance (Spectrum Emission Mask)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 100MHz | 1.5 dB | 1.8 dB | 1.8 dB |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

### 6.5J.3	Spurious emissions for ATG

#### 6.5J.3.0	General

Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products, but exclude out of band emissions unless otherwise stated. The spurious emission limits are specified in terms of general requirements in line with SM.329 [22] and NR operating band requirement to address UE co-existence.

<<< STOP CHANGES >>>