3GPP TSG-RAN WG5 Meeting #107	R5-252420r1

# St. Julian’s, Malta, 19th May 2025 – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-5 | CR | 0095 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of reference sensitivity test cases with variable Tx-Rx spacing for NR NTN bands |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | EchoStar |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN2 specifications allow configuration UL and DL center frequency independently. Normally RAN4 defines minimum requirements with a single default Tx-Rx separation. Satellite industry has a strong need to use different Tx-Rx spacing in different deployments. Therefore, RAN4 specified variable Tx-Rx spacing for NTN bands and added a requirement to validate those different Tx-Rx spacing for reference sensitivity test case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates clauses 5 and 7 per updates made in the core specifications. Adds new clause/test case for reference sensitivity test cases for variable Tx-Rx spacing to support conformance of variable Tx-Rx spacing for NR NTN UEs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformance of variable Tx-Rx spacing cannot be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.4, 7.1, 7.3.2.3, New clause 7.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: i) Updated the test case title; ii) added editor’s note indicating test frequencies and TP analysis specifications need to be updated;ii) updated clause number based on feedback |  |  |  |  |  |  |  |  |

### 5.4.4	TX–RX frequency separation

The default TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation for operating bands is specified in Table 5.4.4-1 for FR1-NTN.

Table 5.4.4-1: UE TX-RX frequency separation (FR1-NTN)

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n256 | 190 MHz1 165 to 215 MHz2 |
| n255 | -101.5 MHz1 -72.5 to -130.5 MHz2 |
| n254 | 862 – 885 MHz |
| NOTE 1: Default Tx-Rx separation NOTE 2: The verification of flexible Tx-Rx frequency separation within this range is limited to reference sensitivity. Further details are specified in clause 7.3.2 |  |

<<NEXT CHANGE >>

## 7.1	General

Unless otherwise stated the receiver characteristics are specified at the antenna connector(s) of the UE. For UE(s) with an integral antenna only, a reference antenna(s) with a gain of 0 dBi is assumed for each antenna port(s). UE with an integral antenna(s) may be taken into account by converting these power levels into field strength requirements, assuming a 0 dBi gain antenna. For UEs with more than one receiver antenna connector, identical interfering signals shall be applied to each receiver antenna port if more than one of these is used (diversity).

The levels of the test signal applied to each of the antenna connectors shall be as defined in the respective clauses below.

With the exception of clause 7.3, the requirements shall be verified with the network signalling value NS_01 configured in Table  6.2.3.3.1-1.

All requirements in this clause are applicable to devices supporting GSO and/or NGSO satellites.

All the parameters in clause 7 are defined using the UL reference measurement channels specified in Annex A.2.2, the DL reference measurement channels specified in Annex A.3.2 and using the set-up specified in Annex C.3.1.

All Rx requirements are verified with default Tx-Rx separation specified in Table 5.4.4-1. Additional Tx-Rx frequency separations specified in clause 7.3.2 do not apply to other requirements.

<<NEXT CHANGE >>

#### 7.3.2.3	Minimum conformance requirements

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annex A3.2, with parameters specified in Table 7.3.2.3-1.

Table 7.3.2.3-1: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| n256 | 15 | -99.5 | -96.3 | -94.5 | -93.3 |
|  | 30 |  | -96.6 | -94.6 | -93.5 |
|  | 60 |  | -97.0 | -94.9 | -93.7 |
| n255 | 15 | -100.0 | -96.8 | -95.0 | -93.8 |
|  | 30 |  | -97.1 | -95.1 | -94.0 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n254 | 15 | -99.5 | -96.3 | -94.5 |  |
|  | 30 |  | -96.6 | -94.6 |  |
|  | 60 |  | -97.0 | -94.9 |  |
| NOTE：The transmitter shall be set to PUMAX as defined in clause 6.2.4. |  |  |  |  |  |

The reference receiver sensitivity (REFSENS) requirement specified in Table 7.3.2.3-1 shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2.3-2 and with default Tx-Rx carrier center frequency separation except for cases specified in Table 7.3.2.3-3.

Table 7.3.2.3-2: Uplink configuration for reference sensitivity

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 5 | 10 | 15 | 20 | Duplex Mode |
|  | 15 | 25 | 50 | 75 | 100 |  |
| n256 | 30 |  | 24 | 36 | 50 | FDD |
|  | 60 |  | 10 | 18 | 24 |  |
|  | 15 | 25 | 50 | 75 | [75] |  |
| n255 | 30 |  | 24 | 36 | [36] | FDD |
|  | 60 |  | 10 | 18 | [18] |  |
| n254 | 15 | 25 | 50 | 75 |  | FDD |
|  | 30 |  | 24 | 36 |  |  |
|  | 60 |  | 10 | 18 |  |  |
| NOTE: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth in Table 5.3.2-1. |  |  |  |  |  |  |

Table 7.3.2.3-3: TX – RX carrier centre frequency separation for REFSENS verification

| Operating Band | Channel bandwidth | TX – RX carrier centre frequency separation for REFSENS verification |
| --- | --- | --- |
| n256 | 5 MHz | 165 MHz, 215 MHz |
|  | 20 MHz | 180 MHz, 200 MHz |
| n255 | 5 MHz | -72.5 MHz, -130.5 MHz |
|  | 20 MHz | -87.5 MHz, -115.5 MHz |

The minimum requirements specified in Table 7.3.2.3-1 shall be verified with the network signalling value NS_01 configured in Table 6.2.3.1-1.

The normative reference for this requirement is TS 38.101-5 [11] clause 7.3.2.

<<NEXT CHANGE >>

### 7.3.2_1	Reference sensitivity power level for flexible Tx-Rx frequency separation

Editor’s Note: This test is incomplete. The following aspects are not yet determined:

- TP analysis needs to be captured in TR 38.905

- Test frequencies have to be specified for this test case

- Clause reference for the test frequencies is TBD

#### 7.3.2_1.1	Test purpose

The test purpose is to verify the ability of the UE to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise for Tx-Rx frequency separation other than the default value.

#### 7.3.2_1.2	Test applicability

The requirements of this test apply to all types of NR Power Class 3 UE release 17 and forward that support satellite access operation.

#### 7.3.2_1.3	Minimum conformance requirements

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annex A3.2, with parameters specified in Table 7.3.2_1.3-1.

#### 7.3.2_1.3	Minimum conformance requirements

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annex A3.2, with parameters specified in Table 7.3.2.3-1.

NOTE：The transmitter shall be set to PUMAX as defined in clause 6.2.4.

The reference receiver sensitivity (REFSENS) requirement specified in Table 7.3.2.3-1 shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2.3-2 and for Tx-Rx carrier center frequency separation specified in Table 7.3.2.3-3.

The minimum requirements specified in Table 7.3.2.3-1 shall be verified with the network signalling value NS_01 configured in Table 6.2.3.1-1.

The normative reference for this requirement is TS 38.101-5 [11] clause 7.3.2.

#### 7.3.2_1.4	Test description

##### 7.3.2_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 7.3.2_1.4.1-1, Table 7.3.2_1.4.1-2, and Table 7.3.2_1.4.1-3 The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2.2 of TS 38.521-1 [2]. Configurations of PDSCH and PDCCH before measurement are specified in TS 38.521-1 [2] Annex C.2.

Table 7.3.2_1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [12] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [12] subclause FFS |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [12] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration |  | Uplink Configuration |  |
|  | Modulation | RB allocation | Modulation | RB allocation |
| 1 | CP-OFDM QPSK | Full RB (NOTE 1) | DFT-s-OFDM QPSK | REFSENS (NOTE 2) |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. |  |  |  |  |

Table 7.3.2_1.4.1-2: Downlink Configuration of each RB allocation

| Channel Bandwidth | SCS(kHz) | LCRBmax | Outer RB allocation / Normal RB allocation |
| --- | --- | --- | --- |
| 5MHz | 15 | 25 | 25@0 |
|  | 30 | 11 | 11@0 |
|  | 60 | N/A | N/A |
| 20MHz | 15 | 106 | 106@0 |
|  | 30 | 51 | 51@0 |
|  | 60 | 24 | 24@0 |
| NOTE: Test Channel Bandwidths are checked separately for each NR band, the applicable channel bandwidths are specified in Table 5.3.5-1. |  |  |  |

Table 7.3.2_1.4.1-3: Uplink configuration for reference sensitivity, LCRB @ RBstart format

| Operating Band | SCS kHz | 5 MHz | 20 MHz |
| --- | --- | --- | --- |
| n256 | 15 | 25@0 | 100@0 |
|  | 30 |  | 50@0 |
|  | 60 |  | 24@0 |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [12] Annex A, Figure A.3.1.1 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to clauses C.0, C.1 and C.2, and uplink signals according to TS 38.521-1 [2] Annex G.0, G.1, G.2, and G.3.1.

4.	The UL and Reference Measurement Channel is set according to Table 7.3.2_1.4.1-1, Table 7.3.2_1.4.1-2, and Table 7.3.2_1.4.1-3.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [12] clause 4.5. Message contents are defined in clause 7.3.2.4.3.

##### 7.3.2_1.4.2	Test procedure.

1.	UE location according to TS 38.508-1 [12] clause 5.6.1 is provided to the UE through any preconfigured means.

2.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 38.508-1 [12] Table 5.6.2.1-1 for GSO if UE supports only GSO or both GSO and NGSO satellites and Table 5.6.2.1-3 for NGSO (LEO-1200) if UE supports only NGSO satellites. Test system shall send same SIB19 information during the duration of the test as defined in TS 38.508-1 [12] clause 5.6.3.1.

3. 	Deactivate UE prediction of satellite trajectory by any preconfigured means.

4.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.3.2_1.4.1-1. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Tables 7.3.2_1.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

6.	Set the Downlink signal level to the appropriate REFSENS value defined in Table 7.3.2_1.5-1 if 2Rx antennas connected.  Send continuously uplink power control "up" commands in the uplink scheduling information to the UE to ensure the UE transmits PUMAX level for at least the duration of the Throughput measurement.

7.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex H.2.

##### 7.3.2_1.4.3	Message contents

Message contents are according to TS 38.508-1 [12] clause 4.6 ensuring Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED for NR band.

Message contents are according to TS 38.508-1[12] subclause 4.6 with the following exceptions for each network signalling value.

SIB19 message contents according to TS 38.508-1 [12] clause 5.6.2.1

7.3.2_1.4.3.1	Message contents exceptions (network signalled value "NS_01")

Message contents according to TS 38.508-1 [12] subclause 4.6 can be used without exceptions.

#### 7.3.2_1.5	Test requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 with reference receive power level specified in Tables 7.3.3.5-1 and parameters specified Tables 7.3.2_1.4.1-1, Tables 7.3.2_1.4.1-2 and Tables 7.3.2_1.4.1-3.

Table 7.3.2_1.5-1: Two antenna port Reference sensitivity QPSK PREFSENS for FDD bands for PC3

| Operating Band | SCS kHz | 5 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- | --- |
| n256 | 15 | -99.5 +TT | -93.3 +TT | FDD |
|  | 30 |  | -93.5 +TT |  |
|  | 60 |  | -93.7 +TT |  |
| NOTE 1: Void. NOTE 2: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 NOTE 3: The requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9-1510.9 MHz. NOTE 4: Void NOTE 5: Void NOTE 6: TT for each frequency and channel bandwidth is specified in Table 7.3.2_1.5-2. |  |  |  |  |

Table 7.3.2_1.5-2: Test Tolerance (TT) for RX sensitivity level

f ≤ 3.0GHz

0.7 dB

<<END OF CHANGES >>