3GPP TSG-RAN5 Meeting #107	R5-252432r1

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3902 | rev | - | Current version: | 18.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR SA FR1 inter-RAT Cell reselection with NTN Carrier for Satellite Access in 14.1.11 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Rel-18 NR SA FR1 inter-RAT Cell reselection with NTN Carrier for Satellite Access in 14.1.11 is currently missing in the specification. This CR adds this test case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new NTN test case 14.1.11 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case would be missing in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.1.0.3, 14.1.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Update the IEs font to italic font. Update the first letter of table to a capital letter ‘T’. Modify the wrong reference numbers of TS 36.133 and TS 38.304. Update Editor’s notes. Update Message contents and Test procedure. |  |  |  |  |  |  |  |  |

<< Start of Changes >>

14.1.0.3	Measurements of inter-RAT NR – E-UTRA cells

This clause applies for the inter-RAT cell re-selection for TN carriers, and NTN carriers if configured. The requirements in this clause apply provided that network provides SIB19 and UE is configured with and one or more TN carriers.

UE is allowed to skip TN neighbour cells measurement in an area where there is no coverage of the frequency based on the provided TN cell coverage information and UE GNSS position information.  Otherwise, UE shall perform TN measurement if its estimated distance to tn-ReferenceLocation is smaller than tn-DistanceRadius. The requirements apply provided that the actual distance between UE to tn-ReferenceLocation is smaller than tn-DistanceRadius – 50m.This clause considers the inter-RAT cell reselection from NTN to E-UTRAN TN in FR1.

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-RAT E-UTRAN layers of higher priority at least every Thigher_priority_search where Thigher_priority_search Thigher_priority_search is described in clause 4.2.2.

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-RAT E-UTRAN layers of higher, lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority inter-RAT E-UTRAN layers shall be the same as that defined below for lower priority RATs.

The requirements in this clause apply to inter-RAT E-UTRAN FDD measurements and E-UTRA TDD measurements. When the measurement rules indicate that inter-RAT E-UTRAN cells are to be measured, the UE shall measure RSRP and RSRQ of detected E-UTRA cells in the neighbour frequency list at the minimum measurement rate specified in this clause.

The parameter NEUTRA_carrier is the number of EUTRA TN carriers indicated by the serving cell, except for the frequency carrier where there is no coverage of that frequency based on the provide TN cell coverage information and UE GNSS position information.

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, UE supports the enhanced inter-RAT E-UTRAN measurement requirements.

The parameter NEUTRA_carrier for a UE configured with idle mode DC measurements (while T331 is running), is the combined number of configured E-UTRA carriers in the neighbour frequency list and E-UTRA carriers configured for idle mode DC measurements.

NOTE:	Combined total number means that if a carrier is an E-UTRA carrier indicated by the serving cell for mobility and additionally a carrier configured for idle mode CA/DC measurements, it only counts as one carrier.

The UE shall filter RSRP and RSRQ measurements of each measured E-UTRA cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least half the minimum specified measurement period.

An inter-RAT E-UTRA cell is considered to be detectable provided the following conditions are fulfilled:

-	the same conditions as for inter-frequency RSRP measurements specified in TS 36.133 [23, Annex B.1.2] are fulfilled for a corresponding Band, and

-	the same conditions as for inter-frequency RSRQ measurements specified in TS 36.133 [23, Annex B.1.2] are fulfilled for a corresponding Band.

-	SCH conditions specified in TS 36.133 [23, Annex B.1.2] are fulfilled for a corresponding Band

The UE shall be able to evaluate whether a newly detectable inter-RAT E-UTRAN cell meets the reselection criteria defined in TS 38.304 [30] within NEUTRA_carrier * Tdetect,EUTRAN when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ when Treselection = 0 provided that the reselection criteria is met by a margin of at least 6dB for RSRP reselections based on absolute priorities or 4dB for RSRQ reselections based on absolute priorities.

Cells which have been detected shall be measured at least every NEUTRA_carrier * Tmeasure,EUTRAN when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ.

When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,EUTRAN. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this clause shall still be met by the UE before it makes any determination that it may stop measuring the cell.

If the UE detects, on an inter-RAT E-UTRAN carrier, a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall not consider an inter-RAT E-UTRA cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For a cell that has been already detected, but has not been reselected to, the filtering shall be such that a UE not configured with eDRX_IDLE cycle shall be capable of evaluating that an already identified inter-RAT E-UTRA cell has met reselection criterion defined in TS 38.304 [30] within NEUTRA_carrier * Tevaluate,EUTRAN when Treselection = 0 as specified in Table 4.2.2.5-1 and 4.2.2.5-2 in TS 38.133 [6] provided that the reselection criteria is met by a margin of at least 6 dB for RSRP reselections based on absolute priorities or 4 dB for RSRQ reselections based on absolute priorities.

For UE not configured with eDRX_IDLE cycle, Tdetect,EUTRAN, Tmeasure,EUTRAN and Tevaluate, E-UTRAN are specified in Table 4.2.2.5-1 and 4.2.2.5-2 in TS 38.133 [6].

The normative reference for this requirement is TS 38.133 [6] clause 4.2C.2.11.

<<Unchanged sections skipped>>

14.1.11 NR SA FR1 inter-RAT Cell reselection with NTN Carrier for Satellite Access

Editor’s Note:

- MU and TT analysis is incomplete

- Call setup and test procedure needs to be updated

- Applicability needs to be updated

- Annex E needs to be updated

14.1.11.1	Test purpose

This test is to verify the requirement for the NR NTN to E-UTRAN TN inter-RAT cell reselection requirements specified in TS 38.133 [6] clause 4.2C.2.11 when the E-UTRAN cell is of higher priority.

14.1.11.2	Test applicability

This test applies to all types of NR UE release 18 and forward supporting satellite access.

14.1.11.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 14.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.14.1.11.

14.1.11.4	Test description

The test scenario comprises of one NR cell and one E-UTRAN cell as given in Tables 14.1.11.4.1-1, 14.1.11.4.1-3, 14.1.11.5-1 and 14.1.11.5-2. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. NR Cell 1 is already identified by the UE prior to the start of the test. E-UTRAN Cell 2 is of higher priority than Cell 1.

14.1.11.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 14.1.11.4.1-1.

Table 14.1.11.4.1-1: Supported test configurations

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |  |

Configure the test equipment and the DUT according to the parameters in Table 14.1.11.4.1-2.

Table 14.1.11.4.1-2: Initial conditions for NR NTN to E-UTRAN cell re-selection

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-2 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.1.11.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.6.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 14.1.11.4.1-3.

2.	Message contents are defined in clause 14.1.11.4.3.

3.	The test scenario comprises of one NR NTN cell and one E-UTRAN cell. Cell 1 is the NR NTN cell and Cell 2 is the E-UTRA neighbour cell. Cell 1 is configured according to Annex C.1.1 and C.1.2, Cell 2 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1.

4.	The initial test environment conditions are setup according to section 14.0.5.

Table 14.1.11.4.1-3:  General test parameters for NR NTN to E-UTRAN cell re-selection test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1 | Cell 1 | The UE camps on Cell 1 in the initial phase and during T2 period the UE reselects to Cell 2. |
| T2 end | Active cell |  | 1 | Cell 2 | The UE shall perform reselection to cell 2 during T2. |
| condition | Neighbour cell |  | 1 | Cell 1 |  |
| T3 end | Active cell |  | 1 | Cell 1 | The UE shall perform reselection to cell 1 during T3 for iteration of the tests. |
| condition | Neighbour cell |  | 1 | Cell 2 |  |
| Access Barring Information |  | - | 1 | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1 | 1.28 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1 | 102 | The detailed configuration is specified in TS 38.211 [6] clause 6.3.3.2 |
| E-UTRAN PRACH configuration index |  |  | 1 | 53 | As specified in Table 5.7.1-2 in TS 36.211 [23] |
| E-UTRAN PRACH |  |  | 1 | 53 | As specified in Table 5.7.1-2 in |
| T1 |  | s | 1 | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2. |
| T2 |  | s | 1 | 40 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| T3 |  | s | 1 | 15 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

14.1.11.4.2	Test procedure

Two cells are deployed in the test, which are one FR1 NR NTN Cell (Cell 1) and an E-UTRA TN neighbour cell (Cell 2). The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. NR cell 1 is already identified by the UE prior to the start of the test. E-UTRAN cell 2 is of higher priority than cell 1.

Before T1 the UE is camped on to cell 1. During T1, cell 2 shall be powered off. At the start of T2 the UE is expected to detect cell 2, send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on cell 2. At the start of T3 cell 2 becomes weaker than cell 1, and the UE reselects to Cell 1.

In the following test procedure “UE responds” means “UE starts transmitting preamble on PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR with Test Mode On according to TS 38.508-1 [14] clause 4.5 on Cell 1.

2.	Set the parameters according to T1 in Table 14.1.11.5-1 and 14.1.11.5-2. T1 starts.

3.	During T1, Cell 2 shall be powered off and set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for one iteration of the test procedure loop.

4.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 14.1.11.5-1 and 14.1.11.5-2. T2 starts.

5.	The SS waits for random access requests information from the UE to perform cell re-selection to a higher priority cell, Cell 2.

6.	If the UE responds on Cell 2 during time duration T2 within 68 seconds from the beginning of time period T2, then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

7.	The SS shall send an RRCRelease message to ensure that the UE is in state RRC_IDLE on Cell 1.

8.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR according to TS 38.508-1 [14] clause 4.5.

9.	Repeat step 2-9 until a test verdict has been achieved.

14.1.11.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 14.1.11.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.3-1 Table H.2.3-2 with Condition SMTC 2 and higher priority Table H.2.3-3 with Condition higher priority |
| Default RRC messages and information elements contents exceptions |  |

14.1.11.5	Test requirement

Tables 14.1.11.5-1 and 14.1.11.5-2 define the primary level settings including test tolerances for higher priority E-UTRA cell re-selection test case.

Table 14.1.11.5-1: Cell specific test parameters for NR Cell 1

| Parameter | Unit | Test configuration | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| TDD configuration |  | 1, | N/A |  |  |
| PDSCH parameters |  | 1, | SR.1.1 FDD |  |  |
| RMSI CORESET parameters |  | 1, | CR.1.1 FDD |  |  |
| Dedicated CORESET parameters |  | 1 | CCR.1.1 FDD |  |  |
| SSB parameters |  | 1 | SSB.1 FR1 |  |  |
| NR SMTC parameters |  | 1 | SMTC.2 |  |  |
| OCNG Pattern |  | 1 | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1 | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1 | ULBWP.0.1 |  |  |
| RLM-RS |  | 1 | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1 | -140 |  |  |
|  | dBm/SCS | 1 | -98 |  |  |
|  | dBm/15 kHz | 1 | -98 |  |  |
| SS-RSRP | dBm/SCS | 1 | -84 | -84 | -84 |
|  | dB | 1 | 14+TT | 14+TT | 14+TT |
|  | dB | 1 | 14+TT | 14+TT | 14+TT |
| Io | dBm/9.36 MHz | 1 | -55.88+TT | -55.88+TT | -55.88+TT |
| Treselection | s | 1 | 0+TT |  |  |
| SnonintrasearchP | dB | 1 | 50+TT |  |  |
| Threshx, highP (Note 2) | dB | 1 | 48+TT |  |  |
| Threshserving, lowP | dB | 1 | 44+TT |  |  |
| Threshx, lowP | dB | 1 | 50+TT |  |  |
| Propagation Condition |  | 1 | AWGN |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: This refers to the value of Threshx, high which is included in NR system information, and is a threshold for the E-UTRA target cell |  |  |  |  |  |

Table 14.1.11.5-2: Cell specific test parameters for E-UTRA Cell 2

| Parameter | Unit | Cell 2 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| OCNG Patterns defined in TS 36.133 [15] clause A.3.2 |  | OP.2 TDD for test configuration 1, 2, 3; OP.2 FDD for test configuration 4, 5, 6 |  |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PCFICH_RB | dB |  |  |  |
| PHICH_RA | dB |  |  |  |
| PHICH_RB | dB |  |  |  |
| PDCCH_RA | dB |  |  |  |
| PDCCH_RB | dB |  |  |  |
| PDSCH_RA | dB |  |  |  |
| PDSCH_RB | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
| Qrxlevmin | dBm | -140 |  |  |
|  | dBm/15 kHz | -98 |  |  |
| RSRP | dBm/15 KHz | -infinity | -86 | -102 |
|  | dB | -infinity | 12+TT | -4+TT |
|  | dB | -infinity | 12+TT | -4+TT |
| TreselectionEUTRAN | s | 0+TT |  |  |
| SnonintrasearchP | dB | Not sent |  |  |
| Threshx, highP | dB | 48+TT |  |  |
| Threshserving, lowP | dB | 44+TT |  |  |
| Threshx, lowP (Note 2) | dB | 50+TT |  |  |
| Propagation Condition |  | AWGN |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: This refers to the value of Threshx, Low which is included in E-UTRA system information, and is a threshold for the NR target cell |  |  |  |  |  |

The cell reselection delay to a higher priority E-UTRAN cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 2.

The cell re-selection delay to a higher priority cell shall be less than 68 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90 %.

NOTE:	The cell re-selection delay to a higher priority cell can be expressed as: Thigher_priority_search + Tevaluate, E-UTRAN + TSI-E-UTRA,

Where:

Thigher_priority_search	See clause 4.2.2.7

Tevaluate, NR_ inter	See Table 4.2C.2.4-1 in clause 4.2C.2.4

TSI-E-UTRA	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 67.68 s, allow 68 s for the cell re-selection delay to a higher priority E-UTRAN cell.

<<End of changes>>