3GPP TSG-RAN5 Meeting #107	R5-252433r2

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3903 | rev | - | Current version: | 18.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR SA FR1-FR1 Cell re-selection with TN carrier for Satellite Access in 14.1.12 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Rel-18 NR SA FR1-FR1 Cell re-selection with TN carrier for Satellite Access in 14.1.12 is currently missing in the specification. This CR adds this test case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new NTN test case 14.1.12 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case would be missing in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.1.0.4, 14.1.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Update two table numbers. Update the first letter of table to a capital letter ‘T’. Update the wrong section number 14.1.11.5 to 14.1.12.5. Update the reference number of TS 38.306,TS 38.133 and TS 38.304. Update Editor’s notes. r2: Remove test steps of cell reselection from TN to NTN as the capability is optional in clause 5.6 in TS 38.306. |  |  |  |  |  |  |  |  |

<< Start of Changes >>

14.1.0.4	Measurements of inter-frequency NR cells with TN carrier

This clause applies for the inter-frequency cell re-selection for TN carriers, and NTN carriers if configured. The requirements in this clause apply provided that network provides SIB19 and UE is configured with and one or more TN carriers.

UE is allowed to skip TN neighbour cells measurement in an area where there is no coverage of the frequency based on the provided TN cell coverage information and UE GNSS position information. Otherwise, UE shall perform TN measurement if its estimated distance to tn-ReferenceLocation is smaller than tn-DistanceRadius. The requirements apply provided that the actual distance between UE to tn-ReferenceLocation is smaller than tn-DistanceRadius – 50m.This clause considers the inter-frequency cell reselection from NTN to TN in FR1.

The UE shall be able to identify new inter-frequency cells and perform SS-RSRP or SS-RSRQ measurements of identified inter-frequency cells if carrier frequency information is provided by the serving cell, even if no explicit neighbour list with physical layer cell identities is provided.

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, and the distance between UE and serving cell reference location is smaller than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2C.2.9 in TS 38.133 [6].

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, or the distance between UE and serving cell reference location is larger than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for and measure inter-frequency layers of higher, equal or lower priority in preparation for possible reselection. The requirements apply provided that the distance exceeds the distanceThresh by a margin of 50 m. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority layers shall be the same as that defined below in this clause.

The UE shall be able to evaluate whether a newly detectable inter-frequency cell meets the reselection criteria defined in TS 38.304 [1] within Kcarrier_TN * Tdetect,NR_Inter_TN +  i=1Kcarrier_NTNKmulti_SMTC,i*Tdetect,NR_Inter_NTN if the UE does not support the feature for enhanced RRM requirements defined in TS 38.306 [11]  or if the enhancedMeasurementLEO-r17 is not enabled, or withinKcarrier_TN * Tdetect,NR_Inter_TN +  i=1Kcarrier_NTNKmulti_SMTC,i*Tdetect,NR_Inter_NTN_enh if the UE supports the feature for enhanced RRM requirements defined in TS 38.306 [11]  and the enhancedMeasurementLEO-r17 is enabled, if at least carrier frequency information is provided for inter-frequency neighbour cells by the serving cells when Treselection = 0 provided that the reselection criteria is met by a margin of at least [5]dB in FR1 for reselections based on ranking or [6]dB in FR1 for SS-RSRP reselections based on absolute priorities or [4]dB in FR1 for SS-RSRQ reselections based on absolute priorities.

The parameter Kmulti_SMTC,i is the scaling factor for measurement of multiple SMTCs or multiple satellites

-	If SMTCs do not overlap with each other,

-	KmultiSMTC,i=1, if GEO satellites are measured on the carrier;

-	KmultiSMTC,i=NLEO,iNLEO,simul, if LEO satellites are measured on the carrier;

-	If SMTCs partially overlap with each other,

-	KmultiSMTC,i=NSMTC,overlap, if only GEO satellites are measured on the carrier;

-	KmultiSMTC,i=i=1NSMTC,overlapNLEO,iNLEO,simul, if only LEO satellites are measured on the carrier;

where

NLEO,i is the number of LEO satellites to be measured within i-th SMTC,

NLEO,simul is the number of LEO satellites that UE can measure in parallel within an SMTC,NSMTC,overlap is the number of SMTCs that partially overlap with each other.

NOTE:	For deriving Kmulti_SMTC,i for Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter of frequency layer i, two SMTCs are considered as overlapping if they overlap in one or more occasions during a single Tdetect,NR_Inter, Tmeasure,NR_Inter or Tevaluate,NR_Inter.

The parameter Kcarrier_TN is the number of NR TN inter-frequency carriers indicated by the serving cell, except for the frequency carrier where there is no coverage of that frequency based on the provide TN cell coverage information and UE GNSS position information.

Tdetect,NR_Inter_TN ,Tmeasure,NR_Inter_TN and Tevaluate,NR_Inter_TN are the NR TN inter-frequency cell re-selection requirement defined in Table 4.2.2.4-1 in TS 38.133[6].

Tdetect,NR_Inter_NTN ,Tmeasure,NR_Inter_NTN and Tevaluate,NR_Inter_NTN are the NR NTN inter-frequency cell re-selection requirement defined in Table 4.2C.2.4-1 in TS 38.133[6].

Tdetect,NR_Inter_NTN_enh, Tmeasure,NR_Inter_NTN_enh and Tevaluate,NR_Inter_NTN_enh are the NR NTN inter-frequency cell re-selection requirement defined in Table 4.2C.2.4-2 in TS 38.133[6].

An inter-frequency cell is considered to be detectable according to the conditions defined in TS 38.133[6] Annex B.1.7 for a corresponding band.

When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,NR_Inter. If, after detecting a cell in a higher priority search, it is determined that re-selection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of re-selection. However, the minimum measurement filtering requirements specified later in this clause shall still be met by the UE before it makes any determination that it may stop measuring the cell. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall measure SS-RSRP or SS-RSRQ at least every Kcarrier_TN * Tmeasure,NR_Inter_TN + i=1Kcarrier_NTNKmultiSMTC,i*Tmeasure,NR_Inter_NTN  (see TS 38.133 [6] Table 4.2C.2.4-1) if the UE does not support the feature for enhanced RRM requirements defined in TS 38.306 [11]  or if the enhancedMeasurementLEO-r17 is not enabled, or every  Kcarrier_TN * Tmeasure,NR_Inter_TN + i=1Kcarrier_NTNKmultiSMTC,i*Tmeasure,NR_Inter_enh_NTN_enh (see TS 38.133 [6] Table 4.2C.2.4-2) if the UE supports the feature for enhanced RRM requirements defined in TS 38.306 [11]  and the enhancedMeasurementLEO-r17is enabled, for identified lower or equal priority inter-frequency cells. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall filter SS-RSRP or SS-RSRQ measurements of each measured higher, lower and equal priority inter-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Inter/2.

The UE shall not consider a NR neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For an inter-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the inter-frequency cell has met reselection criterion defined TS 38.304 [30] within  Kcarrier_TN * Tevaluate,NR_Inter_TN + i=1Kcarrier_NTNKmultiSMTC,i*Tevaluate,NR_Inter_NTN if the UE does not support Enhanced RRM requirements for measurements in IDLE and INACTIVE modes defined in TS 38.306 [11] or if the enhancedMeasurementLEO-r17 is not enabled, or within  Kcarrier_TN * Tevaluate,NR_Inter_TN + i=1Kcarrier_NTNKmultiSMTC,i*Tevaluate,NR_Inter_NTN_enh if the UE supports the feature for enhanced RRM requirements defined in TS 38.306 [11]  and the enhancedMeasurementLEO-r17 is enabled, when Treselection = 0 as specified in Table 4.2C.2.4-1 provided that the reselection criteria is met by

-	the condition when performing equal priority reselection and

when rangeToBestCell is not configured:

-	the cell is at least [5]dB better ranked in FR1 or.

when rangeToBestCell is configured:

-	the cell has the highest number of beams above the threshold absThreshSS-BlocksConsolidation among all detected cells whose cell-ranking criterion R value in TS 38.304 [30] is within rangeToBestCell of the cell-ranking criterion R value of the highest ranked cell.

-	if there are multiple such cells, the cell has the highest rank among them

-	the cell is at least [5]dB better ranked in FR1 if the current serving cell is among them. or

-	[6]dB in FR1 for SS-RSRP reselections based on absolute priorities or

-	[4]dB in FR1 for SS-RSRQ reselections based on absolute priorities.

When evaluating cells for reselection, the SSB side conditions apply to both serving and inter-frequency cells.

The requirements in this clause apply provided that the valid information for the satellite serving the target cell has been provided by the serving cell.

The requirements in this clause apply provided that SSB of neighbour cells are within the time shifted SMTC.

The normative reference for this requirement is TS 38.133 [6] clause 4.2C.2.10.

<<Unchanged sections skipped>>

14.1.12 NR SA FR1-FR1 Cell re-selection with TN carrier for Satellite Access

Editor’s Note:

- MU and TT analysis is incomplete

- Applicability needs to be updated

- Several test parameters and configuration are still in brackets

- Annex E needs to be updated

- Some test configuration of Table 14.1.12.4.1-3 needs to be updated dependent on RAN4 in TS 38.133

14.1.12.1	Test purpose

This test is to verify the requirement for the inter frequency NR NTN to TN cell re-selection requirements specified in clause TS 38.133 [6] clause 4.2C.2.10.

14.1.12.2	Test applicability

This test applies to all types of NR UE release 18 and forward supporting satellite access.

14.1.12.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 14.1.0.4.

The normative reference for this requirement is TS 38.133 [6] clause A.14.1.12.

14.1.12.4	Test description

The test scenario comprises of 2 cells on 2 different NR carriers, including NR NTN cell/carrier and NR TN cell/carrier respectively as given in Tables 14.1.12.4.1-1, 14.1.12.4.1-2 and14.1.12.4.1-3. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. Both Cell 1 and Cell 2 are already identified by the UE prior to the start of the test. Cell 1 and Cell 2 belong to different tracking areas and Cell 2 is of higher priority than Cell 1.

14.1.12.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 14.1.12.4.1-1.

Table 14.1.12.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |

Configure the test equipment and the DUT according to the parameters in Table 14.1.12.4.1-2.

Table  14.1.12.4.1-2: Initial conditions for NR SA FR1-FR1 cell re-selection

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.1.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 14.1.12.4.1-3.

2.	Message contents are defined in clause 14.1.12.4.3.

3. There is two NR carrier and 2 NR Cells specified in the test. Cell 1 and Cell 2 are configured according to Annex C.1.2.

4.	The initial test environment conditions are setup according to section 14.0.5.

Table 14.1.12.4.1-3: General test parameters for inter frequency NR cell re-selection test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1 | Cell 1 |  |
| T2 end condition | Active cell |  | 1 | Cell 2 |  |
|  | Neighbour cells |  | 1 | Cell 1 |  |
| Final condition | Active cell |  | 1 | Cell 1 |  |
|  | Neighbour cells |  | 1 | Cell 2 |  |
| RF Channel Number |  |  | 1 | 1 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
| Access Barring Information |  | - | 1 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC.6 | Configured in SIB4 for Cell 1 and Cell 2 |
| DRX cycle length |  | s | 1 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1 | 102 | The detailed configuration is specified in TS 38.211 [6] clause 6.3.3.2 |
| rangeToBestCell |  |  | 1 | Not configured |  |
| Ephemeris information |  |  | 1 | [TBD] | The detailed configuration is specified in SIB19 |
| T1 |  | s | 1 | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 1 | 40 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| T3 |  | s | 1 | 15 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

14.1.12.4.2	Test procedure

- Ensure the UE is in [state RRC_IDLE with generic procedure parameters Connectivity NR and Test Mode On according to TS 38.508-1 [14] clause 4.5]. Cell 1 is an active cell and Cell 2 is powered off.

- Set the parameters according to T1 in Table 14.1.12.5-1. Propagation conditions are set according to Annex C clause C.2.2. T1 starts.

- During T1, Cell 2 shall be powered off and set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for this iteration of the test procedure loop.

- When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 14.1.12.5-1. T2 starts.

- The SS waits for random access requests information from the UE to perform cell re-selection to a newly detectable cell, Cell 2.

- If the UE responds on Cell 2 during time duration T2 within 68 seconds from the beginning of time period T2, then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

- The SS shall send an RRCRelease message to ensure that the UE is in state RRC_IDLE on Cell 1.

- Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR according to TS 38.508-1 [14] clause 4.5.

- Repeat step 2-8 until a test verdict has been achieved.

14.1.12.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 14.1.12.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.2-1 Table H.2.2-2 with Condition SSB.1 FR1, SMTC.2 and Asynchronous cells for Cell 1 Table H.2.2-2 with Condition SSB.1 FR1, SMTC.6 and Asynchronous cells for Cell 2 Table H.2.2-3 |
| Default RRC messages and information elements contents exceptions |  |

14.1.12.5	Test requirement

Tables 14.1.12.5-1 defines the primary level settings including test tolerances for inter frequency NR cell re-selection test case.

Table 14.1.12.5-1: Cell specific test parameters for inter frequency NR cell re-selection test case

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Satellite information |  | 1 | SSC.1 for GSO test SSC.2 for NGSO test |  |  |  |  |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
| OCNG Pattern |  | 1 | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1 | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1 | -130 |  |  | -130 |  |  |
| Pcompensation | dB | 1 | 0 |  |  | 0 |  |  |
| Qhysts | dB | 1 | 0 |  |  | 0 |  |  |
| Qoffsets, n | dB | 1 | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1 | SS-RSRP |  |  | SS-RSRP |  |  |
|  | dB | 1 | 16+TT | -3.11+TT | 2.79+TT | -infinity | 2.79+TT | -3.11+TT |
| Note2 | dBm/SCS | 1 | -98+TT |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98+TT |  |  |  |  |  |
|  | dB | 1 | 16+TT | 13+TT | 16+TT | -infinity | 16+TT | 13+TT |
| SS-RSRP Note3 | dBm/SCS | 1 | -82+TT | -85+TT | -82+TT | -infinity | -82+TT | -85+TT |
| Io | dBm/9.36 MHz | 1 | -53.94+TT | -52.21+TT | -52.21+TT | Same as parameters specified in Cell 1 columns- |  |  |
| Treselection | s | 1 | 0+TT | 0+TT | 0+TT | 0+TT | 0+TT | 0+TT |
| SintersearchP | dB | 1 | 60+TT |  |  | 60+TT |  |  |
| Propagation Condition |  | 1 | AWGN |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

The cell reselection delay to a higher priority cell is defined as the time from the beginning of time period T2, to the moment when the UE camps again on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on Cell 2.

The cell re-selection delay to a higher priority cell shall be less than 68 s.

The cell reselection delay to a lower priority cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on Cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on Cell 1.

The cell re-selection delay to a lower priority cell shall be less than 8 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90 %.

NOTE:	The cell re-selection delay to a higher priority cell can be expressed as: Thigher_priority_search + Tevaluate, NR_ inter + TSI-NR, and to a lower priority cell can be expressed as: Tevaluate, NR_ inter + TSI-NR,

Where:

Thigher_priority_search	See clause 4.2.2.7

Tevaluate, NR_ inter	See Table 4.2C.2.4-1 in clause 4.2C.2.4

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 67.68 s, allow 68 s for the cell re-selection delay to a higher priority.

<<End of changes>>