3GPP TSG-RAN5 Meeting #107	R5-252490r2

Malta, MT, 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3909 | rev |  | Current version: | 18.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test requirement for L1-RSRP measurement for group-based beam reporting test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR2_multiRX_DL-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test requirement tables for defining the limit in 7.7.14.1 and 7.7.14.2 have been missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The test requirements tables have been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7.14.1, 7.7.14.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: - Corrected the WIC on the cover sheet to resolve 3GU issue - Updated the test requirements values to address TT impact in the future R2: Corrected the test requirements values to address TT impact to 7.7.14.2 |  |  |  |  |  |  |  |  |

-------- Start of 1st change --------

### 7.7.14	L1-RSRP measurement for group-based beam reporting

###### 7.7.14.1	NR SA FR2 SSB based L1-RSRP measurement for group-based beam reporting

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD:

- TT is missing.

-	 Configuration of 3AoA setup and testability of the device is still FFS.

7.7.14.1.1	Test Purpose

The purpose of this test is to verify that the L1-RSRP measurement accuracy for group-based beam reporting is within the specified limits. This test will verify the requirements as defined in clauses 9.5.2 and 10.1.20.1 of TS 38.133[6] for SSB based L1-RSRP measurements.

7.7.14.1.2	Test Applicability

This test applies to all types of NR PC3 UE release 18 onwards that support groupBasedBeamReporting-r17.

7.7.14.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clauses 7.7.4.0.1 and 7.7.4.0.2

The normative reference for this requirement is TS 38.133 [6] clauses A.7.7.14.1

7.7.14.1.4	Test description

7.7.14.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.7.14.1.4.1-1.

Table 7.7.14.1.4.1-1: Supported Test Configurations

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | NR 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

1.	Configure the test equipment and the DUT according to the parameters in Table 7.7.4.1.4.1-2.

2.	Message contents are defined in clause 7.7.14.1.4.3.

3.	The test scenario comprises of 1 NR FR2 Cell. There is no measurement gap configured in the test. Before the test, UE is configured two CSI resource sets with one SSB resource in each set.

4.	UE is configured with groupBasedBeamReporting-r17 for SSB index 0 and SSB index 1.

5.	UE is configured to perform RLM and BFD based on SSB 0 and UE is configured to perform group-based L1-RSRP measurement based on the SSB resources 0 and 1.

7.7.14.1.4.2	Test procedure

Same as in clause 7.7.4.1.4.2 except Step 4 R1 scenario.

4.	The UE shall start sending valid L1-RSRP reports, a valid report shall meet the absolute L1-RSRP requirement for SSB#0 and SSB#1 and the relative L1-RSRP requirement for SSB1. The SS shall check for the following requirements:

- R1: The L1-RSRP values of SSB#0 and SSB#1 are reported by the UE is compared to the expected L1-RSRP values for SSB#0 and SSB#1. If the value is outside the limits in Table 7.7.14.1.5-3 (depending on the test configuration) or the UE fails to report the measurement value for SSB#0 and SSB#1, the number of failed iterations for R1 is increased by one. Otherwise, the number of passed iterations for R1 is increased by one.

7.7.14.1.4.3	Message contents

Message contents are same as in clause 7.7.4.1.4.3 with the following exceptions:

Table 7.7.14.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.6-2 with conditions PERIODIC, SS-RSRP and groupBasedBeamReporting as enabled Table H.3.6-3 with conditions SSB and PERIODIC Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

7.7.14.1.5	Test requirements

Table 7.7.14.1.5-1 and 7.7.14.1.5-2 define the primary level settings including the test tolerances for all tests.

Same as in clause 7.7.4.1.5 with below exceptions:

After 320ms from the beginning of the test, the L1-RSRP measurement accuracy for SSB#0 and SSB#1 of Cell 1 shall fulfil the requirements as defined in clauses 7.7.4.0.1, 7.7.4.0.2 TS 38.133 [6] 10.1.20.1. The following requirements are to be verified:

For Test 1:

Absolute accuracy of SSB0 and SSB1. The UE is deemed to meet the requirement if the reported L1-RSRP is in the range shown in Table 7.7.4.1.3-1.

Relative accuracy of SSB0 compared with SSB1. The UE is deemed to meet the requirement if the difference in reported L1-RSRP meets the requirements in TS 38.133 [6] Table 10.1.20.1.2-1.

For Test 2:

Absolute accuracy of SSB resource reported by UE in L1-RSRP report (SSB0 or SSB1). The UE is deemed to meet the requirement if the reported L1-RSRP is in the range shown in Table 7.7.4.1.3-1.

Relative accuracy of SSB0 compared with SSB1. The UE is deemed to meet the requirement if the difference in reported L1-RSRP meets the requirements in TS 38.133 [6] Table 10.1.20.1.2-1.

Table 7.7.14.1.5-1: FR2 SSB based L1-RSRP general test parameters

| Parameter | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| SSB ARFCN | 1, 2 |  | freq1 | freq1 |
| Duplex mode | 1, 2 |  | TDD | TDD |
| TDD Configuration | 1, 2 |  | TDDConf.3.1 | TDDConf.3.1 |
| BWchannel | 1, 2 | MHz | 100: NPRB,c = 66 | 100: NPRB,c = 66 |
| Data PRBs allocated | 1, 2 |  | 66 | 66 |
| PDSCH Reference measurement channel | 1 |  | SR.3.2 TDD | SR.3.2 TDD |
|  | 2 |  | SR.3.3 TDD | SR.3.3 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD | CR.3.1 TDD |
|  | 2 |  | CR.3.2 TDD | CR.3.2 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD | CCR.3.1 TDD |
|  | 2 |  | CCR.3.7 TDD | CCR.3.7 TDD |
| SSB configuration | 1 |  | SSB.1 FR2 | SSB.1 FR2 |
|  | 2 |  | SSB.2 FR2 | SSB.2 FR2 |
| OCNG Patterns | 1, 2 |  | OP.1 | OP.1 |
| Initial BWP Configuration | 1, 2 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1, 2 |  | DLBWP.1.3 ULBWP.1.3 | DLBWP.1.3 ULBWP.1.3 |
| TRS Configuration | 1, 2 |  | TRS.2.1 TDD | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration | 1, 2 |  | TCI.State.2 | TCI.State.2 |
| SMTC configuration | 1, 2 |  | SMTC.1 | SMTC.1 |
| reportConfigType | 1, 2 |  | periodic | periodic |
| reportQuantity | 1, 2 |  | ssb-Index-RSRP | ssb-Index-RSRP |
| Number of reported RS | 1, 2 |  | 2 | 2 |
| L1-RSRP reporting period | 1, 2 |  | slot320 | slot320 |
| Propagation condition | 1, 2 |  | AWGN | AWGN |
| Antenna configuration | 1, 2 |  | 1x2 | 1x2 |
| EPRE ratio of PSS to SSS | 1, 2 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |

Table 7.7.14.1.5-2: FR2 SSB based L1-RSRP OTA related test parameters

| Parameter | Config | Unit | Test 1 |  | Test 2 NOTE 3 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SSB0 | SSB1 | SSB0 | SSB1 |
| Angle of arrival configuration |  |  | Setup 5 according to A.9.5 |  | Setup 5 according to A.9.5 |  |
| Assumption for UE beamsNote 4 |  |  | Rough |  | Rough |  |
|  | 1~2 | dBm/15kHz | -100+TT |  | n.a. |  |
|  | 1 | dBm/SSB SCS | -91+TT |  | n.a. |  |
|  | 2 |  | -88+TT |  | n.a. |  |
|  | 1~2 | dB | 10+TT | -2+TT | n.a. |  |
| SSB_RPNote1 | 1 | dBm/SCS | -81+TT | -93+TT | As in Table B.2.4-2 |  |
|  | 2 |  | -78+TT | -90+TT | As in Table B.2.4-2 |  |
| IoNote1 | 1~2 | dBm/ 95.04MHz | -51.57+TT |  | SSB_RP+28.98 |  |
|  | 1~2 | dB | 10+TT | -2+TT | n.a. |  |
| Note 1: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: Void Note 3: No additional noise is added by the test system in Test 2. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |

Table 7.7.14.1.5-3: L1-RSRP absolute accuracy requirements for the reported values for test configurations 1 and 2 for the absolute accuracy rules R1

| UE power class 3 |  |  |
| --- | --- | --- |
| Normal Conditions | Test 1 Config1 All bands | Test 1 Config2 All bands |
| Lowest reported value (SSB#0) | 59+TT | 62+TT |
| Highest reported value (SSB#0) | 102+TT | 105+TT |
| Lowest reported value (SSB#1) | 47+TT | 50+TT |
| Highest reported value (SSB#1) | 90+TT | 93+TT |
| Extreme Conditions | Test 1 Config1 All bands | Test 1 Config1 All bands |
| Lowest reported value (SSB#0) | 56+TT | 59+TT |
| Highest reported value (SSB#0) | 105+TT | 108+TT |
| Lowest reported value (SSB#1) | 44+TT | 47+TT |
| Highest reported value (SSB#1) | 93+TT | 96+TT |

For the test to pass, the ratio of successful reported values for each requirement (R1 to R4) shall be more than 90% with a confidence level of 95%. Each requirement is evaluated independently of the others.

-------- End of 1st change --------

-------- Start of 2nd change --------

###### 7.7.14.2	NR SA FR2 CSI-RS based L1-RSRP measurement on resource set with repetition off

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD:

- TT is missing

- Configuration of 3AoA setup and testability of the device is still FFS

7.7.14.2.1	Test Purpose

The purpose of this test is to verify that the L1-RSRP measurement accuracy for group-based beam reporting is within the specified limits. This test will verify the requirements as defined in clauses 9.5.3 and 10.1.20.2 of TS 38.133 [6] for L1-RSRP measurements based on CSI-RS.

7.7.14.2.2	Test Applicability

This test applies to all types of NR UE from Release 17 onwards supporting groupBasedBeamReporting-r17.

7.7.14.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.7.4.0.3 and 7.7.4.0.4.

The normative reference for this requirement is TS 38.133 [6] clauses 9.5.3, 10.1.20.2 and A.7.7.14.2.

7.7.14.2.4	Test description

7.7.14.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.7.14.2.4.1-1.

Table 7.7.14.2.4.1-1: Supported Test Configurations

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz CSI-RS SCS, 100 MHz bandwidth, TDD duplex mode |

Same as in clause 7.7.4.2.4.1 with the following exceptions:

1.	The general test parameter settings are set up according to Table 7.7.4.2.4.1-2.

2.	Message contents are defined in clause 7.7.4.2.4.3.

3.	The test scenario comprises of 1 NR FR2 Cell. There is no measurement gap configured in the test. Before the test, UE is configured two CSI resource sets with one CSI-RS resource in each set.

4.	UE is configured with groupBasedBeamReporting-r17 for CSI-RS0 and CSI-RS1.

5.	UE is configured to perform RLM and BFD based on SSB 0 and 1 and UE is configured to perform group-based L1-RSRP measurement based on the CSI resources.

6.	CSI-RS resources are not transmitted in the same OFDM symbols as SSB. The connection setup is done according to the settings in Annex C.1.1.

7.	The rx beam peak and directions in which the UE meets the EIS spherical coverage criteria have been found with one of the procedures from Annex I.

7.7.14.2.4.2	Test procedure

Same as in clause 7.7.4.2.4.2 except Step 4 R1 scenario.

4.	The UE shall start sending valid L1-RSRP reports, a valid report shall meet the absolute L1-RSRP requirement for CSI-RS0 and the relative L1-RSRP requirement for CSI-RS1. The SS shall check for the following requirements:

- R1: The L1-RSRP values of CSI-RS0 and CSI-RS1 are reported by the UE is compared to the expected L1-RSRP values for CSI-RS0 and CSI-RS1. If the value is outside the limits in Table 7.7.14.2.5-3 (depending on the test configuration) or the UE fails to report the measurement value for CSI-RS0 and CSI-RS1, the number of failed iterations for R1 is increased by one. Otherwise, the number of passed iterations for R1 is increased by one.

7.7.14.2.4.3	Message contents

Same as in clause 7.7.4.2.4.3 with the following exceptions:

Table 7.7.14.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.6-2 with conditions PERIODIC, CSI-RSRP and groupBasedBeamReporting as enabled Table H.3.6-3 with conditions CSI-RS and PERIODIC Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

7.7.14.2.5	Test requirements

Table 7.7.14.2.5-1 and 7.7.14.2.5-2 define the primary level settings including the test tolerances for all tests.

Same as in clause 7.7.4.2.5 with the following exceptions:

After 640ms from the beginning of the test, the L1-RSRP measurement accuracy for CSI-RS#0 and CSI-RS#1 of Cell 1 shall fulfil the requirements as defined in clauses 7.7.4.0.3, 7.7.4.0.4 and in TS 38.133[6] clause 10.1.20.2. The following requirements are to be verified:

For Test 1:

Absolute accuracy of CSI-RS0 and CSI-RS1. The UE is deemed to meet the requirement if the reported L1-RSRP is in the range shown in TS 38.133[6] clause A.7.7.4.2.3 Table A.7.7.4.2.3-1.

Relative accuracy of CSI-RS0 compared with CSI-RS1. The UE is deemed to meet the requirement if the difference in reported L1-RSRP meets the requirements defined in TS 38.133 [6] Table 10.1.20.2.2-1.

. For Test 2:

Absolute accuracy of CSI-RS resource reported by UE in L1-RSRP report (CSI-RS0 or CSI-RS1). The UE is deemed to meet the requirement if the reported L1-RSRP is in the range shown in TS 38.133[6] clause A.7.7.4.2.3 Table A.7.7.4.2.3-1.

Relative accuracy of CSI-RS0 compared with CSI-RS1. The UE is deemed to meet the requirement if the difference in reported L1-RSRP meets the requirements defined in TS 38.133 [6] in Table 10.1.20.2.2-1.

Table 7.7.14.2.5-1: FR2 CSI-RS based L1-RSRP general test parameters

| Parameter | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| SSB ARFCN | 1 |  | freq1 | freq1 |
| Duplex mode | 1 |  | TDD | TDD |
| TDD Configuration | 1 |  | TDDConf.3.1 | TDDConf.3.1 |
| BWchannel | 1 | MHz | 100: NPRB,c = 66 | 100: NPRB,c = 66 |
| PDSCH Reference measurement channel | 1 |  | SR.3.1 TDD | SR.3.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD | CCR.3.1 TDD |
| SSB configuration | 1 |  | SSB.1 FR2 | SSB.1 FR2 |
| OCNG Patterns | 1 |  | OP.1 | OP.1 |
| Initial BWP Configuration | 1 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| TRS Configuration | 1 |  | TRS.2.1 TDD | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration | 1 |  | TCI.State.2 | TCI.State.2 |
| SMTC configuration | 1 |  | SMTC.1 | SMTC.1 |
| CSI-RS resource sets | 1 |  | TBD | TBD |
| reportConfigType | 1 |  | periodic | periodic |
| reportQuantity | 1 |  | cri-RSRP | cri-RSRP |
| Number of reported RS | 1 |  | 2 | 2 |
| L1-RSRP reporting period | 1 |  | slot80 | slot80 |
| Propagation condition | 1 |  | AWGN | AWGN |
| Antenna configuration | 1 |  | 1x2 | 1x2 |
| EPRE ratio of PSS to SSS | 1 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |

Table 7.7.14.2.5-2: FR2 CSI-RS based L1-RSRP OTA related test parameters

| Parameter | Config | Unit | Test 1 |  | Test 2 NOTE 3 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | CSI-RS0 | CSI-RS1 | CSI-RS0 | CSI-RS1 |
| Angle of arrival configuration |  |  | Setup 5 according to A.9.5 |  | Setup 5 according to A.9.5 |  |
| Assumption for UE beamsNote 4 |  |  | Rough |  | Rough |  |
|  | 1~2 | dBm/15kHz | -100+TT |  | n.a. |  |
|  | 1~2 | dBm/SSB SCS | -91+TT |  | n.a. n.a. |  |
|  | 1~2 | dB | 10+TT | -2+TT | n.a. |  |
| CSI-RS-RSRPNote1 | 1~2 | dBm/SCS | -81+TT | -93+TT | As in Table B.2.4-2 |  |
| IoNote1 | 1~2 | dBm/ 95.04MHz | -59.86+TT |  | SS-RSRP+28.98 |  |
|  | 1~2 | dB | -51.57+TT | -2+TT | n.a. |  |
| Note 1: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 3: No additional noise is added by the test system in Test 2. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |

Table 7.7.14.2.5-3: L1-RSRP absolute accuracy requirements for the reported values for the absolute accuracy rules R1 for test configuration 1

| Normal Conditions | Test 1 All bands |
| --- | --- |
| Lowest reported value (CSI-RS0) | 59+TT |
| Highest reported value (CSI-RS0) | 102+TT |
| Lowest reported value (CSI-RS1) | 90+TT |
| Highest reported value (CSI-RS1) | 47+TT |
| Extreme Conditions | Test 1 All bands |
| Lowest reported value (CSI-RS0) | 105+TT |
| Highest reported value (CSI-RS0) | 56+TT |
| Lowest reported value (CSI-RS1) | 93+TT |
| Highest reported value (CSI-RS1) | 44+TT |

For the test to pass, the ratio of successful reported values for each requirement (R1 to R4) shall be more than 90% with a confidence level of 95%. Each requirement is evaluated independently of the others.

-------- End of 2nd change --------