3GPP TSG-RAN5 Meeting #107	R5-252708r1

# Malta, Malta, May.19th – 23rd 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0824 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of PICS for MG_enh RRM test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - To add PICS for following UE capabilities used in MG_enh2 RRM test cases: - interRAT-NeedForGapsNR-r16 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - PICS for the UE capabilites mentioned above are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: To correct editorial issue caused by Word auto correction. - ”Pc_...” to “pc_...” |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.4.3.6	Measurement Capabilities

Table A.4.3.6-1: UE Measurement Capabilities

| Item | UE Measurement Capabilities | Ref. | Release | Mnemonic | M | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support NR measurements and events A triggered reporting | 38.306, 4.2.9 | Rel-15 | pc_eventA_MeasAndReport | Yes |  |  |
| 2 | Support two independent measurement gap configurations for FR1 and FR2 | 38.306, 4.2.9 | Rel-15 | pc_independentGapConfig | No |  |  |
| <Unchanged contents are omitted> |  |  |  |  |  |  |  |
| 105 | Support reporting the measurement gap requirement information for NR target in the UE response to a network configuration RRC message. | 38.306, 4.2.9 | Rel-16 | pc_nr_NeedForGap_Reporting_r16 | No |  |  |
| 105A | Support performing SSB based inter-RAT measurements on each supported NR band without measurement gaps | 36.306, 4.3.6.38 | Rel-16 | pc_interRAT_NeedForGapsNR_r16 | No |  |  |
| 106 | Support reporting the interruption requirement information for SSB based measurement towards NR target without gap in the UE response to a network configuration RRC message | 38.306, 4.2.9 | Rel-18 | pc_nr_NeedForInterruptionReport_r18 | No |  | A UE supporting this feature shall also indicate support of nr-NeedForGap-Reporting-r16 (pc_nr_NeedForGap_Reporting_r16). |
| 107 | Support concurrent inter-RAT measurement on EUTRAN cell in non-DSS and PDCCH or PDSCH reception from the serving cell with a different numerology | 38.306, 4.2.9 | Rel-18 | pc_concurrentMeasCRS_InsideBWP_EUTRA_r18 | No |  | A UE supporting this feature shall also indicate support of eutra-NoGapMeasurementInsideBWP-r18 (pc_eutra_NoGapMeasurementInsideBWP_r18) or eutra-NoGapMeasurementOutsideBWP-r18 (pc_eutra_NoGapMeasurementOutsideBWP_r18). |
| <Unchanged contents are omitted> |  |  |  |  |  |  |  |
| 118 | Support location based RRM measurements of neighbour cells in NTN quasi-Earth fixed cell in RRC_IDLE/RRC_INACTIVE | 38.306, 5.6 | Rel-18 | pc_meas_ntn_quasi_earth_fixed_cell | No |  | If a UE supports this feature, then it supports r18 NTN Enhanced |
| 119 | Support location based RRM measurements of neighbour cells in NTN Earth-moving cell in RRC_IDLE/RRC_INACTIVE | 38.306, 5.6 | Rel-18 | pc_meas_ntn_earth_moving_cell | No |  | If a UE supports this feature, then it supports r18 NTN Enhanced |

<End of Change 1>