3GPP TSG-RAN5 Meeting #107	R5-252792r1

# St Julian’s, Malta, May 19-23, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3385 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test cases MPR, SEM and NR ACLR for 4Tx TxD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_FR1_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The new test cases MPR, SEM and NR ACLR for 4Tx TxD are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the new test cases MPR, SEM and NR ACLR for 4Tx TxD. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MPR, SEM and NR ACLR for 4Tx TxD can’t be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2G.2, 6.2G.2_1(new), 6.5G.2.1, 6.5G.2.1_1(new), 6.5G.2.3.1, 6.5G.2.3_1.1(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TR 38.905 CR 1020 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | The 1st revision: - Correct the wrong table number. |  |  |  |  |  |  |  |  |

<Start of Changes>

6.2G.2	UE maximum output power reduction for Tx Diversity

6.2G.2.1	Test purpose

The number of RB identified in Table 6.2D.2.3-1 and Table 6.2D.2.3-2 is based on meeting the requirements for adjacent channel leakage ratio and the maximum power reduction (MPR) due to Cubic Metric (CM).

6.2G.2.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

NOTE:	Test execution is not necessary if TS 38.521-1 6.5G.2.3.1 is executed.

6.2G.2.3	Minimum conformance requirements

For UE supporting Tx diversity, the allowed MPR for the maximum output power is specified in Table 6.2.2.3-1 for UE power class 3, in Table 6.2D.2.3-1 for UE power class, Table 6.2D.2.3-2 and Table 6.2D.2.3-3 for UE power class 1.5 with 2Tx, in Table 6.2D.2.3-4 and 6.2D.2.3-5 for UE power class 1.5 with 4 Tx. For UE power class 1.5, the allowed maximum power reduction (MPR) defined in Table 6.2D.2.3-3 is in accordance with the indicated modifiedMPR-Behaviour specified in Table L.1-1 for channel bandwidths ≤ 100 MHz. The maximum output power is defined as the sum of the maximum output power at each UE antenna connector. If a UE that supports PC1.5 has to apply the requirements of PC2 according to the rules in clause 6.2.1, the MPR requirements in Table 6.2.2.3-2 apply.

For the UE maximum output power modified by MPR, the power limits specified in subclause 6.2G.4 apply.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2G.2.

6.2G.2.4	Test description

6.2G.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.2G.2.4.1-1, Table 6.2G.2.4.1-2, and Table 6.2G.2.4.1-3. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2G.2.4.1-1: Test Configuration Table for power class 2 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Default | Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 16 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 19 | Default |  | CP-OFDM QPSK | Inner Full |
| 20 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 21 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 22 | Default |  | CP-OFDM QPSK | Outer Full |
| 23 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 24 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 25 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 27 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 28 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 29 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 30 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 31 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 32 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. |  |  |  |  |

Table 6.2G.2.4.1-2: Test Configuration Table for power class 1.5 UE and power class 1.5 FWA (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Default | Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Default |  | DFT-s-OFDM_64_QAM | Inner Full |
| 14 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 15 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 16 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 17 | Default |  | DFT-s-OFDM_256_QAM | Inner Full |
| 18 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 19 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 20 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 21 | Default |  | CP-OFDM QPSK | Inner Full |
| 22 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 23 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 24 | Default |  | CP-OFDM QPSK | Outer Full |
| 25 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 26 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 27 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 28 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 29 | Default |  | CP-OFDM 64 QAM | Inner Full |
| 30 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 31 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 32 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 33 | Default |  | CP-OFDM 256 QAM | Inner Full |
| 34 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 35 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 36 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. |  |  |  |  |

Table 6.2G.2.4.1-2a: Test Configuration Table for power class 3 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 13 | Default | Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 23 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 33 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 43 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 54 | Default |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 64 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 74 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 84 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 10 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 13 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 14 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 15 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 16 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 17 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 18 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 19 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 20 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 21 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 22 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 23 | Default |  | CP-OFDM QPSK | Inner Full |
| 24 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 25 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM QPSK | Outer Full |
| 27 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 28 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 29 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 30 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 31 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 32 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 33 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 34 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 35 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 36 | Default |  | CP-OFDM 256 QAM | Outer Full |
| 374,5 | Low |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Left |
| 384,5 | High |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Right |
| 394,5 | Default |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 3: UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for bands n40, n41, n77, n78 and n79. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n40, n41, n77, n78 and n79. NOTE 5: Applicable to UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16. |  |  |  |  |

Table 6.2G.2.4.1-3: Test Configuration Table for power class 2 and 3 (almost contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation | RB allocation (NOTE 1) |
| 1 | Default |  | CP-OFDM QPSK | Inner Full |
| 2 | Default |  | CP-OFDM QPSK | Outer Full |
| 3 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 4 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 5 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 6 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.2.2.4.1-4, 6.2.2.4.1-4a and 6.2.2.4.1-4b. NOTE 2: Test applies only for UEs which support almost contiguous UL CP-OFDM transmissions. For PC2 UE which support almost contiguous UL CP-OFDM transmissions, test is only applicable for Release 16 and forward. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.2.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2G.2.4.1-1, Table 6.2G.2.4.1-2, Table 6.2G.2.4.1-2a and Table 6.2G.2.4.1-3.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2G.2.4.3.

6.2G.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2G.2.4.1-1, Table 6.2G.2.4.1-2, Table 6.2G.2.4.1-2a and Table 6.2G.2.4.1-3. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE at each antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots. For TDD, only slots consisting of only UL symbols are under test.

4.	For UEs supporting Power Class 1.5 and Power Class 2, repeat steps 1~3 for Test ID 16 and 22 in Table 6.2G.2.4.1-2a on the applicable bands with message exception of P-Max defined in Table 6.2G.2.4.3-2.

5.	For UEs supporting Power Class 1.5, repeat steps 1~3 for all Test IDs in Table 6.2G.2.4.1-1 on the applicable bands with message exception of P-Max defined in Table 6.2G.2.4.3-3.

NOTE 1: When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.2G.2.4.1-1 and Table 6.2G.2.4.1-2, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2G.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 with the following exceptions:

Table 6.2G.2.4.3-1: PUSCH-Config

| Derivation Path: TS 38.508-1 [5] subclause 4.6.3 Table 4.6.3-118 PUSCH-Config |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType0 |  | Almost contiguous allocation |
|  | resourceAllocationType1 |  | Contiguous allocation |
| } |  |  |  |

Table 6.2G.2.4.3-2: P-Max (Step 4)

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | 23 |  | PC2 UE or PC1.5 UE |

Table 6.2G.2.4.3-3: P-Max (Step 5)

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | 26 |  | PC1.5 UE |

6.2G.2.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2G.2.5-1 to Table 6.2G.2.5-12.

The maximum output power, derived in step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2G.2.5-5 to Table 6.2G.2.5-12.

The maximum output power, derived in step 5 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2G.2.5-2b.

Table 6.2G.2.5-1: UE Power Class test requirements (for Bands n34, n39, n40, n41, n77, n78, n79, n98) for Power Class 2 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 0 | 0 | （1.52） | 26.0 | （24.52） | 3.0 |  | 3 | 28.0 + TT | 23.0 - TT | （21.5 - TT2） |
| 2 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 3 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0- TT2） |
| 4 | 26 | 0 | 1 | 0 | （1.52） | 25.0 | （23.52） | 3.0 |  | 3 | 28.0 + TT | 22.0 - TT | （20.5- TT2） |
| 5 | 26 | 0 | 0.5 | 0 | （1.52） | 25.5 | （24.02） | 3.0 |  | 3 | 28.0 + TT | 22.5 - TT | （21.0 - TT2） |
| 6 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 7 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 8 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.5 - TT2） |
| 9 | 26 | 0 | 1.5 | 0 | （1.52） | 24.5 | （23.02） | 3.0 |  | 3 | 28.0 + TT | 21.5 - TT | （20.0 - TT2） |
| 10 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 11 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 12 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （22.02） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.5 - TT | （17.0 - TT2） |
| 13 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 14 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0- TT2） |
| 15 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | （16.5 - TT2） |
| 16 | 26 | 0 | 5.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | （5.02） | 3 | 28.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 17 | 26 | 0 | 5.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | （5.02） | 3 | 28.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 18 | 26 | 0 | 5.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | （5.02） | 3 | 28.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 19 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.5 - TT2） |
| 20 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 21 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5- TT2） |
| 22 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 23 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （22.02） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.5 - TT | （17.0 - TT2） |
| 24 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5 - TT2） |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 26 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 27 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 28 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 29 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 30 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.5 - TT2） |
| 31 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.5 - TT2） |
| 32 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.5 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2G.2.5-4. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2G.2.5-2: UE Power Class test requirements (for Band n34, n39, n40, n41, n77, n78, n79) for Power Class 1.5 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 0.5 | 0 | （1.52） | 28.5 | (27.0²) | 3.0 |  | 3 | 31.0 + TT | 25.5 - TT | (24.0 - TT²) |
| 2 | 29 | 0 | 6 | 0 | （1.52） | 23.0 | (21.5²) | 3.0 | （5.02） | 3 | 31.0 + TT | 20.0 - TT | (16.5 - TT²) |
| 3 | 29 | 0 | 6 | 0 | （1.52） | 23.0 | (21.5²) | 3.0 | （5.02） | 3 | 31.0 + TT | 20.0 - TT | (16.5 - TT²) |
| 4 | 29 | 0 | 2 | 0 | （1.52） | 27.0 | (25.5²) | 3.0 |  | 3 | 31.0 + TT | 24.0 - TT | (22.5 - TT²) |
| 5 | 29 | 0 | 0.5 | 0 | （1.52） | 28.5 | (27.0²) | 3.0 |  | 3 | 31.0 + TT | 25.5 - TT | (24.0 - TT²) |
| 6 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 7 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 8 | 29 | 0 | 2.5 | 0 | （1.52） | 26.5 | (25.0²) | 3.0 |  | 3 | 31.0 + TT | 23.5 - TT | (22.0 - TT²) |
| 9 | 29 | 0 | 1.5 | 0 | （1.52） | 27.5 | (26.0²) | 3.0 |  | 3 | 31.0 + TT | 24.5 - TT | (23.0 - TT²) |
| 10 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 11 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 12 | 29 | 0 | 3.5 | 0 | （1.52） | 25.5 | (24.0²) | 3.0 |  | 3 | 31.0 + TT | 22.5 - TT | (21.0 - TT²) |
| 13 | 29 | 0 | 3.5 | 0 | （1.52） | 25.5 | (24.0²) | 3.0 |  | 3 | 31.0 + TT | 22.5 - TT | (21.0 - TT²) |
| 14 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 15 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 16 | 29 | 0 | 4 | 0 | （1.52） | 25.0 | (23.5²) | 3.0 |  | 3 | 31.0 + TT | 22.0 - TT | (20.5 - TT²) |
| 17 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 18 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 19 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 20 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 21 | 29 | 0 | 2 | 0 | （1.52） | 27.0 | (25.5²) | 3.0 |  | 3 | 31.0 + TT | 24.0 - TT | (22.5 - TT²) |
| 22 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 23 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 24 | 29 | 0 | 4.5 | 0 | （1.52） | 24.5 | (23.0²) | 3.0 |  | 3 | 31.0 + TT | 21.5 - TT | (20.0 - TT²) |
| 25 | 29 | 0 | 2.5 | 0 | （1.52） | 26.5 | (25.0²) | 3.0 |  | 3 | 31.0 + TT | 23.5 - TT | (22.0 - TT²) |
| 26 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 27 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 28 | 29 | 0 | 4.5 | 0 | （1.52） | 24.5 | (23.0²) | 3.0 |  | 3 | 31.0 + TT | 21.5 - TT | (20.0 - TT²) |
| 29 | 29 | 0 | 4.5 | 0 | （1.52） | 24.5 | (23.0²) | 3.0 |  | 3 | 31.0 + TT | 21.5 - TT | (20.0 - TT²) |
| 30 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 31 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 32 | 29 | 0 | 5 | 0 | （1.52） | 24.0 | (22.5²) | 3.0 | （5.02） | 3 | 31.0 + TT | 21.0 - TT | (17.5 - TT²) |
| 33 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | (19.0²) | 6.0 | （5.02） | 3 | 31.0 + TT | 14.5 - TT | (14.0 - TT²) |
| 34 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | (19.0²) | 6.0 | （5.02） | 3 | 31.0 + TT | 14.5 - TT | (14.0 - TT²) |
| 35 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | (19.0²) | 6.0 | （5.02） | 3 | 31.0 + TT | 14.5 - TT | (14.0 - TT²) |
| 36 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | (19.0²) | 6.0 | （5.02） | 3 | 31.0 + TT | 14.5 - TT | (14.0 - TT²) |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2G.2.5-4. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2G.2.5-2a: UE Power Class test requirements (for Band n34, n39, n40, n41, n77, n78, n79) for Power Class 1.5 FWA (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 0 | 0 | （1.53） | 29.0 | (27.53) | 3.0 |  | 3 | 31.0 + TT | 26.0 - TT | (24.5 – TT3) |
| 2 | 29 | 0 | 6 | 0 | （1.53） | 23.0 | (21.53) | 3.0 | (5.03) | 3 | 31.0 + TT | 20.0 - TT | (16.5 – TT3) |
| 3 | 29 | 0 | 6 | 0 | （1.53） | 23.0 | (21.53) | 3.0 | (5.03) | 3 | 31.0 + TT | 20.0 - TT | (16.5 – TT3) |
| 4 | 29 | 0 | 1.5 | 0 | （1.53） | 27.5 | (26.03) | 3.0 |  | 3 | 31.0 + TT | 24.5 - TT | (23.0 – TT3) |
| 5 | 29 | 0 | 0 | 0 | （1.53） | 29.0 | (27.53) | 3.0 |  | 3 | 31.0 + TT | 26.0 - TT | (24.5 – TT3) |
| 6 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 7 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 8 | 29 | 0 | 2 | 0 | （1.53） | 27.0 | (25.53) | 3.0 |  | 3 | 31.0 + TT | 24.0 - TT | (22.5 – TT3) |
| 9 | 29 | 0 | 1 | 0 | （1.53） | 28.0 | (26.53) | 3.0 |  | 3 | 31.0 + TT | 25.0 - TT | (23.5 – TT3) |
| 10 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 11 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 12 | 29 | 0 | 3.5 | 0 | （1.53） | 25.5 | (24.03) | 3.0 |  | 3 | 31.0 + TT | 22.5 - TT | (21.0 – TT3) |
| 13 | 29 | 0 | 3 | 0 | （1.53） | 26.0 | (24.53) | 3.0 |  | 3 | 31.0 + TT | 23.0 - TT | (21.5 – TT3) |
| 14 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 15 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 16 | 29 | 0 | 3.5 | 0 | （1.53） | 25.5 | (24.03) | 3.0 |  | 3 | 31.0 + TT | 22.5 - TT | (21.0 – TT3) |
| 17 | 29 | 0 | 5.5 | 0 | （1.53） | 23.5 | (22.03) | 3.0 |  | 3 | 31.0 + TT | 20.5 - TT | (19.0 – TT3) |
| 18 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 19 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 20 | 29 | 0 | 5.5 | 0 | （1.53） | 23.5 | (22.03) | 3.0 | (5.03) | 3 | 31.0 + TT | 20.5 - TT | (17.0 – TT3) |
| 21 | 29 | 0 | 1.5 | 0 | （1.53） | 27.5 | (26.03) | 3.0 |  | 3 | 31.0 + TT | 24.5 - TT | (23.0 – TT3) |
| 22 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 23 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 24 | 29 | 0 | 4 | 0 | （1.53） | 25.0 | (23.53) | 3.0 |  | 3 | 31.0 + TT | 22.0 - TT | (20.5 – TT3) |
| 25 | 29 | 0 | 2 | 0 | （1.53） | 27.0 | (25.53) | 3.0 |  | 3 | 31.0 + TT | 24.0 - TT | (22.5 – TT3) |
| 26 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 27 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 28 | 29 | 0 | 4 | 0 | （1.53） | 25.0 | (23.53) | 3.0 |  | 3 | 31.0 + TT | 22.0 - TT | (20.5 – TT3) |
| 29 | 29 | 0 | 4 | 0 | （1.53） | 25.0 | (23.53) | 3.0 |  | 3 | 31.0 + TT | 22.0 - TT | (20.5 – TT3) |
| 30 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 31 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 32 | 29 | 0 | 4.5 | 0 | （1.53） | 24.5 | (23.03) | 3.0 |  | 3 | 31.0 + TT | 21.5 - TT | (20.0 – TT3) |
| 33 | 29 | 0 | 7.5 | 0 | （1.53） | 21.5 | (20.03) | 5.0 | （6.03） | 3 | 31.0 + TT | 16.5 - TT | (14.0 – TT3) |
| 34 | 29 | 0 | 7.5 | 0 | （1.53） | 21.5 | (20.03) | 5.0 | （6.03） | 3 | 31.0 + TT | 16.5 - TT | (14.0 – TT3) |
| 35 | 29 | 0 | 7.5 | 0 | （1.53） | 21.5 | (20.03) | 5.0 | （6.03） | 3 | 31.0 + TT | 16.5 - TT | (14.0 – TT3) |
| 36 | 29 | 0 | 7.5 | 0 | （1.53） | 21.5 | (20.03) | 5.0 | （6.03） | 3 | 31.0 + TT | 16.5 - TT | (14.0 – TT3) |
| NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 3: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2G.2.5-4. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2G.2.5-2b: UE Power Class test requirements (for Band n34, n39, n40 n41, n77, n78, n79) for Power Class 1.5 falling back to Power Class 2 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 3 | 0 | 0 | （1.52） | 26.0 | (24.5²) | 3.0 |  | 3 | 28.0 + TT | 23.0 - TT | (21.5 - TT²) |
| 2 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 3 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 4 | 29 | 3 | 0.5 | 0 | （1.52） | 25.5 | (24.0²) | 3.0 |  | 3 | 28.0 + TT | 22.5 - TT | (21.0 - TT²) |
| 5 | 29 | 3 | 0 | 0 | （1.52） | 26.0 | (24.5²) | 3.0 |  | 3 | 28.0 + TT | 23.0 - TT | (21.5 - TT²) |
| 6 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 7 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 8 | 29 | 3 | 1 | 0 | （1.52） | 25.0 | (23.5²) | 3.0 |  | 3 | 28.0 + TT | 22.0 - TT | (20.5 - TT²) |
| 9 | 29 | 3 | 1 | 0 | （1.52） | 25.0 | (23.5²) | 3.0 |  | 3 | 28.0 + TT | 22.0 - TT | (20.5 - TT²) |
| 10 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 11 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 12 | 29 | 3 | 2 | 0 | （1.52） | 24.0 | (22.5²) | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | (17.5 - TT²) |
| 13 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 14 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 15 | 29 | 3 | 2.5 | 0 | （1.52） | 23.5 | (22.0²) | 3.0 | （5.02） | 3 | 28.0 + TT | 20.5 - TT | (17.0 - TT²) |
| 16 | 29 | 3 | 4.5 | 0 | （1.52） | 21.5 | (20.0²) | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | (14.0 - TT²) |
| 17 | 29 | 3 | 4.5 | 0 | （1.52） | 21.5 | (20.0²) | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | (14.0 - TT²) |
| 18 | 29 | 3 | 4.5 | 0 | （1.52） | 21.5 | (20.0²) | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | (14.0 - TT²) |
| 19 | 29 | 3 | 1.5 | 0 | （1.52） | 24.5 | (23.0²) | 3.0 |  | 3 | 28.0 + TT | 21.5 - TT | (20.0 - TT²) |
| 20 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 21 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 22 | 29 | 3 | 3 | 0 | （1.52） | 23.0 | (21.5²) | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | (16.5 - TT²) |
| 23 | 29 | 3 | 2 | 0 | （1.52） | 24.0 | (22.5²) | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | (17.5 - TT²) |
| 24 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 25 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 26 | 29 | 3 | 3 | 0 | （1.52） | 23.0 | (21.5²) | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | (16.5 - TT²) |
| 27 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 28 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 29 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 30 | 29 | 3 | 6.5 | 0 | （1.52） | 19.5 | (18.0²) | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | (13.0 - TT²) |
| 31 | 29 | 3 | 6.5 | 0 | （1.52） | 19.5 | (18.0²) | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | (13.0 - TT²) |
| 32 | 29 | 3 | 6.5 | 0 | （1.52） | 19.5 | (18.0²) | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | (13.0 - TT²) |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2G.2.5-4. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2G.2.5-3: UE Power Class test requirements (for Bands n34, n39, n40, n41, n77, n78, n79, n98) for Power Class 2 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 26 | 0 | 2 | 1.5 | 0 | 1.52 | 22.5 | 21.02 | 5.0 | 5.02 | 3 | 28.0 + TT | 17.5 - TT | 16 - TT2 |
| 15 | 26 | 0 | 2 | 1 | 0 | 1.52 | 23.0 | 21.52 | 3.0 | 5.02 | 3 | 28.0 + TT | 20.0 - TT | 16.5 - TT2 |
| 26 | 26 | 0 | 3.5 | 1.5 | 0 | 1.52 | 21.0 | 19.52 | 5.0 | 5.02 | 3 | 28.0 + TT | 16.0 - TT | 14.5 - TT2 |
| 27 | 26 | 0 | 3.5 | 1 | 0 | 1.52 | 21.5 | 20.02 | 5.0 | 6.02 | 3 | 28.0 + TT | 16.5 - TT | 14.0 - TT2 |
| 34 | 26 | 0 | 2.5 | 1.5 | 0 | 1.52 | 22.0 | 20.52 | 5.0 | 6.02 | 3 | 28.0 + TT | 17.0 - TT | 14.5 - TT2 |
| 35 | 26 | 0 | 2.5 | 1 | 0 | 1.52 | 22.5 | 21.02 | 5.0 | 5.02 | 3 | 28.0 + TT | 17.5 - TT | 16.0 - TT2 |
| 46 | 26 | 0 | 3.5 | 1.5 | 0 | 1.52 | 21.0 | 19.52 | 5.0 | 5.02 | 3 | 28.0 + TT | 16.0 - TT | 14.5 - TT2 |
| 47 | 26 | 0 | 3.5 | 1 | 0 | 1.52 | 21.5 | 20.02 | 5.0 | 6.02 | 3 | 28.0 + TT | 16.5 - TT | 14.0 - TT2 |
| 56 | 26 | 0 | 4.5 | 1.5 | 0 | 1.52 | 20.0 | 18.52 | 6.0 | 5.02 | 3 | 28.0 + TT | 14.0 - TT | 13.5 - TT2 |
| 57 | 26 | 0 | 4.5 | 1 | 0 | 1.52 | 20.5 | 19.02 | 6.0 | 5.02 | 3 | 28.0 + TT | 14.5 - TT | 14.0 - TT2 |
| 66 | 26 | 0 | 8 | 1.5 | 0 | 1.52 | 16.5 | 152 | 5.0 | 5.02 | 3 | 28.0 + TT | 11.5 - TT | 10.0 - TT2 |
| 67 | 26 | 0 | 8 | 1 | 0 | 1.52 | 17.0 | 15.52 | 5.0 | 5.02 | 3 | 28.0 + TT | 12.0 - TT | 10.5 - TT2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2G.2.5-4. NOTE 4: Applicable for CBW/SCS combinations other than CBW=40MHz when SCS=60kHz. NOTE 5: Only applicable for CBW 40MHz when SCS is 60kHz. NOTE 6: Applicable for CBW/SCS combinations other than CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 7: Only applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 8: Test applies only for UEs which support almost contiguous UL CP-OFDM transmissions. For PC2 UE which support almost contiguous UL CP-OFDM transmissions, test is only applicable for Release 16 and forward. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2G.2.5-4: Test Tolerance (Maximum Power Reduction (MPR))

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

Table 6.2G.2.5-5: UE Power Class test requirements(for Bands n1, n2, n3, n5, n7, n8, n12, n13, n14, n20, n25, n26, n30, n34, n38, n39, n40, n41, n50, n51, n53, n54, n65, n66, n70, n74, n85, n98, n109) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | -3 | 0.2 | 0 | （1.52） | 25.8 | （24.32） | 3.0 |  | 2 | 28.0 + TT | 22.8 - TT | （21.3 - TT2） |
| 2 | 23 | -3 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 2 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 3 | 23 | -3 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 2 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 4 | 23 | -3 | 1.2 | 0 | （1.52） | 24.8 | （23.32） | 3.0 |  | 2 | 28.0 + TT | 21.8 - TT | （20.3 - TT2） |
| 5 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 2 | 25.0 + TT | 20.0 - TT | （16.5 - TT2） |
| 6 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 2 | 25.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 7 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 2 | 25.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 8 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 2 | 25.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 9 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 2 | 25.0 + TT | 20.0 - TT | （16.5 - TT2） |
| 10 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 2 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 11 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 2 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 12 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 2 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 13 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 2 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 14 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 2 | 25.0 + TT | 16.0 - TT | （14.5 - TT2） |
| 15 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 2 | 25.0 + TT | 16.0 - TT | （14.5 - TT2） |
| 16 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 2 | 25.0 + TT | 16.0 - TT | （14.5 - TT2） |
| 17 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | （5.02） | 2 | 25.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 18 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | （5.02） | 2 | 25.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 19 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | （5.02） | 2 | 25.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 20 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 5.0 |  | 2 | 25.0 + TT | 13.5 - TT | （12.0 - TT2） |
| 21 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 5.0 |  | 2 | 25.0 + TT | 13.5 - TT | （12.0 - TT2） |
| 22 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 5.0 |  | 2 | 25.0 + TT | 13.5 - TT | （12.0 - TT2） |
| 23 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 2 | 25.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 24 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 2 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 25 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 2 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 26 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 2 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 27 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 2 | 25.0 + TT | 16.0 - TT | （14.5 - TT2） |
| 28 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 2 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 29 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 2 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 30 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 2 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 31 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 2 | 25.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 32 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 2 | 25.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 33 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 2 | 25.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 34 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 2 | 25.0 + TT | 11.5 - TT | （9.0 - TT2） |
| 35 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 2 | 25.0 + TT | 11.5 - TT | （9.0 - TT2） |
| 36 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 2 | 25.0 + TT | 11.5 - TT | （9.0 - TT2） |
| 37 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 2 | 25.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 38 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 2 | 25.0 + TT | 17.5 - TT | (16.0 - TT2) |
| 39 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 2 | 25.0 + TT | 20.0 - TT | （16.5 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n2, n3, n7, n8, n12, n20, n25, n26, n41, n109 transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2G.2.5-4. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2G.2.5-6: UE Power Class test requirements (for Bands n24, n48, n77, n78, n79, n104) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | -3 | 0.2 | 0 | （1.52） | 25.8 | （24.32） | 3.0 |  | 3 | 28.0 + TT | 22.8 - TT | （21.3 - TT2） |
| 2 | 23 | -3 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 3 | 23 | -3 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 4 | 23 | -3 | 1.2 | 0 | （1.52） | 24.8 | （23.32） | 3.0 |  | 3 | 28.0 + TT | 21.8 - TT | （20.3 - TT2） |
| 5 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 3 | 25.0 + TT | 20.0 - TT | （16.5 - TT2） |
| 6 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 25.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 7 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 25.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 8 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 25.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 9 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 3 | 25.0 + TT | 20.0 - TT | （16.5 - TT2） |
| 10 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 11 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 12 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 13 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 14 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 | 25.0 + TT | 16.0 - TT | （14.5 - TT2） |
| 15 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 | 25.0 + TT | 16.0 - TT | （14.5 - TT2） |
| 16 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 | 25.0 + TT | 16.0 - TT | （14.5 - TT2） |
| 17 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 18 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 19 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 20 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 5.0 |  | 3 | 25.0 + TT | 13.5 - TT | （12.0 - TT2） |
| 21 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 5.0 |  | 3 | 25.0 + TT | 13.5 - TT | （12.0 - TT2） |
| 22 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 5.0 |  | 3 | 25.0 + TT | 13.5 - TT | （12.0 - TT2） |
| 23 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 3 | 25.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 24 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 25 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 26 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 27 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 | 25.0 + TT | 16.0 - TT | （14.5 - TT2） |
| 28 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 29 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 30 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 31 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 25.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 32 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 2 | 25.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 33 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 2 | 25.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 34 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 2 | 25.0 + TT | 11.5 - TT | （9.0 - TT2） |
| 35 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 2 | 25.0 + TT | 11.5 - TT | （9.0 - TT2） |
| 36 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 2 | 25.0 + TT | 11.5 - TT | （9.0 - TT2） |
| 37 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 2 | 25.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 38 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 2 | 25.0 + TT | 17.5 - TT | (16.0 - TT2) |
| 39 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 2 | 25.0 + TT | 20.0 - TT | （16.5 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n24, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2G.2.5-4. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2G.2.5-7: UE Power Class test requirements (for Bands n28 with channel bandwidth other than 30MHz, n71, n105) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 23 | 0 | 0 | 0 | 23.0 | 3.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 6 | 23 | 0 | 0.5 | 0 | 22.5 | 5.0 | 2.5 | 25.0 + TT | 17.5 - TT |
| 7 | 23 | 0 | 0.5 | 0 | 22.5 | 5.0 | 2.5 | 25.0 + TT | 17.5 - TT |
| 8 | 23 | 0 | 0.5 | 0 | 22.5 | 5.0 | 2.5 | 25.0 + TT | 17.5 - TT |
| 9 | 23 | 0 | 0 | 0 | 23.0 | 3.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 10 | 23 | 0 | 1 | 0 | 22.0 | 5.0 | 2.5 | 25.0 + TT | 17.0 - TT |
| 11 | 23 | 0 | 1 | 0 | 22.0 | 5.0 | 2.5 | 25.0 + TT | 17.0 - TT |
| 12 | 23 | 0 | 1 | 0 | 22.0 | 5.0 | 2.5 | 25.0 + TT | 17.0 - TT |
| 13 | 23 | 0 | 1 | 0 | 22.0 | 5.0 | 2.5 | 25.0 + TT | 17.0 - TT |
| 14 | 23 | 0 | 2 | 0 | 21.0 | 5.0 | 2.5 | 25.0 + TT | 16.0 - TT |
| 15 | 23 | 0 | 2 | 0 | 21.0 | 5.0 | 2.5 | 25.0 + TT | 16.0 - TT |
| 16 | 23 | 0 | 2 | 0 | 21.0 | 5.0 | 2.5 | 25.0 + TT | 16.0 - TT |
| 17 | 23 | 0 | 2.5 | 0 | 20.5 | 6.0 | 2.5 | 25.0 + TT | 14.5 - TT |
| 18 | 23 | 0 | 2.5 | 0 | 20.5 | 6.0 | 2.5 | 25.0 + TT | 14.5 - TT |
| 19 | 23 | 0 | 2.5 | 0 | 20.5 | 6.0 | 2.5 | 25.0 + TT | 14.5 - TT |
| 20 | 23 | 0 | 4.5 | 0 | 18.5 | 5.0 | 2.5 | 25.0 + TT | 13.5 - TT |
| 21 | 23 | 0 | 4.5 | 0 | 18.5 | 5.0 | 2.5 | 25.0 + TT | 13.5 - TT |
| 22 | 23 | 0 | 4.5 | 0 | 18.5 | 5.0 | 2.5 | 25.0 + TT | 13.5 - TT |
| 23 | 23 | 0 | 1.5 | 0 | 21.5 | 5.0 | 2.5 | 25.0 + TT | 16.5 - TT |
| 24 | 23 | 0 | 3 | 0 | 20.0 | 6.0 | 2.5 | 25.0 + TT | 14.0 - TT |
| 25 | 23 | 0 | 3 | 0 | 20.0 | 6.0 | 2.5 | 25.0 + TT | 14.0 - TT |
| 26 | 23 | 0 | 3 | 0 | 20.0 | 6.0 | 2.5 | 25.0 + TT | 14.0 - TT |
| 27 | 23 | 0 | 2 | 0 | 21.0 | 5.0 | 2.5 | 25.0 + TT | 16.0 - TT |
| 28 | 23 | 0 | 3 | 0 | 20.0 | 6.0 | 2.5 | 25.0 + TT | 14.0 - TT |
| 29 | 23 | 0 | 3 | 0 | 20.0 | 6.0 | 2.5 | 25.0 + TT | 14.0 - TT |
| 30 | 23 | 0 | 3 | 0 | 20.0 | 6.0 | 2.5 | 25.0 + TT | 14.0 - TT |
| 31 | 23 | 0 | 3.5 | 0 | 19.5 | 5.0 | 2.5 | 25.0 + TT | 14.5 - TT |
| 32 | 23 | 0 | 3.5 | 0 | 19.5 | 5.0 | 2.5 | 25.0 + TT | 14.5 - TT |
| 33 | 23 | 0 | 3.5 | 0 | 19.5 | 5.0 | 2.5 | 25.0 + TT | 14.5 - TT |
| 34 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2.5 | 25.0 + TT | 11.5 - TT |
| 35 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2.5 | 25.0 + TT | 11.5 - TT |
| 36 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2.5 | 25.0 + TT | 11.5 - TT |
| 37 | 23 | 0 | 0.5 | 0 | 22.5 | 5.0 | 2.5 | 25.0 + TT | 17.5 - TT |
| 38 | 23 | 0 | 0.5 | 0 | 22.5 | 5.0 | 2.5 | 25.0 + TT | 17.5 - TT |
| 39 | 23 | 0 | 0 | 0 | 23 | 3.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2G.2.5-4. |  |  |  |  |  |  |  |  |  |

Table 6.2G.2.5-8: UE Power Class test requirements (for Band n28 with 30MHz channel bandwidth) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 23 | 0 | 0.5 | 0 | 22.5 | 5 | 2.5 | 25.0 + TT | 17.5-TT |
| 6 | 23 | 0 | 1 | 0 | 22 | 5 | 2.5 | 25.0 + TT | 17.0-TT |
| 7 | 23 | 0 | 1 | 0 | 22 | 5 | 2.5 | 25.0 + TT | 17.0-TT |
| 8 | 23 | 0 | 1 | 0 | 22 | 5 | 2.5 | 25.0 + TT | 17.0-TT |
| 9 | 23 | 0 | 0.5 | 0 | 22.5 | 5 | 2.5 | 25.0 + TT | 17.5-TT |
| 10 | 23 | 0 | 1.5 | 0 | 21.5 | 5 | 2.5 | 25.0 + TT | 16.5-TT |
| 11 | 23 | 0 | 1.5 | 0 | 21.5 | 5 | 2.5 | 25.0 + TT | 16.5-TT |
| 12 | 23 | 0 | 1.5 | 0 | 21.5 | 5 | 2.5 | 25.0 + TT | 16.5-TT |
| 13 | 23 | 0 | 1.5 | 0 | 21.5 | 5 | 2.5 | 25.0 + TT | 16.5-TT |
| 14 | 23 | 0 | 2.5 | 0 | 20.5 | 6 | 2.5 | 25.0 + TT | 14.5-TT |
| 15 | 23 | 0 | 2.5 | 0 | 20.5 | 6 | 2.5 | 25.0 + TT | 14.5-TT |
| 16 | 23 | 0 | 2.5 | 0 | 20.5 | 6 | 2.5 | 25.0 + TT | 14.5-TT |
| 17 | 23 | 0 | 3 | 0 | 20 | 6 | 2.5 | 25.0 + TT | 14.0-TT |
| 18 | 23 | 0 | 3 | 0 | 20 | 6 | 2.5 | 25.0 + TT | 14.0-TT |
| 19 | 23 | 0 | 3 | 0 | 20 | 6 | 2.5 | 25.0 + TT | 14.0-TT |
| 20 | 23 | 0 | 5 | 0 | 18 | 5 | 2.5 | 25.0 + TT | 13.0-TT |
| 21 | 23 | 0 | 5 | 0 | 18 | 5 | 2.5 | 25.0 + TT | 13.0-TT |
| 22 | 23 | 0 | 5 | 0 | 18 | 5 | 2.5 | 25.0 + TT | 13.0-TT |
| 23 | 23 | 0 | 2 | 0 | 21 | 5 | 2.5 | 25.0 + TT | 16.0-TT |
| 24 | 23 | 0 | 3.5 | 0 | 19.5 | 5 | 2.5 | 25.0 + TT | 14.5-TT |
| 25 | 23 | 0 | 3.5 | 0 | 19.5 | 5 | 2.5 | 25.0 + TT | 14.5-TT |
| 26 | 23 | 0 | 3.5 | 0 | 19.5 | 5 | 2.5 | 25.0 + TT | 14.5-TT |
| 27 | 23 | 0 | 2.5 | 0 | 20.5 | 6 | 2.5 | 25.0 + TT | 14.5-TT |
| 28 | 23 | 0 | 3.5 | 0 | 19.5 | 5 | 2.5 | 25.0 + TT | 14.5-TT |
| 29 | 23 | 0 | 3.5 | 0 | 19.5 | 5 | 2.5 | 25.0 + TT | 14.5-TT |
| 30 | 23 | 0 | 3.5 | 0 | 19.5 | 5 | 2.5 | 25.0 + TT | 14.5-TT |
| 31 | 23 | 0 | 4 | 0 | 19 | 5 | 2.5 | 25.0 + TT | 14.0-TT |
| 32 | 23 | 0 | 4 | 0 | 19 | 5 | 2.5 | 25.0 + TT | 14.0-TT |
| 33 | 23 | 0 | 4 | 0 | 19 | 5 | 2.5 | 25.0 + TT | 14.0-TT |
| 34 | 23 | 0 | 7 | 0 | 16 | 5 | 2.5 | 25.0 + TT | 11.0-TT |
| 35 | 23 | 0 | 7 | 0 | 16 | 5 | 2.5 | 25.0 + TT | 11.0-TT |
| 36 | 23 | 0 | 7 | 0 | 16 | 5 | 2.5 | 25.0 + TT | 11.0-TT |
| 37 | 23 | 0 | 1 | 0 | 22 | 5 | 2.5 | 25.0 + TT | 17.0-TT |
| 38 | 23 | 0 | 1 | 0 | 22 | 5 | 2.5 | 25.0 + TT | 17.0-TT |
| 39 | 23 | 0 | 0.5 | 0 | 22.5 | 5 | 2.5 | 25.0 + TT | 17.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2G.2.5-4. |  |  |  |  |  |  |  |  |  |

Table 6.2G.2.5-9: UE Power Class test requirements (for Bands n1, n2, n3, n5, n7, n8, n12, n13, n14, n20, n25, n26, n30, n34, n38, n39, n40, n41, n50, n51, n53, n54, n65, n66, n70, n74, n85, n98, n109) for Power Class 3 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 23 | 0 | 1.5 | 1.5 | 0 | （1.52） | 20 | （18.52） | 6.0 | （5.02） | 2 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 15 | 23 | 0 | 1.5 | 1 | 0 | （1.52） | 20.5 | （192） | 6.0 | （5.02） | 2 | 25.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 26 | 23 | 0 | 3 | 1.5 | 0 | （1.52） | 18.5 | （172） | 5.0 | （5.02） | 2 | 25.0 + TT | 13.5 - TT | （12.0 - TT2） |
| 27 | 23 | 0 | 3 | 1 | 0 | （1.52） | 19 | （17.52） | 5.0 | （5.02） | 2 | 25.0 + TT | 14.0 - TT | （12.5 - TT2） |
| 34 | 23 | 0 | 2 | 1.5 | 0 | （1.52） | 19.5 | （182） | 5.0 | （5.02） | 2 | 25.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 35 | 23 | 0 | 2 | 1 | 0 | （1.52） | 20 | （18.52） | 6.0 | （6.02） | 2 | 25.0 + TT | 14.0 - TT | （12.5 - TT2） |
| 46 | 23 | 0 | 3 | 1.5 | 0 | （1.52） | 18.5 | （172） | 5.0 | （5.02） | 2 | 25.0 + TT | 13.5 - TT | （12.0 - TT2） |
| 47 | 23 | 0 | 3 | 1 | 0 | （1.52） | 19 | （17.52） | 5.0 | （5.02） | 2 | 25.0 + TT | 14.0 - TT | （12.5 - TT2） |
| 56 | 23 | 0 | 3.5 | 1.5 | 0 | （1.52） | 18 | （16.52） | 5.0 | （5.02） | 2 | 25.0 + TT | 13.0 - TT | （11.5 - TT2） |
| 57 | 23 | 0 | 3.5 | 1 | 0 | （1.52） | 18.5 | （172） | 5.0 | （5.02） | 2 | 25.0 + TT | 13.5 - TT | （12.0 - TT2） |
| 66 | 23 | 0 | 6.5 | 1.5 | 0 | （1.52） | 15 | （13.52） | 6.0 | （6.02） | 2 | 25.0 + TT | 9.0 - TT | （7.5- TT2） |
| 67 | 23 | 0 | 6.5 | 1 | 0 | （1.52） | 15.5 | （142） | 6.0 | （6.02） | 2 | 25.0 + TT | 9.5 - TT | （8.0 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n2, n3, n7, n8, n12, n20, n25, n26, n41, n109 transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2G.2.5-4. NOTE 4: Applicable for CBW/SCS combinations other than CBW=40MHz when SCS=60kHz. NOTE 5: Only applicable for CBW 40MHz when SCS is 60kHz. NOTE 6: Applicable for CBW/SCS combinations other than CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 7: Only applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2G.2.5-10: UE Power Class test requirements (for Bands n28 with channel bandwidth other than 30MHz, n71, n105) for Power Class 3 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13 | 23 | 0 | 1.5 | 1.5 | 0 | 20 | 6.0 | 2.5 | 25.0 + TT | 14.0-TT |
| 14 | 23 | 0 | 1.5 | 1 | 0 | 20.5 | 6.0 | 2.5 | 25.0 + TT | 14.5-TT |
| 25 | 23 | 0 | 3 | 1.5 | 0 | 18.5 | 5.0 | 2.5 | 25.0 + TT | 13.5-TT |
| 26 | 23 | 0 | 3 | 1 | 0 | 19 | 5.0 | 2.5 | 25.0 + TT | 14.0-TT |
| 33 | 23 | 0 | 2 | 1.5 | 0 | 19.5 | 5.0 | 2.5 | 25.0 + TT | 14.5-TT |
| 34 | 23 | 0 | 2 | 1 | 0 | 20 | 6.0 | 2.5 | 25.0 + TT | 14.0-TT |
| 45 | 23 | 0 | 3 | 1.5 | 0 | 18.5 | 5.0 | 2.5 | 25.0 + TT | 13.5-TT |
| 46 | 23 | 0 | 3 | 1 | 0 | 19 | 5.0 | 2.5 | 25.0 + TT | 14.0-TT |
| 55 | 23 | 0 | 3.5 | 1.5 | 0 | 18 | 5.0 | 2.5 | 25.0 + TT | 13.0-TT |
| 56 | 23 | 0 | 3.5 | 1 | 0 | 18.5 | 5.0 | 2.5 | 25.0 + TT | 13.5-TT |
| 65 | 23 | 0 | 6.5 | 1.5 | 0 | 15 | 6.0 | 2.5 | 25.0 + TT | 9.0-TT |
| 66 | 23 | 0 | 6.5 | 1 | 0 | 15.5 | 6.0 | 2.5 | 25.0 + TT | 9.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2G.2.5-4. NOTE 3: Applicable for CBW/SCS combinations other than CBW=40MHz when SCS=60kHz. NOTE 4: Only applicable for CBW 40MHz when SCS is 60kHz. NOTE 5: Applicable for CBW/SCS combinations other than CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 6: Only applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. |  |  |  |  |  |  |  |  |  |  |

Table 6.2G.2.5-11: UE Power Class test requirements (for Band n28 with 30MHz channel bandwidth) for Power Class 3 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23 | 23 | 0 | 3.5 | 1.5 | 0 | 18 | 5 | 2.5 | 25.0 + TT | 13.0-TT |
| 24 | 23 | 0 | 3.5 | 1 | 0 | 18.5 | 5 | 2.5 | 25.0 + TT | 13.5-TT |
| 43 | 23 | 0 | 3.5 | 1.5 | 0 | 18 | 5 | 2.5 | 25.0 + TT | 13.0-TT |
| 44 | 23 | 0 | 3.5 | 1 | 0 | 18.5 | 5 | 2.5 | 25.0 + TT | 13.5-TT |
| 53 | 23 | 0 | 4 | 1.5 | 0 | 17.5 | 5 | 2.5 | 25.0 + TT | 12.5-TT |
| 54 | 23 | 0 | 4 | 1 | 0 | 18 | 5 | 2.5 | 25.0 + TT | 13.0-TT |
| 63 | 23 | 0 | 7 | 1.5 | 0 | 14.5 | 6 | 2.5 | 25.0 + TT | 8.5-TT |
| 64 | 23 | 0 | 7 | 1 | 0 | 15 | 6 | 2.5 | 25.0 + TT | 9.0-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2G.2.5-4. NOTE 3: Applicable for CBW=30MHz when SCS=60kHz. NOTE 4: Applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz when SCS=30kHz. |  |  |  |  |  |  |  |  |  |  |

Table 6.2G.2.5-12: UE Power Class test requirements (for Bands n24, n48, n77, n78, n79, n104) for Power Class 3 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 23 | 0 | 1.5 | 1.5 | 0 | （1.52） | 20 | （18.52） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 15 | 23 | 0 | 1.5 | 1 | 0 | （1.52） | 20.5 | （192） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 26 | 23 | 0 | 3 | 1.5 | 0 | （1.52） | 18.5 | （172） | 5.0 | （5.02） | 3 | 25.0 + TT | 13.5 - TT | （12.0 - TT2） |
| 27 | 23 | 0 | 3 | 1 | 0 | （1.52） | 19 | （17.52） | 5.0 | （5.02） | 3 | 25.0 + TT | 14.0 - TT | （12.5 - TT2） |
| 34 | 23 | 0 | 2 | 1.5 | 0 | （1.52） | 19.5 | （182） | 5.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 35 | 23 | 0 | 2 | 1 | 0 | （1.52） | 20 | （18.52） | 6.0 | （6.02） | 3 | 25.0 + TT | 14.0 - TT | （12.5 - TT2） |
| 46 | 23 | 0 | 3 | 1.5 | 0 | （1.52） | 18.5 | （172） | 5.0 | （5.02） | 3 | 25.0 + TT | 13.5 - TT | （12.0 - TT2） |
| 47 | 23 | 0 | 3 | 1 | 0 | （1.52） | 19 | （17.52） | 5.0 | （5.02） | 3 | 25.0 + TT | 14.0 - TT | （12.5 - TT2） |
| 56 | 23 | 0 | 3.5 | 1.5 | 0 | （1.52） | 18 | （16.52） | 5.0 | （5.02） | 3 | 25.0 + TT | 13.0 - TT | （11.5 - TT2） |
| 57 | 23 | 0 | 3.5 | 1 | 0 | （1.52） | 18.5 | （172） | 5.0 | （5.02） | 3 | 25.0 + TT | 13.5 - TT | （12.0 - TT2） |
| 66 | 23 | 0 | 6.5 | 1.5 | 0 | （1.52） | 15 | （13.52） | 6.0 | （6.02） | 3 | 25.0 + TT | 9.0 - TT | （7.5- TT2） |
| 67 | 23 | 0 | 6.5 | 1 | 0 | （1.52） | 15.5 | （142） | 6.0 | （6.02） | 3 | 25.0 + TT | 9.5 - TT | （8.0 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n24, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2G.2.5-4. NOTE 4: Applicable for CBW/SCS combinations other than CBW=40MHz when SCS=60kHz. NOTE 5: Only applicable for CBW 40MHz when SCS is 60kHz. NOTE 6: Applicable for CBW/SCS combinations other than CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 7: Only applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

6.2G.2_1	UE maximum output power reduction for Tx Diversity for UE supporting 4Tx

6.2G.2_1.1	Test purpose

Same test purpose as in clause 6.2G.2.1.

6.2G.2_1.2	Test applicability

This test applies to all types of NR Power Class 1.5 UE release 18 and forward that support 4Tx Tx diversity.

6.2G.2_1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.2G.2.3.

6.2G.2_1.4	Test description

6.2G.2_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.2G.2_1.4.1-1 and Table 6.2G.2_1.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2G.2_1.4.1-1: Test Configuration Table for power class 1.5 UE with 4Tx

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Low | Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 2 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 3 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 4 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 6 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 7 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 8 | Default |  | DFT-s-OFDM 64 QAM | Inner Full |
| 9 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 10 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 11 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 12 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 13 | Default |  | CP-OFDM QPSK | Inner Full |
| 14 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 15 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 16 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 17 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 18 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 19 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. |  |  |  |  |

Table 6.2G.2_1.4.1-2: Test Configuration Table for power class 1.5 UE with 4Tx for FWA

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Low | Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 2 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 3 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 4 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 6 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 7 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 8 | Default |  | DFT-s-OFDM 64 QAM | Inner Full |
| 9 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 10 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 13 | Default |  | CP-OFDM QPSK | Inner Full |
| 14 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 15 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 16 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 17 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 18 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 19 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 20 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 21 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2a for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2G.2_1.4.1-1 and Table 6.2G.2_1.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2G.2_1.4.3.

6.2G.2_1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2G.2_1.4.1-1 and Table 6.2G.2_1.4.1-2. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE at each antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots. For TDD, only slots consisting of only UL symbols are under test.

NOTE 1: When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.2G.2_1.4.1-1 and Table 6.2G.2_1.4.1-2, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2G.2_1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4.

6.2G.2_1.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2G.2_1.5-1 and Table 6.2G.2_1.5-2.

Table 6.2G.2_1.5-1: UE Power Class test requirements (for Band n34, n39, n40, n41, n77, n78, n79) for Power Class 1.5 UEs with 4Tx

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 8.0 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 | 31.0 + TT | 16.0 - TT | （14.52 - TT） |
| 2 | 29 | 0 | 8.0 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 | 31.0 + TT | 16.0 - TT | （14.52 - TT） |
| 3 | 29 | 0 | 3.0 | 0 | （1.52） | 26.0 | （24.52） | 3.0 |  | 3 | 31.0 + TT | 23.0 - TT | （21.52 - TT） |
| 4 | 29 | 0 | 2.0 | 0 | （1.52） | 27.0 | （25.52） | 3.0 |  | 3 | 31.0 + TT | 24.0 - TT | （22.52 - TT） |
| 5 | 29 | 0 | 3.5 | 0 | （1.52） | 25.5 | （24.02） | 3.0 | 5.0 | 3 | 31.0 + TT | 22.5 - TT | （19.02 - TT） |
| 6 | 29 | 0 | 2.5 | 0 | （1.52） | 26.5 | （25.02） | 3.0 |  | 3 | 31.0 + TT | 23.5 - TT | （22.02 - TT） |
| 7 | 29 | 0 | 4.0 | 0 | （1.52） | 25.0 | （23.52） | 3.0 | 5.0 | 3 | 31.0 + TT | 22.0 - TT | （18.52 - TT） |
| 8 | 29 | 0 | 4.5 | 0 | （1.52） | 24.5 | （23.02） | 3.0 | 5.0 | 3 | 31.0 + TT | 21.5 - TT | （18.02 - TT） |
| 9 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 | 31.0 + TT | 15.5 - TT | （14.02 - TT） |
| 10 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 | 31.0 + TT | 15.5 - TT | （14.02 - TT） |
| 11 | 29 | 0 | 4.7 | 0 | （1.52） | 24.3 | （22.82） | 5.0 |  | 3 | 31.0 + TT | 19.3 - TT | （17.82 - TT） |
| 12 | 29 | 0 | 7.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 | 31.0 + TT | 17.0 - TT | （15.52 - TT） |
| 13 | 29 | 0 | 3.5 | 0 | （1.52） | 25.5 | （24.02） | 3.0 |  | 3 | 31.0 + TT | 22.5 - TT | （21.02 - TT） |
| 14 | 29 | 0 | 4.0 | 0 | （1.52） | 25.0 | （23.52） | 3.0 | 5.0 | 3 | 31.0 + TT | 22.0 - TT | （20.02 - TT） |
| 15 | 29 | 0 | 5.0 | 0 | （1.52） | 24.0 | （22.52） | 5.0 | 5.0 | 3 | 31.0 + TT | 19.0 - TT | （17.52 - TT） |
| 16 | 29 | 0 | 7.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | 5.0 | 3 | 31.0 + TT | 17.0 - TT | （15.52 - TT） |
| 17 | 29 | 0 | 9.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 | 5.0 | 3 | 31.0 + TT | 14.5 - TT | （13.02 - TT） |
| 18 | 29 | 0 | 9.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 | 5.0 | 3 | 31.0 + TT | 14.5 - TT | （13.02 - TT） |
| 19 | 29 | 0 | 9.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 | 5.0 | 3 | 31.0 + TT | 14.5 - TT | （13.02 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2G.2_1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2G.2_1.5-2: UE Power Class test requirements (for Band n34, n39, n40, n41, n77, n78, n79) for Power Class 1.5 FWA UEs with 4Tx

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 7.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 | 31.0 + TT | 16.5 - TT | （15.02 - TT） |
| 2 | 29 | 0 | 7.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 | 31.0 + TT | 16.5 - TT | （15.02 - TT） |
| 3 | 29 | 0 | 2.0 | 0 | （1.52） | 27.0 | （25.52） | 3.0 |  | 3 | 31.0 + TT | 24.0 - TT | （22.52 - TT） |
| 4 | 29 | 0 | 0.5 | 0 | （1.52） | 28.5 | （27.02） | 3.0 |  | 3 | 31.0 + TT | 25.5 - TT | （24.02 - TT） |
| 5 | 29 | 0 | 2.5 | 0 | （1.52） | 26.5 | （25.02） | 3.0 |  | 3 | 31.0 + TT | 23.5 - TT | （22.02 - TT） |
| 6 | 29 | 0 | 1.5 | 0 | （1.52） | 27.5 | （26.02） | 3.0 |  | 3 | 31.0 + TT | 24.5 - TT | （23.02 - TT） |
| 7 | 29 | 0 | 3.5 | 0 | （1.52） | 25.5 | （24.02） | 3.0 | 5.0 | 3 | 31.0 + TT | 22.5 - TT | （19.02 - TT） |
| 8 | 29 | 0 | 3.5 | 0 | （1.52） | 25.5 | （24.02） | 3.0 | 5.0 | 3 | 31.0 + TT | 22.5 - TT | （19.02 - TT） |
| 9 | 29 | 0 | 4.0 | 0 | （1.52） | 25.0 | （23.52） | 3.0 | 5.0 | 3 | 31.0 + TT | 22.0 - TT | （18.52 - TT） |
| 10 | 29 | 0 | 8.0 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 | 31.0 + TT | 16.0 - TT | （14.52 - TT） |
| 11 | 29 | 0 | 8.0 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 | 31.0 + TT | 16.0 - TT | （14.52 - TT） |
| 12 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| 13 | 29 | 0 | 2.0 | 0 | （1.52） | 27.0 | （25.52） | 3.0 |  | 3 | 31.0 + TT | 24.0 - TT | （22.52 - TT） |
| 14 | 29 | 0 | 2.5 | 0 | （1.52） | 26.5 | （25.02） | 3.0 |  | 3 | 31.0 + TT | 23.5 - TT | （22.02 - TT） |
| 15 | 29 | 0 | 4.5 | 0 | （1.52） | 24.5 | （23.02） | 3.0 | 5.0 | 3 | 31.0 + TT | 21.5 - TT | （18.02 - TT） |
| 16 | 29 | 0 | 8.0 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 | 31.0 + TT | 16.0 - TT | （14.52 - TT） |
| 17 | 29 | 0 | 8.0 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 | 31.0 + TT | 16.0 - TT | （14.52 - TT） |
| 18 | 29 | 0 | 5.0 | 0 | （1.52） | 24.0 | （22.52） | 5.0 |  | 3 | 31.0 + TT | 19.0 - TT | （17.52 - TT） |
| 19 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 | 31.0 + TT | 15.5 - TT | （14.02 - TT） |
| 20 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 | 31.0 + TT | 15.5 - TT | （14.02 - TT） |
| 21 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 | 31.0 + TT | 15.5 - TT | （14.02 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2G.2_1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2G.2_1.5-3: Test Tolerance (Maximum Power Reduction (MPR))

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

<Unchanged Text Skipped>

6.5G.2	Out of band emission for Tx Diversity

6.5G.2.1	Spectrum emission mask for Tx Diversity

6.5G.2.1.1	Test purpose

Same test purpose as in 6.5.2.2.1.

6.5G.2.1.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.5G.2.1.3	Minimum conformance requirements

For UE supporting Tx diversity, the requirements for Out of band emissions resulting from the modulation process and non-linearity in the transmitters apply to the sum of the emissions from all UE transmit antenna connectors.

The requirements specified in clause 6.5.2 apply.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5G.2.

<Unchanged Text Skipped>

6.5G.2.1_1	Spectrum emission mask for Tx Diversity for UE supporting 4Tx

6.5G.2.1_1.1	Test purpose

Same test purpose as in 6.5G.2.1.1.

6.5G.2.1_1.2	Test applicability

This test applies to all types of NR Power Class 1.5 UE release 18 and forward that support 4Tx Tx diversity.

6.5G.2.1_1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5G.2.1.3.

6.5G.2.1_1.4	Test description

6.5G.2.1_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in table 6.5G.2.1_1.4.1-1 to 6.5G.2.1_1.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Table 6.5G.2.1_1.4.1-1: Test Configuration Table for power class 1.5 UE with 4Tx

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 2 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 3 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 4 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 6 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 9 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 10 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 11 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 12 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 13 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 14 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: It is essential that all test points in this table also exist in Table 6.2G.2_1.4.1-1. |  |  |  |  |

Table 6.5G.2.1_1.4.1-2: Test Configuration Table for power class 1.5 UE with 4Tx for FWA

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 2 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 3 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 4 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 6 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 7 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 8 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 9 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 10 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 11 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 12 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 13 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 14 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 15 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 16 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: It is essential that all test points in this table also exist in Table 6.2G.2_1.4.1-2. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2a for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2 and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5G.2.1_1.4.1-1 to Table 6.5G.2.1_1.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5G.2.1_1.4.3.

6.5G.2.1_1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5G.2.1_1.4.1-1 to Table 6.5G.2.1_1.4.1-2. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously power control “up” commands to the UE until the UE transmits at PUMAX level. Allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE from all transmit antenna connectors in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in clause 6.2G.2_1.5. The period of the measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots. For TDD, only slots consisting of only UL symbols are under test.

4.	Measure the sum of the mean power of the transmitted signal from both transmit antenna connectors with a measurement filter of bandwidths according to table 6.5G.2.1_1.5-1 and using a rms detector. If the sweep count is higher than one, the trace mode shall be average. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

NOTE 1: When switching to DFT-s-OFDM waveform, as specified in the test configuration table 6.5G.2.1_1.4.1-1 to 6.5G.2.1_1.4.1-2, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.5G.2.1_1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6.

6.5G.2.1_1.5	Test requirement

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in clause 6.2G.2_1.5 as appropriate, and the power of any UE emission derived in step 4 shall fulfil requirements in Table 6.5G.2.1_1.5-1.

Table 6.5G.2.1_1.5-1: General NR spectrum emission mask

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- |
|  | 5 | 10, 15, 20, 25, 30, 35, 40, 45 | 50, 60, 70, 80, 90, 100 |  |
| ± 0-1 | -13 + TT | -13 + TT |  | 1 % of channel BW |
| ± 0-1 |  |  | -24 + TT | 30 kHz |
| ± 1-5 | -10 + TT | -10 + TT |  | 1 MHz |
| ± 5-6 | -13 + TT |  |  |  |
| ± 6-10 | -25 + TT |  |  |  |
| ± 5-BWChannel |  | -13 + TT |  |  |
| ± BWChannel-(BWChannel+5) |  | -25 + TT |  |  |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel. Note 4: TT for each frequency and channel bandwidth is specified in Table 6.5G.2.1_1.5-2. |  |  |  |  |

Table 6.5G.2.1_1.5-2: Test Tolerance (Spectrum Emission Mask)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 100MHz | 1.5 dB | 1.8 dB | 1.8 dB |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

<Unchanged Text Skipped>

6.5G.2.3	Adjacent channel leakage ratio for Tx Diversity

If UE indicates IE Tx diversity capability, Adjacent Channel Leakage power Ratio (ACLR) is defined as the ratio of sum of the filtered mean power at each antenna connector centred on the assigned channel frequency to sum of the filtered mean power at each antenna connector centred on an adjacent channel frequency.

The requirements specified in clause 6.5.2 apply.

6.5G.2.3.1	NR ACLR for Tx Diversity

6.5G.2.3.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR).

6.5G.2.3.1.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.5G.2.3.1.3	Minimum conformance requirements

NR adjacent channel leakage power ratio (NRACLR) is the ratio of sum of the filtered mean power at each antenna connector centred on the assigned NR channel frequency to sum of the filtered mean power at each antenna connector centred on an adjacent NR channel frequency at nominal channel spacing.

The assigned NR channel power and adjacent NR channel power are measured with rectangular filters with measurement bandwidths specified in Table 6.5G.2.3.1.3-1.

If the measured adjacent channel power is greater than –50dBm then the NRACLR shall be higher than the value specified in Table 6.5G.2.3.1.3-2.

Table 6.5G.2.3.1.3-1: NR ACLR measurement bandwidth

| Channel bandwidth | (MHz) | 5,10,15,20,25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 | 30 |
| NR ACLR measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |

Table 6.5G.2.3.1.3-2: NR ACLR requirement

|  | Power class 1.5 | Power class 2 | Power class 3 |
| --- | --- | --- | --- |
| NR ACLR | 31 dB | 31 dB | 30 dB |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5G.2.

6.5G.2.3.1.4	Test description

6.5G.2.3.1.4.1	Initial conditions

Same initial conditions as 6.2G.2.4.1.

6.5G.2.3.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to the test configuration tables in clause 6.2G.2.4.1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously power control “up” commands to the UE until the UE transmits at PUMAX level. Allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE at each antenna connector in the channel bandwidth of the radio access mode according to the test configuration, as measured in step 3 of 6.2G.2.4.2, which shall meet the requirements described in clause 6.2G.2.5 as appropriate.

4.	Measure the sum of the rectangular filtered mean power at each antenna connector for the assigned NR channel using a rms detector. If the sweep count is higher than one, the trace mode shall be average.

5.	Measure the sum of the rectangular filtered mean power at each antenna connector of the first NR adjacent channel on both lower and upper side of the assigned NR channel using a rms detector, respectively. If the sweep count is higher than one, the trace mode shall be average.

6.	Calculate the ratios of the power between the values measured in step 4 over step 5 for lower and upper NR ACLR, respectively.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration tables in clause 6.2G.2.4.1, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

<Unchanged Text Skipped>

6.5G.2.3_1	Adjacent channel leakage ratio for Tx Diversity for UE supporting 4T

6.5G.2.3_1.1	NR ACLR for Tx Diversity

6.5G.2.3_1.1.1	Test purpose

Same test purpose as in clause 6.5G.2.3.1.1.

6.5G.2.3_1.1.2	Test applicability

This test applies to all types of NR Power Class 1.5 UE release 18 and forward that support 4Tx Tx diversity.

6.5G.2.3_1.1.3	Minimum conformance requirements

NR adjacent channel leakage power ratio (NRACLR) is the ratio of sum of the filtered mean power at each antenna connector centred on the assigned NR channel frequency to sum of the filtered mean power at each antenna connector centred on an adjacent NR channel frequency at nominal channel spacing.

The assigned NR channel power and adjacent NR channel power are measured with rectangular filters with measurement bandwidths specified in Table 6.5G.2.3_1.1.3-1.

If the measured adjacent channel power is greater than –50dBm then the NRACLR shall be higher than the value specified in Table 6.5G.2.3_1.1.3-2.

Table 6.5G.2.3_1.1.3-1: NR ACLR measurement bandwidth

| Channel bandwidth | (MHz) | 5,10,15,20,25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 | 30 |
| NR ACLR measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |

Table 6.5G.2.3_1.1.3-2: NR ACLR requirement

|  | Power class 1.5 |
| --- | --- |
| NR ACLR | 31 dB |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5G.2.

6.5G.2.3_1.1.4	Test description

6.5G.2.3_1.1.4.1	Initial conditions

Same initial conditions as 6.2G.2_1.4.1.

6.5G.2.3_1.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to the test configuration tables in clause 6.2G.2_1.4.1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously power control “up” commands to the UE until the UE transmits at PUMAX level. Allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE at each antenna connector in the channel bandwidth of the radio access mode according to the test configuration, as measured in step 3 of 6.2G.2_1.4.2, which shall meet the requirements described in clause 6.2G.2_1.5 as appropriate.

4.	Measure the sum of the rectangular filtered mean power at each antenna connector for the assigned NR channel using a rms detector. If the sweep count is higher than one, the trace mode shall be average.

5.	Measure the sum of the rectangular filtered mean power at each antenna connector of the first NR adjacent channel on both lower and upper side of the assigned NR channel using a rms detector, respectively. If the sweep count is higher than one, the trace mode shall be average.

6.	Calculate the ratios of the power between the values measured in step 4 over step 5 for lower and upper NR ACLR, respectively.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration tables in clause 6.2G.2_1.4.1, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.5G.2.3_1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4.

6.5G.2.3_1.1.5	Test requirement

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in clause 6.2G.2_1.5 as appropriate, and if the measured adjacent channel power is greater than –50 dBm then the measured NR ACLR, derived in step 6, shall be higher than the limits in Table 6.5G.2.3_1.1.5-2 with TT applying for the sum of power at each of UE antenna connector.

Table 6.5G.2.3_1.1.5-1: NR ACLR measurement bandwidth

|  | NR channel bandwidth / NR ACLR measurement bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| NR ACLR measurement bandwidth (MHz) | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 28.815 | 33.855 | 38.895 | 48.615 | 58.35 | 68.07 | 78.15 | 88.23 | 98.31 |

Table 6.5G.2.3_1.1.5-2: NR ACLR requirement for Tx Diversity

|  | Power class 1.5 |
| --- | --- |
| NR ACLR | 31 - TT dB |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.5G.2.3_1.1.5-3. NOTE 2: TT for the sum of power at each of UE antenna connector. |  |

Table 6.5G.2.3_1.1.5-3: Test Tolerance (NR ACLR)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 100MHz | 0.8 dB | 0.8 dB | 0.8 dB |

< End of Changes >