3GPP TSG-RAN5 Meeting #108	R5-254010

# Bangalore, India, 25th - 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0678 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of test case 10.17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - "text" feature tag shall be specified consistently in default message contents so that there is no need anymore to specify it in specific message contents. - INVITE requests at steps 2 and 6 are re-INVITE requests. - In SDP answer at step 4 of A.29 the sess-version is hard-coded to 1111111111. - The 200 OK responses at steps 4 and 9 are for re-INVITE and therefore need condition A23. - The 200 OK responses at steps 4 and 9 need Content-Type. - The message sequence of the test case has not much to do with A.29. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - "text" feature tag removed from specific message contents. - Specific message contents for steps 2 and 6 specifying A5 (NOTE: when A5 is applied A7 is not needed). - Specific message content added for 200 OK (step 4) to clarify the sess-version being incremented by one. - A23 added to specific message contents for 200 OK responses. (NOTE: 34.229-1 A.3.1 needs to be changed to allow conditions A23 and A6 to be applied without contradictions) - Content-Type added to 200 OK responses. - References to A.29 removed from Table 10.17.3.2-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect test specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 34.229-1 CR 1622 TS 34.229-1 CR 1615 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 10.17	Emergency Call with emergency registration / add RTT and remove RTT / 5GS

10.17.1	Test Purpose (TP)

(1)

with { UE being emergency registered to IMS and emergency call being established}

ensure that {

when { UE is being made to add real-time text to the emergency call }

then { UE sends re-INVITE with SDP media for both voice and real-time text }

}

(2)

with { UE having sent re-INVITE }

ensure that {

when { UE receives 200 OK for re-INVITE and indication that real-time text has been added to the call }

then { UE sends ACK }

}

(3)

with { UE having successfully added RTT }

ensure that {

when { UE is made to remove RTT again }

then { UE sends re-INVITE with SDP offer indicating that real-time text is being removed from the call }

}

(4)

with { UE having sent re-INVITE }

ensure that {

when { UE receives 200 OK for re-INVITE and indication that real-time text has been removed from the call}

then { UE sends ACK }

}

(5)

with { UE having successfully added real-time text }

ensure that {

when { UE receives BYE  }

then { UE sends a 200 OK response }

}

10.17.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 24.229, clause 5.1.2A.1]:

If this is a request within an existing dialog, and the request includes a Contact header field, then the UE should insert the previously used Contact header field.

...

After the dialog is established the UE may change the dialog capabilities (e.g. add a media or request a supplementary service) if defined for the IMS communication service as identified by the ICSI value using the same dialog. Otherwise, the UE shall initiate a new initial request to the other user.

[TS 24.229 clause 5.1.6.2]:

When the user initiates an emergency call, if emergency registration is needed (including cases described in subclause 5.1.6.2A), the UE shall perform an emergency registration prior to sending the SIP request related to the emergency call.

...

When a UE performs an initial emergency registration the UE shall perform the actions as specified in subclause 5.1.1.2 with the following additions and modifications:

a)	the UE shall include a "sos" SIP URI parameter in the Contact header field as described in subclause 7.2A.13, indicating that this is an emergency registration and that the associated contact address is allowed only for emergency service; and

b)	the UE shall populate the From and To header fields of the REGISTER request with:

-	the first entry in the list of public user identities provisioned in the UE;

-	the default public user identity obtained during the normal registration, if the UE is not provisioned with a list of public user identities, but the UE is currently registered to the IM CN subsystem; and

-	the derived temporary public user identity, in all other cases.

[TS 24.229 clause 5.1.6.3]

Upon receiving the 200 (OK) to the REGISTER request that completes the emergency registration, the UE shall not subscribe to the reg event package of the public user identity specified in the REGISTER request.

[TS 24.229 clause 5.1.6.5]

When a UE performs authentication a UE shall perform the procedures as specified in subclause 5.1.1.5.

[TS 24.229 clause 5.1.6.8.3]

After a successful initial emergency registration, the UE shall apply the procedures as specified in subclause 5.1.2A and 5.1.3 with the following additions:

1)	the UE shall insert in the INVITE request, a From header field that includes the public user identity registered via emergency registration or the tel URI associated with the public user identity registered via emergency registration, as described in subclause 4.2;

2)	the UE shall include a service URN in the Request-URI of the INVITE request in accordance with subclause 5.1.6.8.1;

3)	the UE shall insert in the INVITE request, a To header field with the same emergency service URN as in the Request-URI;

4)	if available to the UE, and if defined for the access type as specified in subclause 7.2A.4, the P-Access-Network-Info header field shall contain a location identifier such as the cell id, line id or the identity of the WLAN access node, which is relevant for routeing the IMS emergency call;

NOTE 1:	The IMS emergency specification in 3GPP TS 23.167 [4B] describes several methods how the UE can get its location information from the access network or from a server. Such methods are not in the scope of this specification.

5)	the UE shall insert in the INVITE request, one or two P-Preferred-Identity header field(s) that include the public user identity registered via emergency registration or the tel URI associated with the public user identity registered via emergency registration as described in subclause 4.2;

NOTE 2:	Providing two P-Preferred-Identity header fields is usually supported by UE acting as enterprise network.

6)	void;

7)	if the UE has its location information available, or a URI that points to the location information, then the UE shall include a Geolocation header field in the INVITE request in the following way:

-	if the UE is aware of the URI that points to where the UE's location is stored, include the URI as the Geolocation header field value, as described in RFC 6442 [89]; or

[Rel-15]

-	if the UE is aware of its location information, include the location information in a PIDF location object, in accordance with RFC 4119 [90], include the location object in a message body with the content type application/pidf+xml, and include a Content ID URL, referring to the message body, as the Geolocation header field value, as described RFC 6442 [89], and include a Content-Disposition header field with a disposition type "render" value and a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

[Rel-16]

-	if the UE is aware of its location information, include the location information in a PIDF location object, in accordance with RFC 4119 [90] and RFC 5491 [267], include the location object in a message body with the content type application/pidf+xml, and include a Content ID URL, referring to the message body, as the Geolocation header field value, as described RFC 6442 [89], and include a Content-Disposition header field with a disposition type "render" value and a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

8)	if the UE includes a Geolocation header field, the UE shall also include a Geolocation-Routing header field with a "yes" header field value, which indicates that the location of the UE can be used by other entities to make routing decisions, as described in RFC 6442 [89];

NOTE 3:	It is suggested that UE's only use the option of providing a URI when the domain part belongs to the current P-CSCF or S-CSCF provider. This is an issue on which the network operator needs to provide guidance to the end user. A URI that is only resolvable to the UE which is making the emergency call is not desirable.

9)	if the UE has neither geographical location information available, nor a URI that points to the location information, the UE shall not insert a Geolocation header field in the INVITE request; and

10)	if support of the current location discovery during an emergency call is allowed in the IP-CAN specific annex and the UE supports the current location discovery during an emergency call, the UE shall include a Recv-Info header field as described in RFC 6086 [25], indicating the g.3gpp.current-location-discovery info package name and shall include an Accept header field indicating the "application/vnd.3gpp.current-location-discovery+xml" MIME type.

NOTE 4:	RFC 3261 [26] provides for the use of the Priority header field with a suggested value of "emergency". It is not precluded that emergency sessions contain this value, but such usage will have no impact on the processing within the IM CN subsystem.

[TS 24.237 clause 7.2]:

When originating an emergency call as specified in 3GPP TS 24.229 [2] and if the SC UE has an IMEI, then the SC UE shall include the sip.instance media feature tag as specified in IETF RFC 5626 [22] with value based on the IMEI as defined in 3GPP TS 23.003 [12] in the Contact header field of the SIP INVITE request according to IETF RFC 3840 [53].

[TS 23.003 clause 13.8]:

An instance-id is a SIP Contact header parameter that uniquely identifies the SIP UA performing a registration.

When an IMEI is available, the instance-id shall take the form of a IMEI URN (see RFC 7254 [79]). The format of the instance-id shall take the form "urn:gsma:imei:<imeival>" where by the imeival shall contain the IMEI encoded as defined in RFC 7254 [79]. The optional <sw-version-param> and <imei-version-param> parameters shall not be included in the instance-id. RFC 7255 [104] specifies additional considerations for using the IMEI as an instance-id. An example of such an instance-id is as follows:

EXAMPLE:	urn:gsma:imei:90420156-025763-0

If no IMEI is available, the instance-id shall take the form of a string representation of a UUID as a URN as defined in IETF RFC 4122 [80]. An example of such an instance-id is as follows:

EXAMPLE:	urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91e6bf6

For more information on the instance-id and when it is used, see 3GPP TS 24.229 [81].

[TS 26.114 clause 6.2.1]

… An MTSI client shall offer at least one RTP profile for each RTP media stream. Multiple RTP profiles may be offered using SDPCapNeg as described in Clause 6.2.1a. For voice and real-time text, the first SDP offer shall include at least the AVP profile.

…

[TS 26.114, clause 6.3]:

During session renegotiation for adding or removing media components, the SDP offerer should continue to use the same media (m=) line(s) from the previously negotiated SDP for the media components that are not being added or removed.

An MTSI client in terminal may support multiple media components including media components of the same media type. An MTSI client in terminal may support adding one or more media components to an on-going session which already contains a media component of the same media type. If an MTSI client in terminal needs to have multiple media components of the same media type in a single MTSI session, then the MTSI client in terminal should use the SDP content attributes as defined in [81] for identifying different media components.

[TS 26.144 clause 7.4.4]

The following RTP payload format shall be used:

T.140 text conversation RTP payload format according to RFC 4103 [31] including the updates from RFC 9071 [185] when the negotiation for support of multiparty real-time text is successful.

Real-time text shall be the only payload type in its RTP stream because the RTP sequence numbers are used for loss detection and recovery. The redundant transmission format shall be used for keeping the effect of packet loss low.

10.17.3	Test description

10.17.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is configured to enable RTT and to register for IMS after switch on.

Preamble:

-	The UE is in test state 1N-A (TS 38.508-1 [21]), emergency registered to IMS by executing annex A.3 and set up Emergency call by executing annex A.6.

10.17.3.2	Test procedure sequence

Table 10.17.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make UE to add RTT to the Emergency call | - | - | - | - |
| 2 | Check: Does the UE send re-INVITE with an SDP offer containing media lines for both voice and real-time text? | --> | INVITE | 1 | P |
| 3 | SS reserves resources for real-time text by executing TS 38.508-1 [21], clause 4.9.46. | - | - | - | - |
| 4 | The SS responds INVITE with 200 OK with the media lines for both voice and real-time text. | <-- | 200 OK | - | - |
| 5 | Check: Does the UE send ACK? | --> | ACK | 2 | P |
| 6 | Make UE to remove RTT from the Emergency call | - | - | - | - |
| 7 | Check: Does the UE send re-INVITE with an SDP offer proposing removal or the real-time text media? | --> | INVITE | 3 | P |
| 8 | SS releases the QoS flow for real-time text by executing TS 38.508-1 [21], clause 4.9.48. | - | - | - | - |
| 9 | The SS responds INVITE with 200 OK indicating that the real-time text component has been removed from the emergency call. | <-- | 200 OK | - | - |
| 10 | Check: Does the UE send ACK? | --> | ACK | 4 | P |
| 11 | Step 1 of annex A.8 (MT Release of Voice Call) | <-- | BYE | - | - |
| 12 | Check: Does the UE send 200 OK for the BYE request and ends the call? Step 2 of annex A.8 (MT Release of Voice Call) | --> | 200 OK | 5 | P |

10.17.3.3	Specific message contents

Table 10.17.3.3-1: INVITE (step 2, table 10.17.3.2-1)

| Derivation Path: TS 34.229-1 [2] Annex A.2.1, Condition A5 |  |
| --- | --- |
| Header/param | Value/remark |
| Message-body | Same SDP offer as specified for step 1 in annex A.29 with the following modifications: Session description: o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) [Note 1] Note 1: The "o=" line is identical to previous SDP sent by UE except that sess-version is incremented by one. |

Table 10.17.3.3-2: 200 OK (step 4, table 10.17.3.2-1)

| Derivation Path: TS 34.229-1 [2] Annex A.3.1, Conditions A6 and A23 |  |
| --- | --- |
| Header/param | Value/remark |
| Content-Type |  |
| media-type | application/sdp |
| Message-body | SDP answer as specified for step 4 in annex A.29 with the following modifications: - "o=" line: o= line of previous SDP sent by SS with sess-version incremented by one. |

Table 10.17.3.3-3: ACK (step 5, table 10.17.3.2-1)

Derivation Path: TS 34.229-1 [2] Annex A.2.7, Condition A5

Table 10.17.3.3-4: INVITE (step 6, table 10.17.3.2-1)

| Derivation Path: TS 34.229-1 [2] Annex A.2.1, Condition A5 |  |
| --- | --- |
| Header/param | Value/remark |
| Message-body | Same as in Step 2 with following exceptions: Session description: o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) [Note 1] Media description (real time text): m=text 0 RTP/AVP (fmt) [Note 2] Note 1: The "o=" line is identical to previous SDP sent by UE except that sess-version is incremented by one. Note 2: There are no requirements on b-, c- or a-lines of the real time text stream. |

Table 10.17.3.3-5: 200 OK (step 9, table 10.17.3.2-1)

| Derivation Path: TS 34.229-1 [2] Annex A.3.1, Conditions A6 and A23 |  |
| --- | --- |
| Header/param | Value/remark |
| Content-Type |  |
| media-type | application/sdp |
| Message-body | SDP message as received in the re-INVITE at step 5 with the following modifications: - "o=" line: o= line of previous SDP sent by SS with sess-version incremented by one. - "c=" lines: connection-address changed to the IP address of the SS. - "m=" line of the media description for audio: transport port changed to the port number of the audio stream at the SS |

Table 10.17.3.3-6: ACK (step 10, table 10.17.3.2-1)

Derivation Path: TS 34.229-1 [2] Table in Annex A.2.7, Condition A5

Table 10.17.3.3-7: BYE (step 11, table 10.16.3.2-1)

Derivation Path: TS 34.229-1 [2] Table in Annex A.2.8, Condition A9