3GPP TSG-RAN5 Meeting #108	R5-254119

# Bengaluru, India, 25th – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0685 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new NR IMS data channel test case 7.44 - MO Video call with precondition and BDC success |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_IMS_dataCH-UEConTest |  |  |  |  |  | Date: |  |  | 2025-07-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This new IMS data channel (IMS DC) test case is added to test: UE send SDP offer with IMS DC and NW reply SDP answer with IMS DC. The UE can establish video call with precondition + IMS DC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new NR IMS data channel test case 7.44 IMS Data Channel / DCMTSI MO Video call / Bootstrap Data Channel success / with preconditions at both originating UE and terminating UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NG_RTC_IMS_dataCH-UEConTest is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.44(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 34.229-2 CR 0393 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

7.44	IMS Data Channel / DCMTSI MO Video call / Bootstrap Data Channel success / with preconditions at both originating UE and terminating UE

7.44.1	Test Purpose (TP)

(1)

with { UE supporting IMS data channel and is allowed to use IMS data channel and UE determined its home network supports the IMS data channel capability }

ensure that {

when { UE is made to establish IMS Video Call }

then { UE shall include SDP media-level attributes "a=dcmap" in data channel media negotiation in initial INVITE request or a re-INVITE request }

}

(2)

with { UE having send (re-)INVITE with preconditions, and (re-)INVITE include SDP media-level attributes "a=dcmap" in data channel media negotiation }

ensure that {

when { receiving SDP answer with media-level attributes "a=dcmap" }

then { UE establish IMS Bootstrap Data Channel associated with Video call }

}

(3)

with { UE supporting IMS data channel and is allowed to use IMS data channel and UE determined its home network supports the IMS data channel capability }

ensure that {

when { UE is made to establish IMS Video Call }

then { UE shall include the media feature tag for supported streaming media type in the Contact header field to the remote UE and use +sip.app-subtype="webrtc-datachannel" in the initial INVITE request }

}

(4)

with { UE supporting IMS data channel and is allowed to use IMS data channel and UE determined its home network supports the IMS data channel capability }

ensure that {

when { UE determines to establish the IMS data channels after the establishment of the Video Call }

then { UE shall include the media feature tag for supported streaming media type in the Contact header field to the remote UE and use +sip.app-subtype="webrtc-datachannel" in the re-INVITE request }

}

7.44.2	Conformance Requirements

Same as 7.39.2.

7.44.3	Test description

7.44.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is configured to register for IMS after switch on.

-	The UE is configured to use preconditions.

-	The UE is configured to use IMS data channel.

Preamble:

-	UE is in state 1N-A (TS 38.508-1 [21]) and registered to IMS.

7.44.3.2	Test procedure sequence

Table 7.44.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE attempt an IMS video call. | - | - | - | - |
| 2-7 | Steps 2-7 of generic procedure specified in Table 4.9.24.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| - | EXCEPTION: In parallel with Step 8, parallel behaviour defined in table 7.44.3.2-2 takes place | - | - | - | - |
| 8 | Check: Does UE send INVITE with the first SDP offer containing media lines for audio, video and IMS data channel, or containing media lines for audio and video? (Step 1 of Annex A.15.1a happens) | --> | INVITE | 1,3 | P |
| 9 | SS sends a 100 Trying provisional response. (Step 2 of Annex A.15.1a happens) | <-- | 100 Trying | - | - |
| 10 | SS sends an SDP answer in 183 Session Progress. (Step 3 of Annex A.15.1a happens) | <-- | 183 Session Progress | - | - |
| 11 | UE acknowledges reception of 183 Session Progress. (Step 4 of Annex A.15.1a happens) | --> | PRACK | - | - |
| 12 | SS responds to PRACK. (Step 5 of Annex A.15.1a happens) | <-- | 200 OK | - | - |
| 13 | Step 10 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 [21] is performed. | - | - | - | - |
| - | EXCEPTION: In parallel to steps 14 and 15 below, step16 occurs. | - | - | - | - |
| 14-15 | Steps 11-12 of generic procedure specified in Table 4.9.24.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| 16 | UE sends a second SDP offer in an UPDATE request. (Step 6 of Annex A.15.1a happens) | --> | UPDATE |  |  |
| 17 | SS responds to UPDATE. (Step 7 of Annex A.15.1a happens) | <-- | 200 OK |  |  |
| 18 | SS sends 180 Ringing reliably. (Step 8 of Annex A.15.1a happens) | <-- | 180 Ringing | - | - |
| 19 | UE acknowledges reception of 180 Ringing. (Step 9 of Annex A.15.1a happens) | --> | PRACK | - | - |
| 20 | SS responds to PRACK. (Step 10 of Annex A.15.1a happens) | <-- | 200 OK | - | - |
| 21 | SS responds to INVITE. (Step 11 of Annex A.15.1a happens) | <-- | 200 OK | - | - |
| - | EXCEPTION: Step 22a1 to 22a3 describe behaviour that depends on events happening prior to their execution; the "lower case letter" identifies a step sequence that take place if a specific prior event takes place. | - | - | - | - |
| 22a1-22a3 | IF INVITE includes SDP offer containing media lines for audio, video and IMS data channel in Step8, THEN Steps 1-3 of generic procedure specified in Table 4.9.49.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| 23 | UE acknowledges. (Step 12 of Annex A.15.1a happens) | --> | ACK | 2 | P |
| - | EXCEPTION: Step 24a1 to 24a7 describe behaviour that depends on events happening prior to their execution; the "lower case letter" identifies a step sequence that take place if a specific prior event takes place. | - | - | - | - |
| 24a1 | IF INVITE in step8 does not include SDP offer containing media lines for IMS data channel, THEN Check: Does the UE send re-INVITE with an SDP offer containing media lines for audio and video and IMS data channel? | --> | INVITE | 1,4 | P |
| 24a2 | SS responds with a 100 Trying provisional response | <-- | 100 Trying | - | - |
| 24a3 | SS responds re-INVITE with 200 OK. | <-- | 200 OK | - | - |
| 24a4-24a6 | Steps 1-3 of generic procedure specified in Table 4.9.49.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| 24a7 | Check: Does the UE acknowledge the receipt of 200 OK for re-INVITE? | --> | ACK | 2 | P |
| 25-26 | SS releases the call. (Annex A.8) | - | - | - | - |
| Note 1: SS ignore the DRB including ClientHello for DTLS handshake sent by UE. |  |  |  |  |  |

Table 7.44.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |

7.44.3.3	Specific message contents

Table 7.44.3.3-1: INVITE (step 8, table 7.44.3.2-1)

| Derivation Path: TS 34.229-5, Step 1 of A.15.1a, with following exceptions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Contact feature-param |  | Feature-param is same as TS 34.229-5, Step 1 of A.15.1a except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |  | TS 24.186 [54] |
| Message-body |  | SDP body is same as TS 34.229-5, Step 1 of A.15.1a or same as TS 34.229-5, Step 1 of A.15.1a except adding at least one Media description for IMS bootstrap data channel as follow: Media description for IMS data channel: m=application (transport port) UDP/DTLS/SCTP webrtc-datachannel c=IN (addrtype) (connection-address for UE) [Note 1] Attributes for media for IMS data channel: a=max-message-size:(max-message-size-value) [Note 2] a=sctp-port:(sctp-port-value) [Note 3] a=fingerprint:(hash-func) (fingerprint) [Note 4] a=setup:actpass a=tls-id:(tls-id-value) a=dcmap: (steam id value) subprotocol="http” [Note 5] Note 1: At least one "c=" field shall be present. Note 2: Optional attribute. If the SDP ”max-message-size” attribute is not present the default value is 64K. Note 3: SCTP port range is between 0 and 65535(both included).Leading zeroes must not be used. Note 4: SHA-1,SHA-224, SHA-256, SHA-384, or SHA-512 is used as hash-func. Note 5: Multiple data channels may be mapped to a single data channel SDP media description, each with a corresponding "a=dcmap" SDP attribute and stream IDs that are unique within that media description. |  | TS 26.114 [33] RFC 8841 [55] RFC 8841 [55] RFC 8122 [56] RFC 4145 [59] RFC 8842 [57] RFC 8864 [58] |

Table 7.44.3.3-2: 183 Session Progress (step 10, table 7.44.3.2-1)

| Derivation Path: TS 34.229-5, Step 3 of A.15.1a, with following exceptions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Contact feature-param |  | Feature-param is same as TS 34.229-5, Step 3 of A.15.1a except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |  | TS 24.186 [54] |
| Message-body | A1 | SDP body is same as TS 34.229-5, Step 3 of A.15.1a except adding one Media description as follow: Media description for IMS data channel: m=application (transport port) UDP/DTLS/SCTP webrtc-datachannel [Note 1] b=AS:500 Attributes for media for IMS data channel: a=max-message-size:64000 a=sctp-port:(sctp-port-value) [Note 2] a=setup:passive a=fingerprint: (hash-func) (fingerprint) [Note 3] a=tls-id: (tls-id-value) [Note 4] a=dcmap: (dcmap-stream-id) subprotocol="http" [Note 5] Note 1: Transport port is the port number of the SS. Note 2: The sctp-port-value is the SCTP port of the SS. Note 3: SHA-1,SHA-224, SHA-256, SHA-384, or SHA-512 is used as hash-func. Note 4: The tls-id-value is random number generated by SS. Note 5: The dcmap-stream-id is the same as the dcmap-stream-id in the INVITE message. |  | TS 26.114 [33] |
|  | A2 | SDP body is same as TS 34.229-5, Step 3 of A.15.1a |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | INVITE includes SDP offer containing media lines for audio, video and IMS data channel in Step 8. |
| A2 | INVITE includes SDP offer containing media lines for audio and video in Step 8. |

Table 7.44.3.3-3: UPDATE (step 16, table 7.44.3.2-1)

| Derivation Path: TS 34.229-5, Step 7 of A.15.1a, with following exceptions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Message-body | A1 | SDP body is same as TS 34.229-5, Step 6 of A.15.1a except adding one Media description as follow: Media description for IMS data channel: Same as it in the INVITE message Attributes for media for IMS data channel: Same as it in the INVITE message with following exception: a = setup: active or actpass |  | TS 26.114 [33] |
|  | A2 | SDP body is same as TS 34.229-5, Step 6 of A.15.1a |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | INVITE includes SDP offer containing media lines for audio, video and IMS data channel in Step 8. |
| A2 | INVITE includes SDP offer containing media lines for audio and video in Step 8. |

Table 7.44.3.3-4: 200 OK for UPDATE (step 17, table 7.44.3.2-1)

| Derivation Path: TS 34.229-5, Step 8 of A.15.1a, with following exceptions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Contact feature-param |  | Feature-param is same as TS 34.229-5, Step 7 of A.15.1a except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |  | TS 24.186 [54] |
| Message-body | A1 | SDP body is same as TS 34.229-5, Step 7 of A.15.1a except adding one Media description as follow: Media description for IMS data channel: Same as it in the 183 Session Progress message. Attributes for media for IMS data channel: Same as it in the 183 Session Progress message. |  | TS 26.114 [33] |
|  | A2 | SDP body is same as TS 34.229-5, Step 7 of A.15.1a |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | INVITE includes SDP offer containing media lines for audio, video and IMS data channel in Step8. |
| A2 | INVITE includes SDP offer containing media lines for audio and video in Step8. |

Table 7.44.3.3-4: 200 OK for INVITE (step 21, table 7.44.3.2-1)

| Derivation Path: TS 34.229-5, Step 11 of A.15.1a, with following exceptions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Contact feature-param |  | Feature-param is same as TS 34.229-5, Step 11 of A.15.1a except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |  | TS 24.186 [54] |

Table 7.44.3.3-3: re-INVITE (step 24a1, table 7.44.3.2-1)

| Derivation Path: TS 34.229-1 [2], Annex A.2.1, condition A5 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Contact feature-param |  | Feature-param is same as TS 34.229-1 [2], Annex A.2.1 except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |  | TS 24.186 [aa] |
| Message-body |  | SDP body is same as TS 34.229-5, Step 6 of A.15.1a (UPDATE) with following exception: Attributes for preconditions of audio media: a=curr:qos remote sendrecv Attributes for preconditions of video media: a=curr:qos remote sendrecv Media description for IMS data channel: m=application (transport port) UDP/DTLS/SCTP webrtc-datachannel c=IN (addrtype) (connection-address for UE) [Note 1] Attributes for media for IMS data channel: a=max-message-size:(max-message-size-value) [Note 2] a=sctp-port:(sctp-port-value) [Note 3] a=fingerprint:(hash-func) (fingerprint) [Note 4] a=setup:actpass a=tls-id:(tls-id-value) a=dcmap: (steam id value) subprotocol="http” [Note 5] Note 1: At least one "c=" field shall be present. Note 2: Optional attribute. If the SDP ”max-message-size” attribute is not present the default value is 64K. Note 3: SCTP port range is between 0 and 65535(both included).Leading zeroes must not be used. Note 4: SHA-1,SHA-224, SHA-256, SHA-384, or SHA-512 is used as hash-func. Note 5: Multiple data channels may be mapped to a single data channel SDP media description, each with a corresponding "a=dcmap" SDP attribute and stream IDs that are unique within that media description. |  | TS 26.114 [33] RFC 8841 [55] RFC 8841 [55] RFC 8122 [56] RFC 4145 [59] RFC 8842 [57] RFC 8864 [58] |

Table 7.44.3.3-4: 100 Trying (step 24a2, table 7.44.3.2-1)

Derivation Path: TS 34.229-1 [2], Annex A.2.2, with condition A1.

Table 7.44.3.3-5: 200 OK for re-INVITE (step 24a3, table 7.44.3.2-1)

| Derivation Path: TS 34.229-1 [2], Annex A.3.1, with condition A23 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Contact feature-param |  | Feature-param is same as TS 34.229-1 [2], Annex A.3.1 except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |  | TS 24.186 [54] |
| Content-Type media-type |  | application/sdp |  |  |
| Content-Length value |  | length of message-body |  |  |
| Message-body |  | SDP body is same as TS 34.229-5, Step 7 of A.15.1a (200OK for UPDATE) with follow exception: Media description for IMS data channel: m=application (transport port) UDP/DTLS/SCTP webrtc-datachannel [Note 1] b=AS:500 Attributes for media for IMS data channel: a=max-message-size:64000 a=sctp-port:(sctp-port-value) [Note 2] a=setup:passive a=fingerprint: (hash-func) (fingerprint) [Note 3] a=tls-id: (tls-id-value) [Note 4] a=dcmap: (dcmap-stream-id) subprotocol="http" [Note 5] Note 1: Transport port is the port number of the SS. Note 2: The sctp-port-value is the SCTP port of the SS. Note 3: SHA-1,SHA-224, SHA-256, SHA-384, or SHA-512 is used as hash-func. Note 4: The tls-id-value is random number generated by SS. Note 5: The dcmap-stream-id is the same as the dcmap-stream-id in the INVITE message. |  | TS 26.114 [33] |

Table 7.44.3.3-6: ACK for re-INVITE (step 24a7, table 7.44.3.2-1)

Derivation Path: TS 34.229-1 [2], Annex A.2.7, with condition A5

<End of modified section 1>