3GPP TSG-RAN5 Meeting #108	R5-254432

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5548 | rev | - | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LTE based 5G broadcast RF Rx test cases added |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Technologies Ireland |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_terr_bcast_bands_part2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Adding new test cases as per the LTE 5G broadcast work plan |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new RF Rx test cases for LTE 5G broadcast |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spec not aligned to work plan |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3H.1 (new), 7.4H.1 (new), 7.5H.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of changes >

## 7.3	Reference sensitivity level

< Unchanged sections skipped >

### 7.3H.1	Reference sensitivity requirements for LTE based 5G terrestrial broadcast

Editors Notes: Test procedure and Test requirement are pending

7.3H.1.1 		Test purpose

Measure the reference sensitivity power level REFSENS as the minimum mean power applied to each one of the UE antenna ports at which the throughput shall meet or exceed the requirements for the specified reference measurement channel. The throughput for the REFSENS test is measured based on the Transmission Mode 1 unless specified otherwise.

7.3H.1.2 		Test applicability

This test case applies to all types of E-UTRA UE release 17 and forward and supporting 5G terrestrial broadcast and 0.37 kHz subcarrier spacing.

7.3H.1.3 		Minimum conformance requirements

The throughput shall be ≥ 95% of the maximum throughput as represented by a reported BLER of <5% for the reference measurement channels as specified in Annexes A.3.18 with parameters specified in Table 7.3H.1.3-1.

Table 7.3H.1.3-1: Reference sensitivity for LTE based 5G terrestrial broadcast

| Operating Band | PMCH bandwidth |  |  | Duplex Mode |
| --- | --- | --- | --- | --- |
|  | 6 MHz (dBm) | 7 MHz (dBm) | 8 MHz (dBm) |  |
| 107, 108, 112, 113 | -99.2 | -98.5 | -97.9 | SDO |
| NOTE1: The signal power is specified per port NOTE2: Values in this table apply only to UE implementations that relies on digital filtering according to the configured broadcast channel bandwidth |  |  |  |  |

7.3H.1.4 		Test description

[TBD]

7.3H.1.5 		Test requirement

[TBD]

< Unchanged sections skipped >

## 7.4	Maximum input level

< Unchanged sections skipped >

### 7.4H.1	Minimum input level for LTE based 5G terrestrial broadcast

Editors Notes: Test procedure and Test requirement are pending

7.4H.1.1 		Test purpose

This is defined as the maximum mean power received at the UE antenna port, at which the specified relative throughput shall meet or exceed the minimum requirements for the specified reference measurement channel.

7.4H.1.2 		Test applicability

This test case applies to all types of E-UTRA UE release 17 and forward and supporting 5G terrestrial broadcast and 0.37 kHz subcarrier spacing.

7.4H.1.3 		Minimum conformance requirements

The throughput shall be ≥ 95% of the maximum throughput as represented by a reported BLER of <5% for the reference measurement channels as specified in Annexes A.3.18 with parameters specified in Table 7.4H.1.3-1.

Table 7.4H.1.3-1: Maximum input level for LTE based 5G terrestrial broadcast

| Rx Parameter | Units | PMCH bandwidth |  |  |
| --- | --- | --- | --- | --- |
|  |  | 6 MHz | 7 MHz | 8 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -22 |  |  |
| NOTE: Reference measurement channel is Annex A.3.18 64QAM, R=3/4 variant |  |  |  |  |

7.4H.1.4 		Test description

[TBD]

7.4H.1.5 		Test requirement

[TBD]

< Unchanged sections skipped >

## 7.5	Adjacent Channel Selectivity (ACS)

< Unchanged sections skipped >

### 7.5H.1	Adjacent channel selectivity for LTE based 5G terrestrial broadcast

Editors Notes: Test procedure and Test requirement are pending

7.5H.1.1 		Test purpose

Measure Adjacent Channel Selectivity (ACS) for LTE based 5G terrestrial broadcast in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s).

7.5H.1.2 		Test applicability

This test case applies to all types of E-UTRA UE release 17 and forward and supporting 5G terrestrial broadcast and 0.37 kHz subcarrier spacing.

7.5H.1.3 		Minimum conformance requirements

The UE shall fulfil the minimum requirement specified in Table 7.5H.1.3-1 for all values of an adjacent channel interferer up to –22 dBm. However, it is not possible to directly measure the ACS, instead the lower and upper range of test parameters are chosen in Table 7.5H.1.3-2 and Table 7.5H.1.3-3 where the throughput shall be ≥ 95% of the maximum throughput as represented by a reported BLER of <5% for the reference measurement channels as specified in Annex A.3.18.

Table 7.5H.1.3-1: Adjacent channel selectivity for LTE based 5G terrestrial broadcast

| Rx Parameter | Units | PMCH bandwidth |  |  |
| --- | --- | --- | --- | --- |
|  |  | 6 MHz | 7 MHz | 8 MHz |
| ACS | dB | 29.0 | 30.5 | 31.5 |
| NOTE: Values in this table apply only to UE implementations that relies on digital filtering according to the configured broadcast channel bandwidth |  |  |  |  |

Table 7.5H.1.3-2: Test parameters for Adjacent channel selectivity, Case 1

| Rx Parameter | Units | PMCH bandwidth |  |  |
| --- | --- | --- | --- | --- |
|  |  | 6 MHz | 7 MHz | 8 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |  |
| PInterferer | dBm | REFSENS +41.5dB | REFSENS +43.0dB | REFSENS +44.0dB |
| BWInterferer | MHz | 5 |  |  |
| FInterferer (offset) | MHz | 5.5+0.0125 / -5.5-0.0125 | 6.0+0.0075 / -6.0-0.0075 | 6.5+0.0025 / -6.5-0.0025 |
| NOTE: The interferer consists of the Reference measurement channel specified in Annex A.3.18. |  |  |  |  |

Table 7.5H.1.3-3: Test parameters for Adjacent channel selectivity, Case 2

| Rx Parameter | Units | PMCH bandwidth |  |  |
| --- | --- | --- | --- | --- |
|  |  | 6 MHz | 7 MHz | 8 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -49.5 | -51 | -52 |
| PInterferer | dBm | -22 |  |  |
| BWInterferer | MHz | 5 |  |  |
| FInterferer (offset) | MHz | 5.5+0.0125 / -5.5-0.0125 | 6.0+0.0075 / -6.0-0.0075 | 6.5+0.0025 / -6.5-0.0025 |
| NOTE: The interferer consists of the Reference measurement channel specified in Annex A.3.18. |  |  |  |  |

7.5H.1.4 		Test description

[TBD]

7.5H.1.5 		Test requirement

[TBD]

< End of changes >