3GPP TSG-RAN5 Meeting #108	R5-254590

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5126 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 12.2.11.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of new test case 12.2.11.2 Inter-concurrent operation / DRX / Reception. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new test case 12.2.11.2 Inter-concurrent operation / DRX / Reception. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification remains incomplete compared to work plan coverage. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.2.11.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-2 CR 0623 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changed Section>

12.2.11.2	Inter-concurrent operation / DRX / Reception

12.2.11.2.1	Test Purpose (TP)

(1)

with { UE having established PC5 RRC connection with peer UE }

ensure that {

when { UE is configured by higher layer to inform its peer UE of the sidelink DRX assistance information for unicast communication and the sidelink DRX assistance information has not been sent previously }

then { UE sends an UEAssistanceInformationSidelink message with sl-PreferredDRX-ConfigList-r17 to indicate the reference sidelink DRX configurations }

}

(2)

with { UE having established PC5 RRC connection with peer UE and has sent an UEAssistanceInformationSidelink message }

ensure that {

when { UE receives RRCReconfigurationSidelink message including the sl-DRX-ConfigUC-PC5 }

then { UE sends an RRCReconfigurationCompleteSidelink message }

}

12.2.11.2.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 38.331 clause 5.8.9.6.1, 5.8.9.6.2, 5.8.9.1.1 and 5.8.9.1.3. Unless otherwise stated these are Rel-17 requirements.

[TS 38.331, clause 5.8.9.6.1]

Figure 5.8.9.6.1-1: Sidelink UE assistance information

The purpose of this procedure is for a UE to inform its peer UE of the sidelink DRX assistance information used to determine the sidelink DRX configuration for unicast communication.

For sidelink unicast, a UE may include its desired sidelink DRX configurations in the UEAssistanceInformationSidelink as the sidelink DRX assistance information which is transmitted to its peer UE.

NOTE:	It is up to UE implementation to determine its desired sidelink DRX configurations for unicast communication.

[TS 38.331, clause 5.8.9.6.2]

For sidelink unicast, if both a RX UE and its peer TX UE for a direction of sidelink communication are capable of sidelink DRX, the RX UE that is interested in sending the sidelink DRX assistance information may send the UEAssistanceInformationSidelink as the sidelink DRX assistance information to its peer UE when the sidelink DRX assistance information has not been sent previously or when the previously transmitted sidelink DRX assistance information has changed.

[TS 38.331, clause 5.8.9.1.1]

Figure 5.8.9.1.1-1: Sidelink RRC reconfiguration, successful

The UE may initiate the sidelink RRC reconfiguration procedure and perform the operation in clause 5.8.9.1.2 on the corresponding PC5-RRC connection in following cases:

…

-	the (re-)configuration of the peer UE to perform sidelink DRX;

[TS 38.331, clause 5.8.9.1.3]

The UE shall perform the following actions upon reception of the RRCReconfigurationSidelink:

…

1>	if the RRCReconfigurationSidelink message includes the sl-DRX-ConfigUC-PC5; and

1>	if the UE accepts the sl-DRX-ConfigUC-PC5:

2>	configure lower layers to perform sidelink DRX operation according to sl-DRX-ConfigUC-PC5 for the associated destination as defined in TS 38.321 [3];

…

1>	else:

2>	set the content of the RRCReconfigurationCompleteSidelink message;

3>	if the UE rejects the sidelink DRX configuration sl-DRX-ConfigUC-PC5 received from the peer UE:

4>	include the sl-DRX-ConfigReject in the RRCReconfigurationCompleteSidelink message;

4>	consider no sidelink DRX to be applied for the corresponding sidelink unicast communication;

3>	submit the RRCReconfigurationCompleteSidelink message to lower layers for transmission;

NOTE 1:	When the same logical channel is configured with different RLC mode by another UE, the UE handles the case as sidelink RRC reconfiguration failure.

NOTE 2: It is up to the UE implementation whether or not to indicate the rejection to the peer UE for a received sidelink DRX configuration.

12.2.11.2.3	Test Description

12.2.11.2.3.1	Pre-test conditions

System Simulator:

-	NR-SS-UE

-	NR-SS-UE1 operates as NR sidelink communication device on the resources that UE is expected to use for transmission and reception via PC5 interface.

-	NR-SS-UE1 uses GNSS as the synchronization reference source.

-	GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

- UE is authorised to perform NR sidelink communication.

- The UE is equipped with a USIM containing default values as per TS 38.508-1 [4] clause 4.8.3.3.3 except for those listed in Table 12.2.11.2.3.1-1.

- The UE uses GNSS as the synchronization reference source.

Table 12.2.11.2.3.1-1: UE/ USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFUST |  | service no. 119 V2X is available |  |
| EFVST |  | Service no.2 V2X policy configuration data over PC5 is supported, i.e. value is '01 02' HEX |  |
| EFV2XP_PC5 |  | SL-PreconfigurationNR field as defined in 38.508-1 [4] Table 4.10.1-1 |  |

Preamble:

-	The UE is in state 4-A as defined in TS 38.508-1 [4], subclause 4.4A, using generic procedure parameter Sidelink (On), test mode (On) and GNSS Sync (On) as defined in TS 38.508-1 [4], subclause 4.5.1 using UE initiated unicast mode NR sidelink communication procedure in subclause 4.9.22.

12.2.11.2.3.2	Test procedure sequence

Table 12.2.11.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | To trigger the user initiates sending the sidelink DRX assistance information by MMI. | - | - | - | - |
| 2 | Check: Does the UE transmit an UEAssistanceInformationSidelink message. | --> | PC5-RRC: UEAssistanceInformationSidelink | 1 | P |
| 3 | The NR-SS-UE1 transmits an RRCReconfigurationSidelink message? | <-- | PC5-RRC: RRCReconfigurationSidelink | - | - |
| 4 | Check: Does the UE transmit an RRCReconfigurationCompleteSidelink message. | --> | PC5-RRC: RRCReconfigurationCompleteSidelink | 2 | P |

12.2.11.2.3.3	Specific message contents

Table 12.2.11.2.3.3-1: UEAssistanceInformationSidelink (Step 3, Table 12.2.11.2.3.2-1)

Derivation path: TS 38.508-1 [4], Table 4.6.1A-9 with condition TX

Table 12.2.11.2.3.3-2: RRCReconfigurationSidelink (Step 3, Table 12.2.11.2.3.2-1)

Derivation path: TS 38.508-1 [4], Table 4.6.1A-3 with condition RX and SL_DRX

Table 12.2.11.2.3.3-3: RRCReconfigurationCompleteSidelink (Step 4, Table 12.2.11.2.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1A-4 with condition TX

<End of Changed Section>