3GPP TSG-RAN5 Meeting #108 	R5-254679

# Bengaluru, India, 25th August 2025 – 29th August  2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 4102 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test Tolerance update for RACH-less Soft Satellite switching for Satellite Access test 14.2.2.5. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Korea |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Test tolerance update for test case 14.2.2.5 needs to be updated |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Test tolerance is added for test cases 14.2.2.5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Test case implementation will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.903 CR 1079 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

### 14.2.2	RRC Connection Mobility Control for SAN

<< Skipped sections >>

#### 14.2.2.5	NR SA FR1 RACH-less Soft Satellite switching with re-synchronization for Satellite Access

14.2.2.5.1	Test purpose

This test is to verify the requirement for RACH-less soft satellite switching with re-synchronization from SAN FR1 to SAN FR1 specified in clause 14.2.2.5.4.

14.2.2.5.2	Test applicability

This test applies to all types of NR UE release 18 and forward supporting satellite access and soft satellite switching with re-synchronization.

14.2.2.5.3	Minimum conformance requirement

The requirements in this clause apply provided that UE has the valid and applicable parameters of ephemeris information, common TA, DL and UL Polarization information, Koffset, and Kmac for target NR SAN cell during Dswitch_unchangedPCI, otherwise interruption time may be longer than the requirements in clause 14.2.2.5.3.2.

Requirements for soft satellite switching are applicable for UEs that support softSatelliteSwitchResyncNTN-r18 [11] when network configures soft satellite switching with resynchronization [13].

14.2.2.5.3.1	Satellite switching delay

When the UE receives a broadcast message implying satellite switching within re-synchronization, the UE shall be ready to start the transmission of the new uplink PRACH channel or transmission of the new uplink PUSCH channel within Dswitch_unchangedPCI msec.

Where:

-	Dswitch_unchangedPCI equals to the interruption time stated in clause 14.2.2.5.3.2.

14.2.2.5.3.2	Interruption time for hard satellite switch with re-sync

The Satellite switch delay is from t-serviceStart to the time instance when UE is ready to receive any DL channel/signal and transmit any UL channel/signal from/to the target satellite.

When intra-frequency soft switch to NR SAN cell is commanded,

the satellite switch time shall be less than Tsoft_switch

Tsoft_switch = max(t-service - t-seviceStart, Tsearch + T∆ + Tmargin) + Tprocessing  ms

Where:

-	Tsearch is the time required to search the target NR SAN cell assuming the target cell is not already known when the handover command is received by the UE. If the target cell Es/Iot ≥ -2 dB, then Tsearch = Tfirst_SSB ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

-	T∆ is same as the one defined in TS 38.133 [6] section 6.1C.2.2.2.1.

-	Tprocessing is time for UE processing. Tprocessing can be up to 10 ms.

-	Tmargin is same as the one defined in TS 38.133 [6] section 6.1C.2.2.2.1.

-	Tfirst_SSB is the time to the end of the first complete SSB burst of target satellite, the location of which is determined by the periodicity and location of SSB of the source satellite, the ssb-TimeOffset and the difference between propagation delay of the serving satellite and the target satellite counted from SSB-TimeOffset reference point as defined in TS 38.331 [13] to UE.

During the time period from t-seviceStart to t-service, scheduling restriction as defined in TS 38.133 [6] clause 9.2C.5.3 is allowed, with the exception that the locations of SSB symbols of target satellite where scheduling restriction applies are determined by the periodicity and location of SSB of the source satellite, the ssb-TimeOffset and the difference between propagation delay of the serving satellite and the target satellite counted from the SSB-TimeOffset reference point as defined in TS 38.331 [13] to UE.

UE is allowed to skip measurements for other cells and satellites than the target satellite and source satellite from t-seviceStart to the satellite switch completion.

The requirement in this clause applies and UE is required to keep the connection (DL and UL) with the source NGSO satellite, under the following conditions:

-	SSBs from the two satellites are spaced apart from each other at least by 1 OFDM symbol in the time domain at UE Rx side.

The normative reference for this requirement is TS 38.133 [6] clause 6.1C.3.

14.2.2.5.4	Test description

The test scenario comprises of 1 NR FDD carrier and 2 cells with same PCI as given in Tables 14.2.2.5.4.1-1, 14.2.2.5.4.1-3, 14.2.2.5.5-1 and 14.2.2.5.5-2. Satellite switching delay is tested.

The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively.

At the start of time duration T1, the UE may not have any timing information of Cell 2. During T1, The SIB19 implying t-service-r17 and target satellite configuration SatSwitchWithReSync-r18 shall be sent to UE. The target satellite configuration is in table 14.2.2.5.5-1. The configured grant PUSCH transmission in the Cell 2 is configured in the RRC message from Cell 1.

At the start of time duration T2, Cell 2 becomes detectable and t-ServiceStart-r18 is fulfilled.

At the start of time duration T3, t-service-r17 of Cell 1 is fulfilled.

The normative reference for this requirement is TS 38.133 [6] clause A.14.2.2.5.

14.2.2.5.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 14.2.2.5.4.1-1.

Table 14.2.2.5.4.1-1: Supported test configurations for NR SA FR1 RACH-less Soft Satellite switching with re-synchronization for Satellite Access

| Configuration | Description |
| --- | --- |
| 14.2.2.5-1 | NGSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |

Configure the test equipment and the DUT according to the parameters in Table 14.2.2.5.4.1-2.

Table 14.2.2.5.4.1-2: Initial conditions for NR SA FR1 RACH-less Soft Satellite switching with re-synchronization for Satellite Access

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.12-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 14.2.2.5.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 14.2.2.5.4.3.

2.	The power levels and settings for Cell 1 and Cell 2 are set according to Annex C.1.2 and C.1.3. Both Cell 1 and Cell 2 are satellite access cells.

3.	The test parameters are given in Table 14.2.2.5.4.1-3.

4.	The initial test environment conditions are setup according to section 14.0.5.

Table 14.2.2.5.4.1-3: General test parameters for NR SA FR1 RACH-less Soft Satellite switching with re-synchronization for Satellite Access

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1 | One NR NTN satellite RF channel |
| Initial conditions | Active cell |  | Cell 1 |  |
| Final condition | Active cell |  | Cell 2 |  |
| UE position (N,S, H) |  |  | (0, 0, 0) | Set by AT command |
| Access Barring Information |  | - | Not barred | No additional delays in random access procedure. |
| timeDomainOffset |  |  | 0 |  |
| timeDomainAllocation |  |  | 0 | PUSCH MappingType A startSymbol S=0 Length L=14 |
| timeReferenceSFN-r16 |  |  | sfn512 |  |
| Periodcity |  |  | sym10x14 |  |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | ms | 100 |  |
| T3 |  | s | 5 |  |

14.2.2.5.4.2	Test Procedure

In this test Cell 1 and Cell 2 have the same physical cell identity. This identity does not change across iterations.

1.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 14.2.2.5.5-1. Propagation conditions are set according to Annex C clause C.2.2. Cell 1 broadcasts SIB19 according to Table 14.2.2.5.4.3-2. T1 starts.

3.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 14.2.2.5.5-1. Cell 2 becomes detectable and t-ServiceStart-r18 is fulfilled on Cell 2, however t-Service-r17 on Cell 1 is not yet fulfilled.

4.	When T2 expires, the SS shall switch the power setting from T2 to T3 as specified in Table 14.2.2.5.5-1. At the same time, t-Service-r17 of Cell 1 expires, causing the UE to trigger a satellite switch.

5.	The UE shall start satellite switching on Cell 2. If the UE transmits the uplink PUSCH channel to Cell 2 less than 130 ms from the beginning of time period T2 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

6.	After T3 expires, the SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

7.	Once the connection is released, the SS shall switch off and then on the UE.

8.	Repeat steps 1-7 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

14.2.2.5.4.3	Message Contents

Message contents are according to TS 38.508-1 [14] clauses 4.6.1 and 7.3 with the following exceptions:

Table 14.2.2.5.4.3-1: Common Exception messages for NR SA FR1 RACH-less Soft Satellite switching with re-synchronization for Satellite Access

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | 38.508-1 [14] Table 4.6.3-15 with condition CG_Config |

Table 14.2.2.5.4.3-2: SIB19 for NR SA FR1 RACH-less Soft Satellite switching with re-synchronization for Satellite Access

| Derivation Path: 38.508-1 Table 7.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17::= SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ntn-UlSyncValidityDuration-r17 | s5 |  |  |
| cellSpecificKoffset-r17 | 14 |  |  |
| kmac-r17 | Not present |  |  |
| ta-Info-r17 SEQUENCE { |  |  |  |
| ta-Common-r17 | 0 |  |  |
| ta-CommonDrift-r17 | 0 |  |  |
| ta-CommonDriftVariant-r17 | 0 |  |  |
| } |  |  |  |
| ntn-PolarizationDL-r17 | linear |  |  |
| ntn-PolarizationUL-r17 | linear |  |  |
| ephemerisInfo-r17 | EphemerisInfo-r17 | 38.508-1 Clause 7.6.2 |  |
| ta-Report-r17 | Not present |  |  |
| } |  |  |  |
| t-service-r17 | 100*T | T is the time difference in seconds between end of T2 and the 00:00:00 UTC on Gregorian calendar date 1 January, 1900. Units in 10ms |  |
| ntn-NeighCellConfig-r17 | Not present |  |  |
| satSwitchWithResync-r18 | SatSwitchWithReSync-r18 | Table 14.2.2.4.4.3-3 |  |
| } |  |  |  |

Table 14.2.2.5.4.3-3: SatSwitchWithReSync-r18 for NR SA FR1 RACH-less Soft Satellite switching with re-synchronization for Satellite Access

| Derivation Path: 38.508-1 Table 4.6.2-18C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SatSwitchWithReSync-r18 ::= SEQUENCE { |  |  |  |
| ntn-Config-r18::= SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ntn-UlSyncValidityDuration-r17 | s5 |  |  |
| cellSpecificKoffset-r17 | 14 |  |  |
| kmac-r17 | Not present |  |  |
| ta-Info-r17 SEQUENCE { |  |  |  |
| ta-Common-r17 | 0 |  |  |
| ta-CommonDrift-r17 | 0 |  |  |
| ta-CommonDriftVariant-r17 | 0 |  |  |
| } |  |  |  |
| ntn-PolarizationDL-r17 | linear |  |  |
| ntn-PolarizationUL-r17 | linear |  |  |
| ephemerisInfo-r17 | EphemerisInfo-r17 | 38.508-1 Clause 7.6.3.2 |  |
| ta-Report-r17 | Not present |  |  |
| } |  |  |  |
| t-ServiceStart-r18 | 100*T | T is the time difference in seconds between end of T1 and the 00:00:00 UTC on Gregorian calendar date 1 January, 1900. Units in 10ms |  |
| ssb-TimeOffset-r18 | 10 |  |  |
| } |  |  |  |

Table 14.2.2.5.4.3-4: ConfiguredGrantConfig for NR SA FR1 RACH-less Soft Satellite switching with re-synchronization for Satellite Access

| Derivation Path: TS 38.508-1 [14], clause 4.6.3-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConfiguredGrantConfig ::= SEQUENCE { |  |  |  |
| periodicity | sym10x14 |  |  |
| rrc-ConfiguredUplinkGrant SEQUENCE { |  |  |  |
| timeDomainOffset | 0 |  |  |
| timeDomainAllocation | 0 |  |  |
| timeReferenceSFN-r16 | sfn512 |  |  |
| } |  |  |  |

Table 14.2.2.5.4.3-5: MAC-CellGroupConfig for NR SA FR1 RACH-less Soft Satellite switching with re-synchronization for Satellite Access

| Derivation Path: TS 38.508-1, Table 4.6.3-68 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-CellGroupConfig ::= SEQUENCE { |  |  |  |
| tag-Config SEQUENCE { |  |  |  |
| tag-ToReleaseList | Not present |  |  |
| tag-ToAddModList SEQUENCE (SIZE (1..maxNrofTAGs)) OF SEQUENCE { | 1 entry |  |  |
| tag-Id[1] | 0 |  |  |
| timeAlignmentTimer[1] | infinity |  |  |
| } |  |  |  |
| } |  |  |  |

14.2.2.5.5	Test Requirement

Table 14.2.2.5.5-1 defines the cell specific primary level settings including test tolerances for NR SA FR1 RACH-less Soft Satellite switching with re-synchronization for Satellite Access.

Table 14.2.2.5.5-1: Cell-specific test parameters for NR SA FR1 RACH-less Soft Satellite switching with re-synchronization for Satellite Access

| Parameter |  | Unit | Cell 1Note1 |  |  | Cell 2Note1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Satellite configurationNote2 |  |  | SSC.2 |  | N/A | N/A | SSC.2 |  |
| BWchannel |  | MHz | 10: NPRB,c = 52 |  |  |  | 10: NPRB,c = 52 |  |
| BWP BW |  | MHz | 10: NPRB,c = 52 |  |  |  | 10: NPRB,c = 52 |  |
| Kmac |  | ms | 0 |  |  |  | 0 |  |
| DRX Cycle |  | ms | Not Applicable |  |  |  | Not Applicable |  |
| PDSCH Reference measurement channel |  |  | SR.1.1 FDD |  |  |  | SR.1.1 FDD |  |
| CORESET Reference Channel |  |  | CR.1.1 FDD |  |  |  | CR.1.1 FDD |  |
| TRS configuration |  |  | TRS.1.1 FDD |  |  |  | TRS.1.1 FDD |  |
| OCNG Patterns |  |  | OP.1 |  |  |  | OP.1 |  |
| SMTC Configuration |  |  | SMTC.1 |  |  |  | SMTC.5 |  |
| SSB Configuration |  |  | SSB.1 FR1 |  |  |  | SSB.5 FR1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 15 kHz |  |  |  | 15 kHz |  |
| PUCCH/PUSCH subcarrier spacing |  | kHz | 15 kHz |  |  |  | 15 kHz |  |
| PRACH configuration |  |  | FR1 PRACH configuration 1 |  |  |  | N/A |  |
| BWP configuration | Initial DL BWP |  | DLBWP.0.1 |  |  |  | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  | DLBWP.1.1 |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |  |  | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  | ULBWP.1.1 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/ 15 kHz | -98 |  |  |  |  |  |
| Note2 |  | dBm/ SCS | -98 |  |  |  |  |  |
|  |  | dB | 4 | -5.51 | -Infinity | -Infinity | 3.54 | 9 |
|  |  | dB | 4 | 4 | -Infinity | -Infinity | 9 | 9 |
| SSB_RP |  | dBm/ SCS | -94 | -94 | -Infinity | -Infinity | -89 | -89 |
| IoNote3 |  | dBm/ 9.36 MHz | -64.59 | -59.46 | -60.53 | -64.59 | -59.46 | -60.53 |
| Propagation condition |  | - | AWGN |  |  |  |  |  |
| NOTE 1: Cell 1 and Cell 2 have same PCI. Satellite serving for Cell 1 and Satellite serving for Cell 2 are two different NGSO satellites. NOTE 2: SSB transmit timing from TE should fit the SSB-timeOffset and the nominal propagation delay difference between serving satellite and target satellite. The nominal propagation delay is counted from the SSB-TimeOffset reference point to UE, which based on satellite locations and UE location known to the TE in this test case. NOTE 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 4: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 5: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |  |  |  |  |  |

Table 14.2.2.5.5-2: Target Satellite configuration pattern for NR SA FR1 RACH-less Soft Satellite switching with re-synchronization for Satellite Access

| Parameter | TSC.2 |
| --- | --- |
| Interval between adjacent epoch time | 2.56 s |
| ntn-UlSyncValidityDuration | 5 s |
| cellSpecificKoffset | 14 slots |
| ta-Common | 0 |
| ta-CommonDrift | 0 |
| ta-CommonDriftVariant | 0 |
| ntn-PolarizationDL | linear |
| ntn-PolarizationUL | linear |
| ephemerisInfo | Detailed ephemeris information is provided in TS 38.508-1 [14] |
| ssb-TimeOffset | 10 |
| t-ServiceStart | T2 |

The UE shall start to transmit the PUSCH to Cell 2 less than 130 ms from the beginning of time period T2.

The rate of correct handovers observed during repeated tests shall be at least 90 %.

NOTE:	The satellite switch with re-sync delay Dswitch_unchangedPCI can be expressed as: Tsoft_switch, where:

Tsoft_switch = max(t-service - t-seviceStart, Tsearch + T∆ + Tmargin) + TIU + Tprocessing  ms

Here: t-service - t-seviceStart= 100ms; Tsearch = 10.5ms; T∆ = 20ms; Tmargin = 2ms, Tprocessing = 10ms.

Besides, interruption uncertainty TIU = 20ms in acquiring the first configured grant based PUSCH transmission resource is needed.

This gives a total of 130 ms.

<< End of Changes >>