Title:	Test Tolerances for Intra frequency handover case for UE Category 1bis

Source:	Bureau Veritas

# 1.	Introduction

This discussion paper relates to the intra frequency E-UTRAN handover test cases 5.1.19 and 5.1.20 in TS 36.521-3.

The test cases in question are:

- 5.1.19, E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis

- 5.1.20, E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis

This Tdoc describes the process to derive the Test Tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. Test cases in TS 36.133

The test conditions are defined in the following extracts from TS 36.133 v15.1.0 .

A.5.1.19	E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis

A.5.1.19.1	Test Purpose and Environment

This test is to verify the requirement for the FDD-FDD intra frequency handover requirements for UE category 1bis specified in clause 5.1.2.1.

The test scenario comprises of 1 E-UTRA FDD carrier and 2 cells as given in tables A.5.1.19.1-1 and A.5.1.19.1-2. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE may not have any timing information of cell 2.

E-UTRAN shall send a RRC message implying handover to cell 2. The RRC message implying handover shall be sent to the UE during period T2, after the UE has reported Event A3. T3 is defined as the end of the last TTI containing the RRC message implying handover.

Table A.5.1.19.1-1: General test parameters for E-UTRAN FDD-FDD intra frequency handover test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| E-UTRA RF Channel Number |  |  | 1 | Only one FDD carrier frequency is used. |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  |  | OFF |
| CP length |  |  | Normal |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| PRACH configuration |  |  | 4 | As specified in table 5.7.1-2 in TS 36.211 |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

Table A.5.1.19.1-2: Cell specific test parameters for E-UTRAN FDD-FDD intra frequency handover test case

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 |  | T2 |  |  | T3 |  |
| E-UTRA RF Channel Number |  | 1 |  |  | 1 |  |  |  |  |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  |  |  |  |  |
| PDSCH Reference Measurement Channel in clause A.3.1.1.1 |  | R.0 FDD | R.0 FDD | - | - | - |  |  | R.0 FDD |  |  |
| PCFICH/PDCCH/PHICH Reference Channel in clause A.3.1.2.1 |  | R.6 FDD |  |  | R.6 FDD |  |  |  |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 and in A.3.2.1.2 |  | OP.1 FDD | OP.1 FDD | OP.2 FDD | OP.2 FDD | OP.2 FDD |  |  | OP.1 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  |  |  |  |  |  |  |  |  |
|  | dB | 8 | 8 | 8 | - Infinity |  |  | 12 |  |  | 12 |
|  | dB | 8 | -4.27 | -4.27 | -Infinity |  |  | 3.36 |  |  | 3.36 |
| RSRP Note 3 | dBm/15 KHz | -90 | -90 | -90 | - Infinity |  | -86 |  |  | -86 |  |
| Io Note 3 | dBm/9MHz | -61.58 | - 56.57 | - 56.57 | Specified in columns for Cell 1 |  |  |  |  |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |  |  |  |  |
| Antenna Configuration |  | 1x1 |  |  | 1x1 |  |  |  |  |  |  |
| Timing offset to Cell 1 Asynchronous cells | ms | - |  |  | 3 |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |

A.5.1.19.2	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 50 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 15 ms and is specified in clause 11.2 in TS 36.331 [2].

Tinterrupt = 35 ms in the test; Tinterrupt is defined in clause 5.1.2.1.2.

This gives a total of 50 ms.

A.5.1.20	E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis

A.5.1.20.1	Test Purpose and Environment

This test is to verify the requirement for the TDD-TDD intra frequency handover requirements for UE category 1bis specified in clause 5.2.2.4.

The test scenario comprises of 1 E-UTRA TDD carrier and 2 cells as given in tables A.5.1.20.1-1 and A.5.1.20.1-2. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE may not have any timing information of cell 2.

E-UTRAN shall send a RRC message implying handover to cell 2. The RRC message implying handover shall be sent to the UE during period T2, after the UE has reported Event A3. T3 is defined as the end of the last TTI containing the RRC message implying handover.

Table A.5.1.20.1-1: General test parameters for E-UTRAN TDD-TDD Intra frequency handover test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| E-UTRA RF Channel Number |  |  | 1 | Only one TDD carrier frequency is used. |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  |  | OFF |
| CP length |  |  | Normal |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211 |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211 |
| PRACH configuration index |  |  | 53 | As specified in table 5.7.1-3 in TS 36.211 |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

Table A.5.1.20.1-2: Cell specific test parameters for E-UTRAN TDD-TDD Intra frequency handover test case

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 |  |  | T2 |  | T3 |  |
| E-UTRA RF Channel Number |  | 1 |  |  | 1 |  |  |  |  |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  |  |  |  |  |
| PDSCH Reference Measurement Channel in clause A.3.1.1.2 |  | R.0 TDD | R.0 TDD | - | - | - |  |  | R.0 TDD |  |  |
| PCFICH/PDCCH/PHICH Reference Channel in clause A.3.1.2.2 |  | R.6 TDD |  |  | R.6 TDD |  |  |  |  |  |  |
| OCNG Patterns defined in A.3.2.2.1 and in A.3.2.2.2 |  | OP.1 TDD | OP.1 TDD | OP.2 TDD | OP.2 TDD | OP.2 TDD |  |  | OP.1 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  |  |  |  |  |  |  |  |  |
|  | dB | 8 | 8 | 8 | - Infinity |  | 12 |  |  |  | 12 |
|  | dB | 8 | -4.27 | -4.27 | -Infinity |  | 3.36 |  |  |  | 3.36 |
| RSRP Note 3 | dBm/15 KHz | -90 | -90 | -90 | - Infinity |  |  | -86 |  | -86 |  |
| Io Note 3 | dBm/9MHz | -61.58 | - 56.57 | - 56.57 | Specified in columns for Cell 1 |  |  |  |  |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |  |  |  |  |
| Antenna Configuration |  | 1x1 |  |  | 1x1 |  |  |  |  |  |  |
| Timing offset to Cell 1 Asynchronous cells | s | - |  |  | 3 |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |

A.5.1.20.2	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 50 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 15 ms and is specified in clause 11.2 in TS 36.331 [2].

Tinterrupt = 35 ms in the test; Tinterrupt is defined in clause 5.2.2.4.2.

This gives a total of 50 ms.

We note that the level and time parameters for FDD and TDD respectively in tables A.5.1.19.1-2 and A.5.1.20.1-2 are identical, so the same treatment for Test Tolerances can be applied for both Test cases. The TDD test case A.5.1.20 is synchronous, meaning that the two cells are time offset by only 3 s. Symbols therefore overlap, and intra-frequency interference has to be considered for TDD. Taking this into account will ensure that the proposed solution can be applied to both FDD and TDD test cases.

# 3. Discussion

The test case has 3 time periods: T1, T2 and T3. To analyse the test case it is helpful to visualise these as pie-charts, showing the distribution of power, as below:

During T1, only cell 1 is present. The nominal Ês/Iot and RSRP will be +8dB and -90 dBm/15kHz respectively.

During T2 and T3, the RSRP ratio for cell 2 compared to cell 1 is calculated as Cell 2 power / Cell 1 power and Cell 2 is 4dB better than Cell 1, so with nominal signal levels the measurement Event A3 with threshold of 0dB is easily met.

However, uncertainties would make the test verdict unreliable. Test Tolerances therefore need to be applied.

# 4. Calculation of Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which original or derived parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters need to be copied from Tables 5.1.19.5-1 and 5.1.20.5-1 in TS 36.521-3. The key parameters are selected as the minimum set to define the cell power levels.

The key parameters appear in section a) of the accompanying spreadsheet. The AWGN used to model the other cells is given a separate set of columns. This allows the spreadsheet calculations to be done in a consistent way.

b) Derived parameters

A number of derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

In these test cases, at the start of T2 Cell 2 becomes detectable, and the UE is expected to detect and send an event triggered measurement report. The criterion is Event A3: Neighbour becomes amount of offset better than serving. The UE makes a measurement of Cell 2 (neighbour) relative to Cell 1 (serving), and compares it to the signalled A3 offset. So further parameter to calculate is RSRP power difference:

- (Cell 2 RSRP – Cell 1 RSRP)

c) Uncertainties

The SS provides AWGN and 2 intra-frequency cells to the UE. We propose to control the following parameters:

- AWGN absolute power, Noc ±1.0 dB

- Ratio of cell 1 signal / AWGN, Ês1 / Noc ±0.3 dB

- Ratio of cell 2 signal / AWGN, Ês2 / Noc ±0.3 dB

This choice forms a minimum set, so the superposition principle can be applied if necessary. For these test cases the signals are not faded, and the values for Ês / Noc are chosen to be similar to equivalent parameters in W-CDMA. The absolute level of Noc is not critical, and is specified as ±1.0 dB.

In addition the UE RSRP relative measurement accuracy needs to be considered, because when the UE makes the (Cell 2 RSRP – Cell 1 RSRP) comparison with the A3 offset, it has some inaccuracy which could affect the test verdict. The UE measurement accuracy is defined in the following extract from TS 36.133 v15.1.0 :

9.1.2.8	Relative Accuracy of RSRP for UE Category 1bis

The relative accuracy of RSRP is defined as the RSRP measured from one cell compared to the RSRP measured from another cell on the same frequency.

The accuracy requirements in Table 9.1.2.8-1 are valid under the following conditions:

Cell specific reference signals are transmitted either from one, two or four antenna ports.

Conditions defined in 36.101 Clause 7.3 for reference sensitivity are fulfilled.

RSRP1,2|dBm according to Annex B.3.8 for a corresponding Band.

Table 9.1.2.8-1: RSRP Intra frequency relative accuracy for UE category 1bis

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 2 | Io Note 1 range |  |  |
|  |  |  | E-UTRA operating band groups Note 5 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 4 | dBm/BWChannel |
| 3 | 4 | -3 dB | FDD_A, TDD_A | -121 | -50 |
|  |  |  | FDD_B1 | -120.5 | -50 |
|  |  |  | FDD_C, TDD_C | -120 | -50 |
|  |  |  | FDD_D | -119.5 | -50 |
|  |  |  | FDD_E, TDD_E | -119 | -50 |
|  |  |  | FDD_F | -118.5 | -50 |
|  |  |  | FDD_G | -118 | -50 |
|  |  |  | FDD_H | -117.5 | -50 |
|  |  |  | FDD_N | -114.5 | -50 |
| 4 | 4 | -6 dB | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter Ês/Iot is the minimum Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

The value of +/-4dB is selected because the test is carried out in normal conditions, and during T2, T3 the Ês/Iot of Cell 1 is > -6 dB, but not > -4 dB.

A similar set of conditions and accuracies apply for TDD.

d) Controlled parameters critical to verdict

The 5 controlled parameters listed in the accompanying spreadsheet have been derived by study of the test case and the relevant clauses in TS 36.133. The reason for each parameter being critical to the test verdict is given briefly in the “Comment” column of section d) in the accompanying spreadsheet.

In general all the values are listed for each time interval T1, T2 and T3 but as cell 2 is not present during T1 those entries are greyed out.

Having identified the parameters which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is just the ratio (effect on a critical parameter y / a test equipment uncertainty x), and is usually in dB/dB. Often it can be derived by inspection as one or zero. For example, an error of 1dB in the absolute AWGN level Noc would cause 1dB error in the Cell 1 RSRP, so the sensitivity factor is 1.000 during T1, T2 and T3. However the same error of 1dB in the absolute AWGN level Noc would cause no change to Cell 2 Ês/Iot, because all other powers are specified relative to Noc, so the sensitivity factor is zero.

In some cases, the sensitivity factor is an intermediate value. For example, the Cell 2 Ês/Noc has an effect on Cell 1 Ês/Iot which depends on ratios of the powers making up the total. In such cases a sensitivity factor value between 0 and 1 results. It is important to calculate these correctly to obtain the overall uncertainty.

For example,

- During T3, the effect of Cell 2 Ês/Noc uncertainty on Cell 1 Ês/Iot  is x 0.941

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, taking into account the uncertainties and sensitivity factors for each parameter that can be set by the test equipment, and the UE RSRP relative measurement accuracy where relevant. This process follows the superposition principle. Uncertainties are calculated separately for T1, T2 and T3.

For the test system uncertainties the normal procedure of combining uncorrelated uncertainties root-sum-square is followed.

When the test system uncertainties and the UE RSRP reporting accuracy are combined, the (root-sum square of test system uncertainties) is added arithmetically to the UE RSRP reporting accuracy. If all the uncertainties were combined root-sum square, the resulting smaller test limits could case a conformant test system to fail a conformant UE, which would be unacceptable.

e) Determine parameters to offset

During T1 the UE is connected to Cell 1. The uncertainties do not take Cell 1 Ês/Iot or RSRP and SCH power outside the allowed range, therefore no offsets are required.

During T2 and T3, the UE performs handover to Cell 2. The UE measures the difference in power between Cell 2 and Cell 1, compares it to the threshold A3, and sends a measurement report. We need to consider the power level conditions and uncertainties to make sure a good UE will send the measurement report.

the Event A3 measurement report trigger requirement in TS 36.331 clause 5.5.4.4 is “Event A3 (Neighbour becomes offset better than serving). Inequality A3-1 (Entering condition) is:

”.

The values in this equation are derived as follows:

- Mn is evaluated for cell 2 at T2 and is nominally -86dBm

- Ofn is the frequency specific offset offsetFreq of the frequency of the neighbour cell, and set to zero if not configured for the neighbour cell

- Ocn is the cell specific offset cellIndividualOffset of the neighbour cell and is assumed to be 0dB

- Hys is the hysteresis parameter for this event, specified as 0dB in TS 36.521-3 Table 5.1.19.4.1-1

- Ms is evaluated for cell 1 at T2 and is nominally -90dBm

- Ofs is the frequency specific offset offsetFreq of the serving frequency specified as 0dB in TS 36.508 Table 4.6.6-2

- Ocs is the cell specific offset cellIndividualOffset of the serving cell and is assumed to be 0dB

- Off is the offset parameter for this event (i.e. A3-Offset), specified as 0dB in TS 36.521-3 Table 5.1.19.4.1-1

The equation therefore reduces to:

Mn > Ms + Off. Putting in values and rearranging gives -86dBm - (-90)dBm > 0dB.

This inequality is satisfied by 3dB under nominal conditions. However, the overall uncertainty for UE Measured (Cell 2 RSRP - Cell 1 RSRP), taking into account both test system and UE measurement uncertainties, is ±4.42dB as calculated in step d) of the accompanying spreadsheet. The inequality may therefore not be satisfied when uncertainties are included.

Furthermore, the uncertainties mean that the Ês/Iot of the Cell 1 control channel used in this test case can fall below the demodulation performance. We’ve noted that same issues are already met in intra-frequency measurement reporting test cases 8.1.1. As a result, serving cell Es/Noc is increased by an amount to ensure that serving cell Es/Iot is larger than the PDCCH demodulation requirements, which is specified at -1.7dB SNR in TS 36.101 Table 8.4.1.1-1 (FDD) or -1.6dB SNR in TS 36.101 Table 8.4.2.1-1 (TDD).

We suggest apply the same method in TC 5.1.19/5.1.20. To be more specific, by lowering Cell 2 Es/Noc to let Cell 1 Es/Iot to remain high enough for control channel demodulation( > -1.7dB). a3-offset is also decreased accordingly to ensure entry condition of event A3 is still satisfied.

f) Parameters modified by Test Tolerances

Based on the decision in e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, but this time modified by the Test Tolerances (applied offsets). During T2 and T3 the Ês/Noc of Cell 2 is decreased by just enough to cover the variability from uncertainties.

From the inequality in step e) we see that (Cell 2 RSRP - Cell 1 RSRP) compared to the A3 offset needs to be at least 0dB. Its nominal value is 0.7dB, but it can vary +/-4.42dB due to uncertainties. So we need to decrease a3-offset by at least 4dB to ensure that the entry condition is met under all conditions.

Re-derived parameters are calculated using the same methods as were used in step b). In general all the values are listed for each time interval T1, T2 and T3 but as cell 2 is not present during T1 those entries are greyed out.

g) Check controlled parameters Min/Max

Using a format similar to that in step d), the nominal value of each controlled parameter is recalculated.

The minimum and maximum values, due to variability from uncertainties, of controlled parameters are then calculated and compared against the requirements (Event A3 condition, Ês/Iot range, RSRP level..). It is not necessary to calculate all parameters during each time interval T1, T2 and T3, so a selection is made of those critical to the test verdict. The critical requirement for each parameter is given briefly in the “Comment” column of section g) in the accompanying spreadsheet. The cases closest to limit (in these test cases, all limits are one-sided) are identified by turquoise cells in the spreadsheet. If all requirements are met, then the exercise is complete.

It can be seen that with the uncertainty values and Test Tolerances proposed, all the requirements are met.

# 5. Treatment of E-UTRAN TDD – TDD Intra frequency case 5.1.20

TS 36.521-3 also contains similar TDD-TDD test case in clause 5.1.20. The TDD test case 5.1.20 is synchronous, meaning that the two cells are time offset by 3µs. As intra-frequency interference has already been considered for the FDD case, it will be no worse for TDD.

The TDD test case in 5.1.20 has the same 3 time intervals T1, T2 and T3 and the signal levels are similar, so Agilent propose to apply the same level uncertainties and Test Tolerances approach as for FDD. A single tab on the spreadsheet provided is used to cover both 5.1.19 and 5.1.20 as all the numeric values are identical.