3GPP TSG-RAN5 Meeting #108	R5-254691

# Bengaluru, India, Aug.25th – 29rd 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.903 | CR | 0472 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating TT analysis for LTE Cat 1bis RRM test case 5.1.19 and 5.1.20 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, LTE_UE_cat_1RX-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | PCell (Cell 1) Es/Iot level in T2/T3 is only -4.74dB based on TT analysis results given in 36.903 attachments (“5.1.19+5.1.20 TT.zip”), which is too low for PDCCH/PDSCH demodulation reception. As a result, the UE may, - fail to decode PDCCH carrying UL grant, then the UE will fail to send the required measurement report message; - fail to decode the Status report PDU sent by TE on SRB#1. As a result, the UE will trigger a RLF when retransmission number of RLC PDU exceeds the maximum number threshold. either one will lead to failing the test. During RAN5#107 online discussion we have we’d received comments that side condition used in TT analysis shall not be changed. After some checking we realized that this issue is a common issue in LTE intra-frequency measurement test cases 36.521-3 TC 8.1.x. And the existed solution is to increase serving cell Es/Iot by an amount to ensure serving cell Es/Iot is larger than the that PDCCH demodulation specified in 36.101. Take TC 8.1.3 as an example, Cell Es/Iot is increased to -1.69dB (> PDCCH demod requirement -1.7dB) in TT analysis (see “8.1.1+8.1.9 TT_v4”): We suggest do the same updating to TT analysis for 5.1.19 and 5.1.20. |  |  |  |  |  |  |  |  |
| Summary of change: |  | TT analysis for 5.1.19 and 5.1.20 is updated: - Cell 2 Es/Noc in T2 and T3 is decreased by an amount to ensure that Cell 1 Es/Iot is high enough for PDCCH demodulation - a3-offset is decreased by an amount to ensure that entry condition of event A3 is satisfied. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.19, 5.1.20 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.521-3 CR 3100 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Associated TC CR: R5-254569 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

Following .zip files are removed from the attachments of TR 36.903:

“5.1.19+5.1.20 TT.zip”

Following .zip files are added to the attachments of TR 36.903:

“5.1.19+5.1.20 TT_v2.zip”

<End of Change 1>