3GPP TSG-RAN5 Meeting #108 	R5-254846

# Bengaluru, India, 25th August 2025 – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3595 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to SIB19 configuration for NR-NTN test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM Europe Inc. - Spain |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current message contents defined for SIB19 are incomplete and are missing several IE containers and sub-containers. In addition, it makes it difficult to configure the NTN-Config of the neighbor cells in a single table. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Split the message contents from Table 7.6.3.1-1 into two tables: one for SIB19 IEs, and another for NTN-Config IEs. The latter can be reused for all neighbor cells contained in ntn-NeighCellConfigList-r17 IE. - Added missing IEs to SIB19 config. - Corrected ntn-NeighCellConfigList-r17 IE, and added missing IEs inside it. - Corrected the table mapping references. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case configuration would be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.2, 7.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

## 7.6	Test environment for NTN testing

This section defines the test environment which applies to all RRM test cases executed for NR NTN UEs, unless otherwise specified.

### 7.6.1	UE location

Same as in section 5.6.1.

### 7.6.2	Ephemeris Information

Ephemeris information in this section has been generated as described in clause 5 in TR38.905 [69].

#### 7.6.2.0	Assumptions for Ephemeris generation

Same as in section 5.6.2.0.

#### 7.6.2.1	Ephemeris for elevation angle 30º

All NR NTN test cases defined in in section 14 in TS 38.533 [18], unless otherwise stated, shall use ephemeris configuration in tables 7.6.2.1-1a to 7.6.2.1-3b, corresponding to an elevation angle of 30º for the type of satellite under test, keeping such information constant throughout the duration of the test:

-	In case UE supports only NGSO satellites, ephemeris values in Table 7.6.2.1-3a or 7.6.2.1-3b shall be used.

Table 7.6.2.1-1a: SIB19- NR NTN serving cell Ephemeris information (stateVectors format) for GSO satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 7.6.3.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -17129898 |  |  |
| positionY-r17 | 27505094 |  |  |
| positionZ-r17 | 1413939 |  |  |
| velocityVX-r17 | 116 |  |  |
| velocityVY-r17 | 378 |  |  |
| velocityVZ-r17 | -5944 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30 degrees, one-way delay equal to 128.77 ms and Doppler equal to -0.15 ppm. NOTE 2: stateVectors ephemeris format shall be used in NR NTN RRM test cases listed in Table A.3.36.3-1 in TS38.133 [13]. |  |  |  |

Table 7.6.2.1-1b: SIB19- NR NTN serving cell Ephemeris information (orbitalParameters format) for GSO satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 7.6.3.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| orbital-r17 SEQUENCE { |  |  |  |
| semiMajorAxis-r17 | 8393544277 |  |  |
| eccentricity-r17 | 0 |  |  |
| periapsis-r17 | 266532360 |  |  |
| longitude-r17 | 224998499 |  |  |
| inclination-r17 | 2125399 |  |  |
| meanAnomaly-r17 | 118496066 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30 degrees, one-way delay equal to 128.77 ms and Doppler equal to -0.15 ppm. NOTE 2: orbitalParameters ephemeris format shall be used in NR NTN RRM test cases listed in Table A.3.36.3-2 in TS38.133 [13]. |  |  |  |

Table 7.6.2.1-1c: SIB19- NR NTN neighbour cells Ephemeris information (stateVectors format) for GSO satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 7.6.3.1-1 and Table 7.6.3.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-NeighCellConfigList-r17 SEQUENCE (SIZE(1..maxCellNTN-r17)) OF NTN-NeighCellConfig-r17{ | 1 entry for test cases requiring only one neighbour cell 2 entries in test cases requiring two neighbour cells |  | 2CELLS OR 3CELLS |
| NTN-NeighCellConfig-r17 [1] SEQUENCE { |  | entry 1 | 2CELLS OR 3CELLS |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -25235934 |  |  |
| positionY-r17 | 20178903 |  |  |
| positionZ-r17 | 2814664 |  |  |
| velocityVX-r17 | 298 |  |  |
| velocityVY-r17 | -241 |  |  |
| velocityVZ-r17 | 4398 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NTN-NeighCellConfig-r17 [2] SEQUENCE { |  | entry 2 | 3CELLS |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -6457459 |  |  |
| positionY-r17 | 31531426 |  |  |
| positionZ-r17 | 4004156 |  |  |
| velocityVX-r17 | -387 |  |  |
| velocityVY-r17 | -66 |  |  |
| velocityVZ-r17 | -104 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Neighbour cell 1 Satellite-UE elevation angle equal to 30 degrees, one-way delay equal to 128.77 ms and Doppler equal to +0.11 ppm. Neighbour cell 2 Satellite-UE elevation angle equal to 32.17 degrees, one-way delay equal to 128.14 ms and Doppler equal to 0 ppm. NOTE 2: stateVectors ephemeris format shall be used in NR NTN RRM test cases listed in Table A.3.36.3-1 in TS38.133 [13]. |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2CELLS | Test cases where one serving cell and one neighbour cell are required |
| 3CELLS | Test cases where one serving cell and two neighbour cells are required |

Table 7.6.2.1-1d: SIB19- NR NTN neighbour cells Ephemeris information (orbitalParameters format) for GSO satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 7.6.3.1-1 and Table 7.6.3.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-NeighCellConfigList-r17 SEQUENCE (SIZE(1..maxCellNTN-r17)) OF NTN-NeighCellConfig-r17 { | 1 entry for test cases requiring only one neighbour cell 2 entries in test cases requiring two neighbour cells |  | 2CELLS OR 3CELLS |
| NTN-NeighCellConfig-r17 [1] SEQUENCE { |  | entry 1 | 2CELLS OR 3CELLS |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| orbital-r17 SEQUENCE { |  |  |  |
| semiMajorAxis-r17 | 8393544277 |  |  |
| eccentricity-r17 | 0 |  |  |
| periapsis-r17 | 137910436 |  |  |
| longitude-r17 | 105383807 |  |  |
| inclination-r17 | 67072560 |  |  |
| meanAnomaly-r17 | 32981819 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NTN-NeighCellConfig-r17 [2] SEQUENCE { |  | entry 2 | 3CELLS |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| orbital-r17 SEQUENCE { |  |  |  |
| semiMajorAxis-r17 | 8393544277 |  |  |
| eccentricity-r17 | 0 |  |  |
| periapsis-r17 | 249232221 |  |  |
| longitude-r17 | 228396393 |  |  |
| inclination-r17 | 12328441 |  |  |
| meanAnomaly-r17 | 67231401 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Neighbour cell 1 Satellite-UE elevation angle equal to 30 degrees, one-way delay equal to 128.77 ms and Doppler equal to +0.11 ppm. Neighbour cell 2 Satellite-UE elevation angle equal to 32.17 degrees, one-way delay equal to 128.14 ms and Doppler equal to 0 ppm. NOTE 2: orbitalParameters ephemeris format shall be used in NR NTN RRM test cases listed in Table A.3.36.3-2 in TS38.133 [13]. |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2CELLS | Test cases where one serving cell and one neighbour cell are required |
| 3CELLS | Test cases where one serving cell and two neighbour cells are required |

Table 7.6.2.1-2a: Void

Table 7.6.2.1-2b: Void

Table 7.6.2.1-3a: SIB19- NR NTN serving cell Ephemeris information (stateVectors format) for NGSO (LEO-1200) satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 7.6.3.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2935552 |  |  |
| positionY-r17 | 4895460 |  |  |
| positionZ-r17 | 1188458 |  |  |
| velocityVX-r17 | 8506 |  |  |
| velocityVY-r17 | -23314 |  |  |
| velocityVZ-r17 | 118253 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30.03 degrees, one-way delay equal to 6.67 ms and Doppler equal to 17.55 ppm. NOTE 2: stateVectors ephemeris format shall be used in NR NTN RRM test cases listed in Table A.3.36.3-1 in TS38.133 [13]. |  |  |  |

Table 7.6.2.1-3b: SIB19- NR NTN serving cell Ephemeris information (orbitalParameters format) for NGSO (LEO-1200) satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 7.6.3.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| orbital-r17 SEQUENCE { |  |  |  |
| semiMajorAxis-r17 | 273626880 |  |  |
| eccentricity-r17 | 769300 |  |  |
| periapsis-r17 | 198830479 |  |  |
| longitude-r17 | 89834718 |  |  |
| inclination-r17 | 65473992 |  |  |
| meanAnomaly-r17 | 7822790 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30.03 degrees, one-way delay equal to 6.67 ms and Doppler equal to 17.55 ppm. NOTE 2: orbitalParameters ephemeris format shall be used in NR NTN RRM test cases listed in Table A.3.36.3-2 in TS38.133 [13]. |  |  |  |

Table 7.6.2.1-3c: SIB19- NR NTN neighbour cells Ephemeris information (stateVectors format) for NGSO (LEO-1200) satellites (serving cell 30º elevation angle conditions)

| Derivation Path: TS 38.508-1 [12], Table 4.6.2-18C, Table 7.6.3.1-1 and Table 7.6.3.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-NeighCellConfigList-r17 SEQUENCE (SIZE(1..maxCellNTN-r17)) OF NTN-NeighCellConfig-r17{ | 1 entry for test cases requiring only one neighbour cell 2 entries in test cases requiring two neighbour cells |  | 2CELLS OR 3CELLS |
| NTN-NeighCellConfig-r17 [1] SEQUENCE { |  | entry 1 | 2CELLS or 3CELLS |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2470752 |  |  |
| positionY-r17 | 3858366 |  |  |
| positionZ-r17 | 3624634 |  |  |
| velocityVX-r17 | 31542 |  |  |
| velocityVY-r17 | -67516 |  |  |
| velocityVZ-r17 | 94597 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NTN-NeighCellConfig-r17 [2] SEQUENCE { |  | entry 2 | 3CELLS |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2771542 |  |  |
| positionY-r17 | 4509243 |  |  |
| positionZ-r17 | 2455572 |  |  |
| velocityVX-r17 | 20837 |  |  |
| velocityVY-r17 | -46176 |  |  |
| velocityVZ-r17 | 109528 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Neighbour cell 1 Satellite-UE elevation angle equal to 150.02 degrees, one-way delay equal to 6.79 ms and Doppler equal to -17.65 ppm. Neighbour cell 2 Satellite-UE elevation angle equal to 89.54 degrees, one-way delay equal to 4.04 ms and Doppler equal to 0 ppm. NOTE 2: stateVectors ephemeris format shall be used in NR NTN RRM test cases listed in Table A.3.36.3-1 in TS38.133 [13]. |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2CELLS | Test cases where one serving cell and one neighbour cell are required |
| 3CELLS | Test cases where one serving cell and two neighbour cells are required |

Table 7.6.2.1-3d: SIB19- NR NTN neighbour cells Ephemeris information (orbitalParameters format) for NGSO (LEO-1200) satellites (serving cell 30º elevation angle conditions)

| Derivation Path: TS 38.508-1 [12], Table 4.6.2-18C, Table 7.6.3.1-1 and Table 7.6.3.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-NeighCellConfigList-r17 SEQUENCE (SIZE(1..maxCellNTN-r17)) OF NTN-NeighCellConfig-r17{ | 1 entry for test cases requiring only one neighbour cell 2 entries in test cases requiring two neighbour cells |  | 2CELLS OR 3CELLS |
| NTN-NeighCellConfig-r17 [1] SEQUENCE { |  | entry 1 | 2CELLS OR 3CELLS |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| orbital-r17 SEQUENCE { |  |  |  |
| semiMajorAxis-r17 | 272189533 |  |  |
| eccentricity-r17 | 715030 |  |  |
| periapsis-r17 | 199909097 |  |  |
| longitude-r17 | 89257064 |  |  |
| inclination-r17 | 64358188 |  |  |
| meanAnomaly-r17 | 27947120 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NTN-NeighCellConfig-r17 [2] SEQUENCE { |  | entry 2 | 3CELLS |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 ::= CHOICE { |  |  |  |
| orbital-r17 SEQUENCE { |  |  |  |
| semiMajorAxis-r17 | 273057241 |  |  |
| eccentricity-r17 | 744972 |  |  |
| periapsis-r17 | 197978669 |  |  |
| longitude-r17 | 89682433 |  |  |
| inclination-r17 | 65034798 |  |  |
| meanAnomaly-r17 | 17169725 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Neighbour cell 1 Satellite-UE elevation angle equal to 150.02 degrees, one-way delay equal to 6.79 ms and Doppler equal to -17.65 ppm. Neighbour cell 2 Satellite-UE elevation angle equal to 89.54 degrees, one-way delay equal to 4.04 ms and Doppler equal to 0 ppm. NOTE 2: orbitalParameters ephemeris format shall be used in NR NTN RRM test cases listed in Table A.3.36.3-2 in TS38.133 [13]. |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2CELLS | Test cases where one serving cell and one neighbour cell are required |
| 3CELLS | Test cases where one serving cell and two neighbour cells are required |

#### 7.6.2.2	Ephemeris for NR NTN RRM UL timing accuracy test cases

Table 7.6.2.2-1: SIB19- NR NTN serving cell Ephemeris information for GSO satellites (minimum Doppler)

| Derivation Path: Table 7.6.3.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -16944523 |  |  |
| positionY-r17 | 27362209 |  |  |
| positionZ-r17 | 4019393 |  |  |
| velocityVX-r17 | -336 |  |  |
| velocityVY-r17 | -208 |  |  |
| velocityVZ-r17 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 35.04 degrees, one-way delay equal to 127.33 ms and Doppler equal to 4.89E-06 ppm |  |  |  |

Table 7.6.2.2-2: SIB19- NR NTN serving cell Ephemeris information for NGSO (LEO-1200) satellites (minimum Doppler)

| Derivation Path: Table 7.6.3.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2771542 |  |  |
| positionY-r17 | 4509243 |  |  |
| positionZ-r17 | 2455572 |  |  |
| velocityVX-r17 | 20837 |  |  |
| velocityVY-r17 | -46176 |  |  |
| velocityVZ-r17 | 109528 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.54 degrees, one-way delay equal to 4.04 ms and Doppler equal to 3.6E-3 ppm. |  |  |  |

Table 7.6.2.2-3: SIB19- NR NTN serving cell Ephemeris information for GSO satellites (maximum positive Doppler)

| Derivation Path: Table 7.6.3.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| SIB19-r17 ::= SEQUENCE { |  |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |  |
| epochTime-r17 | Not present |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |  |
| positionX-r17 | -16989970 |  |  |  |
| positionY-r17 | 27591785 |  |  |  |
| positionZ-r17 | 1413211 |  |  |  |
| velocityVX-r17 | 390 |  |  |  |
| velocityVY-r17 | -65 |  |  |  |
| velocityVZ-r17 | 5945 |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30 degrees, one-way delay equal to 128.77 ms and Doppler equal to 0.15 ppm |  |  |  |  |

Table 7.6.2.2-4: SIB19- NR NTN serving cell Ephemeris information for NGSO (LEO-600) satellites (maximum positive Doppler)

| Derivation Path: Table 7.6.3.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| SIB19-r17 ::= SEQUENCE { |  |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |  |
| epochTime-r17 | Not present |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |  |
| positionX-r17 | -2654549 |  |  |  |
| positionY-r17 | 4386991 |  |  |  |
| positionZ-r17 | 1594205 |  |  |  |
| velocityVX-r17 | 14581 |  |  |  |
| velocityVY-r17 | -34487 |  |  |  |
| velocityVZ-r17 | 120182 |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30.11 degrees, one-way delay equal to 3.60 ms and Doppler equal to 19.83 ppm. NOTE 2: This configuration can be skipped as it is the same as default 30º elevation angle |  |  |  |  |

Table 7.6.2.2-5: SIB19- NR NTN serving cell Ephemeris information for GSO satellites (maximum negative Doppler)

| Derivation Path: Table 7.6.3.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| SIB19-r17 ::= SEQUENCE { |  |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |  |
| epochTime-r17 | Not present |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |  |
| positionX-r17 | -17129898 |  |  |  |
| positionY-r17 | 27505094 |  |  |  |
| positionZ-r17 | 1413939 |  |  |  |
| velocityVX-r17 | 116 |  |  |  |
| velocityVY-r17 | 378 |  |  |  |
| velocityVZ-r17 | -5944 |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30 degrees, one-way delay equal to 128.77 ms and Doppler equal to -0.15 ppm. NOTE 2: This configuration can be skipped as it is the same as default 30º elevation angle. |  |  |  |  |

Table 7.6.2.2-6: SIB19- NR NTN serving cell Ephemeris information for NGSO (LEO-600) satellites (maximum negative Doppler)

| Derivation Path: Table 7.6.3.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| SIB19-r17 ::= SEQUENCE { |  |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |  |
| epochTime-r17 | Not present |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |  |
| positionX-r17 | -2401021 |  |  |  |
| positionY-r17 | 3830217 |  |  |  |
| positionZ-r17 | 2907041 |  |  |  |
| velocityVX-r17 | 28879 |  |  |  |
| velocityVY-r17 | -61410 |  |  |  |
| velocityVZ-r17 | 105774 |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 149.91 degrees, one-way delay equal to 3.66 ms and Doppler equal to -19.95 ppm |  |  |  |  |

### 7.6.3	NR NTN message contents for RRM tests

#### 7.6.3.1	SIB19 IE values

The ntn-config-r17 values including the EphemerisInfo-r17 values in SIB19 shall be periodically signalled to the UE during test based on the Ephemeris information in clause 7.6.2 (kept constant throughout the duration of each the test).

Table 7.6.3.1-1: SIB19 for RRM tests

| Derivation Path: Table 4.6.2-18C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark |  |  | Comment |  | Condition |  |
| SIB19-r17 ::= SEQUENCE { |  |  |  |  |  |  |  |
| ntn-Config-r17 |  | NTN-Config-r17 |  |  | Serving NTN-Config, Table 7.6.3.1-2 |  | 1CELL or 2CELLS or 3CELLS |
| t-Service-r17 |  | Not present |  |  |  |  |  |
| referenceLocation-r17 |  | Not present |  |  |  |  |  |
| distanceThresh-r17 |  | Not present |  |  |  |  |  |
| ntn-NeighCellConfigList-r17 |  | Not present |  |  |  |  | 1CELL |
| ntn-NeighCellConfigList-r17 SEQUENCE { |  |  |  |  |  |  |  |
| ntn-NeighCellConfig-r17[1] SEQUENCE { |  |  |  |  |  |  | 2CELLS |
| ntn-Config-r17 |  | NTN-Config-r17 |  |  | Cell2 NTN-Config, Table 7.6.3.1-2 |  |  |
| carrierFreq-r17 |  | ARFCN-ValueNR |  |  | Freq of Cell2 |  |  |
| physCellId-r17 |  | PhysCellId |  |  | PCI of Cell2 |  |  |
| } |  |  |  |  |  |  |  |
| ntn-NeighCellConfig-r17 [2] SEQUENCE { |  | See cl. 7.6.2 |  |  |  |  | 3CELLS |
| ntn-Config-r17 |  | NTN-Config-r17 |  |  | Cell3 NTN-Config, Table 7.6.3.1-2 |  |  |
| carrierFreq-r17 |  | ARFCN-ValueNR |  |  | Freq of Cell3 |  |  |
| physCellId-r17 |  | PhysCellId |  |  | PCI of Cell3 |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| 1CELL | Test cases where only one cell (serving cell) is required |
| 2CELLS | Test cases where one serving cell and one neighbour cell are required |
| 3CELLS | Test cases where one serving cell and two neighbour cells are required |

Table 7.6.3.1-2: NTN-Config for RRM tests

| Derivation Path: Table 4.6.3-84C |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark |  |  | Comment | Condition |
| NTN-Config-r17 ::= SEQUENCE { |  |  |  |  |  |  |
| epochTime-r17 |  | Not present |  |  |  |  |
| ntn-UlSyncValidityDuration-r17 |  | s900 |  |  |  | GSO |
|  |  | s5 |  |  |  | NGSO |
| cellSpecificKoffset-r17 |  | 258 |  |  |  | GSO |
|  |  | 14 |  |  |  | NGSO |
| kmac-r17 |  | Not present |  |  |  |  |
| ta-Info-r17 SEQUENCE { |  |  |  |  |  |  |
| ta-Common-r17 |  | 0 |  |  |  |  |
| ta-CommonDrift-r17 |  | 0 |  |  |  |  |
| ta-CommonDriftVariant-r17 |  | 0 |  |  |  |  |
| } |  |  |  |  |  |  |
| ntn-PolarizationDL-r17 |  | linear |  |  |  |  |
| ntn-PolarizationUL-r17 |  | linear |  |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |  |  | TC_EF_SV |
| positionX-r17 |  | See cl. 7.6.2 |  |  |  | 1CELL or 2CELLS or 3CELLS |
| positionY-r17 |  | See cl. 7.6.2 |  |  |  | 1CELL or 2CELLS or 3CELLS |
| positionZ-r17 |  | See cl. 7.6.2 |  |  |  | 1CELL or 2CELLS or 3CELLS |
| velocityVX-r17 |  | See cl. 7.6.2 |  |  |  | 1CELL or 2CELLS or 3CELLS |
| velocityVY-r17 |  | See cl. 7.6.2 |  |  |  | 1CELL or 2CELLS or 3CELLS |
| velocityVZ-r17 |  | See cl. 7.6.2 |  |  |  | 1CELL or 2CELLS or 3CELLS |
| } |  |  |  |  |  |  |
| orbital-r17 SEQUENCE { |  |  |  |  |  | TC_EF_OP |
| semiMajorAxis-r17 |  | See cl. 7.6.2 |  |  |  | 1CELL or 2CELLS or 3CELLS |
| eccentricity-r17 |  | See cl. 7.6.2 |  |  |  | 1CELL or 2CELLS or 3CELLS |
| periapsis-r17 |  | See cl. 7.6.2 |  |  |  | 1CELL or 2CELLS or 3CELLS |
| longitude-r17 |  | See cl. 7.6.2 |  |  |  | 1CELL or 2CELLS or 3CELLS |
| inclination-r17 |  | See cl. 7.6.2 |  |  |  | 1CELL or 2CELLS or 3CELLS |
| meanAnomaly-r17 |  | See cl. 7.6.2 |  |  |  | 1CELL or 2CELLS or 3CELLS |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| ta-Report-r17 |  | Not present |  |  |  |  |
| } |  |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| TC_EF_SV | Test cases using ephemeris with Position Velocity format according to section 7.6.2 |
| TC_EF_OP | Test cases using ephemeris with Orbital format according to section 7.6.2 |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |
| 1CELL | Test cases where only one cell (serving cell) is required |
| 2CELLS | Test cases where one serving cell and one neighbour cell are required |
| 3CELLS | Test cases where one serving cell and two neighbour cells are required |

<< End of Changes >>