3GPP TSG-RAN5 Meeting #108				R5-254877

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

Agenda item:	5.3.26.7

Source: 	Qualcomm Inc

Title: 	Minimum test time analysis for FR1 PDSCH absolute phy layer tput test cases

Document for:	Discussion and Endorsement

## 1.	Introduction

TS 38.521-4 [3] introduced PDSCH absolute physical layer throughput performance for FR1 FDD and TDD 2Rx and 4Rx based on the requirements defined in [4]. This paper discusses the simulation results to derive the minimum test time for these PDSCH absolute physical layer throughput cases.

## 2.	Discussion

From [1] and [2], we can recall the procedure used to come up with the minimum test time for PDSCH throughput test cases.

- Run the PDSCH simulations for an extended period of time at the SNR of interest for multiple seeds.

- The minimum test time is determined by the time it took the throughput curve to settle within +/- 2% of the target throughput value across all seeds.

- If the throughput does not settle within the +/- 2% throughput limit within a reasonable amount of time for any seed, expand the threshold to +/- 3.5% of the target throughput value.

- To account for the additional 1.5% throughput variation, determine from the Throughput vs SNR curve, what is the SNR delta corresponding to this 1.5 % throughput increase.

- That SNR value is the added SNR uncertainty due to finite test time.

PDSCH simulations for all the defined demod test cases in [3] were run. Pasting the test requirements table below.

Table 5.6.2.1.1_1.4-2: Test requirements

| Test num. | Fraction of maximum throughput (%) | SNR (dB) |
| --- | --- | --- |
| Test 1 | 35% | 20.3 |
| Note 1: The maximum throughout is defined as with TBS corresponding to CQI index 15 with rank 2. |  |  |

Table 5.6.2.2.1_1.4-2: Test requirements

|  | Fraction of maximum throughput (Note 1) | SNR(dB) |
| --- | --- | --- |
| Test 1 | 35% | 21.4 |
| Note 1: The maximum throughput is defined as the throughput using the TBS corresponding to CQI index 15 with rank 2. |  |  |

Table 5.6.3.1.1.0-2: Minimum requirements

|  | Fraction of maximum throughput (Note 1) | SNR (dB) |
| --- | --- | --- |
| Test 1 | 50% | 18.0 |
| Note 1: The maximum throughput is defined as the throughput using the TBS corresponding to CQI index 15 with rank 2. |  |  |

Table 5.6.3.1.1_1.4-2: Test requirements

|  | Fraction of maximum throughput (Note 1) | SNR (dB) |
| --- | --- | --- |
| Test 1 | 50% | 19 |
| Note 1: The maximum throughput is defined as the throughput using the TBS corresponding to CQI index 15 with rank 2. |  |  |

Each scenario was simulated using 30 different seeds. The time it took for the throughput to converge within +/2% of the reference throughput was noted and is captured in the below table. The below table also captures the reference throughput numbers for each test case.

Table 2-1: Throughput convergence time

| Test case | Test Num | TPUT [%] of Maximum | Throughput at Reference [Mbps] | Requirement SNR | Lower 2% of Ref Tput | Upper 2% of Ref Tput | Tput convergence time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5.6.2.1.1_1 | 1 | 35 | 24.2 | 20.3 | 23.7 | 24.7 | 50 seconds |
| 5.6.2.2.1_1 | 1 | 35 | 76 | 21.4 | 74.5 | 77.5 | 50 seconds |
| 5.6.3.1.1_1 | 1 | 50 | 34.6 | 18.0 | 33.9 | 35.3 | 50 seconds |
| 5.6.3.2.1_1 | 1 | 50 | 108.4 | 19.0 | 106.2 | 110.6 | 50 seconds |

For all the 4 scenarios, PDSCH throughput converged within +/- 2% of the reference throughput within 50 seconds.

Proposal1: Use MNAS (minimum number of active subframes) of 50 seconds for all the above mentioned PDSCH Phy layer throughput scenarios.

## 3.	Conclusion

Proposal1: Use MNAS (minimum number of active subframes) of 50 seconds for all the above mentioned PDSCH Phy layer throughput scenarios.

## 4.		References

[1]	R5-191250, Discussion paper on statistical tput calculation for demod test cases, Qualcomm

[2]	R5-194687, Discussion on minimum test time for PDSCH and PDCCH with more simulation results, Qualcomm

[3]	TS 38.521-4 v18.7.0

[4]	TS 38.101-4 v18.8.0