3GPP TSG-RAN5 Meeting #108	R5-255112

# Bengaluru, India, 25th Aug 2025 – 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5110 | rev | 2 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new UAV Test Case 8.1.3.5.2 Test Case for event H2 reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM JAPAN LLC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UAV_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-09-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test case 8.1.3.5.2 needs to be added as per NR UAV_plus_CT1-UEConTest WP |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case 8.1.3.5.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This aspect of WP will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.3.5.2 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.523-2 CR0616, TS/TR 38.508-1 CR3549 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from original R5-254276, R5-255042 |  |  |  |  |  |  |  |  |

### <Unchanged Sections Omitted>

##### 8.1.3.5.2	NR UAV / Measurement configuration control and reporting / Event H2

8.1.3.5.2.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and configured for event H2 with event based periodical reporting }

ensure that {

when { Aerial UE height becomes lower than the absolute threshold minus hysteresis }

then { UE sends MeasurementReport message at regular intervals while entering conditions for event H2 are satisfied }

}

(2)

with { UE in RRC_CONNECTED state and periodical measurement reporting triggered by event H2 ongoing }

ensure that {

when { Aerial UE height becomes higher than the absolute threshold plus hysteresis }

then { UE stops sending MeasurementReport message }

}

8.1.3.5.2.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.331, clauses 5.3.5.3, 5.5.4.1, 5.5.4.22 and 5.5.5.1 Unless otherwise stated these are Rel-18 requirements.

[TS 38.331, clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration, upon execution of the conditional reconfiguration (CHO, CPA, CPC, or subsequent CPAC), or upon execution of an LTM cell switch:

1>	if the RRCReconfiguration message includes the aerial-Config:

2>	(re)configure the aerial parameters in accordance with the included aerial-Config;

[TS 38.331, clause 5.5.4.1]

If AS security has been activated successfully, the UE shall:

- for each measId included in the measIdList within VarMeasConfig:

…

2>	else if the reportType is set to eventTriggered and if the eventId is set to eventD1 or eventD2 or eventH1 or eventH2 and if the entering condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include a measurement reporting entry for this measId:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

- initiate the measurement reporting procedure, as specified in 5.5.5;

…

[TS 38.331, clause 5.5.4.22]

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition H2-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition H2-2, as specified below, is fulfilled;

Inequality H2-1 (Entering condition)

Ms + Hys < Thresh

Inequality H2-2 (Leaving condition)

Ms – Hys > Thresh

The variables in the formula are defined as follows:

Ms is the Aerial UE altitude relative to the sea level.

Hys is the hysteresis parameter for this event (i.e. h2-Hysteresis as defined within reportConfigNR for this event).

Thresh is the threshold parameter for this event (i.e. h2-Threshold as defined within reportConfigNR for this event).

Ms, Hys, Thresh are expressed in meters.

[TS 38.331, clause 5.5.5.1]

The UE shall, for each entry in the VarMeasReportList:

1>	if the eventH1 or eventH2 is configured in the corresponding reportConfig associated with the measId that triggered the measurement reporting:

2>	for all the entries in the VarMeasReportList for which the measurement reporting procedure was triggered and the corresponding reportConfig is configured with the same eventID and simulMultiTriggerSingleMeasReport set to true:

3>	except for the entry corresponding to the event for which the difference between the configured altitude threshold and the altitude of the UE is the smallest, remove all other measurement reporting entries from the VarMeasReportList, if any, and stop the associated periodical reporting timer(s), if running;

…

For the measId for which the measurement reporting procedure was triggered, the UE shall set the measResults within the MeasurementReport message as follows:

1>	set the measId to the measurement identity that triggered the measurement reporting;

1>	for each serving cell configured with servingCellMO:

…

1>	increment the numberOfReportsSent as defined within the VarMeasReportList for this measId by 1;

1>	stop the periodical reporting timer, if running;

1>	if the numberOfReportsSent as defined within the VarMeasReportList for this measId is less than the reportAmount as defined within the corresponding reportConfig for this measId:

2>	start the periodical reporting timer with the value of reportInterval as defined within the corresponding reportConfig for this measId;

8.1.3.5.2.3	Test description

8.1.3.5.2.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

-	1 GNSS simulator

UE:

- None.

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

8.1.3.5.2.3.2	Test procedure sequence

Table 8.1.3.5.2.3.2-1 illustrates the simulated aerial height above sea level at various time instants of the test execution.. Row marked "T0" denotes the conditions after the preamble, while rows marked "T1 and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.3.5.2.3.2-1: Simulated aerial height

| Time | Parameter | Unit | Value (Note 2) | Remark (Note 1) |
| --- | --- | --- | --- | --- |
| T0 | Simulated aerial height above sea level (Ms) | m | 500 | Simulated aerial height above sea level (Ms) is such that entry condition for event H2 is not satisfied: Ms - Hys – Offset < Thresh |
| T1 |  |  | 480 | Simulated aerial height above sea level (Ms) is such that entry condition for event H2 is satisfied: Ms - Hys – Offset < Thresh |
| T2 |  |  | 680 | Simulated aerial height above sea level (Ms) is such that exit condition for event H2 is satisfied: Ms + Hys – Offset > Thresh |
| Note 1: Value of Hys and Offset correspond to values of h2-Threshold-r18 and h2-Hysteresis-r18 in Table 8.1.3.5.2.3.3-3. Value of Thresh corresponds to value of heightThreshRef-r15 in Table 8.1.3.5.2.3.3-2. Note 2: An allowance of +/-90m has been used to generate the aerial height values. This allowance takes into account both the UE measurement accuracy and the test tolerances in the simulation. |  |  |  |  |

Table 8.1.3.5.2.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Trigger the UE to reset UTC time in order that the UE accepts the time generated by the GNSS simulator and also uses only GNSS for position calculation in the subsequent test procedure, as defined in TS 38.509 [25] clause 5.6.3. (Note1) | - | - | - | - |
| 2 | SS configures the GNSS simulator for step 1 of Scenario #1 as defined in TS 36.508 [7] Table 4.12.2-1 to simulate a location with the height above sea level given for time T0 in Table 8.1.3.5.2.3.2-1. | - | - | - | - |
| 3 | SS transmits a RRReconfiguration message including MeasConfig to setup intra NR measurement and reporting for event H2. | <-- | RRCReconfiguration | - | - |
| 4 | The UE transmits a RRCReconfigrationComplete message. | --> | RRCReconfigurationComplete | - | - |
| 5 | SS configures GNSS simulator for step 2 of Scenario #1 as defined in TS 36.508 [7] Table 4.12.2-1 to simulate the UE altitude moving to a height above sea level given for time T1 in Table 8.1.3.5.2.3.2-1. | - | - | - | - |
| 6 | Check: Does the UE transmit a MeasurementReport message to report event H2 with the measured aerial height value? | --> | MeasurementReport | 1 | P |
| - | EXCEPTION: Step 5 below is repeated until 3 MeasurementReport messages are received from the UE. | - | - | - | - |
| 7 | Check: Does the UE transmit a MeasurementReport message to report event H2 with the measured aerial height value? | --> | MeasurementReport | 1 | P |
| 8 | SS configures GNSS simulator for step 4 of Scenario #1 as defined in TS 36.508 [7] Table 4.12.2-1 to simulate the UE altitude moving to a height above sea level given for time T2 in Table 8.1.3.5.2.3.2-1. | - | - | - | - |
| 9 | Wait and ignore MeasurementReport messages for 50 s (as detailed in TS 36.508 [7] Table 4.12.2-1) to allow simulated change of aerial height value to be measured by the UE. | - | - | - | - |
| 10 | Check: Does the UE attempt to transmit an uplink message within the next 10s? | - | - | 2 | F |
| Note 1: The UTC time reset may be performed by MMI or AT command (+CUTCR). |  |  |  |  |  |

8.1.3.5.2.3.3	Specific message contents

Table 8.1.3.5.2.3.3-1: RRCReconfiguration (step 3, Table 8.1.3.5.2.3.2-2)

Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 with condition NR_MEAS and UAV

Table 8.1.3.5.2.3.3-2: MeasConfig (step 3, Table 8.1.3.5.2.3.3-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR | Table 8.1.3.5.2.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-EventH2 | Table 8.1.3.5.2.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.5.2.3.3-3: MeasObjectNR (Table 8.1.3.5.2.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 1 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |

Table 8.1.3.5.2.3.3-4: ReportConfigNR-EventH2 (Table 8.1.3.5.2.3.3-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-142 with condition EVENT_H2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventH2-r18 SEQUENCE { |  |  |  |
| h2-Threshold-r18 | 580 (580m) |  |  |
| h2-Hysteresis-r18 | 10 (10m) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportAmount | infinity |  |  |
| } |  |  |  |

Table 8.1.3.5.2.3.3-5: MeasurementReport (step 6, Table 8.1.3.5.2.3.2-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-5A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| measurementReport SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServingMOList SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResultServMO { | 1 entry |  |  |
| MeasResultServMO[1] SEQUENCE { |  | entry 1 |  |
| servCellId | ServCellIndex of NR Cell 1 |  |  |
| measResultServingCell SEQUENCE { |  |  |  |
| physCellId | PCI of NR Cell 1 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not present |  |  |
|  | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| altitudeUE-r18 | (-420..10000) |  |  |
| } |  |  |  |

<Unchanged Sections Omitted>